| Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document JVT-D042 File: JVT-D042.doc Generated: 2002-06-10 |
| --- | --- |

| Title: | Collocated Blocks and Associated Forward Reference Frames / MVs for Direct Mode MB in AFF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone and Limin Wang Motorola, Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-3618 krit@motorola.com |
| Source: | Motorola, Inc., BCS |  |  |

Abstract

This documentation provides the definition of the collocated blocks, and the associated reference frames and MVs for direct mode MB in B for AFF [3].

Summary

In JVT CD [1], a MB of 16x16 can be further divided into one of 7 block patterns, that is, 16x16, 16x8, 8x16, 8x8, 8x4, 4x8 and 4x4. If coded in inter mode (forward), JVT CD allows each 16x16, 16x8, 8x16, 8x8, 8x4, 4x8 and 4x4 MC block have its own MV, and each 16x16, 16x8, 8x16 and 8x8 MC block have its own reference frame. Hence, for CD, a MB can have up to 16 MVs and up to 4 reference frames.

Direct mode MB in B uses bi-directional prediction. Forward and backward MVs of a direct mode MB are derived from the forward MVs of its collocated MB in the future reference of I/P. Forward reference frames are the reference frames pointed by the MVs of its collocated MB. In the current JVT CD, a direct mode MB in B, as its collocated MB, can have up to 16 MVs and up to 4 reference frames. With adaptive switch of frame/field coding at picture level [3], a B frame and its future reference frame can be coded in either frame structure or field structure. Hence, in terms of frame/field coding modes, there can be four different combinations for a pair of a MB in B and its collocated MB in the future reference frame of I/P.

JVT WD [2] provides the detailed definitions of the collocated MB and the associated forward reference frame and MV for direct mode MB in B. But, JVT WD allows only one forward reference frame per MB, which is different from the current JVT CD. Those definitions therefore need to be modified accordingly. Since JVT CD allows each 16x16, 16x8, 8x16 and 8x8 MC block have its own reference frame, the concept of “collocated” can be moved down to the block level as well, that is, collocated blocks.

When a B and its future reference frame of I/P are in the same coding mode (frame or field), the collocated blocks of a direct mode MB in B are defined as the blocks in the same geometrical locations in the future reference I/P. When a B and its future reference frame of I/P are in different coding structure, the collocated blocks of a direct mode MB in B are defined as follows:

- When the current B in field coding and the future reference P in frame coding (Fig. 1): Let ycurrent and ycollocated be the vertical indexes of the blocks in the current MB and in its collocated MB respectively. Then, ycollocated=2ycurrent.  The blocks in the current MB and its collocated MB have the same horizontal indexes. The forward reference field is the same-parity field of the reference frame pointed by the MV of collocated block.

- When the current B in frame coding and the future reference P in field coding (Fig. 2): Let ycurrent and ycollocated be the vertical indexes of the blocks in the current MB and in its collocated MB respectively. Then, ycollocated=ycurrent2. The blocks in the current MB and its collocated MB have the same horizontal indexes. Note that like JVT WD, the collocated blocks are from field 1 of the future reference frame of I/P. The forward reference frame is the frame with one of its fields pointed by the MV of the collocated block.

With the above definitions of collocated blocks for direct mode MB, the forward and backward MVs of  the blocks of a direct mode MB can be calculated based upon the same rules in JVT WD [2].

The above have been implemented in JVT test mode SW - jm2.1.

Fig. 1. Current B picture is field coded while future P picture is frame coded.

Fig. 2. Current B picture is frame coded while future P picture is field coded.

Reference

- T. Wiegand, “Committee Draft of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)”, June, 2002.

- T. Wiegand and G. Sullivan, “Working draft of JVT standard”, April, 2002.

- L. Wang, K. Panusopone, R. Gandhi, Y. Yue and A. Luthra, “Adaptive frame/field coding for JVT”, JVT-B071, Jan., 2002.

- (Append for Proposal Documents)

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Motorola Inc. |  |  |
| Mailing address | # 6450 Sequence Drive San Diego, CA 92121 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Limin Wang |  |  |
| Telephone | 858-404-3893 |  |  |
| Fax | 858-404-2501 |  |  |
| Email | liwang@gi.com |  |  |
| Place and date of submission | San Diego, CA., USA, 05/01/02 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Collocated Blocks and Associated Forward Reference Frames / MVs for Direct Mode MB in AFF |  |
| Contribution number |  | JVT-D042 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| Without having conducted a search or investigation, the submitter/author(s) individually believe(s) they or General Instrument Corporation (Patent Holder) may have granted, pending or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. see comments below regarding Baseline Profile |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | Comment to Section 2.2. To support the proposed royalty free JVT Baseline Profile, and in addition to the statement of 2.2 above, Patent Holder is also willing to grant a royalty free license to patents essential to JVT Baseline Profile decoder applications on the condition that any and all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: