| Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document JVT-D046 File: JVT-D046.doc Generated: 2002-06-10 |
| --- | --- |

| Title: | Direct Prediction for Predictive (P) Picture in Field Coding mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone and Limin Wang Motorola, Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-3618 krit@motorola.com |
| Source: | Motorola, Inc., BCS |  |  |

Abstract

This documentation proposes direct prediction mode for predictive (P) picture in field coding mode. This new mode is an extension of the direct prediction mode, which has been considered in JVT standard at the Fairfax meeting.

The Proposed Scheme

In JVT CD [1], direct mode is allowed in B picture for both field and frame coding modes. Direct mode is efficient in interpolating motion effect between the two anchor pictures. Direct mode MB derives both forward and backward MVs based on the forward MV of its collocated block in the future anchor pictures. Direct mode MB also takes the same reference picture referred to by its collocated MB as the past anchor picture.

Field direct mode, proposed in [2], is based on the same idea as frame direct mode. Direct mode MB in the top field picture uses its collocated MB from the top field of its immediate future anchor picture. Similarly, direct mode MB in the bottom field picture uses its collocated MB from the bottom field of its immediate future anchor picture. Direct mode MB takes the same reference field picture referred to by its collocated MB as the past anchor picture. Details of the scaling process when its collocated MB is coded in a different picture type are given in [2].

Direct mode has recently been proposed as a new coding mode in P picture [3]. Direct mode MB saves bits by not sending any motion information. MV of direct mode MB in P picture is derived based on MV of the collocated position of its previously encoded MB (motion projection). It has been shown [3] that a combination of direct and motion_copy mode provides improvement of either 0.2 dB PSNR gain or 4% bitrate reduction.

The document proposes direct mode to be used in P picture in field coding mode. Similar to [3], forward MV of field direct mode MB is a projection of MV of its collocated MB. With adaptive switch of frame/field coding at picture level, both P picture and its past reference picture can be coded as one P frame picture or two B field pictures. Hence, in terms of frame/field coding modes, there can be four different combinations for a pair of a MB in P and its collocated MB in the past reference. Calculations of the MV in direct mode are slightly different for the four cases.

Case 1: Both the current MB and its collocated are in frame mode

Both the current B and its past reference are in frame structure. This case is the same as in progressive sequence. The forward reference is the frame pointed to by the same ref_idx_fwd number as the one used in the collocated MB. Forward MV of frame direct mode MB,MVF, is the same as MV  of its collocated MB.

Case 2: Both the current MB and its collocated MB are in field mode

Both the current P frame and its past reference are in field structure. MV of direct mode MB in P field is calculated from the forward MV of the collocated MB in the past reference field of the same parity. The forward reference field for the direct mode MB is the field that is marked by the same ref_idx_fwd number as the one used by the forward MV of the collocated MB. The forward field MV, MVF,i, for the direct mode MB is the forward MV of the collocated MB in field i of the past reference frame.

Case 3:The current MB is field mode and its collocated MB in frame mode

The current P is in field structure while its past reference is in frame structure. Let ycurrentMB and ycollocatedMB be the notations of the vertical indexes of the current MB and its collocated MB respectively. Then, ycollocatedMB=2ycurrentMB. Field MV of direct mode MB is calculated from the forward MV of the collocated MB in the past reference frame. Since the collocated MB is frame coded, fieldMV is derived by dividing the associated frame-based MV by 2 in vertical direction. Forward reference field of direct mode MB is the same parity field in the reference frame that is marked by the same ref_idx_fwd number as the one used by the forward MV of the collocated MB.

Case 4: The current MB is in frame mode while its collocated MB in fieldmode

The current P is in frame structure while its past reference is in field structure. Let ycurrentMB and ycollocatedMB be the notations of the vertical indexes of the current MB and its collocated MB respectively. Then, ycollocatedMB=ycurrentMB2. The two fields of the collocated MB in the past reference may be coded in different modes. Since field 1 of the past reference is temporally close to the current B, the collocated MB in field 1 of the past reference provides certain basis for the current MB. Hence, the collocated MB in field 1 of the past reference is used in calculating the MVs and determining the references for direct mode. MVF of direct mode MB is derived by doubling the field-based MV of the collocated MB in field 1 of the past reference in vertical direction.

Reference

- T. Wiegand, “Committee Draft of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)”, June, 2002.

- L. Wang, K. Panusopone, R. Gandhi, and A. Luthra, “Adaptive frame/field coding for JVT”, JVT-B071, Jan., 2002.

- A. M. Tourapis, F. Wu and S. Li, “Direct Prediction for Predictive (P) and Bidirectionally Predictive (B) frames in Video Coding”, JVT-C128, May, 2002.

- (Append for Proposal Documents)

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Motorola Inc. |  |  |
| Mailing address | # 6450 Sequence Drive San Diego, CA 92121 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Limin Wang |  |  |
| Telephone | 858-404-3893 |  |  |
| Fax | 858-404-2501 |  |  |
| Email | liwang@gi.com |  |  |
| Place and date of submission | San Diego, CA., USA, 05/01/02 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Direct Prediction for Predictive (P) Picture in Field Coding mode |  |
| Contribution number |  | JVT-D046 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| Without having conducted a search or investigation, the submitter/author(s) individually believe(s) they or General Instrument Corporation (Patent Holder) may have granted, pending or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. see comments below regarding Baseline Profile |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | Comment to Section 2.2. To support the proposed royalty free JVT Baseline Profile, and in addition to the statement of 2.2 above, Patent Holder is also willing to grant a royalty free license to patents essential to JVT Baseline Profile decoder applications on the condition that any and all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: