| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D059 Filename: JVT-D059.doc |
| --- | --- |

| Title: | Low Complexity Diagonal Mode for Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Greg Conklin 2601 Elliott Ave Seattle, WA 98121 USA | Tel: Email: | 206.674.2337 gregc@real.com |
| Source: | RealNetworks, Inc. |  |  |

_____________________________

1.  Introduction

This document examines the computational complexity of forming the Intra prediction modes as defined in the CD of the JVT standard.  It finds that one mode (mode 7) has a much higher computational requirement than the others.  Therefore, a new definition of mode 7 is proposed that reduces its computational complexity -- and improves coding performance up to 1.6%, although typically a gain of about 0.5% is found.

2.  Computational Complexity

Table 1 presents the number of operations (adds and shifts) required to form the diagonal Intra prediction modes (modes 3-8).  The number of operations under the “naïve” column were found merely by counting the adds and shifts shown in the mode definition in the CD.  However, by reusing intermediate calculations, the total number of operations can be reduced, and is calculated under the “smart” column. These numbers show that Mode 7 requires significantly more operations than other modes.

TABLE 1: Number of operations to form Intra prediction modes

| Mode | Direction | Naive |  |  | Smart |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Adds | Shifts | Total | Adds | Shifts | Total |
| 3 | SE | 21 | 14 | 35 | 22 | 7 | 29 |
| 4 | NE | 40 | 13 | 53 | 35 | 7 | 42 |
| 5 | SSE | 24 | 16 | 40 | 23 | 10 | 33 |
| 6 | NNE | 31 | 16 | 47 | 28 | 10 | 38 |
| 7 | ENE | 45 | 18 | 63 | 47 | 10 | 57 |
| 8 | ESE | 26 | 16 | 42 | 23 | 10 | 33 |

It is therefore proposed that Mode 7 be simplified to the following:

New Mode 7 - DIAG_PRED_ENE

a 		= (2*(C + I + J) + B + D + 4) / 8;

b 		= (2*(D + J) + C + E + I + K + 4) / 8;

c,e 		= (J + K + 1) / 2;

d,f 		= (J + 2*K + L + 2) / 4;

g,i 		= (K + L + 1) / 2;

h,j 		= (K + 2*L + M + 2) / 4;

k,m 		= (L + M + 1) / 2;

l,n 		= (L + 2*M + N + 2) / 4;

o 		= (M + N + 1) / 2;

p 		= (M + 2*N + O + 2) / 4;

This version of Mode 7 requires 31 adds and 16 shifts (47 total) in the naive implementation and 28 adds and 10 shifts (38 total) in the smart implementation.

Figure 1 shows a block diagram implementation of the new prediction mode (this diagram does not show the final add and shift required to properly scale the final results).  This figure is being provided to demonstrate a smart implementation that requires 39 operations.  Blocks a-p represent the current 4x4 block of pixels that are to be predicted by surrounding pixels A-E, I-O that have already been decoded by previous operations.

Figure 1: Block Diagram of New Diagonal Mode (Mode 7)

3.  Coding Performance

Table 2 shows the relative PSNR and Bitrate improvement that is achieved using the proposed mode 7.  Results show a reduction in bitrate between 0.3% - 1.6%.

TABLE 2: Bitrate and PSNR improvement using new diagonal mode

| Sequence | Image Size | Frame-Skip | Frames | BD-PSNR | BD-Bits |
| --- | --- | --- | --- | --- | --- |
| Akiyo | QCIF | 2 | 100 | 0.035 dB | -0.48% |
| Container* | QCIF | 2 | 100 | 0.064 dB | -0.97% |
| Foreman* | QCIF | 2 | 100 | 0.097 dB | -1.57% |
| Mother | QCIF | 0 | 300 | 0.017 dB | -0.31% |
| News* | QCIF | 2 | 100 | 0.029 dB | -0.36% |
| Silent* | QCIF | 1 | 150 | 0.032 dB | -0.57% |
| Akiyo | CIF | 0 | 300 | 0.045 dB | -0.67% |
| Container | CIF | 0 | 300 | 0.033 dB | -0.58% |
| Foreman | CIF | 0 | 300 | 0.059 dB | -1.12% |
| Hall | CIF | 0 | 300 | 0.064 dB | -0.96% |
| Mother | CIF | 0 | 300 | 0.018 dB | -0.34% |
| Mobile* | CIF | 0 | 300 | 0.029 dB | -0.34% |
| News | CIF | 0 | 300 | 0.032 dB | -0.43% |
| Paris* | CIF | 1 | 150 | 0.029 dB | -0.38% |
| Silent | CIF | 1 | 150 | 0.027 dB | -0.58% |
| Table | CIF | 0 | 300 | 0.025 dB | -0.46% |
| Tempete* | CIF | 0 | 260 | 0.026 dB | -0.38% |

*common conditions

4.  Conclusion

It is proposed that Mode 7 be redefined in order to reduce the worst-case computational complexity required to for the intra prediction.  The proposed mode reduces the number of operations from 57 operations to 39 operations and also slightly improves coding efficiency.

Appendix A – Number of operations for diagonal prediction modes

Appendix A presents the current 6 diagonal modes (modes 3-8) as they are defined in the current Committee Draft (CD) of the JVT standard (MPEG-4, part 10 / ITU H.26L), and the operation counts for each.  Additionally, re-definitions of these modes are given to illustrate the ability to reuse intermediate calculations to reduce the total number of operations to implement each mode.

Mode 3 - DIAG_PRED_SE

m 		= (L + 2*K + J + 2) / 4;

i,n 		= (K + 2*J + I + 2) / 4;

e,j,o 	= (J + 2*I + X + 2) / 4;

a,f,k,p 	= (I + 2*X + A + 2) / 4;

b,g,l		= (X + 2*A + B + 2) / 4;

c,h		= (A + 2*B + C + 2) / 4;

d 		= (B + 2*C + D + 2) / 4;

21 adds, 14 shifts.

m 		= ((L + K) + (K + J) + 2) / 4;

i,n 		= ((K + J) + (J + I) + 2) / 4;

e,j,o 	= ((J + I) + (I + X) + 2) / 4;

a,f,k,p 	= ((I + X) + (X + A) + 2) / 4;

b,g,l		= ((X + A) + (A + B) + 2) / 4;

c,h		= ((A + B) + (B + C) + 2) / 4;

d 		= ((B + C) + (C + D) + 2) / 4;

22 adds, 7 shifts.

Mode 4 - DIAG_PRED_NE

a 		= (A + C + I + K + 2*(B + J) + 4) / 8;

b,e 		= (B + D + J + L + 2*(C + K) + 4) / 8;

c,f,i		= (C + E + K + M + 2*(D + L) + 4) / 8;

d,g,j,m	= (D + F + L + N + 2*(E + M) + 4) / 8;

h,k,n		= (E + G + M + O + 2*(F + N) + 4) / 8;

l,o		= (F + H + N + P + 2*(G + O) + 4) / 8;

p		= (G + O + H + P + 2) / 4;

40 adds, 13 shifts.

a 		= ((A + B + I + J) + (B + C + J + K) + 4) / 8;

b,e 		= ((B + C + J + K) + (C + D + K + L) + 4) / 8;

c,f,i		= ((C + D + K + L) + (D + E + L + M) + 4) / 8;

d,g,j,m	= ((D + E + L + M) + (E + F + M + N) + 4) / 8;

h,k,n		= ((E + F + M + N) + (F + G + N + O) + 4) / 8;

l,o		= ((F + G + N + O) + (G + H + O + P) + 4) / 8;

p		= ((G + H + O + P) + 2) / 4;

35 adds, 7 shifts.

Mode 5 - DIAG_PRED_SSE

a,j 		= (X + A + 1) / 2;

b,k		= (A + B + 1) / 2;

c,l		= (B + C + 1) / 2;

d 		= (C + D + 1) / 2;

e,n 		= (I + 2*X + A + 2) / 4;

f,o		= (X + 2*A + B + 2) / 4;

g,p		= (A + 2*B + C + 2) / 4;

h 		= (B + 2*C + D + 2) / 4;

i		= (X + 2*I + J + 2) / 4;

m		= (I + 2*J + K + 2) / 4;

24 adds, 16 shifts.

a,j 		= ((X + A) + 1) / 2;

e,n 		= ((I + X) + (X + A) + 2) / 4;

b,k		= ((A + B) + 1) / 2;

f,o		= ((X + A) + (A + B) + 2) / 4;

c,l		= ((B + C) + 1) / 2;

g,p		= ((A + B) + (B + C) + 2) / 4;

d 		= ((C + D) + 1) / 2;

h 		= ((B + C) + (C + D) + 2) / 4;

i		= ((X + I) + (I + J) + 2) / 4;

m		= ((I + J) + (J + K) + 2) / 4;

23 adds, 10 shifts

Mode 6 - DIAG_PRED_NNE

a 		= (2*(A + B + K) + J + L + 4) / 8;

b,i 		= (B + C + 1) / 2;

c,j 		= (C + D + 1) / 2;

d,k 		= (D + E + 1) / 2;

l 		= (E + F + 1) / 2;

e 		= (2*(B + L) + A + C + K + M + 4) / 8;

f,m 		= (B + 2*C + D + 2) / 4;

g,n 		= (C + 2*D + E + 2) / 4;

h,o 		= (D + 2*E + F + 2) / 4;

p 		= (E + 2*F + G + 2) / 4;

31 adds, 16 shifts

a 		= (((A + B) + (K + L)) + (J + K) + (A + B) + 4) / 8;

e 		= (((A + B) + (K + L)) + (B + C) + (L + M) + 4) / 8;

b,i 		= ((B + C) + 1) / 2;

c,j 		= ((C + D) + 1) / 2;

f,m 		= ((B + C) + (C + D) + 2) / 4;

d,k 		= ((D + E) + 1) / 2;

g,n 		= ((C + D) + (D + E) + 2) / 4;

l 		= ((E + F) + 1) / 2;

h,o 		= ((D + E) + (E + F) + 2) / 4;

p 		= ((E + F) + (F + G) + 2) / 4;

28 adds, 10 shifts

Mode 7 – DIAG_PRED_ENE

a 		= (2*(C + I + J) + B + D + 4) / 8;

b 		= (2*(D + J) + C + E + I + K + 4) / 8;

c,e 		= (2*(E + J + K) + D + F + 4) / 8;

d,f 		= (2*(F + K) + E + G + J + L + 4) / 8;

g,i 		= (2*(G + K + L) + F + H + 4) / 8;

h,j 		= (2*(H + L) + G + H + K + L + 4) / 8;

l,n 		= (L + 2*M + N + 2) / 4;

k,m 		= (G + H + L + M + 2) / 4;

o 		= (M + N + 1) / 2;

p 		= (M + 2*N + O + 2) / 4;

45 adds, 18 shifts

a 		= ((B + C) + (C + D) + 2*(I +J) + 4) / 8;

b 		= ((C + D) + (D + E) + (I + J) + (J + K) + 4) / 8;

c,e 		= ((D + E) + (E + F) + 2*(J + K) + 4) / 8;

d,f 		= ((E + F) + (F + G) + (J + K) + (K + L) + 4) / 8;

g,i 		= ((F + G) + (G + H) + 2*(K + L) + 4) / 8;

h,j 		= ((G + H) + 2*H + (K + L) + 2*L + 4) / 8;

l,n 		= ((L + M) + (M + N) + 2) / 4;

k,m 		= ((G + H) + (L + M) + 2) / 4;

o 		= ((M + N) + 1) / 2;

p 		= ((M + N) + (N + O) + 2) / 4;

47 adds, 10 shifts

Mode 8 - DIAG_PRED_ESE

a,g 		= (X + I + 1) / 2;

b,h 		= (I + 2*X + A + 2) / 4;

c 		= (X + 2*A + B + 2) / 4;

d 		= (A + 2*B + C + 2) / 4;

e,k 		= (I + J + 1) / 2;

f,l 		= (X + 2*I + J + 2) / 4;

i,o 		= (J + K + 1) / 2;

j,p 		= (I + 2*J + K + 2) / 4;

m 		= (K + L + 1) / 2;

n 		= (J + 2*K + L + 2) / 4;

26 adds, 16 shifts

d 		= ((A + B) + (B + C) + 2) / 4;		// 4,1

c 		= ((X + A) + (A + B) + 2) / 4;

b,h 		= ((I + X) + (X + A) + 2) / 4;

a,g 		= ((X + I) + 1) / 2;

f,l 		= ((X + I) + (I + J) + 2) / 4;

e,k 		= ((I + J) + 1) / 2;

j,p 		= ((I + J) + (J + K) + 2) / 4;

i,o 		= ((J + K) + 1) / 2;

n 		= ((J + K) + (K + L) + 2) / 4;

m 		= ((K + L) + 1) / 2;

23 adds,  10 shifts

Appendix B – Number of operations for the proposed diagonal prediction mode

The following is the definition of the new prediction mode for mode 7.

New Mode 7 - DIAG_PRED_ENE

a 		= (2*(C + I + J) + B + D + 4) / 8;

b 		= (2*(D + J) + C + E + I + K + 4) / 8;

c,e 		= (J + K + 1) / 2;

d,f 		= (J + 2*K + L + 2) / 4;

g,i 		= (K + L + 1) / 2;

h,j 		= (K + 2*L + M + 2) / 4;

k,m 		= (L + M + 1) / 2;

l,n 		= (L + 2*M + N + 2) / 4;

o 		= (M + N + 1) / 2;

p 		= (M + 2*N + O + 2) / 4;

31 adds, 16 shifts

The following is a re-definition of the new prediction mode for mode 7 that helps illustrate how to reuse intermediate operations to reduce the total number of operations.

New Mode 7 - DIAG_PRED_ENE

a 		= (((C + D) + (I + J)) + (B + C) + (I + J) + 4) / 8;

b 		= (((C + D) + (I + J)) + (D + E) + (J + K) + 4) / 8;

c,e 		= ((J + K) + 1) / 2;

d,f 		= ((J + K) + (K + L) + 2) / 4;

g,i 		= ((K + L) + 1) / 2;

h,j 		= ((K + L) + (L + M) + 2) / 4;

k,m 		= ((L + M) + 1) / 2;

l,n 		= ((L + M) + (M + N) + 2) / 4;

o 		= ((M + N) + 1) / 2;

p 		= ((M + N) + (N + O) + 2) / 4;

28 adds, 10 shifts

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | RealNetworks, Inc. |  |  |
| Mailing address | P.O. Box 91123 Seattle, WA 98111-9223 |  |  |
| Country | USA |  |  |
| Contact person | Greg Conklin |  |  |
| Telephone | +1 206 674 2337 |  |  |
| Fax | +1 206 674 3580 |  |  |
| Email | gregc@real.com |  |  |
| Place and date of submission | Klagenfurt, Austria, July, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Low Complexity Diagonal Mode for Intra Prediction |  |
| Contribution number |  | JVT-D059 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | Gregory Conklin |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: For purposes of this contribution, the terms of 2.2.1 above [and as detailed in document VCEG-O54 submitted by Gary Sullivan] are agreed to only as necessary to comply with the required portions of a “baseline” JVT video profile.