| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D069-d2 Filename: JVT-D069-d2.doc |
| --- | --- |

| Title: | Prediction based Directional Refinement (PDR) algorithm for Fractional Pixel Motion Search Strategy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhibo Chen, Yun He Dept. of Electronic Engineering, Tsinghua University, Beijing, China Yidong Chen SVA Group, Co.Ltd, Shanghai, China | Tel: Email: | 86-10-62781413 chenzb@ieee.org hey@tsinghua.edu.cn |
| Source: | Tsinghua University, China and SVA Group, China |  |  |

1. Introduction

Hybrid coding framework has been successfully adopted in various moving picture coding standards, such as H.261, MPEG-1, MPEG-2, H.263, and MPEG-4. Comparing with H.263, some significant changes have been adopted in H.26L[1]: the residue coding is 4x4 block based, the DCT is replaced by integer transform, universal VLC table is used, 1/4 and 1/8 pixel accuracy motion estimation is proposed, etc. A PSNR analysis shows the superiority of the H.26L results of averages more than 2 dB better in luma PSNR across all test cases, relative to ISO-MPEG’s chosen MPEG-4 “Advanced Simple” profile anchor performance. However, H.26L is essentially still based on hybrid coding framework, motion estimation and compensation take even more important parts in achieving the high coding efficiency.

Generally motion estimation is conducted into two steps: the first step is integer-pel motion estimation; and the second is fractional pixel motion estimation. Algorithms on fast motion estimation are always hot research topics, especially the fast integer-pel motion estimation. The fact is that the full 1/2-pel motion estimation which needs eight checking points takes a trivial part in the total computation load of motion estimation. However, the development of the fast integer-pel motion estimation algorithms has decreased the checking points down to ten [2][3]. As a result, the computation load for fractional pixel motion estimation takes a major portion in the total ME, because 16 or 24 checking points are needed each in 1/4 or 1/8 pixel motion estimation if a full search strategy is used. From our knowledge, there may not have relative works investigating the fast fractional pixel motion estimation algorithms in the proposals, hence, this proposal will present the method, test results and shows both necessary and significant on this topic.

By using our proposed method, in any cases (1/4 pixel or 1/8 pixel resolution) the computation load of fractional-pel motion estimation can be reduced to the 17.4%～34.7% of the original full fractional-pel search algorithm. And experimental results show that our method keeps a better performance in preserving image quality and makes little influence on the bit rate.

Technical description and experimental results are given in the following sections.

2. Technical Description

2.1 Full Fractional Pixel Motion Estimation algorithm in H.26L

In H.26L, for each block or macro-block the motion vector is obtained using full search on integer-pel resolution followed by fractional-pel refinement. The fractional-pel refinement is done also by full fractional-pel search algorithm.

To retell the reference method, let’s fist redraw the fractional-pel positions in Fig.1, where the capital letters represent integer pixel positions, the Roma numbers represent 1/2-pel positions, the lower case letters represent 1/4-pel positions and the Arabian numbers represent 1/8-pel positions. In a motion compensated prediction scheme, the motion vector and the block-size pattern (e.g. 16x16, four 8x8, etc.) have to be estimated for each macroblock, which is done by the following 5 steps:

- Make an integer-pel search to find the best integer-pel motion vector;

- Check the eight 1/2-pel candidates, I ~ VIII around the best integer-pel C (correspondent to the integer-pel motion vector) ; decide the best 1/2-pel (correspondent to the 1/2-pel motion vector) V in this case, subject to the minimal cost among the 1/2 -pel candidates;

- Check the eight 1/4-pel candidates, a ~ h around the best 1/2-pel V, decide the best 1/4-pel (correspondent to the 1/4-pel motion vector ) h in this case, subject to the minimal cost among the 1/4-pel candidates;

- Check the eight 1/8-pel candidates, 1 ~ 8 around the best 1/4-pel h, decide the best1/8-pel (correspondent to the 1/8-pel motion vector ), subject to the minimal cost among the 1/8-pel candidates;

- Select the motion vector and block-size pattern, which produces the lowest cost.

In generation of these fractional pixel positions, a 6 tap filter: (1,-5,20,20,-5,1)/32 is used to produce the 1/2-pel positions; 1/4-pel positions is produced by linear interpolation; and an 8-tap filters is used in providing a 1/8-pel accuracy prediction.

Fig.1 Full Fractional Pixel Motion Estimation

2.2 Prediction based Search Strategy for 1/2-pel motion estimation

In H.26L, SAD (Sum of Absolute Difference) is used as the basic cost function, which is described as:

F(p)=L(p)−L(c)         		(1)

where p is an integer-pel or a fractional-pel in the previous reconstructed picture, c is the pixel in the current picture, L(⋅) represents the luma of the pixel. The summation in (1) is executed on the different block-size pattern in H.26L. As Fig.1 shows, suppose C is the best integer-pel, the integer-pels around C are H1, H2, V1, V2 and their cost function values are F(C), F(H1), F(H2), F(V1), F(V2) respectively. In our method it’s assumed that all these 5 values are available in the integer pixel motion estimation. This assumption can be satisfied by the fast integer pixel motion estimation algorithms with a diamond search pattern such as Advanced Prediction Diamond Search(APDZS)[3], Diamond Search (DS)[4], Nearest-Neighbors Search (NNS)[5] and Unrestricted Center-Biased Diamond Search (UCBDS)[6]. For other algorithms while couldn’t satisfy this assumption, we can calculate the coast functions of required positions.

The strategy can be described in several steps:

1) Obtain the minimum value SADmin and sub-minimum value SADsub among SAD(V1)，SAD(V2)，SAD(H1)and SAD(H2), PminandPsubare the according pixel positions of SADmin and SADsub, where:

Pmin=argminj(SAD(Pj)), subject to Pj∈Ω

Psub=arg minj(SAD(Pj)), subject to Pj∈(Ω-Pmin)

={V1，V2，H1，H2}

2) if Pminand Psub are in the same horizontal or vertical line, i.e XPmin=XPsub or YPmin=YPsub, select two 1/2-pels on the line segment Pmin-Psub as well as C  as the 1/2-pel candidates (for example in Fig.1, if V1 and V2 are the minimum and sub-minimum points respectively, II, IIV and C will be the candidates for 1/2-pel motion estimation.)

3) if XPmin≠XPsub and YPmin≠YPsub, a 1/2-pel on line segment Pmin-C, a 1/2 –pel on line segment Psub-C, a 1/2-pel between line segments Pmin-C and Pmin-Psub, as well as C are chosen as the 1/2-pel candidates (for example in fig.1, if V1 and H1 are the minimum and sub-minimum points respectively, I，II，IV and C will be the candidates for 1/2-pel motion vector decision making).

4) Among the candidates points, choose the 1/2 -pel with the minimum cost function as the best 1/2-pel, and the 1/2-pel with the sub-minimum cost function as the sub-best 1/2-pel.

2.3 Direction Refinement Search Strategy for 1/4- or 1/8-pel motion estimation

As described in the previous section 2.2, the best 1/2-pel and the sub-best 1/2-pel have been chosen. By comparing the relative positions of the best 1/2-pel and the sub-best 1/2-pel, two search patterns are shown up in Fig.2 (a) and (b), where A is the best 1/2-pel and B is the sub-best 1/2-pel. In case (a), A and B are in the same horizontal or vertical line, three 1/4-pels in block font style between A and B can be chosen as the checking pixels or candidates for 1/4-pel motion estimation, in this case they are 1/4 pels 3,5,8. In case (b), A and B are lining in the same diagonal direction, then the three 1/4-pels between A and B, in this case the 1/4-pels 2,3,5 in block font style, can be chosen as the candidates. Then the best 1/4-pel and the sub-best 1/4-pel are obtained from the three 1/4-pel candidates and the best 1/2-pel by minimum and sub-minimum cost calculations.  In the same way, the 1/8-pel motion vector can be decided as that of the 1/4-pel.

Fig.2 Two patterns in prediction refinement

2.4 Half-Stop judgment:

In our fractional-pel motion search strategy, we add a half-stop judgment rule to stop the search process when the current minimum cost is below a certain threshold. That is:

if    min_cost<T  , stop search

Here we should choose different T for different block-size, and this T can be derived from the experiment or from the transform and quantization method in H.26L.

3. Experiment Results:

The proposed fractional fast motion estimation scheme is implemented with an integer fast motion scheme.

# Testing conditions:

4 tests are:

test 1: 	JM2.0

test 2: 	JM2.0 + JVT-D069 (our proposal)

test 3. 	JM2.0 + JVT-D117

test 4. 	JM2.0 + JVT-D117 + JVT-D069(our proposal)

This fast motion vector search software is modified from JVT-D117_software.zip, a motion vector search range prediction scheme proposed by JVT-D117. A fast integer and full fractional motion vector search are added. The encoding source code is in JVT-D069-software.zip, the test results are in JVT-D069-result-020722.xls.

4 testing sequences are:

Container	CIF		260 frames skip 0

News		QCIF	300 frames skip 2

Silence		QCIF	300 frames skip 1

Tempete		QCIF	300 frames skip 2

Encoding parameters:

Search range(or start search range): (-32,32)

Inter search mode: 16x16

MV  resolution: 1/4 pel

Number of Reference Frames: 1

Symbol mode: CABAC

Inter search mode: 16x16

RD Optimization: used

Loop Filter: used

Testing platform:

Intel Celeron 800Mhz

64MRAM

Windows 2000

5. Conclusions:

Compare test 2 with test 1: The computation saving for the encoder is 40%~60%, PSNR loss is less than 0.05dB.

Compare test 4 with test 3: The computation saving for the encoder is 15%~28%, PSNR loss is less than 0.07dB

It is suggested to first use JVT-D117 to predict the search range, and then use JVT-D069’s integer and fractional fast motion search to further save computation time. The fast fractional motion search software will be provided late.

Reference:

[1] “H.26L Test Model Long Term Number 8 (TML-8) draft0”, ITU VCEG, June, 2001

[2] P.I.Hosur and K.K.Ma, “Motion Vector Field Adaptive Fast Motion Estimation”, Second Inter. Conf. On Information Communications and Signal Processing(ICICS’99). Singapore. Dec’99.

[3] Tourapis, A.M.; Au, O.C.; Liou, M.L. “New results on zonal based motion estimation algorithms-advanced predictive diamond zonal search”, Circuits and Systems, 2001. ISCAS 2001. The 2001 IEEE International Symposium on , Volume: 5 , 2001 Page(s): 183 -186 vol. 
[4] Shan Zhu and Kai-Kuang Ma, “A New Diamond Search Algorithm for Fast Block-Matching Motion Estimation”, IEEE Trans. Image Processing, vol. 92, no. 2, pp. 287 -290, Feb. 2000

[5] M. Gallant, F. Kossentini, "A computation constrained motion vector search algorithm for block-based motion estimation," IEEE conference on signal, systems, and computers, Nov. 1998

[6] J. Y. Than, S. Ranganath, M. Ranganath, and A. A. Kassim, "A Novel Unrestricted Center-Biased Diamond Search Algorithm for Block Motion Estimation," IEEE Trans. Circuits and System for Video Technology, vol. 8, no. 4, pp. 369-377, Aug. 1998

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Tsinghua University, China and SVA Group, China |  |  |
| Mailing address | 100084 425# East mail building, Tsinghua University, Beijing, China |  |  |
| Country | China |  |  |
| Contact person | Zhibo Chen |  |  |
| Telephone | +86-10-62781413 |  |  |
| Fax | +86-10-62770317 |  |  |
| Email | chenzb@video.mdc.tsinghua.edu.cn |  |  |
| Place and date of submission | 7/15/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Prediction based Directional Refinement (PDR) algorithm for Fractional Pixel Motion Search Strategy |  |
| Contribution number |  | JVT-C034 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: