| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D071 Filename: JVT-D071.doc |
| --- | --- |

| Title: | # Transmission of Auxiliary Chroma Information for Upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez 5030 Sugarloaf Parkway Lawrenceville, GA 30042 USA | Tel: Email: | +1 (770) 236-6483 aar@sciatl.com |
| Source: | Scientific-Atlanta |  |  |

_____________________________

Summary

This contribution proposes transmission of ancillary chroma information for chroma upsampling to perform the closest reverse filtering that matches the chroma subsampling performed by the encoder. We propose transmission of a minimal amount of ancillary information.

Introduction

Encoders often accept 4:2:2 interlace or 4:2:2 progressive video as input.  Methods of subsampling from 4:2:2 to 4:2:0 tend to differ between encoders from different manufacturers since they tend to perform proprietary noise reduction and picture downscaling operations.  It is common to combine chroma subsampling with these operations in the pre-processing stage.  Noise filtering in turn depends on the specific method employed for noise level estimation and likely controlled by the encoder’s proprietary rate control strategy.

These factors amount to potential differences in chroma subsampling for progressive pictures.  However, chroma subsampling schemes on interlaced pictures tend to differ to a greater extent.  Differences stem from subsampling philosophies that are possibly justified empirically or as viable trade-offs between quality and cost.

For interlace pictures, some encoders place subsampled chroma at a quarter pixel shift toward the top in the top field and a quarter pixel shift toward the bottom in the bottom field.  Other encoders place a sub-pixel shift in one type of field and a different or no phase-shift in the opposite field.  Some encoders do not implement a subpixel shift at all.

The advent of personal video recorders in consumer devices introduces yet another consideration.   When an NTSC or PAL decoded video signal is presented as input to an encoder, filtering in the pre-processing stage and chroma subsampling leverage from the fact that the input video signal is band-limited. Encoders in these devices produce video streams to be solely consumed and decoded by the resident decoder.  The resident decoder must decode broadcast streams as well, as is the case in a set-top box.  However, the encoder’s chroma subsampling tends to differ from the chroma subsampling in the “assumed” encoder producing the broadcast video streams.

Encoders in products capable of decoding multiple video formats will be required to encode video from another format to H.26L video format (i.e., perform a transcode operation).   Consequently, an H.26L encoder may be presented through shared memory or through a physical input port a digitized 4:2:0 video format with a different chroma position.  Since the resident decoder is the sole consumer, it would be advantageous to preserve the original chroma location.

As evident from the aforementioned, chroma upsampling can benefit from transmission of ancillary information to perform the closest reverse filtering that matches the chroma subsampling performed by the encoder.  This should result in the elimination of chroma artifacts that often manifest as jagged edges and horizontal streaks in certain color transitions and especially in synthetic like colors.

Proposal

Rather than a display process be informed of the progressive or interlace nature of the frame to effect one of two chroma upsampling generalized filters, we propose a minimal aggregate number of bits to carry ancillary chroma information.   We propose transmission of ancillary chroma information in the Supplemental Enhancement Information (SEI) payload of H.26L to enhance chroma upsampling.   Alternatively, the table could be transmitted as any element in the hierarchy of layers of syntactical elements in H.26L that the body of video experts deemed suitable.

In all three proposals below, ancillary chroma information should be transmitted every 0.5 or 1.0 second.  Alternatively, it can be transmitted at the as frequent as SEI is transmitted.

### Proposal A:  Index Transmission in SEI field

A table of filters can be pre-specified to the decoder.   Each entry in the table can comprise of an upsampling filter or alternatively a pair of filters, one for each respective field type.  An index to the table is transmitted in a finite size field of the SEI payload.  Each filter or pair of filters can comprise a certain number of taps.  Obviously, a display operator may not be capable of implementing the required filter due to limited number of line buffers in its display functionality.  However, an index to a filter (or pair of filters) in the table merely connotes three pieces of information:

- A subpixel shift from a set of agreed and predetermined set of subpixel shifts,

- A level of low pass filtering from a set of agreed and predetermined levels, and

- A filter to top-field or bottom-field association.

Therefore, an index merely allows the display operation to perform the most appropriate corresponding measure at its disposal for chroma upsampling.  Some decoders may opt to ignore the index and perform customary generalized chroma upsampling.  Other decoders may beneficially perform the appropriate chroma upsampling.

A single index may be sufficient to signal the pixel phase-shift for one type of field and a different phase-shift in the opposite field.

Proposal B:  One-bit field and optional index transmission in SEI

A field in the SEI payload as small as one-bit can be transmitted to signal to the decoder that ancillary chroma information is to be employed for chroma upsampling.  No aggregate information (i.e., index) is transmitted when the flag is inactive.  When the flag is active or enabled, an aggregate field carrying the index is transmitted.  The aggregate information can be as small as four bits (or a nibble) to specify one of sixteen possible filter pairs (or chroma upsampling choices) for both top and bottom fields.  Alternatively, two small fields can carry indices for chroma upsampling for the top and bottom fields, respectively.

Even when the aggregate field is transmitted, a decoder may opt to ignore the index and perform its customary generalized chroma upsampling.

Proposal C:  Two-bit field and less frequent index transmissionin SEI

A two-bit field in the SEI can be transmitted to signal to the decoder one of three possible states:

- active and no immediate transmission of ancillary chroma information,

- active and immediate transmission of information in aggregate index field (nibble), or

- not active (disabled).

The aggregate field may alternatively comprise of two smaller indices.  Even when the aggregate index field is transmitted, a decoder may opt to ignore the index and perform its customary generalized chroma upsampling.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Scientific-Atlanta, Inc. |  |  |
| Mailing address | 5030 Sugarloaf Parkway Lawrenceville, GA 30042 USA |  |  |
| Country | USA |  |  |
| Contact person | K. M. Massaroni |  |  |
| Telephone | 770.236.4717 |  |  |
| Fax | 770.236.4837 |  |  |
| Email | Ken.massaroni@sciatl.com |  |  |
| Place and date of submission | 4th (Klagenfurt, Austria) JVT meeting, 22-26 July 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | # Transmission of Auxiliary Chroma Information for Upsampling |  |
| Contribution number |  | JVT-D071 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: