| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D074 Filename: JVT-D074.doc |
| --- | --- |

| Title: | Alternative Skip Mode in B-picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hideaki Kimata NTT Cyber Space Laboratories 1-1 Hikari-no-oka, Yokosuka JAPAN | Tel: Email: | +81 468 59 3124 kimata.hideaki@lab.ntt.co.jp |
| Source: | NTT |  |  |

_____________________________

- Introduction

In the last JVT meeting, B-picture was enhanced to select two temporally forward reference pictures in the bi-predictive mode. And skip mode MC for P-picture was adopted in terms of bug fixing. So B-picture should have the same functionality as for the skip MB of the P-picture.

And there is another bug in direct mode definition described in the current CD, where there is no clear description about direct mode when collocated block is intra, or collocated block’s ref_idx_forward is not zero. Skip MB of B-picture is currently defined as direct mode, so the same problem exists for skip MB.

This document proposes the scheme that the semantic of skip MB in B-picture is adaptively changed, in order to fix the above bugs. Proposed modification does not change the current syntax, and does not change the complexity of encoder and decoder.

- Proposed Adaptively Changing Prediction Mode at Skip MB

In the proposed method, each semantic for 8x8 block in Skip MB is determined by the following flow.

(s1) If there is any direct mode block around the current 8x8 block,

and collocated block is not Intra,

and the ref_idx_forward of collocated block is zero,

the semantic of the block is set to direct mode.

(s2) Ohterwise, counting candidate MV for forward or backward prediction, which is available for ref_idx_forward=0 or ref_idx_backward=0.

(s3) Then the semantic of the 8x8 block is set as following, where

Nf is the number of candidate MV for forward prediction

Nb is the number of candidate MV for backward prediction

(s3-1) If Nf=0 and Nb=0,

and co-located block is not Intra

and the ref_idx_forward of collocated block is zero,

the semantic is set to direct mode

(s3-2) If Nf=0 and Nb=0,

and co-located block is Intra,

or the ref_idx_forward of collocated block is not zero,

the semantic is set to bi-predictive mode

(s3-3) If Nf>0 and Nb=0,

the semantic is set to forward mode

(s3-4) If Nf=0 and Nb>0,

the semantic is set to backward mode

(s3-5) If Nf>0 and Nb>0,

the semantic is set to bi-predictive

In (s1), the surrounding blocks are depicted in Figure 1. The prediction modes of four surrouding blocks A,B,C and D are checked, where the current block is positioned in E. When the current block E is upper right or bottom right position in MB, three positions A,B and D are checked.

Figure 1. The illustration of four surrouding blocks A,B,C,D around the current block E.

- Evaluation

  - Experimental conditions

Experiments were conducted under almost the common test conditions specified in VCEG-N77r1. The base software is JM1.9+new CABAC code, which is delivered by Detlev Marpe. And JVT-C127 and motion copy in P-picture is integrated into, and RD optimization of the skip MB in B-picture is implemented. We just name this integrated software as JM1.9+ in this document.

The search range of motion estimation is set to –32 to +32. The IBBP structure is experimented, where the backward prediction in B-picture is on the future picture reference.

  - Proposal total efficiency

Table 1 shows BD-bitsaving in UVLC cases, and Table 2 shows BD-bitsaving in CABAC cases.

All bitrate and PSNR data and PSNR plots are illustrated in JVT-D074.xls.

Table 1: BD-bitsaving with the proposed method in UVLC case

| [%] | Cont(Q) | News(Q) | Fore(Q) | Sile(Q) | Stef(Q) | Pari | Mobi | Temp | Coas | Stef | Fore | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 ref | -0.40 | 1.51 | 4.90 | 1.82 | 2.16 | 2.10 | 3.86 | 0.83 | 7.00 | 4.77 | 6.51 | 3.19 |
| 1 ref | -0.48 | 1.06 | 4.13 | 0.68 | 1.68 | 1.11 | 0.43 | 0.44 | 4.73 | 4.06 | 4.10 | 1.99 |

Table 2: BD-bitsaving with the proposed method in CABAC case

| [%] | Cont(Q) | News(Q) | Fore(Q) | Sile(Q) | Stef(Q) | Pari | Mobi | Temp | Coas | Stef | Fore | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 ref | -0.67 | 1.22 | 3.47 | 1.57 | 1.55 | 1.73 | 3.51 | 0.68 | 5.95 | 3.94 | 5.53 | 2.59 |
| 1 ref | -0.37 | 0.75 | 2.89 | 0.69 | 1.53 | 0.92 | 0.37 | 0.57 | 4.55 | 3.29 | 3.67 | 1.71 |

From these results, the proposed method is generally effective and especially more in high motion scene, such as Stefan, Foreman, and Coast.

  - Efficiency of surrounding direct mode blocks checks

To investigate the efficiency of surrounding direct mode blocks checks, another experiment is done where step (s1) is modified as follows. We name this scheme as P2 in this document.

(s1#2) If collocated block is not Intra,

and the ref_idx_forward of collocated block is zero,

the semantic of the block is set to direct mode.

(s2) Ohterwise, …

Table 3: BD-bitsaving with P2 in UVLC case

| [%] | Cont(Q) | News(Q) | Fore(Q) | Sile(Q) | Stef(Q) | Pari | Mobi | Temp | Coas | Stef | Fore | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 ref | -0.07 | 1.60 | 2.12 | 1.82 | 0.54 | 1.70 | 3.97 | 0.93 | 5.09 | 1.61 | 3.99 | 2.12 |
| 1 ref | -0.22 | 1.13 | 0.86 | 0.47 | 0.13 | 0.46 | 0.30 | 0.30 | 2.64 | 0.88 | 1.34 | 0.75 |

Table 3 shows BD-bitsaving in UVLC cases. Comparing with Table 1, we can see the bitsaving with P2 is smaller than with the proposed method. From this result, the procedure of surrounding direct mode blocks checks is proved to be effective.

  - Efficiency of checking collocated block’s type and ref_idx_forward

And to investigate the efficiency of changing direct mode itself, another experiment is done where step (s1) and (s3-2) is modified as follows. We name this scheme as P3 in this document.

(s1#3) If there is any direct mode block around the current 8x8 block,

the semantic of the block is set to direct mode.

(s2) Ohterwise, …

(s3)

(s3-2#3) If Nf=0 and Nb=0,

and co-located block is Intra,

or the ref_idx_forward of collocated block is not zero,

the semantic is set to direct mode

Table 4: BD-bitsaving with P3 in UVLC case

| [%] | Cont(Q) | News(Q) | Fore(Q) | Sile(Q) | Stef(Q) | Pari | Mobi | Temp | Coas | Stef | Fore | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 ref | -0.09 | 0.44 | 4.14 | 0.76 | 1.98 | 1.03 | 0.64 | 0.27 | 3.10 | 4.33 | 4.40 | 1.91 |
| 1 ref | -0.06 | 0.54 | 3.88 | 0.45 | 1.73 | 0.86 | 0.14 | 0.15 | 2.76 | 3.94 | 3.56 | 1.63 |

Table 4 shows BD-bitsaving in UVLC cases. Comparing with Table 1, we can see the dependency on image is similar, but the bitsaving with P3 is smaller than with the proposed method. From this result, the procedure of checking collocated block’s type and ref_idx_forward is proved to be effective.

  - Efficiency of changing direct mode itself

And to investigate the efficiency of changing direct mode itself, one more another experiment is done where in addition to changing the semantic of skip MB like Proposal, the semantic of direct mode block is changed in the same flow following (s2) and (s3). (s1) is not done for direct mode block besides skip MB. We name this scheme as P4 in this document.

Table 5: BD-bitsaving with P4 in UVLC case

| [%] | Cont(Q) | News(Q) | Fore(Q) | Sile(Q) | Stef(Q) | Pari | Mobi | Temp | Coas | Stef | Fore | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 ref | -1.74 | 0.98 | 4.64 | 0.62 | 1.56 | 1.51 | -2.22 | -4.62 | 5.52 | 3.60 | 5.18 | 1.37 |
| 1 ref | -1.94 | 0.91 | 4.10 | 0.08 | 1.11 | 0.73 | -4.42 | -1.25 | 3.94 | 3.07 | 2.67 | 0.82 |

Table 5 shows BD-bitsaving in UVLC cases. Comparing with Table 1, we can see the bitsaving with P4 is smaller than with the proposed method. In addition, the severe degradation is seen in Mobile and Tempete. From this result, changing direct mode itself is proved not to be effective in the scheme, where just the semantic of the direct mode is alternatively changed to bi-predictive or forward or backward.

- Conclusion

We propose an alternative skip mode which is adaptively changing prediction mode in B-picture. The proposed method achieves the better coding efficiency especially for motion scene both in UVLC and CABAC. It also fixes bugs about direct mode issues in terms of skip MB.

From other experiments, both the procedure of surrounding direct mode checks, and checking collocated block’s type and ref_idx_forward in the proposal are proved to be effective. And it is also proved that changing the semantic of all direct mode blocks would degrade the coding efficiency in some cases.

Proposed method does not change the semantic of direct mode, just change that of skip MB. I propose to adopt this alternative skip MB scheme. As for the semantic of direct mode itself, other techniques could be applied in combination with my proposal.

- References

[1] Jani Lainema, “Skip mode motion compensation,” JVT-C027, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

[2] Alexis Michael Tourapis, “Motion Vector Prediction in Bidirectionally Predictive (B) frames with regards to Direct Mode, “JVT-C127, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

[3] “Joint Committee Draft (CD),” JVT-C167, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NTT |  |  |
| Mailing address | 1-1 Hikari-no-oka, Yokosuka, Kanagawa |  |  |
| Country | JAPAN |  |  |
| Contact person | Yoshiyuki Yashima |  |  |
| Telephone | +81 468 59 2831 |  |  |
| Fax | +81 468 59 2829 |  |  |
| Email | yashima.yoshiyuki@lab.ntt.co.jp |  |  |
| Place and date of submission | Klagenfurt, Austria, 22-26 July, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Alternative Skip Mode in B-picture |  |
| Contribution number |  | JVT-D074 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | Hideaki Kimata, Yoshiyuki Yashima |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: