| # Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-C091 Filename: JVT-C091.doc |
| --- | --- |

| Title: | On Level Definitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland | Tel: Email: | +358 40 5212845 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Summary

The contribution proposes proposes values for maximum bit-rate and HRD/VBV buffer size for some levels defined in the JVT committee draft. A different definition of the peak bit-rate is proposed for low-latency and high-latency applications. In low-latency applications, the peak bit-rate is equal to the peak transmission bit-rate, whereas in high-latency applications, the peak bit-rate is equal to the peak video bit-rate.

Proposed agenda category: 2) issues previously identified for further study/verification

Keywords for agenda allocation: profiles & levels

# Maximum Video Bit-rate

## Transmission Bit-Rate versus Video Bit-Rate

Internet and mobile streaming applications are characterized by the fact that relatively much data (typically a couple of seconds) is buffered prior to decoding. Encoders can then allow instantaneous video bit-rate to fluctuate in order to achieve a stable picture quality. Consequently, the prevailing transmission/reception bit-rate of data may differ significantly from the prevailing decoding rate of data. For example, a complex scene transition may allocate remarkably more bit-rate than the average bit-rate and the transmission rate of the stream.

Low-delay applications, such as video conferencing, are characterized in that data is decoded (nearly) as soon as it is received. Consequently, the prevailing transmission/reception bit-rate is approximately equal to the decoding bit-rate. Low-delay applications are also characterized by the fact that decoding timestamps may not be used.

Earlier standards have included the peak transmission bit-rate in the definition of levels. We have understood that the reason for specifying the peak transmission bit-rate was to limit the processing constraints of the decoder. This suits low-delay applications, where the transmission/reception bit-rate is approximately equal to the video bit-rate. However, as discussed in the paragraph above, the peak transmission bit-rate may not correspond to the peak video bit-rate especially in streaming applications. Thus, we propose level definitions based on the peak transmission bit-rate for the low-delay applications and based on the peak video bit-rate otherwise. The parameter set in use shall define whether the stream is intended for low-delay use.

There is yet another reason to use the peak video bit-rate as a level constraint: Many of today’s networks, such as the Internet, do not inherit an absolute maximum throughput per connection. Moreover, the packet-switched channels of the third generation mobile network, UMTS, can be reserved so that temporary exceeding of guaranteed connection bit-rate is allowed. Thus, there are transmission systems that allow fluctuating transmission bit-rate. Limiting the maximum transmission rate in a video standard is not practical in such transmission systems.

The peak video bit-rate of a coded data stream is defined as the maximum of the sum of the coded size of data units whose decoding time is within any one-second window.

The instantaneous transmission bit-rate is defined as the amount of data transmitted at once divided by the period of time until the next piece of data is transmitted. The maximum transmission bit-rate is equal to the maximum instantaneous bit-rate.

We considered that the same bit-rates for both the peak video bit-rate and the peak transmission bit-rate are sufficient for level definitions. Thus, no distinction between the two definitions of peak bit-rates is made in the level definitions. In order to let the receiving decoder know, which definition of the peak bit-rate was used, we propose that an appropriate indication is inserted in the parameter set structure. We call this indication as the buffering mode indication. It can have two values: low-delay mode where the peak transmission bit-rate is valid, and normal mode where the peak video bit-rate is valid in the level definitions.

## Proposed Levels

As we do not think that we have enough expertise to design the levels above “mobile” bit-rates, we only propose the first three levels. The peak bit-rates are selected using a bit-rate that is from twice bigger than the bit-rate of the previous level.

| Level number | Peak Bit-Rate (bits/second) |
| --- | --- |
| 1 | 64,000 |
| 1.1 | 128,000 |
| 1.2 | 256,000 |

Open issue: The maximum number of active parameter sets should be limited according to the level in use. We did not have enough insight into the issue to propose appropriate maximum values, though.

# Buffer Size

The size of the VBV buffer for MPEG-4 Visual Simple Profile Level 1, 20480 bytes, was used as a basis of the memory requirements of the pre-decoder buffer in level 1. The pre-decoder buffer size for other primary target picture sizes was selected to be linearly proportional to the proposed maximum bit-rate.

| Level number | HRD/VBV Buffer Size |
| --- | --- |
| 1 | 20,480 |
| 1.1 | 40,960 |
| 1.2 | 81,920 |

A streaming profile is proposed in JVT-D085. Buffer sizes larger than the default ones could be used in streaming

- to enable larger video bit-rate variation and obtain more stable image quality, and

- to enable sending data in advance of its needed time. Critical data (e.g., some I-frames) could be sent early using an ARQ scheme to make sure it arrives before it is needed. Alternatively, some data can be pre-delivered to smooth over fluctuations in the bit rate needed to code the video to a given fidelity. See the related document on NAL unit order (JVT-D093) for further details.

We propose that the proposed streaming profile would have HRD/VBV buffers that are double the size of the buffers for other profiles.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | On Level Definitions |  |
| Contribution number |  | JVT-D091 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: