| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4rd Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D117 Filename: JVT-D117.doc |
| --- | --- |

| Title: | Further Improvement of Motion Search Range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min-Cheol Hong Soongsil University School of Electronic Engineering Seoul, Korea Chul-Woo Kim McubeWorks Korea Design Center, Yatap, Bundang Korea Kyoung Seok In 13F, Seoul Finance Center, 84, Taepyungro 1-ga, Chung-gu, Seoul 100-768, Korea | Tel: Email: Tel: Email: Tel: Email: | +82-2-820-0716 mhong@e.ssu.ac.kr +82-31-601-5875 charlie@mcubeworks.com +82-2-6323-3934 seok@sktelecom.com |
| Source: | Soongsil University, Mcube Works |  |  |

_____________________________

 Introduction

This contribution was originally proposed in Fairfax meeting (JVT-C065). The purpose of the submission is to verify the result of cross check and revision. Motion search range decision was proposed and adopted in Geneva meeting, 2002 [1]. Motion search range decision plays as a role of a pre-processing step of a motion estimation to reduce encoder complexity. Search range is differently determined at each macro-block by using the local statistics of its neighbouring blocks. In this contribution, we proposed the modified algorithm to further reduce the complexity of motion estimation.

Figure 1. Reference block location for search range decision

Let assume that E is a macro-block under motion estimation, and A, B, C are its neighboring 44 blocks, as shown in Fig. 1. Also, assume that motion vectors of A, B, and C are (MV_A_x, MV_A_y), (MV_B_x, MV_B_y), and (MV_C_x, MV_C_y), and input search range is defined as input_search_range. With the condition, a new search range decision is determined as following steps;

- step 1: check the number of neighboring 44 blocks. If the number of available neighboring blocks is more than 1, go to step 2, otherwise go to step 4.

- step 2: from the given motion vectors of neighboring blocks, determine the local maximum absolute horizontal and vertical component value of motion vector, respectively. It is

max_MV_E_i=max(abs(MV_A_i),max(abs(MV_B_i),abs(MV_C_i))   for i=x,y   (1)

In Eq. (1), if a neighboring block is not available, ‘0’ is assigned to a vector component.

- step 3: determine a local statistics of the neighboring blocks. The statistics is written as

local_search_range_i=max(k_i,  2×max_MV_E_i),   (2)

wherer local statistics, kiis defined as

k_i=

- step 4: The proposed algorithm has a constraint such in a way that the maximum search range is subject to input search range, and therefore, the motion search range is constrained as

new_search_range_i=min(input_search_range, local_search_range_i)   (4)

when the number of the available neighboring blocks is more than 1. Otherwise, the ‘new_search_range_i’ is defined as

new_search_range_i=input_search_range   (5)

# Experimental results

The modified search range decision algorithm has been tested with several sequences, resolutions, frame rates, and bit rates. In order to evaluate MV encoding saving, the following criterion is defined;

encoding saving (%)=100-encoding time of JM2 +  search range decisionencoding time of JM2×100   (6)

Also, the test condition is

MV Resolution: 1/4 pel

Number Reference Frames: 1

Slice Mode: not used

Symbol Mode: UVLC

Input Search Range: 32

RD Optimization: used

Loop Filter: used

Plat-form used:

Pentium IV-1.7 GHz  /  512 MB RAM

Detailed performance comparison is in JVT-D117.xls The results show that encoding time is dramatically reduced as much as more than 60 (%) (additional 12(%) saving over JVT-B022) without significant loss in visual quality. Details about simulation procedure and verification software can be found in JVT-D117_software.zip and JVT-D117-Sim.doc.

[Reference]

1. M.-C. Hong and H. H. Oh, “Range Decision for Motion Estimation of VCEG-N33,’’ JVT-B022, Feb. 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Soongsil University/ Mcube Works/ SK Telecom |  |  |
| Mailing address | Soongsil University Sangdo-Dong, Dongjak-Gu Seoul |  |  |
| Country | Korea |  |  |
| Contact person | Min-Cheol Hong |  |  |
| Telephone | +82-2-820-0716 |  |  |
| Fax | +82-2-821-7653 |  |  |
| Email | mhong@e.ssu.ac.kr |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT, H.263, MPGE4 (v2), H.261, MPEG2, and MPGE1 |  |
| Title |  | Further improvement of motion search range |  |
| Contribution number |  | JVT-D117 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | On pending |  |
| Inventor(s)/Assignee(s) |  | Min-Cheol Hong and Chul Woo Kim |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: