| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D128 Filename: JVT-D128.doc |
| --- | --- |

| Title: | Editorial Simplification of nal_unit-to-EBSP Syntax Interface |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal (Editorial) |  |  |
| Author(s) or Contact(s): | Gary Sullivan One Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 (425) 703-5308 garysull@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Summary

This contribution proposes an editorial change to the interface between the NAL unit and EBSP syntax.  In the current syntax tables, there is reported to be some difficulty clearly showing the interface between nal_unit() and rbsp_extraction().  This contribution proposes a way to address this issue.  We attribute the idea for the method of clarification to Mike Nilsson.

# Current Syntax (JVT-D015d5)

### 7.3.1	NAL unit syntax

| nal_unit( NumBytesInEBSP ) { | Category | Descriptor |
| --- | --- | --- |
| error_flag |  | u( 1 ) |
| nal_unit_type |  | u( 5 ) |
| picture_header_flag |  | u( 1 ) |
| non_stored_content_flag |  | u( 1 ) |
| for( i=0; i<NumBytesInEBSP; i++ ) |  |  |
| ebsp[ i ] |  | b( 8 ) |
| } |  |  |

### 7.3.2	Raw and encapsulated byte sequence payloads

#### 7.3.2.1	RBSP from EBSP extraction syntax

| rbsp_extraction( NumBytesInEBSP ) { /* external framing */ | Category | Descriptor |
| --- | --- | --- |
| NumBytesInRBSP = 0 |  |  |
| for( i=0; i<NumBytesInEBSP; i++ ) { |  |  |
| if( next_bits( ) = = 0x0003 ) { |  |  |
| rbsp[ NumBytesInRBSP++ ] |  | b( 8 ) |
| i++ |  |  |
| emulation_prevention_byte /* = = 0x03 */ |  | f( 8 ) |
| } else |  |  |
| rbsp[ NumBytesInRBSP++ ] |  | b( 8 ) |
| } |  |  |
| } |  |  |

# Proposed Change (Revision Marked)

### 7.3.1	NAL unit syntax

| nal_unit( NumBytesInEBSP ) { /* External Framing */ | Category | Descriptor |
| --- | --- | --- |
| error_flag |  | u( 1 ) |
| nal_unit_type |  | u( 5 ) |
| picture_header_flag |  | u( 1 ) |
| non_stored_content_flag |  | u( 1 ) |
| ebsp( NumBytesInEBSP ) |  |  |
| } |  |  |

### 7.3.2	Raw and encapsulated byte sequence payloads

#### 7.3.2.1	Encapsulated byte sequence payload syntax

| ebsp( NumBytesInEBSP ) { | Category | Descriptor |
| --- | --- | --- |
| NumBytesInRBSP = 0 |  |  |
| for( i=0; i<NumBytesInEBSP; i++ ) { |  |  |
| if( next_bits( ) = = 0x0003 ) { |  |  |
| rbsp[ NumBytesInRBSP++ ] |  | b( 8 ) |
| i++ |  |  |
| emulation_prevention_byte /* = = 0x03 */ |  | f( 8 ) |
| } else |  |  |
| rbsp[ NumBytesInRBSP++ ] |  | b( 8 ) |
| } |  |  |
| } |  |  |

# Semantics Clarification

Corresponding minor changes should be made to the semantics section.

Also, (if the picture header structure remains the same as in the current design) it should be clearly noted in the draft (if it isn't already) that if the picture_header_flag is 1, the ebsp() syntax actually consists of the concatenation of two EBSP's: one for the picture header RBSP and one for the slice layer RBSP, and parsing of the picture header is necessary to determine the location of the border between these two.

Additional clarification of the interface between ebsp( ) and the various xxx_rbsp() tables may be needed.

# Closing Remarks

We believe the above small alteration shown above significantly improves the clarity of the design description.

# Acknowledgement

This method of clarifying this part of the syntax is from remarks by Mike Nilsson on the JVT email reflector.