| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, July 22-26, 2002 | Document: JVT-D136 Filename: JVT-C136.doc |
| --- | --- |

| Title: | Flexible Data Partitioning Mode for Streaming Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jong Chul Ye and Yingwei Chen Philips Research 345 Scarborough Road Briarcliff Manor, NY 10510 | Tel: Email: | (914) 945-6133 jong.ye@philips.com |
| Source: | Philips Research USA |  |  |

_____________________________

# Abstract

We propose modifications needed to enable a two-partition mode that allows flexible splitting of AC DCT coefficients for the streaming profile. We first describe an unequal error protection scheme that can effectively prioritize base and enhancement layer packets and how the flexible two-partition mode can be used in such video streaming systems. We then specify the small changes needed to enable this mode. Finally we present simulation results.

# Unequal Error Protection and Data Partitioning

Data partitioning is an effective robustness tool. The advantages of data partitioning as a robustness tool include transparent quality at the full bit rate due to the minimal bit rate overhead, small decoding complexity increase as a result of only the partition-merging operation, and good error resilience performance when some enhancement layer packets are lost.

However there are many doubts over how effective data partitioning is in real streaming systems. It is argued that unequal error protection is either impossible to implement, or incur a large overhead in bit rate, or may not be effective at all because the required up-to-date channel information is not available. While this may be true to some extent in large diverse networks such as the Internet and mobile networks, effective unequal error protection can be achieved in more localized networks such as wireless LANs. Below one such transmission scheme is described.

# Proposed Modifications for Flexible Data Partitioning

The current JVT specification supports one data partition mode that supports three partitions: A, B and C. Partition A contains header and motion vector information, Partition B contains intra block information, and partition C contains the rest. This partitioning mode yields a fixed partition ratio hence quality levels achievable from reconstructing the various partitions, and hence leaves no flexibility in a high-quality streaming system for optimization.

More specifically, there are 20 syntax elements (SE) in the current implementation:

0  SE_HEADER,

1  SE_PTYPE,

2  SE_MBTYPE,

3  SE_REFFRAME,

4  SE_INTRAPREDMODE,

5  SE_MVD,

6  SE_CBP_INTRA,

7  SE_LUM_DC_INTRA,

8  SE_CHR_DC_INTRA,

9  SE_LUM_AC_INTRA,

10  SE_CHR_AC_INTRA,

11  SE_CBP_INTER,

12  SE_LUM_DC_INTER,

13  SE_CHR_DC_INTER,

14  SE_LUM_AC_INTER,

15  SE_CHR_AC_INTER,

16  SE_DELTA_QUANT_INTER,

17  SE_DELTA_QUANT_INTRA,

18  SE_BFRAME,

19  SE_EOS,

and the partition A, B, and C are composed of the following SEs.

partition A = {0,1,2,3,5,18,19}

partition B = {4,6,7,8,9,10,17}

partition C = {11,12,13,14,15,16}

In the proposed flexible data partitioning, we break SE_LUM_AC_INTRA, SE_CHR_AC_INTRA, SE_LUM_AC_INTER, SE_CHR_AC_INTER into two partitions, respectively:

SE_LUM_AC_INTRA = {SE_LUM_AC_INTRA_L,SE_LUM_AC_INTRA_H}  = {9L,9H}

SE_CHR_AC_INTRA = {SE_CHR_AC_INTRA_L,SE_CHR_AC_INTRA_H} = {10L,10H}

SE_LUM_AC_INTER = {SE_LUM_AC_INTER_L,SE_LUM_AC_INTER_H}  = {14L,14H}

SE_CHR_AC_INTER = {SE_CHR_AC_INTER_L,SE_CHR_AC_INTER_H}  = {15L,15H}

where those break points between low frequency  AC and high frequency AC are signaled for each slice by slice by coding a priority break point (PBP). The PBP value is encoded using UVLC at each slice header.

Then, the proposed partition map is given by

partition A (base layer) = {0,1,2,3,4,5,6,7,8,9L,10L,11,12,13,14L,15L,16,17,18,19}

partitoin B (enh layer)  = {9H,10H,14H,15H}

Note that by splitting the AC coefficient flexibly, the ratio between each partition can be adjusted according to the channel throughput variations. Furthermore, by signaling the PBP value for each slice,  the sets of optimal PBP values in rate-distortion sense can be computed.

Special Treatment of Intra Macroblocks

In the current implementation, there exists several intra coding modes for each macroblock: i.e.  Intra Prediction for 4x4 mode for luma, and Intra prediction for 16x16 mode for luma, and Prediction in intra coding of chroma blocks.

In particular, if the entropy coding is UVLC and the image QP value is less than 24, then the transform coefficients of the intra4x4 mode for luma is doubly scanned rather than zig-zag single scanning.  We found that if the double scanning is used, then the loss of some part of  transform coefficients significantly reduces the quality of the video. Hence, for such mode,  we includes every transform coefficients into the partition A and provide more error protection.  While this treatment includes most of the I-frame into the partition A (about 80-90% of the total bit for the corresponding I-frame), the proposed partitioning scheme still allows wide range of partitioning ratio for the P and B frames.

# Simulation Results

We performed simulations to support our proposal. Because data partitioning as a robustness tool has already been established in previous meetings, the purpose of these simulations was to see how much room flexible data partitioning provides for optimization for high-quality video streaming. We performed tests on a subset of the common test conditions.

| Sequences | Mobile (cif), tempete (cif), paris (cif) |
| --- | --- |
| QP | 14, 16, 20 |
| Entropy coding | UVLC |
| Motion compensation precision | ¼ pel |
| Rate distortion optimization | On |
| Search range | 32 |
| Intra period | 45 |
| Frame skip | 0 |
| B frames | 2 |
| Slice mode | 2 (fixed #of bytes in slice) |
| Slice argument | 1500 bytes |
| Use constrained intra prediction | No |
| Restrict serach range | no restriction |

Table 1. Test conditions

The simulation results have been summarized in Excel format and attached as a separate file (JVT-D136.xls). For each simulation, the PBP value is fixed for all frames and the all the enhancement layer packets are assumed lost.

Since the optimization of the PBP values for each slice level can be done in rate-distortion framework, and most of the practical network do not lose every enhancement packets, the simulation results, therefore, should be understood as a worst case scenario.

I

# Conclusions

Only very small changes are needed to enable flexible data partitioning that can give video streaming systems power to optimize video quality under channel constraints. We propose to incorporate these changes into the CD and the streaming profile.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Philips Research USA |  |  |
| Mailing address | 345 Scarborough Road Briarcliff Manor, NY 10510 |  |  |
| Country | USA |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Flexbile Data Partitioning for Improved Robustness Performance |  |
| Contribution number |  | C125 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: