| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D153 Filename: JVT-D153.doc |
| --- | --- |

| Title: | A Computational Complexity Comparison of MPEG4 and JVT Codecs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Massimo Ravasi Marco Mattavelli Christophe Clerc | Tel: Email: Tel: Email: Tel: Email: | + 41 21 693 69 78 Massimo.Ravasi@epfl.ch +41 21 693 6979 Marco.Mattavelli@leg.de.epfl.ch + 41 21 693 69 78 Christophe.Clerc.epfl.ch |
| Source: | Swiss Federal Institute of Technology (EPFL) |  |  |

_____________________________

# Introduction

This document presents the initial results of a computational complexity analysis of the generically optimized MPEG4 codec (simple profile) [1] and of the JVT codec [2]. The analysis is based on the number of operations performed during program execution in real working conditions. The results provided with this analysis are used to compare the performance of the two codecs. According to the recommendations specified in document N4571, the settings for the testbench are copied from the configuration files appended in document N4570.

This analysis was carried out by means of the Software Instrumentation Tool (SIT), an automatic tool for algorithmic complexity analysis of complex systems, currently developed at the Swiss Federal Institute of Technology (EPFL).

# Testbench Definition

The reference softwares used for this complexity analysis are the MPEG-4 Verification Model FPDAM1-020414 [3], generically optimized version for simple profile (OptSimple), and the JVT Joint Model JM 2.1 [4].

The proposed testbench consists of three sequences (Table 1). Mother and Daughter is a low complexity head and shoulders sequence. Foreman has a medium complexity and Calendar and Mobile is a high complexity sequence, with a lot of movement including rotation.

Table 1: Characteristics of the test sequences

| Test Case | Frame rate (fps) |
| --- | --- |
| Mother & Daughter | 30 |
| Foreman | 25 |
| Calendar & Mobile | 15 |

JVT JM has been tested with two different encoding settings:

- “Simple” settings: in this configuration JM performance is similar, in terms of PSNR and bitrate, to that of MPEG-4 simple profile VM. This configuration is meant to compare the algorithmic complexity of the two codecs in similar working conditions.

- “Complex” settings: this configuration activates all JM’s functionalities. The analysis will show how the improvement of JM’s performance, due to “complex” settings, increases the algorithmic complexity of the overall coding process.

The following tables show the different settings and performance of the three considered cases. MPEG-4 and JVT in “simple” configuration were run on the same number of frames, 100, and with the same settings for the three sequences. As shown in Table 2 and Table 3, we can see that the performance of the VM and the JM are slightly different, depending on the coded sequence. Because of simulation time constraints, the JM in “complex” configuration was tested on 9 frames only, as shown in Table 4. To compare the complexity of the JM in “complex” configuration versus the VM and the JM in “simple” configuration, a projection of the results of Table 4, on a virtual 100 frames sequence is shown in Table 6. Because of the fact that, independently on the number of frames, the VM “complex” configuration codes input frames according to a IBPBP[…] sequence, the projected bitrates of Table 6 are computed by adding to the I frame bits the projections of the P and B frame bits (Table 5) according to following equation and considering that a 100 frames sequence would have 49 P frames and 50 B frames:

TotBits100=bits9,I+TotBits9,Pframes9,P⋅frames100,P+TotBits9,Bframes9,Bframes100,B	(1)

Concerning the PSNR, the values for the 100 frames sequence can be assumed to be close to those of the 9 frames sequence, because the PSNR values of each single I, P and B frame differs of few tenths of dB and consequently the average PSNR of the whole sequence does not change sensibly with the number of frames.

Table 6 shows that the “complex” configuration sensibly reduces the bitrate with respect to “simple” configuration and provides a PSNR gain of about 2dB. As it will be shown at paragraph , the price of such performance improvements corresponds to a computational complexity increase of more one order of magnitude for the encoder and of about three times for the decoder.

Table 2: MPEG-4 VM frame rate and performance

| Test Case | Number of Frames | Bitrate (kbps) | PSNR Y (dB) | PSNR U (dB) | PSNR V (dB) |
| --- | --- | --- | --- | --- | --- |
| Mother & Daughter | 100 | 82 | 34.689 | 39.798 | 40.514 |
| Foreman | 100 | 167 | 30.912 | 35.197 | 35.620 |
| Calendar & Mobile | 100 | 195 | 23.182 | 28.405 | 27.256 |

Table 3: JVT “simple” frame rate and performance

| Test Case | Number of Frames | Bitrate (kbps) | PSNR Y (dB) | PSNR U (dB) | PSNR V (dB) |
| --- | --- | --- | --- | --- | --- |
| Mother & Daughter | 100 | 46 | 32.372 | 40.130 | 41.076 |
| Foreman | 100 | 141 | 30.900 | 38.313 | 39.718 |
| Calendar & Mobile | 100 | 243 | 26.537 | 31.928 | 31.261 |

Table 4: JVT “complex” frame rate and performance

| Test Case | Number of Frames | Bitrate (kbps) | PSNR Y (dB) | PSNR U (dB) | PSNR V (dB) |
| --- | --- | --- | --- | --- | --- |
| Mother & Daughter | 9 | 55 | 34.213 | 40.145 | 41.238 |
| Foreman | 9 | 135 | 32.241 | 38.761 | 40.606 |
| Calendar & Mobile | 9 | 244 | 27.537 | 33.102 | 32.649 |

Table 5: JVT “complex” I, P, B frames total bits

| Test Case | Number of Frames | I frames |  | P frames |  | B frames |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # | TotBits | # | TotBits | # | TotBits |
| Mother & Daughter | 9 | 1 | 14152 | 4 | 1576 | 4 | 712 |
| Foreman | 9 | 1 | 22480 | 4 | 22240 | 4 | 3904 |
| Calendar & Mobile | 9 | 1 | 99144 | 4 | 36992 | 4 | 10384 |

Table 6: JVT “complex” performance projected on 100 frames

| Test Case | Number of Frames | Bitrate (kbps) | Bitrate vs “simple” cfg | PSNR Y (dB) | PSNR U (dB) | PSNR V (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Mother & Daughter | 100 | 13 | 23.6% | 34.213 | 40.145 | 41.238 |
| Foreman | 100 | 86 | 63.7% | 32.241 | 38.761 | 40.606 |
| Calendar & Mobile | 100 | 102 | 41.8% | 27.537 | 33.102 | 32.649 |

# SIT Analysis

The Software Instrumentation Tool is an automatic tool for complexity analysis of algorithms specified in C. SIT complexity analysis consists basically of two steps. First, the algorithm under test is instrumented and compiled to produce the instrumented executable. Second, the instrumented executable is run with different input data and configurations to produce the complexity analysis results that can be explored by means of a graphical tool. The results of this analysis are in terms of number of operations and data types, corresponding to C operators and data types. The results are organized on a context tree, where a “context” corresponds to a function or compound statement and the context tree maps how these contexts are traversed during the execution. The analysis aims to be as independent as possible from the compilation and the underlying architecture chosen to run the software: assuming the C code to be an abstract implementation of the algorithm for validation purposes, the real input of the analysis are the algorithm itself and the input data influencing its behaviour.

In the following paragraphs the results of the complexity analysis of the different test cases are presented and compared.

## Encoder

### Preliminary Computational Complexity

Table 7 shows the complexity analysis results collected for MPEG-4 VM encoder with “mother & daughter” sequence. These results are relative to context “function main” and all its subtrees, thus to the whole encoding process. Data types correspond to C data types as they are represented on the workstation on which the VM was tested.

Table 7: Detailed results for MPEG-4 VM encoder, function ‘main’, “mother & daughter” sequence

Considering the results for the other coders and sequences, a preliminary coarse comparison of the three coders is shown in Figure 1 and Table 8. The operations are grouped in the following four sets:

- Comparison operations: they correspond to C operators “==”, “!=”, “>”, “>=”, “<” and “<=”.

- Logical operations:  they correspond to C operators “!”, “&&”, “||”.

- Memory operations:  they correspond to C operators “=”, “[]”, “->” and the pointer dereferencing operator “*”.

- Arithmetic operations: all the other operations.

Concerning the results for the JM with “complex” configuration, scaled values of the results obtained with the 9 frames sequence simulation are considered. While the VM and the JM in “simple” configuration present almost the same computational complexity, the cost for the best performance of the JM in “complex” configuration, specifically a reduction of the bitrate of about 50% and a small PSNR gain, is an increase of more than one order of magnitude of the computational complexity.

The different visual complexity of the test sequences does not alter sensibly the computational complexity of all the coders. Furthermore in all the studied cases it is clear that the most important role is played by “arithmetic” and “memory” operations.

Figure 1: Encoders - Computational complexity

Table 8: Encoders - Computational complexity comparison. The table shows the number of operations and their ratios vs MPEG-4 VM.

|  | MPEG-4 VM | JVT JM, “simple” cfg |  | JVT JM, “complex” cfg |  |
| --- | --- | --- | --- | --- | --- |
|  | # ops | # ops | ratio vs VM | # ops | ratio vs VM |
| Arithmetic ops. | 2.76E+10 | 6.52E+10 | 2.36 | 2.77E+11 | 10.01 |
| Comparison ops. | 7.58E+09 | 6.31E+09 | 0.83 | 6.16E+10 | 8.12 |
| Logical ops. | 1.15E+08 | 2.46E+08 | 2.15 | 8.99E+09 | 78.43 |
| Memory ops. | 2.72E+10 | 5.71E+10 | 2.10 | 5.36E+11 | 19.71 |
| All ops. | 6.26E+10 | 1.29E+11 | 2.06 | 8.84E+11 | 14.13 |

## Decoder

### Preliminary Computational Complexity

Similarly to the comparison described at paragraph  for the encoders, following Figure 2 and Table 9 show the computational complexity analysis for the decoders in the three cases. The computational complexity of the JM in “simple” configuration is about the double of that of the VM, as in the case of the encoder. On the contrary, the computational complexity of the JM in “complex” configuration is about three times higher than that of the VM, while in the case of the encoder the increase was of more than one order magnitude. Furthermore the results clearly show that the computational complexity of decoding process, in all the three cases, does depend on the visual complexity of the sequences.

Figure 2: Decoders - Computational complexity (results for “mother and daughter” sequence with JM in “simple” configuration are not available).

Table 9: Decoders - Computational complexity comparison. The table shows the number of operations and their ratios vs MPEG-4 VM.

|  | MPEG-4 VM | JVT JM, “simple” cfg |  | JVT JM, “complex” cfg |  |
| --- | --- | --- | --- | --- | --- |
|  | # ops | # ops | ratio vs VM | # ops | ratio vs VM |
| Arithmetic ops. | 1.42E+09 | 2.97E+09 | 2.09 | 5.04E+09 | 3.54 |
| Comparison ops. | 4.33E+08 | 8.80E+08 | 2.03 | 1.59E+09 | 3.67 |
| Logical ops. | 2.33E+06 | 3.33E+07 | 14.30 | 3.37E+07 | 14.48 |
| Memory ops. | 1.52E+09 | 2.64E+09 | 1.74 | 3.51E+09 | 2.31 |
| All ops. | 3.38E+09 | 6.53E+09 | 1.93 | 1.02E+10 | 3.01 |

# Conclusions

This document presented a preliminary computational complexity comparison of the optimized MPEG-4 simple profile VM and JVT JM codecs. With respect to the MPEG-4 VM, the analysis showed that activating all the functionalities of JVT encoder, in order to achieve better compression ratios, results in a quite higher computational complexity of the overall encoding process, namely more than one order of magnitude, and in a smaller increase of about three times for the decoding process.

# References

- ISO/IEC JTC1/SC29WG11, “AHG report on editorial convergence of MPEG-4 reference software”, m8041, Jeju, March 2002.

- Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, “Joint Model Number 1, Revision 1 (JM-1r1)”, JVT_A003r1, Pattaya, December 2001

- http://megaera.ee.nctu.edu.tw/mpeg/Optimized_Ref_Software/MoMuSys-FPDAM1-1.0-020414_nctu.zip

- http://bs.hhi.de/~suehring/tml/download/jm21.zip