| Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E011 Filename: input.doc Generated: 2002-10-16 |
| --- | --- |

| Title: | AHG Report: JM Reference Encoding Method Development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chul-Woo Kim Rm. 406 Korea Design Center 344-1 Yatap, Bundang, Seongnam Seoul Korea | Tel: Email: | +82-31-788-7652 charlie@mcubeworks.com |
| Source: | McubeWorks Inc. |  |  |

_____________________________

- Introduction

The mandate of this Ad-Hoc Group is to review the proposals in the test model, encoder optimization category. JM-REMD (JM Reference Encoding Method Development) AHG has been continuing activity improving the result of rate control from Klagenfurt meeting as well as looking for efficient motion estimation algorithm. Currently following 5 contributions are listed in the Geneva document registration list. Three of them belong to motion estimation category, and the other 2 to rate control area.

Table 1. Contributions in the Encoder Optimization Category at Geneva meeting

| Doc. No. | Author(s) | Title |
| --- | --- | --- |
| JVT Geneva Meeting |  |  |
| JVT-E023 | Cheong et. al. | Fast Motion Estimation for the JVT Codec |
| JVT-E024 | Lim et. al. | New Rate Control Algorithm |
| JVT-E031 | Kerofsky | Verif. of Fast Motion Estimation |
| JVT-E045 | Chen et. al. | Fast Integer and Fractional Pel |
| JVT-E069 | Ma et. al. | Improved Rate Control Algorithm |
| JVT-E106 | Chen et. al | Verif. Fast Frac.-Pel Motion Estimation |

- Motion Estimation Proposals

  - Overview

There are two contributions in this category, JVT-E023 and JVT-E045 which both are verified by another contribution. But JVT-E106 which is the verification of JVT-E045 is not available yet. JVT-E023 uses EZPS algorithm and shows the speed up of average 5.91 when 5 reference frames were used. The speed up ration of JVT-E045 was converted by the author, for it used another measure for the performance of speed up. Table 2 is the brief comparison of both proposals. Speed up ratio used in Table 2 is calculated as following:

JM_encoding_time

Speed_up_ratio  =  -------------------------------------

Proposed_encoding_time

Table 2. Comparison of JVT-E023 & JVT-E045

|  |  | JVT-E023 | JVT-E045 |
| --- | --- | --- | --- |
| JM version |  | JM 4.0 | JM 2.0 |
| Reference SW |  | Executables only | Uploaded (late) |
| Algorithm |  | EPZS | Fractional Pel |
| Test Sets |  | VCEG-N81 | VCEG-N81 except Foreman & Tempete |
| Speed Up | 1 Ref. F. | 2.94 | 2.50 (JM 2.0) |
|  | 5 Ref. F. | 5.91 | 5.00 (JM 2.0) |
| Performance Loss (1 Ref. Frames) | BD-BRS | 0.62 [%] | Not Available Yet |
|  | BD-SNR | -0.023 [dB] | Not Available Yet |
| Performance Loss (5 Ref. Frames) | BD-BRS | 0.77 [%] | Not Available Yet |
|  | BD-SNR | -0.028 [dB] | Not Available Yet |
| Cross Verification |  | JVT-E031 | JVT-E106 (not present) |
| IPR Statement |  | 2.2.1 | 2.2.1 |
| Comment |  | Source code absent | Implemented on JM 2.0 |
| Current Status |  |  |  |

JVT-D023 contribution is an informative one. The result for JVT-E045 in the above table was obtained from the implementation based on JM 2.0. The result based on JM 4.2 was presented at the break-out meeting. Detailed description of the results is in the meeting report.

- Rate Control Proposals

  - Overview

There are two contributions in this category too, JVT-E024 and JVT-E069. JVT-E069 is the contribution to improve JVT-D030 which was decided to be incorporated into JM for rate control feature. JVT-E024 uses fluid-flow model and was tested on MsFGS-V4_2.0 (VBR case) and  TMN 3.2 (CBR case). But neither experimental results on JM 4.x nor reference source code is available yet. Table 3 is the brief comparison of both contributions.

Table 3. Brief comparison of JVT-E024 & JVT-E069

|  | JVT-E024 | JVT-E069 |
| --- | --- | --- |
| JM version | MsFGS 4.0 & TMN 3.2 | JM 3.9 (AHM 1.0) |
| Reference SW | Not available | Uploaded (late) |
| Algorithm | Fluid-flow Traffic model | Parameter tuning (JVT-D30) |
| Test Sets | Children, Container, Coastguard & Carphone | VCEG-N81 |
| Cross Verification | None | Partly by AHG Chair |
| IPR Statement | 2.2.1 | 2.2.1 |
| Comment | Not a JVT issue | Performance verification required |
| Current Status |  |  |

- Comments

Two half-day separate sessions over two days would be desirable so that experts can discuss  between the sessions. Verification and test will be required during the meeting days to compare the performance of each contribution precisely. Revised AHG report will be uploaded after finalizing the experts’ opinion.