| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E012 Filename: JVT-E012.doc |
| --- | --- |

| Title: | AHG Report: Level Constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoichi Yagasaki, Lowell Winger Sony Corp., Videolocus Inc. | Tel: Email: | +81-354353669 yagasaki@av.crl.sony.co.jp +1-725-9797 x304 lwinger@videolocus.com |
| Source: | Sony Corp., Videolocus Inc. |  |  |

____________________________

## 1. Introduction

An adhoc group was formed in the Klagenfurt meeting to study various constraints related with level

- Test and verify Limits on MVs.

- Test and verify Reduced Peak Bus Bandwidth

## 2. Report

JVT email reflector was used to conduct the discussion. Group agreed to perform two kinds of test and verifiacrion. And also agreed to use HD sequences. the test conditions as given in Annex A and Annex B. And the test sequences are proposed in Annex C.

JVT-E052 Prop. [Zhong+] Limits on MVs for Practical Compliant HD Decoding

JVT-E065 Info. [Shen] Verif. JVT-D134 MV Count Limits for HD

JVT-E028 Info. [Tourapis+] Verif. Results on Number of MVs & Block Sizes

JVT-E072 P2.0/3.1 [Suzuki+] Restriction of MC Block Size for HDTV Coding

JVT-E037 Info. [Winger] HD Temporal Direct-Mode Verification & Text

JVT-E102 Info. [Suzuki+] Verif. Reduced Peak Bus Bandwidth JVT-C114

JVT-E074 Prop.(N-N) [Suzuki+] Proposed HDTV Material for Verification Tests

ANNEX A

“Limits on MVs” Experiments

## The first category of experiments is to collect MB pair histograms for the number of MVs. The following are some encoding conditions used by the authors and the other co-verification parties

Table 1: Test Conditions

Coding structure is IBPBPBP…

ME search range is +/- 63 in both directions.

Unrestricted block sizes for ME/MC.

Mixed Inter/Intra MBs are disabled, unless explicitly stated otherwise.

CABAC is used for entropy coding.

RD-optimization is on.

ABT is off.

Quantization scales for I, P, and B pictures are indicated below.

Frame or field coding is indicated below.

Table 2: Test Sequences

| Sequence | Frames | Qp (I, P, B) | Frame/field coded |
| --- | --- | --- | --- |
| Raven (720P) | 0-119 | 24, 24, 25 | Frame |
| Crew (720P) | 0-119 | 24, 24, 25 | Frame |
| BigShips (720P) | 0-119 | 24, 24, 25 | Frame |
| Jets (720P) | 0-119 | 24, 24, 25 | Frame |
| City (720P) | 0-119 | 24, 24, 25 | Frame |
| ShuttleStart (720P) | 300-419 | 16, 24, 25 | Frame |
| Kayak (1080I) | 0-59 | 31, 32, 33 | Field |
| Flamingo (1080I) | 0-59 | 31, 32, 33 | Field |
| Fireworks* (1080I) | 0-59 | 39, 40, 41 | Field |
| Mountain (1080I) | 0-59 | 31, 32, 33 | Frame |

## The second type of experiments have also been conducted by completely disallowing any smaller than 8x8 MC blocks, not only for the bi-predicted MBs, but also for any other MB in P or B pictures, while the other coding conditions are kept

Table 3: Test Sequences

| Sequence | Frames | Qp (I, P, B) | Frame/field coded |
| --- | --- | --- | --- |
| Raven (720P) | 0-119 | 24, 24, 25 | Frame |
| BigShips (720P) | 0-119 | 24, 24, 25 | Frame |
| Jets (720P) | 0-119 | 24, 24, 25 | Frame |
| City (720P) | 0-119 | 24, 24, 25 | Frame |
| Fireworks (1080I) | 0-59 | 39, 40, 41 | Field |
| Kayak (1080I) | 0-59 | 31, 32, 33 | Field |
| Flamingo (1080I) | 0-59 | 31, 32, 33 | Field |
| Mountain (1080I) | 0-59 | 31, 32, 33 | Frame |

Experimenters

Broadcom, MicroSoft, Cisco, Sony

ANNEX B

“Reduced Decoder Peak Bus Bandwidth” Experiments

In Fairfax, the restriction of MC block size for direct mode was proposed. In this contribution, the results in JVT-C115 were verified using HDTV materials.

Table 3: Test Conditions

Software: JM41

GOP: IBBPBBP (Only the first frame is I)

QP=QI=QB

RDopt on

SR: +-39

1 reference frame

CABAC, No ABT, Frame based fr/fi adaptive

Test Sequences

Table 4: Test Sequences

|  | Resolution |
| --- | --- |
| Fireworks | 1080@30i |
| Flamingo | 1080@30i |
| Kyayak | 1080@30i |
| Mountain | 1080@30i |
| Asakusa Shop | 1080@30i |
| Running | 1080@30i |

Experimenters

Videolucas, Sony

ANNEX C

# New 1080i Materials

1080i test materials were distributed from Sony for the experiments within AHG on Level Constraints.

The materials are 1080@30i captured by CCD camera and stored with NO compression by 1-inch VTR.

The list of test materials is as follows.

| Name | Number of frames | Description |
| --- | --- | --- |
| Fireworks | 60 | Detail texture, Random motion |
| Kayak | 60 | Detail texture, Fast motion |
| Flamingo | 60 | Detail texture, Random motion |
| Mountain | 60 | Slow panning, Many trees |
| Train* | 60 | Panning, Flat shade area |
| School* | 60 | Zoom up, Texture, Dhade area |
| Asakusa Shop* | 60 | Detail texture, |
| Running* | 60 | Panning, Fast motion |

* Those sequences are available for other ad hoc group, and will be available for this ad hoc group upon request.