| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E015 Filename: JVT-E015.doc |
| --- | --- |

| Title: | AHG Report: Special Position and Motion Compensation Memory Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ajay Luthra Motorola, Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-3470 aluthra@motorola.com |
| Source: | Motorola, Inc. |  |  |

____________________________

1. Introduction

An adhoc group was formed in the Klagenfurt meeting to develop test conditions to

- See whether the Special Position processing helps in improving visual quality of the video in the current design of the JVT codec and

- Test and verify the Motion Compensation related proposals (D029, D080, D106, D110) presented in the Klagenfurt meeting.

2. Report

JVT email reflector was used to conduct the discussion. Many emails were exchanged. Group agreed to the test conditions as given in Annex A and Annex B.

3. Acknowledgment

I would like to thank Dr. Sherman Chen for providing me his help in coordinating this effort while I was away on vacation for three weeks.

ANNEX A

“Special Position” Experiments

## Test Conditions

Table 1: Test Conditions

| Evaluation Method | A: JM 4.0d B: JM 4.0d without Special Position (i.e., always use 6-tap filter for quarter-pel MC) |
| --- | --- |
| Sequence | Sequences as specified in Table 2 |
| GOP Structure | IPPP… IBBP… |
| QP | QPI=QPP= QPB= 24,28,32,36,40 ** (as defined in FCD) |
| Entropy Coding | UVLC/CAVLC |
| RD Optimization | ON |
| Hadamard | YES |
| Motion Compensation | A: 1/4-pel accuracy with Special Position feature B: 1/4-pel accuracy without Special Position feature |
| Reference Frames | Maximum number of reference frames as specified in Levels (e.g. 5 for CIF) |
| MV Search Range | 32x32 |
| Restricted Search Range | 2 |
| Resolution | QCIF, CIF, HHR (optional)* |

** For BD calculation use lower 4 Qp values.

## Test Sequences

Table 2: Test Sequences

|  | Resolution | PSNR[dB] | BRS[%] |
| --- | --- | --- | --- |
| News | QCIF |  |  |
| Container | QCIF |  |  |
| Silent | QCIF |  |  |
| Foreman | QCIF, CIF |  |  |
| Paris | CIF |  |  |
| Mobile | CIF |  |  |
| Tempete | CIF |  |  |
| Football | HHR* (Optional) |  |  |
| Bus | HHR* (Optional) |  |  |
| Rugby | HHR* (Optional) |  |  |
| Canoa | HHR* (Optional) |  |  |
| F1 car | HHR* (Optional) |  |  |

*Experiments for HHR are not required for this testing. However, if HHR sequences are used they should be generated as follows:

- Take right most 352x480/576 pixels

The test for HHR resolution sequences should be performed as follows:

- Create a 704x480/576 clip by placing the 352x480/576 original on the left and 352x480/576 codec-processed clip on the right (or vice versa). This is also known as side-by-side comparison.

Experimenters

Mitsubishi (Yamada-san, Sekiguchi-san), TI ?, Conexant (D. Hoang), Harmonics (Andrew Johnson)

ANNEX B

“MC Memory Analysis” Experiments

Test Conditions

Table 3: Test Conditions

| Reference Software Version | JM 4.0d |
| --- | --- |
| Sequence | Sequences as specified in Table 2 |
| GOP Structure | IPPP… IBBP… |
| QP | QPI=QPP= QPB= 24,28,32,36,40 ** (as defined in FCD) |
| Entropy Coding | UVLC/CAVLC |
| RD Optimization | ON |
| Hadamard | YES |
| Number of Reference Frames | QCIF: 3 CIF: 3 HHR: 2 |
| MV Search Range | 32x32 |
| Restricted Search Range | 2 |
| Resolution | QCIF,CIF,HHR* |

** For BD calculation use lower 4 Qp values.

Test Sequences

Table 4: Test Sequences

|  | Resolution | PSNR[dB] | BRS[%] |
| --- | --- | --- | --- |
| Foreman | QCIF |  |  |
| Container | QCIF |  |  |
| Silent | QCIF |  |  |
| News | QCIF |  |  |
| Paris | CIF |  |  |
| Mobile | CIF |  |  |
| Tempete | CIF |  |  |
| Bus | HHR* |  |  |
| Canoa | HHR* |  |  |

*HHR sequences are generated as follows:

- Take right most 352x480/576 pixels

The test for HHR resolution sequences should be performed as follows:

- Create a 704x480/576 clip by placing the 352x480/576 original on the left and 352x480/576 codec-processed clip on the right (or vice versa). This is also known as side-by-side comparison.

- For subjective assessment use Qp = 24 and 36.

ABT may be used just for Inter in all the HHR cases (-p AdaptiveBlockTransforms = 1) and may be disabled completely for CIF and QCIF.

Experimenters

Ulrich, Yagasaki-san, Suzuki-san, Sekiguchi-san, Jani