| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E026 Filename: JVT-E026.doc |
| --- | --- |

| Title: | Performance comparison of Temporal and Spatial Direct mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Feng Wu, and Shipeng Li 3F Sigma Center, No.49 Zhichun Road, Haidian District, Beijing, 100080, China | Tel: Email: | (86-10) 6261-7711 ext 3108 alexismt@ieee.org |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Introduction

In this document we present a performance analysis of the two Bi-predictive picture Direct modes (Spatial and Temporal Direct Modes). From this analysis we observe that both modes are beneficial for the JVT design since, apart from the different architectural benefits that each one may provide, they tend to perform distinctively different for different types of sequences. A combination of these modes using Rate Distortion Optimization selection is also presented which demonstrates that their combination could yield better performance compared to using either direct mode separately.

Some issues with the current FCD text related to both Spatial and Temporal Direct Modes are also addressed.

# Spatial Direct Mode

During our implementation of the Spatial Direct mode, we have observed that with the current description we allowed reference frames to change even at a 4x4 block size due to the consideration of quasi-stationary motion in the collocated block. This could potentially cause problems into some decoder designs, thus during the implementation a different approach was used to avoid this problem, by only making such a consideration if the preliminary reference frame is the same as the first picture in list 1. It is though necessary to correct the FCD text as well, which is provided here:

#### 10.3.3.1 Spatial technique of obtaining the direct mode motion parameters

The first step in the spatial technique for direct mode prediction is the determination of the reference picture index for each list (list 0 and list 1).

The reference picture indices for the neighbouring blocks A, B, C, and D within the current slice for the 16x16 current luma block E, as described in subclause 8.4.1.1 and shown in Error: Reference source not found, shall be used to determine the reference picture index for each list.  The reference picture index for each list (list 0 and list 1) shall be the minimum reference picture index among the reference picture indices used from the same list for the prediction or bi-prediction of the neighbouring blocks.  If no neighbouring blocks are present within the current slice that use prediction from the same list, either for prediction or bi-prediction, the reference index for that list shall be interpreted as not existing.

If the reference index exists for either list, decision of the associated motion vector values for that list for each 4x4 block of the current macroblock depends on the coded parameters of the co-located 4x4 blocks in the first picture in list 1. If the reference index of either list is zero, the first picture in list 1 is a short-term picture, and if all lines of the co-located 4x4 block were predicted using list 0 prediction with reference picture index 0 and motion vector components in the range of -1 to 1, in sub-pixel units, inclusive, then the motion vector for this 4x4 block is set to (0, 0). Otherwise, the associated motion vector shall be obtained, using the 16x16 block motion vector prediction, as described in subclause 8.4.1 by using the reference picture index for each list.

If both reference picture indices exist, then the block is predicted as a bi-prediction block using the corresponding reference picture index and associated motion vector for each list. Otherwise, if a reference picture index exists for only one of the two lists, the block shall be predicted by single-list prediction using the reference picture index and associated motion vector for the existing candidate. Finally, if neither reference picture index exists, bi-prediction shall be used with reference picture index zero and associated motion vector (0, 0) for both lists.

# Temporal Direct Mode

As it was pointed out on the reflector, the current text for the temporal direct mode appears to have not considered the case of the collocated block has only a list 1 motion vector, but also seems to poorly consider the case that both lists point to the same direction (currently the temporal distance is always considered as a unsigned value).

We believe that the second problem is easy to fix with the consideration of a signed value for the temporal distance (basically considering temporal distance as a vector), which would allow us to properly scale and invert if necessary our motion vectors. As an alternative we may also consider both list 0 and list 1 motion vectors of the collocated block for the calculation of the direct parameters, and copy and scale them accordingly. A related proposal from Toshiba is also presented and thus we will not elaborate on this further.

If the collocated block has only a list 1 motion vector, then we may consider one of the following alternatives.

- Similar to the motion vector prediction process, if only a list 1 motion vector is available then the motion vectors are set to zero, and the list 0 reference picture for the direct mode is the most recent temporally preceding stored picture.

- The list 1 motion vector is used instead of the list 0 (since it is not available). In this case the list 1 reference picture does not need to change, while the list 0 reference picture for the direct mode is the most recent temporally preceding stored picture. As an alternative, single-list prediction maybe used instead of bi-prediction using the list 1 motion vector.

- Motion vectors are calculated using the spatial direct method for this block.

From the above we think that (a) is the simplest in concept, even though (b) also seems to be an interesting approach.

# Spatial and Temporal Direct Mode comparisons

We have used the testing conditions described in [3], and included some additional sequences, to analyze the performance of the Spatial and Temporal Direct modes. Furthermore we have also performed some additional simulations to evaluate the performance of the new RDO for B pictures that was introduced in [5-6]. Furthermore, considering that both temporal and spatial modes might have different properties and give different performance benefits for different types of sequences, we have combined these two schemes using a picture level Rate Distortion Optimization selection. This selection was done by minimizing the equation:

J=SSE+λ∗Rate

where λ=4×0.85×2QP−123.

We need to point out that this selection was not performed within the current software but after the sequences were encoded using the two different direct mode methods, but we believe it is legitimate enough since B pictures were not used as references themselves to code any other picture. The selection could easily be ported within the software, as is also currently done for the picture level Adaptive Field/Frame coding. Detailed results can also be found within the additional excel documents provided. The results are for the entire sequence and not for B pictures exclusively.

Table 1: Performance comparison of Spatial and combined method versus current Temporal (JM4.1d)

| Scheme | Entr. | Seq. | ForQ | Cont | News | Paris | Sil | Mob. | Temp. | ForC | StefQ | StefC | CoastC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spatial | CAVLC1 | PSNR | 0.149 | -0.219 | -0.047 | -0.028 | -0.013 | -0.021 | -0.174 | 0.048 | 0.100 | 0.174 | 0.125 |
|  |  | Bitrate % | 3.63 | -6.21 | -1.19 | -0.68 | -0.34 | -0.54 | -4.92 | 1.31 | 2.14 | 4.00 | 3.45 |
|  | CAVLC5 | PSNR | 0.175 | -0.215 | -0.038 | 0.009 | 0.028 | 0.036 | -0.201 | 0.131 | 0.106 | 0.200 | 0.172 |
|  |  | Bitrate % | 4.24 | -6.20 | -0.91 | 0.21 | 0.77 | 0.72 | -5.07 | 3.63 | 2.29 | 4.54 | 4.79 |
| Comb. | CAVLC1 | PSNR | 0.165 | 0.011 | 0.029 | 0.013 | 0.031 | 0.085 | 0.005 | 0.149 | 0.112 | 0.192 | 0.167 |
|  |  | Bitrate % | 3.97 | 0.26 | 0.70 | 0.34 | 0.82 | 1.99 | 0.11 | 4.36 | 2.36 | 4.39 | 4.67 |
|  | CAVLC5 | PSNR | 0.186 | 0.03 | 0.034 | 0.031 | 0.057 | 0.126 | 0.008 | 0.192 | 0.119 | 0.212 | 0.207 |
|  |  | Bitrate % | 4.50 | 0.09 | 0.081 | 0.79 | 1.59 | 2.77 | 0.19 | 5.37 | 2.57 | 4.79 | 5.73 |
| oldRDO | CAVLC1 | PSNR | -0.086 | -0.530 | -0.169 | -0.483 | -0.225 | -0.889 | -0.450 | -0.312 | -0.113 | X | X |
|  |  | Bitrate % | -2.10 | -14.27 | -4.16 | -11.52 | -6.05 | -20.98 | -12.28 | -9.76 | -2.61 | X | X |
|  | CAVLC5 | PSNR | -0.103 | -0.536 | -0.154 | -0.425 | -0.194 | -0.522 | -0.350 | -0.280 | -0.115 | -0.190 | -0.351 |
|  |  | Bitrate % | -2.67 | -15.26 | -3.71 | -10.94 | -5.39 | -11.82 | -8.28 | -8.28 | -2.80 | -4.70 | -10.00 |
| Spatial | CABAC1 | PSNR | 0.084 | -0.216 | -0.044 | -0.035 | -0.021 | -0.035 | -0.181 | X | X | X | X |
|  |  | Bitrate % | 2.26 | -6.27 | -1.18 | -0.82 | -0.61 | -0.70 | -4.90 | X | X | X | X |
|  | CABAC5 | PSNR | 0.115 | -0.210 | -0.030 | 0.004 | 0.013 | 0.012 | -0.177 | X | X | X | X |
|  |  | Bitrate % | 2.77 | -5.97 | -0.86 | 0.10 | 0.37 | 0.53 | -4.86 | X | X | X | X |
| Comb. | CABAC1 | PSNR | 0.099 | 0.014 | 0.025 | 0.013 | 0.024 | 0.085 | 0.004 | X | X | X | X |
|  |  | Bitrate % | 2.65 | 0.32 | 0.61 | 0.32 | 0.61 | 1.88 | 0.11 | X | X | X | X |
|  | CABAC5 | PSNR | 0.125 | 0.008 | 0.035 | 0.031 | 0.042 | 0.111 | 0.006 | X | X | X | X |
|  |  | Bitrate % | 3.01 | 0.19 | 0.80 | 0.73 | 1.12 | 2.60 | 0.13 | X | X | X | X |

From the above results, an immediate observation is that the new RDO gives considerable benefits in performance (up to 15%) compared to the old RDO. Furthermore, as we have discussed previously, the Spatial and Temporal methods tend to give different benefits for different sequences.

For smoother,small, and regular motion the temporal method seems to be considerably better, while the spatial method gives better results when motion is irregular in terms of time. In addition, we observe from our analytical results that the temporal technique seems to be usually better at larger QP values. This is relatively affected by the RD selection since for larger QPs the spatial method tends to rely more on the motion vectors generated within the same picture which are considerably affected by the larger  value. The temporal method instead, since it can use motion vectors from P pictures (which themselves use a much lower  value), is affected in a lesser degree by the larger  of pictures. If though a picture level RDO selection, or some other method is used within the encoder for selecting which direct method is more appropriate to encode for the current block/macroblock, it is rather obvious that much better performance can be achieved compared to the performance of both methods. We observe for example that for Foreman at CIF resolution the spatial method gives about 1.31% smaller bitstreams on average, while the combination method can reduce the bitstream size by an additional 3% (4.36% compared to the temporal method).  This also seems to suggest that a similar combining could also be used for P pictures as well with the addition of a similar flag as direct_spatial_mv_pred_flag, which could allow a different type of skip mode (always zero or a direct method)

Apart from some of the benefits of the spatial method with regards to memory storage and division operations, this method is also more error resilient than the temporal method. This property is in particular important when Bi-predictive pictures are used instead of P pictures, since an error within one picture will have less impact on the remaining.  Thus it is our opinion that both methods are useful and should remain within the JVT standard.

# References

- Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, “Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC) – Joint Final Committee Draft”, document JVT-E022d6.doc, September 2002.

- JVT Reference Software, unofficial Version 4.1d, http://bs.hhi.de/~suehring/tml/download/Unofficial/

- G. Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, document VCEG-N81, Sep. 2001.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, document VCEG-M33, Mar. 2001

- Alexis Michael Tourapis, Feng Wu, Shipeng Li, and Gary Sullivan, “ Timestamp Independent Motion Vector Prediction for P and B frames with Division Elimination”, document JVT-D040, JVT meeting, Klagenfurt, July 2002

- Alexis Michael Tourapis, Feng Wu, and Shipeng Li, “ Performance Analysis of Lagrangian Parameter Selection in JVT”, document JVT-D041, JVT meeting, Klagenfurt, July 2002

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft |  |  |
| Mailing address | Microsoft Research Asia, 3F Sigma, No49 Zhichun Rd, Haidian Beijing 100080, China |  |  |
| Country | China/USA |  |  |
| Contact person | Alexis Michael Tourapis |  |  |
| Telephone | (86-10) 6261-7711 ext 3108 |  |  |
| Fax | (86-10) 8809 7306 |  |  |
| Email | alexismt@ieee.org |  |  |
| Place and date of submission | Geneva, 4/10/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Performance comparison of Temporal and Spatial Direct mode |  |
| Contribution number |  | E026 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: