| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E027 Filename: JVT-E027.doc |
| --- | --- |

| Title: | Reference Frame Selection for Skip Mode and Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Feng Wu, and Shipeng Li 3F Sigma Center, No.49 Zhichun Road, Haidian District, Beijing, 100080, China | Tel: Email: | (86-10) 6261-7711 ext 3108 alexismt@ieee.org |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Introduction

In this document, we present an alternative motion vector prediction method which, for each candidate motion vector also considers the temporal distance of its corresponding reference picture. Furthermore, similar to the Spatial Direct Method in the FCD, we introduce a reference frame selection process that can be used within the Skip motion vector macroblock mode. Our results show that we can achieve additional benefits by considering the combination of these two methods within the JVT standard.

### 2. Motion Vector Prediction

Motion Vector Prediction within the JVT standard is essentially Median Biased. If multiple reference encoding is used in order to improve performance, the current FCD also considers the case where if only one of the candidate motion vectors used for the motion vector prediction uses the same reference picture as the current block, then only this candidate is used instead of the median value. Apparently though no consideration was made for the case where only 1 candidate or all three use a different reference picture. In such cases, the median calculation is still used, which could potentially lead to a bad motion vector prediction.

To resolve this problem, we propose also considering the temporal distance of each motion vector within the motion vector prediction calculation. More specifically, instead of having to compute the median predictor as:

MVpred=MedianMVA,MVB,MVC

if any of the motion vectors points to a different picture than the current reference and is not intra, then the median calculation can be calculated as :

MVpred=TDref×MedianMVATDA,MVBTDB,MVCTDC

where TDref is the temporal distance of the current reference picture, and TDA, TDB, TDC, are the corresponding temporal distances of the three motion vector candidates. Similar to the Direct mode motion parameter calculation, the above division operation could be replaced with a shift calculation as:

ZA=(TDref×256)TDA

ZB=(TDref×256)TDB

ZC=(TDref×256)TDC

MVpred=Median(ZA×MVA+128)>>8,(ZB×MVB+128)>>8,(ZC×MVC+128)>>8

The changes in both the text and the software are extremely minor for introducing the above concept. In particular, in section 8.4.1.1 we may add the following additional rule:

If any predictor is using a different reference picture than E and is not intra then this predictor is scaled according to the temporal distance of its reference picture and the temporal distance of the reference picture of E, prior to the median calculation. Scaling for a predictor X is performed as follows

ZX=(TDE×256)TDX

MVXscaled=(ZX×MVX+128)>>8

If any predictor is using a different reference picture than E and is pointing to a long term reference picture, then this predictor shall be set to zero prior to the median calculation.

### 2. Skip Mode Reference Picture Selection

The existing skip mode in the FCD standard always uses the last decoded picture as its reference. This though might not always be appropriate and could impair performance especially for some special types of sequences (such as news sequences). The spatial direct mode apart from the motion vector calculation process, also introduced a method for selecting the reference pictures of the direct coded block. We propose using the same concept for selecting the reference picture for the skip mode. Similar to the direct mode approach, the process is done as follows:

“The reference picture indices for the neighbouring blocks A, B, C, and D within the current slice for the 16x16 current luma block E, as described in subclause 8.4.1.1 and shown in Error: Reference source not found, shall be used to determine the reference picture index of the skip mode macroblock.  The reference picture index shall be the minimum reference picture index among the reference picture indices used for the prediction of the neighbouring blocks.  If no neighbouring blocks are present within the current slice (i.e. are all intra), the reference index for that list shall be the last decoded picture. “

Again modifications within the code are relatively minor while this process does not affect.

### 3. Simulation results

The proposed methods were incorporated within the JVT reference software, version 4.0c. We have used the testing conditions recommended in [3], while also adding some additional sequences.

|  | Foreman QCIF | Container | News | Paris | Silence | Mobile | Tempete | Foreman CIF | Stefan QCIF | Stefan CIF | Coastguard | Bus |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSNR | 0.012 | 0.035 | -0.023 | 0.004 | -0.008 | 0.225 | 0.134 | 0.063 | 0.080 | 0.131 | 0.038 | 0.172 |
| bitrate % | 0.22 | 0.70 | -0.39 | 0.06 | -0.17 | 4.89 | 3.15 | 1.36 | 1.38 | 2.52 | 1.13 | 3.58 |

We observe that even though our modifications are very minor, we can achieve up to 0.225dB PSNR gain, or equivalently 4.89% bitrate reduction. We have only experienced loss in two sequences, News and Silence, which could be considered quite minor. As we have also mentioned previously, we expect other sequences which might be switching back and forth to the same scenes to perform even better than the above. If time allows, we will try to provide additional evidence on this claim. In general, we recommend that these methods are accepted within the standard since they could provide additional quality benefits, while only requiring minor modifications. It is also important to point out, that these do not affect in any way the encoder/decoder if only 1 reference frame is used.

We have also examined one additional case within the motion vector prediction process, which though gave worse performance than the current FCD and thus we do not wish to propose it. According to this case, if 2 of the motion vector candidates are intra, and only one candidate is not intra but uses a different reference picture, only the non-intra candidate is used within the motion vector prediction calculation.

References

- Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, “Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC) – Joint Final Committee Draft”, document JVT-E022d6.doc, September 2002.

- JVT Reference Software, unofficial Version 4.0c, http://bs.hhi.de/~suehring/tml/download/Unofficial/

- G. Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, document VCEG-N81, Sep. 2001.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, document VCEG-M33, Mar. 2001

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft |  |  |
| Mailing address | Microsoft Research Asia, 3F Sigma, No49 Zhichun Rd, Haidian Beijing 100080, China |  |  |
| Country | China/USA |  |  |
| Contact person | Alexis Michael Tourapis |  |  |
| Telephone | (86-10) 6261-7711 ext 3108 |  |  |
| Fax | (86-10) 8809 7306 |  |  |
| Email | alexismt@ieee.org |  |  |
| Place and date of submission | Geneva, 4/10/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Reference Frame Selection for Skip Mode and Motion Vector Prediction |  |
| Contribution number |  | E027 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: