| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: input.doc Filename: input.doc.doc |
| --- | --- |

| Title: | Matrix IDCT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | Louis Kerofsky Sharp Labs of America 5750 NW Pacific Rim Blvd. Camas WA 98607 | Tel: Email: | 360-817-7644 lkerofsky@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Introduction

The JVT standard normatively defines the IDCT calculation using a shift/add implementation.  This implementation requires only 16-bits for all intermediate calculation.  A matrix calculation is presented giving a mathematically identical result.  This calculation uses a 32-bit accumulator but all multiplication operations and memory accesses are within 16-bits.  This implementation was included in JM4.0 as a compile option.  The decoded image is identical to that of the shift/add implementation.   The code in the first version of this document may have a difference with the shift/add implementation for QP<11.  A rounding offset can be included to eliminate this difference at fine QP.  A modification to the code that fixes this problem is indicated below.

# Matrix IDCT

In the code segment below, if MATRIX_IDCT is defined, the inverse transform is computed using a 4x4 integer matrix multiply rather than a shift add implementation.

***********************************************************************

* \brief

*    Inverse 4x4 transformation, transforms cof to m7

***********************************************************************

*/

void itrans(struct img_par *img, //!< image parameters

int ioff,            //!< index to 4x4 block

int joff,            //!<

int i0,              //!<

int j0)              //!<

{

#ifdef MATRIX_IDCT

short i,j,i1,j1;

const short A[4][4]={{2,2,2,1},{2,1,-2,-2},{2,-1,-2,2},{2,-2,2,-1}};

short b[BLOCK_SIZE][BLOCK_SIZE];

long x;

// horizontal

for (j=0;j<BLOCK_SIZE;j++)

{

for (i=0;i<BLOCK_SIZE;i++)

{

x=1;

for(i1=0;i1<4;i1++)

x+=A[i][i1]*img->cof[i0][j0][i1][j];

b[i][j]=(short int)(x>>1);

}

}

// vertical

for (i=0;i<BLOCK_SIZE;i++)

{

for (j=0;j<BLOCK_SIZE;j++)

{

x=1;

for(j1=0;j1<4;j1++)

x+=A[j][j1]*b[i][j1];

img->m7[i][j] =mmax(0,mmin(255,(x+(img->mpr[i+ioff][j+joff] <<(DQ_BITS+1))+(DQ_ROUND<<1))>>(DQ_BITS+1)));

}

}

#else

int i,j,i1,j1;

int m5[4];

int m6[4];

// horizontal

for (j=0;j<BLOCK_SIZE;j++)

{

for (i=0;i<BLOCK_SIZE;i++)

{

m5[i]=img->cof[i0][j0][i][j];

}

m6[0]=(m5[0]+m5[2]);

m6[1]=(m5[0]-m5[2]);

m6[2]=(m5[1]>>1)-m5[3];

m6[3]=m5[1]+(m5[3]>>1);

for (i=0;i<2;i++)

{

i1=3-i;

img->m7[i][j]=m6[i]+m6[i1];

img->m7[i1][j]=m6[i]-m6[i1];

}

}

// vertical

for (i=0;i<BLOCK_SIZE;i++)

{

for (j=0;j<BLOCK_SIZE;j++)

m5[j]=img->m7[i][j];

m6[0]=(m5[0]+m5[2]);

m6[1]=(m5[0]-m5[2]);

m6[2]=(m5[1]>>1)-m5[3];

m6[3]=m5[1]+(m5[3]>>1);

for (j=0;j<2;j++)

{

j1=3-j;

img->m7[i][j] =mmax(0,mmin(255,(m6[j]+m6[j1]+(img->mpr[i+ioff][j+joff] <<DQ_BITS)+DQ_ROUND)>>DQ_BITS));

img->m7[i][j1]=mmax(0,mmin(255,(m6[j]-m6[j1]+(img->mpr[i+ioff][j1+joff]<<DQ_BITS)+DQ_ROUND)>>DQ_BITS));

}

}

#endif

}