| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E041 Filename: JVT-E041.doc |
| --- | --- |

| Title: | More Results on Restriction on Vector Spread-out in 8x8 Partitioning Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal for Level Definition |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Incorporated 12500 TI Boulevard, MS 8649 Dallas, Texas 75243, USA Jani Lainema, Marta Karczewicz Nokia Inc 6000 Connection Drive Irving, Texas 75039, USA | Tel: Email: | +1 214 480-3816 zhou@ti.com jani.lainema@nokia.com |
| Source: | Texas Instruments |  |  |

# Introduction

In the 8x8 partitioning mode, an 8x8 block can have one 8x8, two 4x8, 8x4 or four 4x4 vectors. The current H.26L profile@level definition allows the minimum vector block size down to 4x4. Therefore, in the worst case a macroblock can have sixteen 4x4 vectors. Since there is no restriction on the vector spread-out, those vectors can point to anywhere in the reference pictures. For the hardware/software implementation, this is a big challenge especially on the decoder side. Because in the worst case a decoder has to load in total 48 (3 x 16) reference blocks for the luma and chroma motion compensation.

One typical example is on DSPs, in which reference pictures are stored in the off-chips memory. DMAs are normally used for loading the reference blocks from off-chip to on-chip memory. Time spent on the DMA transfer has two parts: the cycles used for the DMA setup and issuing the DMA request, and the cycles consumed by DMA data transfer itself. Running the DMA transfer in background can normally hide the later part of time, but not the time spent on DMA setup. The DMA setup leads to a significant overhead for a video decoder if there are too many blocks to be transferred for a macroblock. For example, in the worst case the DMA setup overhead alone on 100 MHz DSPs amounts to around 20-23% of total cycle budget for 30 frame/s CIF size decoding.

Another problem is the high number of DMA channels required. Having the “unrestricted” 4x8, 8x4, 4x4 vectors in the 8x8 partitioning mode requires in total 48 DMA setups in the worst case, and a maximum of 48 DMA channels to enable a full background DMA reference block transfer. For platforms with limited amount of DMA channels, 48 DMA channels might not be available, a full background DMA transfer is then impossible. Consequently, the overhead spent on DAM setup and data transfer can greatly decrease the decoder performance.

- Proposed Restriction

There are three ways to resolve the problem addressed above:

- Restrict the minimum vector block size to 8x8

- Put a cap on the maximum percentage of sub-8x8 vectors per frame

- Constrain the reference blocks for motion compensation of an 8x8 block into a box area with reasonable size.

Removing all sub-8x8 vectors might not be a good idea because it could affect visual quality and coding efficiency. However, it is worth looking at the possibility of imposing a cap on maximum percentage of sub-8x8 vectors.  Table 1 lists the percentage of sub-8x8 vectors over the H.26L common test sequences measured under the common test conditions [2] with JM4.1. As shown in Table 1, the percentage depends on both the picture content and bit-rate. The percentage is higher at high bit-rates (low QPs) and varies from sequence to sequence.  Therefore, it is hard to choose a reasonable cap.

Table 1. Percentage of sub-8x8 vectors on the H.26L common test sequences

| QP | container | foreman | mobile | news | paris | silent | tempete |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 2.813% | 11.215% | 12.236% | 6.462% | 20.500% | 5.432% | 11.117% |
| 32 | 1.693% | 5.131% | 7.979% | 3.930% | 4.986% | 2.372% | 5.565% |
| 36 | 0.866% | 1.796% | 3.843% | 2.032% | 2.662% | 0.801% | 1.830% |
| 40 | 0.268% | 0.482% | 1.542% | 0.756% | 0.938% | 0.185% | 0.488% |

In order to limit the number of reference block transfers per macroblock in the motion compensation, we propose to impose the following restriction on the vector spread-out of the 8x8 partitioning mode for the Baseline profile, level 2 and below.

In the case of multiple vectors in an 8x8 block, The rectangular block of memory which encompasses all memory required to perform motion compensation on all of the vectors within the block must contain no more than CONSTRAINED_MEMSIZE bytes. NOTE: The dimensions of the rectangular block of memory are not restricted by this limitation, only the area.

The byte limit CONSTRAINED_MEMSIZE will be determined by the experimental results in the next section. The minimum byte limit should be 324 bytes, which is equal to the total luma reference block size for motion compensation of an 8x8 block, if it uses four 4x4 vectors and reference blocks are loaded independently. The byte limit with 324 bytes provides 3 pixel freedom in both horizontal/vertical directions for sub-8x8 vector estimation around the initial center vector (e.g. the 8x8 vector).

As shown in the figure above, this restriction tends to constrain all the reference blocks for an 8x8 block in a box marked by red color. Only one reference block loading is needed for motion compensation of each 8x8 block if the advantage of this restriction is taken on the decoder side. With this restriction, it effectively reduces the maximum number of reference block transfer from 48 to 12 per macroblock in the worst case.

- Experimental Results

The proposed restriction was implemented in the JM 4.1. Comparison was made between the JM4.1 with and without the restriction (denoted by JM4.r and JM4.1, respectively). The simulation results are listed in Table 2 in terms of Bjontegaard Delta bitrate savings in %. The common test conditions from VCEG-N81 were followed. The detailed results can be found in Appendix A.

Two memory constrained motion estimation algorithms were tested. One is to estimate the 8x8 vector first, then all the sub-8x8 vectors (4x8, 8x8, 4x4 vectors) of the 8x8 block are estimated by a vector refinement around the 8x8 vector (denoted by Top2down in Table 2) with a refinement search range derived by the byte limit. The other is to estimate the sub-8x8 vectors independently as what is being done in JM4.1, then the memory size of the rectangle box which contains all the reference blocks of the sub-8x8 vectors is checked. If the memory size exceeds the pre-defined byte limit (e.g. 484 bytes), the estimated sub-8x8 vectors are discarded and replaced by vectors estimated around the 8x8 block vector with the search range derived from the byte limit (denoted by Hybrid in Table 2). Two byte limits, namely 324 and 484 bytes, were used, which resulted in four test sets when combined with two memory constrained motion estimation algorithms. The horizontal/vertical  3 and  5 refinement search ranges were used for simulation with byte limits 324 bytes and 484 bytes, respectively.

Table 2: Performance Comparison (Bjontegaard Delta bitrate savings in %) between JM4.1 and JM4.1 with proposed restriction

|  | container | foreman | mobile | news | paris | silent | tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Top2down_324 | 0.035% | 0.734% | 0.078% | 0.557% | 0.228% | 0.270% | 0.136% | 0.291% |
| Top2down_484 | 0.016% | 0.357% | 0.014% | 0.796% | 0.271% | 0.437% | 0.017% | 0.272% |
| Hybrid_324 | 0.171% | 0.213% | -0.032% | 0.655% | -0.052% | 0.258% | 0.096% | 0.187% |
| Hybrid_484 | 0.000% | -0.351% | 0.066% | 0.362% | -0.122% | 0.195% | 0.173% | 0.046% |

The simulation results revealed that there is hardly any impact on the coding efficiency as expected. Also, no significant difference was measured between byte limit 324 and 484 bytes.

- Conclusions

Constraining the vector spread-out in 8x8 partitioning mode is an effective way to reduce the overhead used for the reference block loading without harming the coding efficiency. It is proposed to impose a byte limit of 324 bytes (400 and 484 bytes are acceptable as well, but seems not necessary) to facilitate the H.26L adoption in mobile applications.

- References

[1]  H.26L Joint Video Team Committee Draft (JVT-CD), 3rd JVT meeting, Fairfax, VA, May 4 – May 11, 2002

[2] Recommended Simulation Conditions for H.26L, ITUT-T VCEG-N81

Appendix A:

Table 3: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM4.1 and JM4.1 with proposed restriction (byte limit 324 bytes, top-to-down estimation)

| Sequence Name | # of frames | QP | JM4.1 PSNR | JM4.1 Bitrate | JM4.r PSNR | JM4.r Bitrate | % change in Bitrate | # change in PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 36.03 | 23.19 | 36.03 | 23.08 | 0.035% | -0.001[dB] |
|  |  | 32 | 33.23 | 12.92 | 33.23 | 12.94 |  |  |
|  |  | 36 | 30.54 | 7.65 | 30.54 | 7.68 |  |  |
|  |  | 40 | 27.89 | 4.75 | 27.89 | 4.71 |  |  |
| foreman | 100 | 28 | 35.95 | 73.29 | 35.95 | 73.56 | 0.734% | -0.038[dB] |
|  |  | 32 | 33.39 | 43.21 | 33.32 | 43.45 |  |  |
|  |  | 36 | 30.87 | 26.70 | 30.88 | 26.65 |  |  |
|  |  | 40 | 28.53 | 16.77 | 28.58 | 16.98 |  |  |
| mobile | 300 | 28 | 34.03 | 1490.12 | 34.03 | 1489.45 | 0.078% | -0.004[dB] |
|  |  | 32 | 30.64 | 731.97 | 30.64 | 731.24 |  |  |
|  |  | 36 | 27.58 | 353.20 | 27.57 | 353.12 |  |  |
|  |  | 40 | 24.66 | 189.39 | 24.65 | 189.90 |  |  |
| news | 100 | 28 | 36.82 | 44.36 | 36.83 | 44.51 | 0.557% | -0.032[dB] |
|  |  | 32 | 33.75 | 27.30 | 33.72 | 27.44 |  |  |
|  |  | 36 | 30.83 | 16.77 | 30.84 | 16.87 |  |  |
|  |  | 40 | 28.14 | 10.41 | 28.14 | 10.39 |  |  |
| paris | 150 | 28 | 35.50 | 311.17 | 35.49 | 311.78 | 0.228% | -0.011[dB] |
|  |  | 32 | 32.42 | 180.38 | 32.41 | 180.64 |  |  |
|  |  | 36 | 29.51 | 100.87 | 29.52 | 100.97 |  |  |
|  |  | 40 | 26.85 | 57.15 | 26.83 | 57.28 |  |  |
| silent | 150 | 28 | 35.82 | 56.40 | 35.83 | 56.72 | 0.270% | -0.013[dB] |
|  |  | 32 | 32.98 | 34.05 | 32.97 | 34.10 |  |  |
|  |  | 36 | 30.36 | 20.02 | 30.37 | 20.11 |  |  |
|  |  | 40 | 28.03 | 11.71 | 28.02 | 11.68 |  |  |
| tempete | 260 | 28 | 35.00 | 1130.14 | 35.00 | 1129.05 | 0.136% | -0.006[dB] |
|  |  | 32 | 31.85 | 546.95 | 31.84 | 547.16 |  |  |
|  |  | 36 | 29.01 | 270.45 | 29.01 | 270.60 |  |  |
|  |  | 40 | 26.33 | 145.92 | 26.32 | 145.75 |  |  |
| Average |  |  |  |  |  |  | 0.291% | -0.015[dB] |

Table 4: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM4.1 and JM4.1 with proposed restriction (byte limit 484 bytes, top-to-down estimation)

| Sequence Name | # of frames | QP | JM4.1 PSNR | JM4.1 Bitrate | JM4.r PSNR | JM4.r Bitrate | % change in Bitrate | # change in PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 36.03 | 23.19 | 36.03 | 23.07 | 0.016% | 0.000[dB] |
|  |  | 32 | 33.23 | 12.92 | 33.23 | 12.92 |  |  |
|  |  | 36 | 30.54 | 7.65 | 30.54 | 7.69 |  |  |
|  |  | 40 | 27.89 | 4.75 | 27.89 | 4.71 |  |  |
| foreman | 100 | 28 | 35.95 | 73.29 | 35.96 | 73.55 | 0.357% | -0.019[dB] |
|  |  | 32 | 33.39 | 43.21 | 33.35 | 43.42 |  |  |
|  |  | 36 | 30.87 | 26.70 | 30.87 | 26.53 |  |  |
|  |  | 40 | 28.53 | 16.77 | 28.55 | 16.90 |  |  |
| mobile | 300 | 28 | 34.03 | 1490.12 | 34.04 | 1488.00 | 0.014% | -0.001[dB] |
|  |  | 32 | 30.64 | 731.97 | 30.64 | 731.35 |  |  |
|  |  | 36 | 27.58 | 353.20 | 27.57 | 353.06 |  |  |
|  |  | 40 | 24.66 | 189.39 | 24.65 | 189.46 |  |  |
| news | 100 | 28 | 36.82 | 44.36 | 36.82 | 44.49 | 0.796% | -0.047[dB] |
|  |  | 32 | 33.75 | 27.30 | 33.71 | 27.42 |  |  |
|  |  | 36 | 30.83 | 16.77 | 30.82 | 16.86 |  |  |
|  |  | 40 | 28.14 | 10.41 | 28.12 | 10.43 |  |  |
| paris | 150 | 28 | 35.50 | 311.17 | 35.49 | 311.63 | 0.271% | -0.014[dB] |
|  |  | 32 | 32.42 | 180.38 | 32.43 | 181.04 |  |  |
|  |  | 36 | 29.51 | 100.87 | 29.50 | 100.96 |  |  |
|  |  | 40 | 26.85 | 57.15 | 26.84 | 57.30 |  |  |
| silent | 150 | 28 | 35.82 | 56.40 | 35.83 | 56.57 | 0.437% | -0.021[dB] |
|  |  | 32 | 32.98 | 34.05 | 32.97 | 34.28 |  |  |
|  |  | 36 | 30.36 | 20.02 | 30.36 | 20.08 |  |  |
|  |  | 40 | 28.03 | 11.71 | 28.05 | 11.72 |  |  |
| tempete | 260 | 28 | 35.00 | 1130.14 | 35.01 | 1130.14 | 0.017% | -0.001[dB] |
|  |  | 32 | 31.85 | 546.95 | 31.85 | 547.36 |  |  |
|  |  | 36 | 29.01 | 270.45 | 29.00 | 269.86 |  |  |
|  |  | 40 | 26.33 | 145.92 | 26.33 | 145.97 |  |  |
| Average |  |  |  |  |  |  | 0.272% | -0.014[dB] |

Table 5: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM4.1 and JM4.1 with proposed restriction (byte limit 324 bytes, hybrid estimation)

| Sequence Name | # of frames | QP | JM4.1 PSNR | JM4.1 Bitrate | JM4.r PSNR | JM4.r Bitrate | % change in Bitrate | # change in PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 36.03 | 23.19 | 36.03 | 23.11 | 0.171% | -0.008[dB] |
|  |  | 32 | 33.23 | 12.92 | 33.23 | 12.93 |  |  |
|  |  | 36 | 30.54 | 7.65 | 30.54 | 7.70 |  |  |
|  |  | 40 | 27.89 | 4.75 | 27.90 | 4.74 |  |  |
| foreman | 100 | 28 | 35.95 | 73.29 | 35.96 | 73.33 | 0.213% | -0.012[dB] |
|  |  | 32 | 33.39 | 43.21 | 33.34 | 43.26 |  |  |
|  |  | 36 | 30.87 | 26.70 | 30.89 | 26.64 |  |  |
|  |  | 40 | 28.53 | 16.77 | 28.58 | 16.94 |  |  |
| mobile | 300 | 28 | 34.03 | 1490.12 | 34.04 | 1488.49 | -0.032% | 0.002[dB] |
|  |  | 32 | 30.64 | 731.97 | 30.64 | 731.94 |  |  |
|  |  | 36 | 27.58 | 353.20 | 27.57 | 352.89 |  |  |
|  |  | 40 | 24.66 | 189.39 | 24.66 | 188.77 |  |  |
| news | 100 | 28 | 36.82 | 44.36 | 36.82 | 44.36 | 0.655% | -0.037[dB] |
|  |  | 32 | 33.75 | 27.30 | 33.71 | 27.41 |  |  |
|  |  | 36 | 30.83 | 16.77 | 30.84 | 16.84 |  |  |
|  |  | 40 | 28.14 | 10.41 | 28.05 | 10.36 |  |  |
| paris | 150 | 28 | 35.50 | 311.17 | 35.50 | 311.42 | -0.052% | 0.002[dB] |
|  |  | 32 | 32.42 | 180.38 | 32.43 | 180.37 |  |  |
|  |  | 36 | 29.51 | 100.87 | 29.52 | 101.13 |  |  |
|  |  | 40 | 26.85 | 57.15 | 26.85 | 57.16 |  |  |
| silent | 150 | 28 | 35.82 | 56.40 | 35.82 | 56.46 | 0.258% | -0.011[dB] |
|  |  | 32 | 32.98 | 34.05 | 32.95 | 34.08 |  |  |
|  |  | 36 | 30.36 | 20.02 | 30.39 | 20.13 |  |  |
|  |  | 40 | 28.03 | 11.71 | 28.05 | 11.76 |  |  |
| tempete | 260 | 28 | 35.00 | 1130.14 | 35.00 | 1129.68 | 0.096% | -0.004[dB] |
|  |  | 32 | 31.85 | 546.95 | 31.84 | 546.31 |  |  |
|  |  | 36 | 29.01 | 270.45 | 29.01 | 270.86 |  |  |
|  |  | 40 | 26.33 | 145.92 | 26.33 | 145.90 |  |  |
| Average |  |  |  |  |  |  | 0.187% | -0.010[dB] |

Table 6: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM4.1 and JM4.1 with proposed restriction (byte limit 484 bytes, hybrid estimation)

| Sequence Name | # of frames | QP | JM4.1 PSNR | JM4.1 Bitrate | JM4.r PSNR | JM4.r Bitrate | % change in Bitrate | # change in PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 36.03 | 23.19 | 36.03 | 23.19 | 0.000% | -0.000[dB] |
|  |  | 32 | 33.23 | 12.92 | 33.23 | 12.92 |  |  |
|  |  | 36 | 30.54 | 7.65 | 30.54 | 7.65 |  |  |
|  |  | 40 | 27.89 | 4.75 | 27.89 | 4.75 |  |  |
| foreman | 100 | 28 | 35.95 | 73.29 | 35.97 | 73.35 | -0.351% | 0.017[dB] |
|  |  | 32 | 33.39 | 43.21 | 33.36 | 43.21 |  |  |
|  |  | 36 | 30.87 | 26.70 | 30.90 | 26.44 |  |  |
|  |  | 40 | 28.53 | 16.77 | 28.54 | 16.80 |  |  |
| mobile | 300 | 28 | 34.03 | 1490.12 | 34.03 | 1489.41 | 0.066% | -0.003[dB] |
|  |  | 32 | 30.64 | 731.97 | 30.64 | 732.12 |  |  |
|  |  | 36 | 27.58 | 353.20 | 27.58 | 353.46 |  |  |
|  |  | 40 | 24.66 | 189.39 | 24.65 | 189.64 |  |  |
| news | 100 | 28 | 36.82 | 44.36 | 36.84 | 44.30 | 0.362% | -0.021[dB] |
|  |  | 32 | 33.75 | 27.30 | 33.71 | 27.40 |  |  |
|  |  | 36 | 30.83 | 16.77 | 30.85 | 16.88 |  |  |
|  |  | 40 | 28.14 | 10.41 | 28.16 | 10.36 |  |  |
| paris | 150 | 28 | 35.50 | 311.17 | 35.49 | 310.94 | -0.122% | 0.006[dB] |
|  |  | 32 | 32.42 | 180.38 | 32.43 | 180.28 |  |  |
|  |  | 36 | 29.51 | 100.87 | 29.51 | 100.69 |  |  |
|  |  | 40 | 26.85 | 57.15 | 26.84 | 57.16 |  |  |
| silent | 150 | 28 | 35.82 | 56.40 | 35.84 | 56.42 | 0.195% | -0.009[dB] |
|  |  | 32 | 32.98 | 34.05 | 32.97 | 34.09 |  |  |
|  |  | 36 | 30.36 | 20.02 | 30.37 | 20.11 |  |  |
|  |  | 40 | 28.03 | 11.71 | 28.01 | 11.68 |  |  |
| tempete | 260 | 28 | 35.00 | 1130.14 | 35.00 | 1129.51 | 0.173% | -0.007[dB] |
|  |  | 32 | 31.85 | 546.95 | 31.84 | 546.90 |  |  |
|  |  | 36 | 29.01 | 270.45 | 28.99 | 269.62 |  |  |
|  |  | 40 | 26.33 | 145.92 | 26.33 | 146.18 |  |  |
| Average |  |  |  |  |  |  | 0.046% | -0.002[dB] |

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Texas Instruments Inc |  |  |
| Mailing address | 12500 TI Blvd., Dallas, TX-75243 |  |  |
| Country | USA |  |  |
| Contact person | Minhua Zhou |  |  |
| Telephone | +1 214 480 3816 |  |  |
| Fax | +1 972 671 6969 |  |  |
| Email | zhou@ti.com |  |  |
| Place and date of submission | Klangenfurt, Austria, July 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | More Results on Restriction on Vector Spread-out in 8x8 Partitioning Mode |  |
| Contribution number |  | JVT-E041 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX 75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 972 894 4188 |  |  |
| Fax | +1 972 894 4589 |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | Klangenfurt, Austria, July 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | More Results on Restriction on Vector Spread-out in 8x8 Partitioning Mode |  |
| Contribution number |  | JVT-E041 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: