| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E049 Filename: JVT-E049.doc |
| --- | --- |

| Title: | Clarification of FMO Mode 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information and Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dae-Sung Cho, and Shi-Hwa Lee Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | wskim@sait.samsung.co.kr |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

In the 4th JVT meeting, FMO mode 3 is adopted in the JVT FCD [1] to support the rectangular slice structure [2].  FMO mode 3 describes that the rectangular slice structure can be useful for the coding efficiency.  With use of FMO mode 3 in this document, we show how the rectangular slice structure can be effectively used to provide the region of interest (ROI) coding scheme which have been proposed since the 1st JVT meeting [3][4][5]. In this scheme, the overlapped MB allocation map is used to reduce the boundary effects.

Another usage of the overlapped MB allocation map is proposed to improve the redundant coding method for the purpose of the error robustness. The ROI concept is applied for the better subjective quality in the error prone environment.

We also propose the omission of the MB allocation map to improve the coding efficiency.

- ROI coding using FMO mode 3 (Information)

In JVT-C096 [5], the sub-picture coding method can remarkably improve the subjective quality. It is due to the ROI concept that human eyes are more sensitive to the ROI than any other regions. In the sub-picture coding, the foreground region is coded in high quality by using a low QP value and the background region is coded in low quality by using a high QP value, and the foreground region is better protected to the network channel error than the background region. The QP value is gradually increased from foreground to background region to reduce the boundary effects.

Since the MB allocation map overlapping is allowed in the FMO mode 3, the overlapped MB allocation map can be formed as shown in Fig. 1 (a). Note that the overlapped area is coded only once in the inner slice group, which is indicated by the inside rectangular shape in Fig. 1 (a). Only the not overlapped area is coded in the outer slice group. The inner slice group should have the smaller slice group id than the outer one to be consistent with the JVT FCD. To reduce the boundary effects, the QP value(s) of the slice(s) of the outer slice group should be higher than that of the inner slice group and lower than that of the leftover slice group.

(a) included case                     (b) not included case

Fig. 1. Overlapped MB allocation map

- Redundant coding using overlapped rectangular slices

According to the current JVT FCD, when the MB allocation map is overlapped, the MBs in the overlapped area always belong to the slice group of smaller id.  Fig. 1 shows two cases of the overlapped MB allocation map: Fig. 1 (a) shows the case that one map is completely included in the other and Fig. 1 (b) shows the case that the maps are partially overlapped, which is used by the term of “not included”.

In case of Fig. 1 (a), suppose that the id of the inner slice group is bigger than that of the outer one, then no MB will be assigned to the inner slice group, which seems to be of no use. The Fig. 2 (b) case would seldom be used either.  Therefore, we propose that in both cases the overlapped area should be coded redundantly. In both cases, the ROI region is redundantly coded when it is overlapped.  Since the ROI region is better protected, this method will result in the improvement of the subjective quality.

- Omission of the MB allocation map

The amount of the bits for the MB allocation map is large especially in case of FMO mode 2. With QCIF image, the amount of the bits for the MB allocation map of the FMO mode 2 would be 85% of the whole amount of the bits for the picture parameter set. It would be 95% in case of the CIF image. In some cases, the MB allocation map would not change frame by frame.

Therefore, we propose to add a one-bit flag to indicate the omission of the MB allocation map. In case of the omission, the MB allocation map of the most recently decoded frame should be used.  The one-bit flag can be located right before the mb_allocation_map_type to omit the MB allocation map regardless of MB allocation map type. This can be limited only to the FMO mode 2 by locating it if mb_allocation_map_type is equal to 2.

- Conclusions

We introduce the method to implement the ROI coding by using FMO mode 3 in this document.  The annoying boundary effects between the foreground and the background region are reduced by using the MB allocation map overlapping. In addition, we propose the redundant coding scheme to clarify the FMO mode 3 especially when the MB allocation maps are overlapped.  Since the ROI is better protected as the result of redundant coding, the subjective quality is improved in error prone environment.  We also propose the method to omit the MB allocation map, which can be burdensome to the picture parameter set.

# References

[1] Joint Final Committee Draft (JFCD) of Joint Video Specification, ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-D157, Jul. 2002.

[2] Miska M. Hannuksela, Ye-Kui Wang, and Michael Horowitz, “Enhancements to FMO”, ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-D095, Klagenfurt, Austria, Jul. 2002.

[3] Miska M. Hannuksela and Ye-Kui Wang, “New image segmentation method”, ITU-T Q6/SG16, Document VCEG-O46, Pattaya, Thailand, Dec. 2001.

[4] Woo-Shik Kim and Dae-Sung Cho, “Selection of quantization parameter for sub-picture coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-B087, Geneva, CH, Jan. 2002.

[5] Woo-Shik Kim, Miska M. Hannuksela, and Ye-Kui Wang, “Sub-picture coding with the reduced boundary effect”, ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-C096, Fairfax, Virginia, May 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Electronics Co. Ltd. |  |  |
| Mailing address |  |  |  |
| Country | Korea |  |  |
| Contact person | Woo-Shik Kim |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email | wskim@sait.samsung.co.kr |  |  |
| Place and date of submission | Geneva, CH, 9-17 October, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Clarification of FMO Mode 3 |  |
| Contribution number |  | JVT-E049 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: