| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E051 Filename: JVT-E051.doc |
| --- | --- |

| Title: | Intra Prediction Enhancements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dae-Sung Cho, and Kyung-Wook Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | wskim@sait.samsung.co.kr |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

In the current JVT FCD [1], the intra prediction of luma component consists of two methods: 4x4 prediction and 16x16 prediction. We propose simple schemes in order to improve both 4x4 and 16x16 intra prediction. For the intra 4x4 prediction, we reduce the reference pels from 17 to 9 for the memory bandwidth reduction. We also enhance the intra 4x4 mode coding method. For the intra 16x16, the order of MBtype is changed for the coding efficiency.

- Proposed methods

  - Memory bandwidth reduction in intra 4x4 prediction

The memory bandwidth reduction for the concurrent processing is an open issue. We use all 9 prediction modes. In the case of mode 3, 7, and 8, which uses the pels of upper right and down left block, we simply replace those pels with the right bottom pel in the upper block and the right bottom pel in the left block as shown in Fig. 1 (b).

  - Intra 4x4 mode coding improvement

We propose two algorithms for Intra 4x4 mode coding in this document. According to the current JVT FCD, the minimum mode number between the upper block and left block is referred as most_probable_mode. First, in our experiments, it is shown that the horizontal prediction mode is the most frequently used mode, especially in the interlace sequence. The mode order should be changed in order to obtain more most_probable_modes.

| Q | A | B | C | D | E | F | G | H |  |  | Q | A | B | C | D | D | D | D | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | a | b | c | d |  |  |  |  |  |  | I | a | b | c | d |  |  |  |  |
| J | e | f | g | h |  |  |  |  |  |  | J | e | f | g | h |  |  |  |  |
| K | I | j | k | l |  |  |  |  |  |  | K | I | j | k | l |  |  |  |  |
| L | m | n | o | p |  |  |  |  |  |  | L | m | n | o | p |  |  |  |  |
| M |  |  |  |  |  |  |  |  |  |  | L |  |  |  |  |  |  |  |  |
| N |  |  |  |  |  |  |  |  |  |  | L |  |  |  |  |  |  |  |  |
| O |  |  |  |  |  |  |  |  |  |  | L |  |  |  |  |  |  |  |  |
| P |  |  |  |  |  |  |  |  |  |  | L |  |  |  |  |  |  |  |  |

(a) current method                                            (b) proposed method

Fig. 1. Reference pels

Therefore, we propose to change mode order such that mode 0 and mode 1 are assigned to the horizontal mode and vertical mode, respectively.

Second, the “outside” case is described. For the cases where either the upper block or the left block is outside, the most_probable_mode is set to the prediction mode 2 in the current JVT FCD.  Since “outside” cases always happen in the slice or the image boundary area, we propose to assign the mode 2 to the “outside” block and use the minimum mode number between the upper block and the left block as the most_probable_mode.

  - Intra 16x16 mode coding improvement.

In the current JVT FCD, the intra 16x16 mode is used to be coded with the combination of nc and AC. The order of this combination is (imode, nc, AC) as shown in Table 1, where imode means intra 16x16 mode. However, we found that the nc value 2 is the least frequently used as shown in Table 2. Therefore, we propose to change the order of the combination as (imode, AC, nc) as shown in Table 3.

Table 1. Macroblock types for I slices, (imode, nc, AC)

| Value of mb_type | Name of mb_type | Value of mb_type | Name of mb_type |
| --- | --- | --- | --- |
| 0 | Intra_4x4 | 13 | Intra_16x16_0_0_1 |
| 1 | Intra_16x16_0_0_0 | 14 | Intra_16x16_1_0_1 |
| 2 | Intra_16x16_1_0_0 | 15 | Intra_16x16_2_0_1 |
| 3 | Intra_16x16_2_0_0 | 16 | Intra_16x16_3_0_1 |
| 4 | Intra_16x16_3_0_0 | 17 | Intra_16x16_0_1_1 |
| 5 | Intra_16x16_0_1_0 | 18 | Intra_16x16_1_1_1 |
| 6 | Intra_16x16_1_1_0 | 19 | Intra_16x16_2_1_1 |
| 7 | Intra_16x16_2_1_0 | 20 | Intra_16x16_3_1_1 |
| 8 | Intra_16x16_3_1_0 | 21 | Intra_16x16_0_2_1 |
| 9 | Intra_16x16_0_2_0 | 22 | Intra_16x16_1_2_1 |
| 10 | Intra_16x16_1_2_0 | 23 | Intra_16x16_2_2_1 |
| 11 | Intra_16x16_2_2_0 | 24 | Intra_16x16_3_2_1 |
| 12 | Intra_16x16_3_2_0 |  |  |

Table 2. nc frequency (%)

| nc | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Progressive | 48.47 | 47.26 | 4.27 |
| Interlace | 59.10 | 34.76 | 6.14 |

Table 3. Proposed table for the macroblock types for I slices, (imode, AC, nc)

| Value of mb_type | Name of mb_type | Value of mb_type | Name of mb_type |
| --- | --- | --- | --- |
| 0 | Intra_4x4 | 13 | Intra_16x16_0_1_1 |
| 1 | Intra_16x16_0_0_0 | 14 | Intra_16x16_1_1_1 |
| 2 | Intra_16x16_1_0_0 | 15 | Intra_16x16_2_1_1 |
| 3 | Intra_16x16_2_0_0 | 16 | Intra_16x16_3_1_1 |
| 4 | Intra_16x16_3_0_0 | 17 | Intra_16x16_0_0_2 |
| 5 | Intra_16x16_0_1_0 | 18 | Intra_16x16_1_0_2 |
| 6 | Intra_16x16_1_1_0 | 19 | Intra_16x16_2_0_2 |
| 7 | Intra_16x16_2_1_0 | 20 | Intra_16x16_3_0_2 |
| 8 | Intra_16x16_3_1_0 | 21 | Intra_16x16_0_1_2 |
| 9 | Intra_16x16_0_0_1 | 22 | Intra_16x16_1_1_2 |
| 10 | Intra_16x16_1_0_1 | 23 | Intra_16x16_2_1_2 |
| 11 | Intra_16x16_2_0_1 | 24 | Intra_16x16_3_1_2 |
| 12 | Intra_16x16_3_0_1 |  |  |

- Experimental results

The common test condition [2] is used except that all frames are coded as intra frame. Both CABAC and UVLC are tested with low QP = 12, 16, 20, 24 and high QP = 28, 32, 36, 40. The Bjontegaard Delta bitrate is calculated for Y component. Additionally, the results for the higher resolution sequences are included where all of 50 frames are coded as intra frame with the rate of 3 frame skip. In the tables below, the negative sign means that the bitrate is reduced, which implies the better performance of the proposed method.

  - Results of the Section 2.1, Memory bandwidth reduction in intra 4x4 prediction

Results of the common condition sequences (Bjontegaard Delta bitrate (%))

|  | Qp | Container | Foreman | Mobile | News | Paris | Silent | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC | Low | 0.11% | 0.38% | 0.12% | 0.08% | 0.10% | 0.70% | 0.14% | 0.23% |
|  | High | 0.20% | 1.46% | 0.53% | 0.54% | 0.40% | 2.02% | 0.68% | 0.83% |
| UVLC | Low | 0.11% | 0.33% | 0.05% | 0.07% | 0.11% | 0.64% | 0.07% | 0.20% |
|  | High | 0.10% | 1.75% | 0.53% | 0.73% | 0.48% | 3.10% | 0.82% | 1.07% |

Results of the interlace sequences (Bjontegaard Delta bitrate (%))

|  | Qp | Canoa | F1 car | Rugby | Mobile | Football | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC | Low | 0.04% | 0.08% | 0.13% | 0.14% | 0.09% | 0.06% | 0.09% |
|  | High | 0.25% | 0.23% | 0.27% | 0.55% | 0.36% | 0.71% | 0.40% |
| UVLC | Low | -0.06% | 0.04% | -0.01% | 0.04% | 0.01% | 0.07% | 0.02% |
|  | High | 0.31% | 0.20% | 0.39% | 0.49% | 0.36% | 0.87% | 0.44% |

  - Results of the Section 2.2, Intra 4x4 mode coding enhancements

Results of the common condition sequences (Bjontegaard Delta bitrate (%))

|  | Qp | Container | Foreman | Mobile | News | Paris | Silent | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC | Low | -0.03% | -0.12% | 0.00% | 0.03% | -0.04% | -0.14% | -0.04% | -0.05% |
|  | High | 0.15% | -0.51% | -0.04% | 0.12% | -0.03% | -0.35% | -0.21% | -0.12% |
| UVLC | Low | 0.01% | -0.18% | 0.00% | 0.05% | -0.06% | -0.13% | -0.03% | -0.05% |
|  | High | 0.10% | -0.58% | -0.03% | 0.08% | -0.17% | -0.17% | -0.16% | -0.13% |

Results of the interlace sequences (Bjontegaard Delta bitrate (%))

|  | Qp | Canoa | F1 car | Rugby | Mobile | Football | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC | Low | -0.30% | -0.12% | -0.17% | 0.01% | -0.15% | -0.08% | -0.14% |
|  | High | -1.03% | -0.30% | -0.45% | -0.14% | -0.48% | -0.42% | -0.47% |
| UVLC | Low | -0.15% | -0.05% | -0.09% | -0.03% | -0.09% | -0.03% | -0.07% |
|  | High | -0.62% | -0.21% | -0.28% | -0.11% | -0.34% | -0.23% | -0.30% |

  - Results of the Section 2.3, Intra 16x16 mode coding enhancements

Since this modification affects the UVLC case, only the result of the UVLC case is shown below.

Results of the common condition sequences (Bjontegaard Delta bitrate (%))

| Qp | Container | Foreman | Mobile | News | Paris | Silent | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.03 | -0.01 |
| High | -0.03 | -0.06 | -0.01 | -0.05 | -0.02 | -0.11 | 0.01 | -0.04 |

Results of the interlace sequences (Bjontegaard Delta bitrate (%))

| Qp | Canoa | F1 car | Rugby | Mobile | Football | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| High | -0.01 | -0.19 | 0.01 | -0.18 | -0.22 | -0.22 | -0.14 |

  - Results of the total intra 4x4 prediction enhancement (Section 2.1 and 2.2 are combined)

Results of the common condition sequences (Bjontegaard Delta bitrate (%))

|  | Qp | Container | Foreman | Mobile | News | Paris | Silent | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC | Low | 0.07 | 0.31 | 0.13 | 0.07 | 0.06 | 0.56 | 0.11 | 0.19 |
|  | High | 0.38 | 0.88 | 0.49 | 0.64 | 0.29 | 1.66 | 0.40 | 0.68 |
| UVLC | Low | 0.12 | 0.16 | 0.05 | 0.06 | 0.04 | 0.52 | 0.04 | 0.14 |
|  | High | 0.22 | 1.16 | 0.49 | 0.81 | 0.35 | 2.85 | 0.65 | 0.93 |

Results of the interlace sequences (Bjontegaard Delta bitrate (%))

|  | Qp | Canoa | F1 car | Rugby | Mobile | Football | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC | Low | -0.28 | -0.02 | -0.07 | 0.12 | -0.09 | 0.04 | -0.05 |
|  | High | -0.85 | 0.03 | -0.14 | 0.38 | -0.16 | 0.30 | -0.07 |
| UVLC | Low | -0.16 | -0.04 | -0.10 | 0.02 | -0.04 | 0.04 | -0.05 |
|  | High | -0.31 | 0.06 | 0.05 | 0.43 | 0.10 | 0.65 | 0.16 |

- Conclusions

We propose to restrict the reference blocks to the upper block and the left block. This scheme will benefit the concurrent processing of the 4x4 blocks. In our simulations, the proposed method provides almost the same performance as the current JVT FCD does in the most cases of progressive and interlace sequences as shown in Table 3.1.  In addition, We also introduce the enhanced intra 4x4 mode coding scheme.  Regarding intra 4x4 prediction, the change of mode order is proposed.  Table 3.2 indicates that the change of mode order leads to the better coding efficiency than the current JVT FCD does.

With the combination of two proposed methods for the concurrent processing and improved coding efficiency, Table 3.4 shows that the combined methods will maintain the same quality as the current JVT FCD and improve the coding efficiency in the most cases of interlace sequences.

Our proposal includes the change of intra 16x16 table in this document. Since it is much less probable of having nc value of 2 as our experiment shows in Table 2, it is more reasonable that it should be located at the last of intra macroblock type table to achieve better coding efficiency as shown in Table 3. In our experiment, the slight change order of nc value can provide the better coding efficiency as shown in Section 3.3.

References

[1] Joint Final Committee Draft (JFCD) of Joint Video Specification, ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/SG16, Document JVT-D157, Jul. 2002.

[2] Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material, ITU-T Q6/SG16, Document VCEG-N81, Santa Barbara, CA, Sep. 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Electronics Co. Ltd. |  |  |
| Mailing address |  |  |  |
| Country | Korea |  |  |
| Contact person | Woo-Shik Kim |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email | wskim@sait.samsung.co.kr |  |  |
| Place and date of submission | Geneva, CH, 9-17 October, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Intra Prediction Enhancements |  |
| Contribution number |  | JVT-E051 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: