Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG 5th Meeting: Geneva, Switzerland, 9-17 October, 2002; Document: JVT-E067 File: JVT-E067.doc Generated: 2002-10-03. Title:; Verification Results on MB-level Adaptive Frame/Field Coding. Status:; Input Document to JVT. Contact:; Peter Borgwardt VideoTele.com 6000 SW Meadows Road, Suite 200, Lake Oswego, OR 97035, U.S.A Tel: 503-594-1303, peter.borgwardt@videotele.com. Purpose:; Information.

_____________________________

1. Introduction

Macroblock-level frame/field adaptive coding was adopted into the JVT Final Committee Draft [1] conditioned upon verifying it as a useful alternative to picture-level frame/field adaptive coding. This contribution provides the results of our simulation experiments supporting the value of MB-level field/frame adaptive coding in the JVT standard.  We compared macroblock-level and picture-level frame/field adaptive coding on two sequences (Mobile & Calendar and World News), coding both without B-frames and with B-frames.  The percent savings in bit rates were calculated with software supplied by G. Bjontegaard [2].

2. Simulation Results Without B-frames

| Codec | JM4.2 (PICT-Level Frame/Field Adaptive) JM4.2 (MB-Level Frame/Field Adaptive) |
| --- | --- |
| Sequences | “Mobile & Calendar” (720x480x30i, 150 frames) “World News” (720x480x30i, 150 frames) |
| GOP | IPPP… (1 I-Frame, the rest are P-Frames) |
| QP | QPI = QPP = 28, 32, 36, 40 |
| Entropy Coding | CVLC |
| RD Optimization | On |
| Hadamard | On |
| Motion Compensation Accuracy | 1/4-pel |
| Reference Frames | 2 |
| MV Search Range | ±16 |
| Restricted Search Range | 2 |

Table 2.1.  Simulation conditions without B-Frames.

Fig 2.1.  Simulation Result for Mobile and Calendar Sequence without B-frames.

## Fig 2.2.  Simulation Result for News Sequence without B-frames.

| Sequence | Mobile | News |
| --- | --- | --- |
| % | 14.45 | 18.11 |

Table 2.2.  Bit Rate savings (%)using MB level adaptive coding over picture level adaptive coding

3.  Simulation Results With B-frames

| Codec | JM4.2 (PICT-Level Frame/Field Adaptive) JM4.2 (MB-Level Frame/Field Adaptive) |
| --- | --- |
| Sequences | “Mobile & Calendar” (720x480x30i, 148 frames) “World News” (720x480x30i, 148 frames) |
| GOP | IBBPBBP… (1 I-Frame, the rest are 2 B-Frames then a P-Frame, 2 B-Frames, then P-Frame, etc.) |
| QP | QPI = QPP = 28, 32, 36, 40; QPB = QPP + 2 |
| Entropy Coding | CVLC |
| RD Optimization | On |
| Hadamard | On |
| Motion Compensation Accuracy | 1/4-pel |
| Reference Frames | 2 |
| MV Search Range | ±16 |
| Restricted Search Range | 2 |

Table 3.1.  Simulation Conditions with B-Frames.

Fig 3.1.  Simulation Result for Mobile and Calendar Sequence without B-frames.

## Fig 3.2.  Simulation Result for News Sequence without B-frames.

| Sequence | Mobile | News |
| --- | --- | --- |
| % | 11.16 | 11.06 |

Table 3.2.  Bit Rate savings (%)using MB level adaptive coding over picture level adaptive coding

4. Conclusion

Our simulation results support that macroblock-level frame/field adaptive coding is a useful technique in the JVT standard, giving a bit savings of 11% to 18% on the two sequences tested.

References

[1] “Joint Final Committee Draft (JFCD) of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)”, Thomas Wiegand, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, JVT-C157, August 2002

[2] “Calculation of average PSNR difference between RD-curves”, G. Bjontegaard, VCEG-M33, March 2001