| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E089 Filename: JVT-E089.doc |
| --- | --- |

| Title: | Simplified Chroma Deblocking (revisited) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cristina Gomila 2 Independence Way Princeton, NJ 08540 USA Anthony Joch 400-1788 West 5th Avenue Vancouver, B.C., Canada V6J 1P2 | Tel: Email: Tel: Email: | + 1 609 734 9505 GomilaC@tce.com +1 604 737 2426 .33 anthony@ubvideo.com |
| Source: | Thomson multimedia Inc. and UB Video Inc. |  |  |

_____________________________

## Overview

Two documents [2][3] were presented at the last meeting as valid options to adjust the deblocking filter to the particularities of chroma.  In both cases, the computation of the clipping threshold C, as specified by the standard, was pointed out as inconsistent an overly complex for chroma filtering.

As a part of JVT-D037, an alternative computation of C was considered. Hence, instead of computing C = C0 + ap + aq as it is done for luma, it was proposed to compute C = C0 + 1 in case of chroma. With this small modification, the complexity of chroma filtering when Bs<4 was reduced by approximately 20%, without affecting the visual quality of the coded sequences.

Within JVT-D049, an alternative method was proposed. In this case, the strong filter that applies when both ap and aq are smaller than , was pointed out as a valid default filter for chroma.  Following this approach, not only the computation of ap and aq was avoided, but also the computation of the different clippings.  On that basis, the complexity reduction was even stronger at the expense of a small drop off on the chroma PSNR values.

As both proposals seemed to have their own merits, no decision was taken as a result of their presentation at the Klagenfurt meeting.  Their further study and comparison was reported to an ad-hoc group, leaving the simplification of the chroma deblocking as an open issue for the next meeting in Geneva.

The goal of this contribution is to propose a common solution that merges and improves the previous approaches.

## Common proposal

With this common proposal, we ratify the importance of simplifying the computation of the chroma deblocking.  Furthermore, some of the solutions that were initially conceived for the default chroma filtering (Bs<4), have now been extended also to the computation of the strong chroma filtering (Bs=4).  Let us detail them.

2.1.  Default Chroma Filtering (Bs<4)

The followed approach when Bs<4 corresponds to the one presented in JVT-D037.  According to that, the computation of the clipping threshold is simplified, but the use of the clipping function retained. With this modification, there is no need to perform the calculations of ap and aq, and therefore no need to load p2 and q2. As a result of this change there is a reduction in complexity for the default chroma filtering that goes up to a 20%. This approximate value is based on counting the total number of required operations in the filtering, as well as analyzing optimized implementations on a media processor and a general-purpose processor. There is no objective or subjective quality loss.

Current standard:

ap=|p2-p0| < 

aq=|q2-q0| < 

C = C0

C = (ap< ) ? C + 1: C

C = (aq< ) ? C + 1: C

Proposal:

C = C0 + 1

Changes on the standard specifications:

This proposal affects only the last sentence of Section 8.7.3.   In this case, a differentiation between luma and chroma on the computation of the threshold C is required.

### 8.7.3. Filtering of edges with Bs < 4

[…]

For luma, C is determined by setting it equal to C0 and then incrementing it by one if ap< and again by one if aq<. For chroma, C is computed directly as C0 + 1.

2.2.  Strong Chroma Filtering (Bs=4)

Accepting the previous change, we estimate that the computation of ap and aq should also be reconsidered for the case of strong chroma filtering.  This approach goes in line with JVT-D049, where the role of the simplest strong filter was emphasized. Hence, we would like to propose the following simplification:

Current standard:

ap=|p2-p0| < 

aq=|q2-q0| < 

If ( ap && |p0-q0|< ((>>2) + 2) )

Filtering according to the Equations 8.72, 8.73

Otherwise

Filtering according to the Equation 8.75

If ( aq && |p0-q0|< ((>>2) + 2) )

Filtering according to the Equations 8.77, 8.78

Otherwise

Filtering according to the Equation 8.80

Proposal:

In all cases:

Filtering according to the Equations 8.75 and 8.80

Changes on the standard specifications:

The inclusion of this proposal tends to simplify the text in Section 8.7.4, leaving it as follows:

### 8.7.4. Filtering of edges with Bs = 4

For luma, when Bs is equal to 4 and the following condition holds:

ap <   && |p0 – q0| < (( >> 2) + 2)		(8-71)

filtering of the left/upper side of the block edge is specified by Equations 8-72, 8-73 and 8-74.

P0 = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4) >> 3	(8-72)

P1 = ( p2 + p1 + p0 + q0 + 2) >> 2		(8-73)

P2 = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4) >> 3	(8-74)

For chroma, and when the condition of 8-71 does not hold for luma, the filter in Equation 8-75 is applied.

P0 = ( 2*p1 + p0 + q1 + 2) >> 2		(8-75)

Similarly, for filtering luma on the right/lower side of the edge, if the following condition holds:

aq <   && |p0 – q0| < (( >> 2) +2)		(8-76)

filtering is defined by Equations 8-77, 8-78 and 8-79.

Q0 = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4) >> 3	(8-77)

Q1 = ( p0 + q0 + q1 + q2 + 2) >> 2		(8-78)

Q2 = ( 2*q3 + 3*q2 + q1 + q0 + p0 + 4) >> 3	(8-79)

For chroma, and when the condition of 8-76 does not hold for luma, the filter in Equation 8-80 is applied:

Q0 = ( 2*q1 + q0 + p1  + 2) >> 2		(8-80)

As a result of this simplification, the length of the strong filter is reduced but a chroma filter without clipping is still available.  Results have shown that there is no significant loss in quality, neither on the PSNR values nor on the subjective tests.

The second modification, to Bs=4 filtering, similarly eliminates the loads of p2 and q2. Additionally, fewer samples are filtered, reducing the number of stores. The conditional logic for selecting a filtering mode is removed and the number of operations required for the filtering equations in the worst-case is reduced significantly, by over 50%. Although it is hardware-dependent, we estimate the overall complexity reduction the Bs=4 chroma filtering to be 50% or more.

## 3. Experimental results

Experimental results are provided on seven test sequences using the JM4.3 reference software. The following parameters have been selected to configure the encoder:

Intra period:			0

QP:				28, 32, 36, 40

Motion vector resolution: 	1/4 pel

Number of reference frames: 	2

Number of B frames:		0

Entropy coding method:	CAVLC

Test sequences:

- Container, News and Foreman (Frame Skip  2)

- Paris and Silent (Frame Skip 1)

- Mobile and Tempete (Frame Skip 0)

For more details of the effects on coding efficiency, we are sending jointly with this contribution a summary of our results in a spreadsheet (JVT_E089.xls).

Since the proposed changes only affect the chroma deblocking, PSNR for the Y component remains unchanged. PSNRU and PSNRV gain 0.08 dB in average.

| Sequence | Bjontegaard Delta PSNR U (dB) | Bjontegaard Delta PSNR V (dB) |
| --- | --- | --- |
| Foreman | 0.003 | 0.002 |
| Container | 0.002 | -0.008 |
| Silent | 0.079 | 0.034 |
| # News | -0.036 | 0.000 |
| Mobile | 0.002 | -0.004 |
| Paris | -0.003 | 0.021 |
| Tempete | 0.000 | 0.028 |
| # Overall average | 0.006 | 0.010 |

Table 1.   Bjontegaard delta PSNR for QP=28, 32, 36, 40 for the proposed changes compared to JM4.3.

In most cases visual quality remains unchanged. In some particular sequences with large uniform zones slight differences can be detected due to the smaller size of the strong filter (see Figure 1).  On other sequences, the capability of preserving chroma details clearly improves the current filter and reduces double-edge artifacts  (see Figure 2).

In order to let other experts to judge by themselves, all the coded video sequences will be available for display at the Geneva meeting.

## 4. Conclusions

We present two simplifications to the deblocking filtering for chroma. The first modification reduces the complexity of the default-mode filtering by approximately 20% by removing some redundant calculations. There is no loss in quality, objective or subjective. The second modification greatly simplifies the strong-mode chroma filtering. Changes in the visual quality due to this modification are strongest for intra-coded frames and, even here, nearly imperceptible.

References

[1] Thomas Wiegand, "Joint Final Committee Draft", document JVT-D157.doc, JVT of ISO/IEC MPEG & ITU-T VCEG, Klagenfurt meeting, Austria, July 22-26, 2002.

[2] Cristina Gomila, "Adjustments for chroma deblocking", document JVT-D049, Klagenfurt meeting, Austria, July 22-26, 2002.

[3] Anthony Joch, "Loop Filter simplification and improvement", document JVT-D037, Klagenfurt meeting, Austria, July 22-26, 2002.

(a1) JM 4.3

(b1) JM 43 with new chroma filtering.

Figure 1. - Detail on Tempete sequence, inter frame 240.

(a1) JM 4.3

(b1) JM 43 with new chroma filtering.

Figure 2. - Detail on News sequence, inter frame 41.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson multimedia, Inc. |  |  |
| Mailing address | 2 Independence Way Princeton, NJ 08540 |  |  |
| Country | USA |  |  |
| Contact person | Cristina Gomila |  |  |
| Telephone | +1 609 987 73 11 |  |  |
| Fax | +1 609 987 72 99 |  |  |
| Email | GomilaC@tce.com |  |  |
| Place and date of submission | Geneva, Switzerland October 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Simplified Chroma Deblocking (revisited) |  |
| Contribution number |  | JVT-E089 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | UB Video Inc. (see below) |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | UBVideo Inc. |  |  |
| Mailing address | 400-1788 West 5th Ave. Vancouver, BC V6J 1P2 |  |  |
| Country | Canada |  |  |
| Contact person | Anthony Joch |  |  |
| Telephone | +1 604 737 2426 |  |  |
| Fax | +1 604 737 1514 |  |  |
| Email | anthony@ubvideo.com |  |  |
| Place and date of submission | Geneva, Switzerland October 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Simplified Chroma Deblocking (revisited) |  |
| Contribution number |  | JVT-E089 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| x | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  | Applies to modification of Bs<4 filtering only. |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: