| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E100 Filename: JVT-E100.doc |
| --- | --- |

| Title: | Decoded Picture Buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chong-Soon Lim Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Teck-Wee Foo Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Sheng-Mei Shen Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Shinya Kadono 1006 Kadomo, Kadomo, Osaka 573-8501 Japan | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | (65) 6550-5487 cslim@psl.com.sg (65) 6550-5474 twfoo@psl.com.sg (65) 6550-5466 shen@psl.com.sg +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Panasonic Singapore Laboratories Pte. Ltd. |  |  |

_____________________________

# Summary

The purpose of this proposal is to present a possible design for the decoded picture buffer.  In the Klagenfurt Meeting, the JVT group has agreed to merge the reference picture buffer and the reordering picture buffer to form a decoded picture buffer.  However the design on how to merge these two buffers was not discussed in detail.  In this proposal, we present two possible ways to merge these buffers and compare their advantages and disadvantages.

# Flexible Partitioning of Buffers

The first method to merge the two buffers, reference picture buffer and reordering picture buffer, is using a flexible partitioning approach.  In this approach, the boundary that separates the reference frame buffer and reordering picture buffer is not fixed but varies depending on the occupancy of the reordering buffer.  In another words, the size of the reference picture buffer will varies depending on the total size of the decoded picture buffer minus the number of pictures stored to be reordered.  Thus at different picture interval, the size of the reference picture buffer will varies with the number of pictures in the reordering picture buffer.

This approach is best in maximizing the physical memory allocated for the decoded picture buffer.  However the design for reference picture buffer will be affected by this approach.  The reason is because the reference picture buffer is made up of short term and long-term buffer.  So whenever the size of the reference picture buffer increases or decreases by one due to the reordering picture buffer occupancy, this picture space has to be from either the short-term or long-term buffer.  This is one issue that needs to be resolved for the reference picture buffer.  In additional, if an MMCO command is sent to adjust the size of the long-term buffer for this reason, the short-term buffer should not be reduced or increased by the same amount as described in the current FCD document.

# Fixed Partitioning of Buffers

The second approach will be the fixed partition approach for the two buffers.  This means that in the Sequence parameter set, besides the num_of_ref_frames, an additional parameter, num_of_reorder_frames, will be used to indicate the size of the reordering buffer.  Therefore, as long as the sum of num_of_ref_frames and num_of_reorder_frames does not exceed the level definitions, the decoder will be able to allocate physical memory space for these two buffers.  So within the same video sequence, the partition of the two buffers will be fixed even though physically the two buffers belong to a single buffer.

This approach may not be as effective as the first approach, but the design is simple and it does not affect the reference picture buffer design.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Category | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | ue(v) |
| level_idc | 0 | ue(v) |
| seq_parameter_set_id | 0 | ue(v) |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| offset_for_non_stored_pic | 0 | se(v) |
| num_stored_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_stored_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_stored_frame[ i ] | 0 | se(v) |
| } |  |  |
| num_of_ref_frames | 0 | ue(v) |
| num_of_reorder_frames | 0 | ue(v) |
| required_frame_num_update_behaviour_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_mbs_minus1 | 0 | ue(v) |
| filter_parameters_flag | 0 | u(1) |
| constrained_intra_pred_flag | 0 | u(1) |
| mb_frame_field_adaptive_flag | 0 | u(1) |
| vui_seq_parameters_flag | 0 | u(1) |
| if( vui_seq_parameters_flag ) |  |  |
| vui_seq_parameters( ) | 0 |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

Semantics

num_of_reorder_frames specifies the total number of pictures in the reordering picture buffer.

# Conclusion

Two possible designs for merging the reference picture buffer and the reordering picture buffer have been presented in this document.  The flexible partitioning approach is effective in memory usage but may be quite complicated considering the changes to the design for the reference picture buffer and the decoder’s implementation.  We would like the JVT group to consider the second approach, the fixed partition approach, as the design approach for the decoded picture buffer.  With the fixed partitioning approach, the change to the current JVT FCD text will be minimum.

# References

[1]	Thomas Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Draft 7”, JVT-E022d7, JVT Meeting, Geneva, Switzerland, October 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | 5th Meeting: Geneva, CH, 9-17 Oct., 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Decoded Picture Buffer |  |
| Contribution number |  | JVT-E100 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: