| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E101 Filename: JVT-E101.doc |
| --- | --- |

| Title: | Adaptive Macroblock Scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chong-Soon Lim Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Zhong Xue Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Teck-Wee Foo Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Sheng-Mei Shen Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Shinya Kadono 1006 Kadomo, Kadomo, Osaka 573-8501 Japan | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | (65) 6550-5487 cslim@psl.com.sg (65) 6550-5474 twfoo@psl.com.sg (65) 6550-5466 shen@psl.com.sg +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Panasonic Singapore Laboratories Pte. Ltd. |  |  |

_____________________________

# Summary

The purpose of this proposal is to present an adaptive macroblock scanning method to improve the coding efficiency of Intra Prediction.  We discovered that when the foreman sequence is inverted from left to right, the coding efficiency of Intra Prediction drops by close to 10%.  This shows that the current 9 modes of Intra Prediction may not be efficient for some sequences like the inverted foreman sequences.  Therefore, we propose an adaptive macroblock scanning method where the macroblock can also be scanned from right to left depending on the properties of the picture to improve the coding efficiency of the current Intra Prediction modes.

# Introduction

It is well known that the transfer sequence of the Macroblocks of a picture is always from left to right, and from top to bottom. Because of this reason, the intra prediction methods, such as the Intra4x4 and Intra16x16 prediction, are often based on the left and above available Macroblocks.

Therefore, the current intra prediction methods describe the vertical, horizontal and diagonal directions well, however, they are not suitable for predicting the anti-diagonal direction, the reason is that according to the transfer sequence of MB or block, the right MB always presents after decoding the current MB or block. This non-symmetric prediction makes the existing intra frame coding methods effective for some kinds of pictures, while they may not effective for other kinds of images. This is because different pictures have different properties, such as the orientation of edges, etc. To improve this shortcoming, for example, in the current JVT (MPEG-4 Part 10) model, the original 6-mode intra prediction was replaced by a more complex 9-mode intra prediction, which tried to predict in the anti-diagonal direction.  However, the improvement of the 9-mode intra prediction is limited because of lacking of information.

We performed two simple experiments on the “foreman” sequence using the JM40d software. The first one encodes the original foreman sequence (see Fig. 1), while the second encodes the reversed foreman sequence, which is scanned in the reverse direction (from right to left) (see Fig. 2).

Figure 1		The first picture of “foreman” sequence (CIF)

Figure 2		The reversed foreman pictures, scanned from right to left.

Figure 3 shows the comparative results of these two experiments. It can be seen that even using the 9-model Intra4x4 prediction, the difference between two scanning directions is quite large (about 10% in bitrate when SNRY is 34).

Figure 3		Comparison between the two sequences using the current 9-model intra prediction.

From Fig. 3, it can be seen that the current JM becomes worse when we inverse the pictures of the foreman sequence, thus the intra prediction in the current JM is not symmetric. This experiment indicates that the current JM may not be efficient to some image sequences

# Adaptive Macroblock Scanning

In this proposal, we proposed an adaptive macroblock scanning (AMBS) method, which allows the transfer sequence of the macroblock adaptively changed, i.e. for a certain group of macroblocks (GMB), they can be either transferred from left to right, or from right to left, and based on this transfer sequence, the scanning within the macroblocks as well as the prediction of the luminance and chrominance pixel values can be based on the left/above pixels or based on the right/above pixels. The encoder will decide which transfer sequence is better (from left to right, or from right to left) by RD. In this way, although the prediction is not symmetric, it can be adaptively chosen according to the characteristics of the encoding pictures.

We propose to apply this AMBS (adaptive macroblock scanning) method in intra coding. Moreover, we will study the used of AMBS in coding all the pictures (inter pictures).

The proposed AMBS consists of the following concepts and operations:

A picture/frame is constructed by a number of slices.

A slice consists of a number of Groups of MBs (GMBs).

The MBs of a Group of MB (GMB) are always in the same slice.

The size of GMB can be fixed-size or flexible-size. In this proposal we used the AMBS of fixed-size GMB.

Other restrictions of the AMBS include (note the term “first MB” and “last MB” in this proposal is based on the normal scanning direction):

The last MB of a slice should be the last MB of a GMB,

The first MB of a slice should be the first MB of a slice.

The first MB and the last MB of a line are always the first MB the last MB of GMBs respectively.

If a GMB is inversely scanned, i.e. scanned from its last MB to the first MB, the last MB is always predicted using DC or vertical prediction depending on the availability of the MBs.

The proposed method can be conveniently implemented, which does not affect the existing intra prediction modules by properly arranging the memory/buffer.

# Syntax Changes

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Category | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 1 | ue(v) |
| seq_parameter_set_id | 1 | ue(v) |
| entropy_coding_mode | 1 | ue(v) |
| motion_resolution | 1 | ue(v) |
| adaptive_block_size_transform_flag | 1 | u(1) |
| adaptive_macroblock_scanning_flag | 1 | u(1) |
| if(adaptive_macroblock_scanning_flag) |  |  |
| group_of_macroblock_size | 1 | ue(v) |
| num_slice_groups_minus1 | 1 | ue(v) |
| if( num_slice_groups_minus1 > 0 ) { |  |  |
| mb_allocation_map_type | 1 | ue(v) |
| if( mb_allocation_map_type = = 0 ) |  |  |
| for( i = 0; i <= num_slice_groups_minus1; i++ ) |  |  |
| run_length | 1 | ue(v) |
| else if( mb_allocation_map_type = = 2 ) |  |  |
| for( i = 0; i < num_mbs_in_pic; i++ ) |  |  |
| slice_group_id [Ed. Note: how many bits ?] | 1 | u(v) |
| else if( mb_allocation_map_type = = 3 ) |  |  |
| for( i = 0; i < num_slice_groups_minus1; i++ ) { |  |  |
| top_left_mb | 1 | u(v) |
| bottom_right_mb | 1 | u(v) |
| } |  |  |
| else if( mb_allocation_map_type = = 4 ¦ ¦ mb_allocation_map_type = = 5 ¦ ¦ mb_allocation_map_type = = 6) { |  |  |
| slice_group_change_direction | 1 | u(1) |
| slice_group_change_rate_minus1 | 1 | ue(v) |
| } |  |  |
| } |  |  |
| num_ref_idx_l0_active_minus1 | 1 | ue(v) |
| num_ref_idx_l1_active_minus1 | 1 | ue(v) |
| weighted_pred_flag | 1 | u(1) |
| weighted_bipred_explicit_flag | 1 | u(1) |
| weighted_bipred_implicit_flag | 1 | u(1) |
| slice_qp_minus26 /* relative to 26 */ | 1 | se(v) |
| slice_qp_s_minus26 /* relative to 26 */ | 1 | se(v) |
| redundant_slice_flag | 1 | u(1) |
| vui_pic_parameters_flag | 1 | u(1) |
| if( vui_pic_parameters_flag ) { |  |  |
| vui_pic_parameters( ) | 1 |  |
| } |  |  |
| rbsp_trailing_bits( ) | 1 |  |
| } |  |  |

Semantics

group_of_macroblock_size specifies the number of macroblocks in a group of macroblock.

### Slice data syntax

| slice_data( ) { | Category | Descriptor |
| --- | --- | --- |
| if( mb_frame_field_adapative_flag && ( pic_structure = = 0 ¦ ¦ pic_structure = = 3 ¦ ¦ pic_structure = = 4 ) ) { |  |  |
| MbPairY = first_mb_in_slice / (pic_width_in_mb_minus1 + 1) |  |  |
| MbPairX = first_mb_in_slice % (pic_width_in_mb_minus1+1) |  |  |
| MbNum = ( MbPairY << 1 ) * (pic_width_in_mb_minus1 + 1) + MbPairX |  |  |
| } else |  |  |
| MbNum = first_mb_in_slice |  |  |
| MoreDataFlag = 1 |  |  |
| do { |  |  |
| if( slice_type( ) ! = Intra && slice_type( ) ! = SIntra ) |  |  |
| if( entropy_coding_mode = = 0 ) { |  |  |
| mb_skip_run | 4 | ue(v) |
| MoreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| mb_skip_flag | 4 | ae(v) |
| MoreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| if( MoreDataFlag ) { |  |  |
| if( mb_frame_field_adaptive_flag && ( pic_structure = = 0 ¦ ¦ pic_structure = = 3 ¦ ¦ pic_structure = = 4 ) && ( slice_type( ) ! = BiPred ) ) { |  |  |
| if( MbNum % 2 = = 0 ) |  |  |
| MbFieldDecodingFlag = 1 |  |  |
| if( MbFieldDecodingFlag ) { |  |  |
| mb_field_decoding_flag | 4 | u(1) ¦ ae(v) |
| MbFieldDecodingFlag = 0 |  |  |
| } else |  |  |
| mb_field_decoding_flag = 0 |  |  |
| } |  |  |
| if (adaptive_macroblock_scanning_flag && MbNum == first_mb_in_GMB) |  |  |
| scanning_direction_flag | 4 | u(1) |
| if( adaptive_block_size_transform_flag = = 0 ) |  |  |
| macroblock_layer( ) | 4 ¦ 5 ¦ 6 |  |
| else |  |  |
| macroblock_layer_abt( ) | 4 ¦ 5 ¦ 6 |  |
| if( entropy_coding_mode = = 0 ) |  |  |
| MoreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| if( MbNum < MaxMbAddress ) { |  |  |
| if( mb_frame_field_adaptive_flag && ( pic_structure = = 0 ¦ ¦ pic_structure = = 3 ¦ ¦ pic_structure = = 4 ) && |  |  |
| MbNum % 2 ! = 0 ) |  |  |
| MoreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 4 | ae(v) |
| MoreDataFlag = !end_of_slice_flag |  |  |
| } else |  |  |
| MoreDataFlag = 0 |  |  |
| } |  |  |
| MbNum++ [Ed. Note: is this correct with FMO ?] |  |  |
| } while( MoreDataFlag ) |  |  |
| } |  |  |

Semantics

## scanning_direction_flag

According to the adaptive MB scanning method, the scanning direction of a GMB can be adjusted, i.e. it can be scanned from left to right (the normal direction) or from right to left (the inverse direction).

The scanning direction of the pixels within the MB corresponds to the scanning direction of the GMB. If the scanning direction is normal direction, the prediction of each MB of the GMB is often based on its left and above previously decoded MBs. If the scanning direction is inverse direction, the prediction of each MB of the GMB is often based on its right and above preciously decoded MBs.

The scanning direction of each GMB is given by a codeword, scanning_direction_flag, before the first MB of each GMB.

# Experimental Results

Experiments are carried out to evaluate the proposed method. In this experiment, for simplicity, the size of GMBs is set as a line. The results are presented in the following figures.  All the frames are coded as Intra frames.  C	AVLC is used as the entropy-coding scheme.

| Sequence Name | DPSNR | %Bitrate |
| --- | --- | --- |
| Inverted Foreman Sequence (CIF) | 0.346 dB | -6.16% |
| Inverted Foreman Sequence (QCIF) | 0.492 dB | -7.26% |
| Inverted Foreman Sequence (QCIF) (First 50 frames only) | 0.620 dB | -9.13% |
| Container (QCIF) | 0.138 dB | -2.00% |
| Mobile (CIF) | 0.022 dB | -0.25% |
| Silent (QCIF) | 0.211 dB | -3.62% |
| News (QCIF) | 0.095 dB | -1.14% |
| News (CIF) | 0.133 dB | -1.76% |

Figure 4		Result of contrainer sequence A.

Figure 5		Results of “news” (c, cif)

Figure 6		Result of “news” (A)

Figure 7		Results of inversed foreman (CIF)

Figure 8		Results of inversed foreman (QCIF)

# Conclusion

This document describes an adaptive MB scanning method for intra coding. In the AMBS, the scanning direction of a group of MBs can be adaptively changed, and the scanning and prediction within the MBs are corresponded to the scanning direction of GMBs. The rate distortion is used to determine the scanning direction of the GMBs.

The advantages of the method include: i) by using the adaptive scanning method, bit rates for intra and inter frames are reduced; ii) all the existing intra prediction algorithms and modules (including spatial and spectrum prediction) are not changed, and to implement the AMBS one may need to use the image buffer and re-arrange the image data according to the scanning direction before input the data into the prediction module.

The method is implemented using JM40d, and experiments show that up to 6.16% bitrate saving can be achieved for the inversed foreman CIF sequence and up to 7.26% for the inversed foreman QCIF sequence.

# References

[1]	Thomas Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Draft 7”, JVT-E022d7, JVT Meeting, Geneva, Switzerland, October 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | 5th Meeting: Geneva, CH, 9-17 Oct., 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive Macroblock Scanning |  |
| Contribution number |  | JVT-E101 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: