| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E112 Filename: JVT-E112.doc |
| --- | --- |

| Title: | Reducing Pixel Dependencies for Intra-Prediction with ABT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Greg Conklin 2601 Elliott Ave Seattle, WA 98101 USA | Tel: Email: | +1 206 674 2337 gregc@real.com |
| Source: | RealNetworks, Inc. |  |  |

_____________________________

1.  Introduction

This document examines Intra prediction modes when using ABT block sizes of 8x4, 4x8 and 8x8.  This document proposed new intra prediction modes for all ABT block sizes that use less surrounding pixels for prediction, and thus, present fewer memory access problems.  In addition, these new modes reduce the worse-case computational complexity by 50%, while leaving the compression performance unharmed.

2.  Intra Prediction Modes

Figures 1, 2 and 3 show 8x8, 8x4 and 4x8 blocks, respectively, and the surrounding pixels used for prediction.  The proposal uses fewer surrounding pixels, particularly along the left side of the block. In only 2 of the 9 modes are pixels t8 - t15 (in the 8x8 and 8x4 case) and t4 - t7 (in the 4x8 case) used.

p00

p10

p20

p30

p40

p50

p60

p70

p01

p11

p21

p31

p41

p51

p61

p71

p02

p12

p22

p32

p42

p52

p62

p72

p03

p13

p23

p33

p43

p53

p63

p73

p04

p14

p24

p34

p44

p54

p64

p74

p05

p15

p25

p35

p45

p55

p65

p75

p06

p16

p26

p36

p46

p56

p66

p76

p07

p17

p27

p37

p47

p57

p67

p77

l0

l1

l2

l3

l4

l5

l6

l7

q

t0

t1

t2

t3

t4

t5

t6

t7

t8

t9

t10

t11

t12

t13

t14

t15

Figure 1:  8x8 prediction block, pij, and the surround predictor pixels (shaded)

p00

p10

p20

p30

p01

p11

p21

p31

p02

p12

p22

p32

p03

p13

p23

p33

p04

p14

p24

p34

p05

p15

p25

p35

p06

p16

p26

p36

p07

p17

p27

p37

l0

l1

l2

l3

q

t0

t1

t2

t3

t4

t5

t6

t7

t8

t9

t10

t11

t12

t13

t14

t15

Figure 2:  8x4 prediction block, pij, and the surround predictor pixels (shaded)

p00

p10

p20

p30

p40

p50

p60

p70

p01

p11

p21

p31

p41

p51

p61

p71

p02

p12

p22

p32

p42

p52

p62

p72

p03

p13

p23

p33

p43

p53

p63

p73

l0

l1

l2

l3

l4

l5

l6

l7

q

t0

t1

t2

t3

t4

t5

t6

t7

Figure 3:  4x8 prediction block, pij, and the surround predictor pixels (shaded)

3.  Compression Results

Tables 1 and 2 show the resulting PSNR and bitrate changes with the proposed mode changes.  For high QPs (28-40), other than the 1.7% loss given by Tempete, favorable compression performance is achieved.  While some sequences provide little difference in compression performance, some sequences show a gain of up to 3%.  For low QPs (12-24), the difference in compression performance is small.

TABLE 1: Bitrate and PSNR improvement using new diagonal modes

| Sequence | Image Size | High QPs (28-40) |  |
| --- | --- | --- | --- |
|  |  | BD-PSNR | BD-Bits |
| Akiyo | QCIF | -0.01 dB | 0.13% |
| Container* | QCIF | 0.09 dB | -1.28% |
| Foreman* | QCIF | -0.04 dB | 0.61% |
| Mother | QCIF | 0.18 dB | -3.09% |
| News* | QCIF | 0.04 dB | -0.55% |
| Silent* | QCIF | 0.01 dB | -0.23% |
| Akiyo | CIF | -0.02 dB | 0.26% |
| Container | CIF | 0.03 dB | -0.51% |
| Foreman | CIF | -0.03 dB | 0.56% |
| Hall | CIF | 0.21 dB | -2.85% |
| Mother | CIF | 0.11 dB | -1.82% |
| Mobile* | CIF | -0.05 dB | 0.57% |
| News | CIF | 0.01 dB | -0.17% |
| Paris* | CIF | 0.08 dB | -1.08% |
| Silent | CIF | 0.01 dB | -0.24% |
| Table | CIF | -0.03 dB | 0.56% |
| Tempete* | CIF | -0.12 dB | 1.70% |
| Canoa | CCIR | 0.03 dB | -0.45% |

*common conditions

TABLE 2: Bitrate and PSNR improvement using new diagonal modes

| Sequence | Image Size | Low QPs (12-24) |  |
| --- | --- | --- | --- |
|  |  | BD-PSNR | BD-Bits |
| Container* | QCIF | 0.04 dB | -0.41% |
| Foreman* | QCIF | -0.06 dB | 0.66% |
| News* | QCIF | 0.03 dB | -0.35% |
| Silent* | QCIF | -0.03 dB | 0.29% |
| Canoa | CCIR | 0.05 dB | -0.70% |

*common conditions

4.  Complexity Analysis

Tables 3, 4 and 5 provide the number of operations needed to form each of the prediction modes for each of the three ABT block sizes.  In some cases the number of operations for the proposed modes drops to zero adds and shifts.  The reduction in the worst case number of operations is about 50% for each of the block sizes, as shown in Table 6.

TABLE 3: Number of operations for JM4.0 and Proposed modes for 8x8 blocks

| Mode | JM4.0c |  |  | Proposed |  |  | Percent Reduction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adds | Shifts | Total | Adds | Shifts | Total |  |
| 0 | 25 | 8 | 33 | 0 | 0 | 0 | 100% |
| 1 | 25 | 8 | 33 | 0 | 0 | 0 | 100% |
| 2 | 16 | 1 | 17 | 16 | 1 | 17 | 0% |
| 3 | 92 | 45 | 137 | 46 | 15 | 61 | 55% |
| 4 | 46 | 15 | 61 | 46 | 15 | 61 | 0% |
| 5 | 54 | 23 | 77 | 44 | 22 | 66 | 14% |
| 6 | 54 | 23 | 77 | 44 | 22 | 66 | 14% |
| 7 | 48 | 23 | 71 | 35 | 22 | 57 | 20% |
| 8 | 48 | 23 | 71 | 23 | 14 | 37 | 48% |

TABLE 4: Number of operations for JM4.0 and Proposed modes for 8x4 blocks

| Mode | JM4.0c |  |  | Proposed |  |  | Percent Reduction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adds | Shifts | Total | Adds | Shifts | Total |  |
| 0 | 25 | 8 | 33 | 0 | 0 | 0 | 100% |
| 1 | 13 | 4 | 17 | 0 | 0 | 0 | 100% |
| 2 | 13 | 3 | 16 | 12 | 1 | 13 | 19% |
| 3 | 68 | 33 | 101 | 34 | 11 | 45 | 55% |
| 4 | 34 | 11 | 45 | 34 | 11 | 45 | 0% |
| 5 | 42 | 19 | 61 | 32 | 18 | 50 | 18% |
| 6 | 41 | 14 | 55 | 31 | 14 | 45 | 18% |
| 7 | 32 | 15 | 47 | 29 | 18 | 47 | 0% |
| 8 | 32 | 15 | 47 | 23 | 14 | 37 | 21% |

TABLE 5: Number of operations for JM4.0 and Proposed modes for 4x8 blocks

| Mode | JM4.0c |  |  | Proposed |  |  | Percent Reduction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adds | Shifts | Total | Adds | Shifts | Total |  |
| 0 | 13 | 4 | 17 | 0 | 0 | 0 | 100% |
| 1 | 25 | 8 | 33 | 0 | 0 | 0 | 100% |
| 2 | 13 | 3 | 16 | 12 | 1 | 13 | 19% |
| 3 | 68 | 33 | 101 | 34 | 11 | 45 | 55% |
| 4 | 34 | 11 | 45 | 34 | 11 | 45 | 0% |
| 5 | 38 | 15 | 53 | 32 | 14 | 46 | 13% |
| 6 | 45 | 20 | 65 | 32 | 18 | 50 | 23% |
| 7 | 40 | 19 | 59 | 23 | 14 | 37 | 37% |
| 8 | 40 | 19 | 59 | 23 | 14 | 37 | 37% |

TABLE 6: Worst-case reduction is operations

| Block Size | Worst-Case Reduction |
| --- | --- |
| 8x8 | 52% |
| 8x4 | 50% |
| 4x8 | 50% |

4.  Intra Prediction Mode Definitions

The following tables describe the proposed prediction modes using the notation shown in figures 1, 2, and 3.  Note from figures 1, 2 and 3 that the pixels used for prediction for a MxN block do not extend beyond 2xM horizontally above and N vertically down from the block being predicted.  Therefore, in the 8x4 block case, and when reference to l4-l7 is given, the implementation shall replace these values with l3.  Likewise, in the 4x8 block case, and when reference to t8-t15 is given, the implementation shall replace these values with t7.

Note that unlike the current Intra Prediction method for these block sizes, no predictor pixel filtering is done prior to forming each of the prediction modes.  This itself reduces the required computational complexity significantly.

Mode 0 – Vertical Prediction

For a prediction block size of MxN, where all t0 – tN are available for prediction, then

Pij = tj;

Where i = 0 - (N-1), j = 0 – (M-1).

Mode 2 – Horizontal Prediction

For a prediction block size of MxN, where all l0 – lM are available for prediction, then

Pij = li;

Where i = 0 - (N-1), j = 0 – (M-1).

Mode 2 – DC Prediction

For a prediction block size of MxN,

If t0 – tN and l0 – lM are available for prediction, then

pij = (t0 + … + tN + l0 + … + lM + N/2 + M/2) / (M + N);

Else if only t0 – tN are available, then

pij = (t0 + … + tN + N/2) / N;

Else if only l0 – lM are available, then

pij = (l0 + … + lM + M/2) / M;

Else

pij = 128;

Where i = 0 - (N-1), j = 0 – (M-1).

Mode 3 – Diagonal Left/Down

For a prediction block size of MxN, where all t0 – tN and l0 – lM are available for prediction, then

| 4x8 block samples | 8x4 block samples | 8x8 block samples | predicted by |
| --- | --- | --- | --- |
| p00 | p00 | p00 | ( t2 + t1<<1 + t0 + 2 ) >> 2 |
| p01, p10 | p01, p10 | p01, p10 | ( t3 + t2<<1 + t1 + 2 ) >> 2 |
| p02, p11, p20 | p02, p11, p20 | p02, p11, p20 | ( t4 + t3<<1 + t2 + 2 ) >> 2 |
| p03, p12, p21, p30 | p03, p12, p21, p30 | p03, p12, p21, p30 | ( t5 + t4<<1 + t3 + 2 ) >> 2 |
| p13, p22, p31, p40 | p04, p13, p22, p31 | p04, p13, p22, p31, p40 | ( t6 + t5<<1 + t4 + 2 ) >> 2 |
| p23, p32, p41, p50 | p05, p14, p23, p32 | p05, p14, p23, p32, p41, p50 | ( t7 + t6<<1 + t5 + 2 ) >> 2 |
| p33, p42, p51, p60 | p06, p15, p24, p33 | p06, p15, p24, p33, p42, p51, p60 | ( t8 + t7<<1 + t6 + 2 ) >> 2 |
| p43, p52, p61, p70 | p07, p16, p25, p34 | p07, p16, p25, p34, p43, p52, p61, p70 | ( t9 + t8<<1 + t7 + 2 ) >> 2 |
| p53, p62, p71 | p17, p26, p35 | p17, p26, p35, , p44, p53, p62, p71 | ( t10 + t9<<1 + t8 + 2 ) >> 2 |
| p63, p72 | p27, p36 | p27, p36, p45, p54, p63, p72 | ( t11 + t10<<1 + t9 + 2 ) >> 2 |
| p73 | p37 | p37, p46, p55, p64, p73 | ( t12 + t11<<1 + t10 + 2 ) >> 2 |
| - | - | p47, p56, p65, p74 | ( t13 + t12<<1 + t11 + 2 ) >> 2 |
| - | - | p57, p66, p75 | ( t14 + t13<<1 + t12 + 2 ) >> 2 |
| - | - | p67, p76 | ( t15 + t14<<1 + t13 + 2 ) >> 2 |
| - | - | p77 | ( t15 + t15<<1 + t14 + 2 ) >> 2 |

Mode 4 – Diagonal Right/Down

For a prediction block size of MxN, where all t0 – tN and l0 – lM are available for prediction, then

| 4x8 block samples | 8x4 block samples | 8x8 block samples | predicted by |
| --- | --- | --- | --- |
| - | p07 | p07 | ( t5 + t6<<1 + t7 + 2 ) >> 2 |
| - | p06, p17 | p06, p17 | ( t4 + t5<<1 + t6 + 2 ) >> 2 |
| - | p05, p16, p27 | p05, p16, p27 | ( t3 + t4<<1 + t5 + 2 ) >> 2 |
| - | p04, p15, p26, p37 | p04, p15, p26, p37 | ( t2 + t3<<1 + t4 + 2 ) >> 2 |
| p03 | p03, p14, p25, p36 | p03, p14, p25, p36, p47 | ( t1 + t2<<1 + t3 + 2 ) >> 2 |
| p02, p13 | p02, p13, p24, p35 | p02, p13, p24, p35, p46, p57 | ( t0 + t1<<1 + t2 + 2 ) >> 2 |
| p01, p12, p23 | p01, p12, p23, p34 | p01, p12, p23, p34, , p45, p56, p67 | ( q + t0<<1 + t1 + 2 ) >> 2 |
| p00, p11, p22, p33 | p00, p11, p22, p33 | p00, p11, p22, p33, p44, p55, p66, p77 | ( l0 + q<<1 + t0 + 2 ) >> 2 |
| p10, p21, p32, p43 | p10, p21, p32 | p10, p21, p32, , p43, p54, p65, p76 | ( l1 + l0<<1 + q + 2 ) >> 2 |
| p20, p31, p42, p53 | p20, p31 | p20, p31, p42, p53, p64, p75 | ( l2 + l1<<1 + l0 + 2 ) >> 2 |
| p30, p41, p52, p63 | p30 | p30, p41, p52, p63, p74 | ( l3 + l2<<1 + l1 + 2 ) >> 2 |
| p40, p51, p62, p73 | - | p40, p51, p62, p73 | ( l4 + l3<<1 + l2 + 2 ) >> 2 |
| p50, p61, p72 | - | p50, p61, p72 | ( l5 + l4<<1 + l3 + 2 ) >> 2 |
| p60, p71 | - | p60, p71 | ( l6 + l5<<1 + l4 + 2 ) >> 2 |
| p70 | - | p70 | ( l7 + l6<<1 + l5 + 2 ) >> 2 |

Mode 5 – Vertical-Right

For a prediction block size of MxN, where all t0 – tN and l0 – lM are available for prediction, then

| 4x8 block samples | 8x4 block samples | 8x8 block samples | predicted by |
| --- | --- | --- | --- |
| - | p07 | p07 | ( t6 + t7 + 1 ) >> 1 |
| - | p17 | p17 | ( t5 + t6<<1 + t7 + 2 ) >> 2 |
| - | p06, p27 | p06, p27 | ( t5 + t6 + 1 ) >> 1 |
| - | p16, p37 | p16, p37 | ( t4 + t5<<1 + t6 + 2 ) >> 2 |
| - | p05, p26 | p05, p26, p47 | ( t4 + t5 + 1 ) >> 1 |
| - | p15, p36 | p15, p36, p57 | ( t3 + t4<<1 + t5 + 2 ) >> 2 |
| - | p04, p25 | p04, p25, p46, p67 | ( t3 + t4 + 1 ) >> 1 |
| - | p14, p35 | p14, p35, p56, p77 | ( t2 + t3<<1 + t4 + 2 ) >> 2 |
| p03 | p03, p24 | p03, p24, p45, p66 | ( t2 + t3 + 1 ) >> 1 |
| p13 | p13, p34 | p13, p34, p55, p76 | ( t1 + t2<<1 + t3 + 2 ) >> 2 |
| p02, p23 | p02, p23 | p02, p23, p44, p65 | ( t1 + t2 + 1 ) >> 1 |
| p12, p33 | p12, p33 | p12, p33, p54, p75 | ( t0 + t1<<1 + t2 + 2 ) >> 2 |
| p01, p22, p43 | p01, p22 | p01, p22, p43, p64 | ( t0 + t1 + 1 ) >> 1 |
| p11, p32, p53 | p11, p32 | p11, p32, p53, p74 | ( q + t0<<1 + t1 + 2 ) >> 2 |
| p00, p21, p42, p63 | p00, p21 | p00, p21, p42, p63 | ( q + t0 + 1 ) >> 1 |
| p10, p31, p52, p73 | p10, p31 | p10, p31, p52, p73 | ( l0 + q<<1 + t0 + 2 ) >> 2 |
| p20, p41, p62 | p20 | p20, p41, p62 | ( l1 + l0<<1 + q + 2 ) >> 2 |
| p30, p51, p72 | p30 | p30, p51, p72 | ( l2 + l1<<1 + l0 + 2 ) >> 2 |
| p40, p61 | - | p40, p61 | ( l3 + l2<<1 + l1 + 2 ) >> 2 |
| p50, p71 | - | p50, p71 | ( l4 + l3<<1 + l2 + 2 ) >> 2 |
| p60 | - | p60 | ( l5 + l4<<1 + l3 + 2 ) >> 2 |
| p70 | - | p70 | ( l6 + l5<<1 + l4 + 2 ) >> 2 |

Mode 6 – Horizontal-Down

For a prediction block size of MxN, where all t0 – tN and l0 – lM are available for prediction, then

| 4x8 block samples | 8x4 block samples | 8x8 block samples | predicted by |
| --- | --- | --- | --- |
| - | p07 | p07 | ( t4 + t5<<1 + t6 + 2 ) >> 2 |
| - | p06 | p06 | ( t3 + t4<<1 + t5 + 2 ) >> 2 |
| - | p05, p17 | p05, p17 | ( t2 + t3<<1 + t4 + 2 ) >> 2 |
| - | p04, p16 | p04, p16 | ( t1 + t2<<1 + t3 + 2 ) >> 2 |
| p03 | p03, p15, p27 | p03, p15, p27 | ( t0 + t1<<1 + t2 + 2 ) >> 2 |
| p02 | p02, p14, p26 | p02, p14, p26 | ( q + t0<<1 + t1 + 2 ) >> 2 |
| p01, p13 | p01, p13, p25, p37 | p01, p13, p25, p37 | ( l0 + q<<1 + t0 + 2 ) >> 2 |
| p00, p12 | p00, p12, p24, p36 | p00, p12, p24, p36 | ( q + l0 + 1 ) >> 1 |
| p10, p22 | p10, p22, p34 | p10, p22, p34, p46 | ( l0 + l1 + 1) >> 1 |
| p11, p23 | p11, p23, p35 | p11, p23, p35, p47 | ( q + l0<<1 + l1 + 2 ) >> 2 |
| p20, p32 | p20, p32 | p20, p32, p44, p56 | ( l1 + l2 + 1) >> 1 |
| p21, p33 | p21, p33 | p21, p33, p45, p57 | ( l0 + l1<<1 + l2 + 2 ) >> 2 |
| p30, p42 | p30 | p30, p42, p54, p66 | ( l2 + l3 + 1) >> 1 |
| p31, p43 | p31 | p31, p43, p55, p67 | ( l1 + l2<<1 + l3 + 2 ) >> 2 |
| p40, p52 | - | p40, p52, p64, p76 | ( l3 + l4 + 1) >> 1 |
| p41, p53 | - | p41, p53, p65, p77 | ( l2 + l3<<1 + l4 + 2 ) >> 2 |
| p50, p62 | - | p50, p62, p74 | ( l4 + l5 + 1) >> 1 |
| p51, p63 | - | p51, p63, p75 | ( l3 + l4<<1 + l5 + 2 ) >> 2 |
| p60, p72 | - | p60, p72 | ( l5 + l6 + 1) >> 1 |
| p61, p73 | - | p61, p73 | ( l4 + l5<<1 + l6 + 2 ) >> 2 |
| p70 | - | p70 | ( l6 + l7 + 1) >> 1 |
| p71 | - | p71 | ( l5 + l6<<1 + l7 + 2 ) >> 2 |

Mode 7 – Vertical-Left

For a prediction block size of MxN, where all t0 – tN and l0 – lM are available for prediction, then

| 4x8 block samples | 8x4 block samples | 8x8 block samples | predicted by |
| --- | --- | --- | --- |
| p00 | p00 | p00 | ( t0 + t1 + 1) >> 1 |
| p10 | p10 | p10 | ( t0 + t1<<1 + t2 + 2 ) >> 2 |
| p01, p20 | p01, p20 | p01, p20 | ( t1 + t2 + 1) >> 1 |
| p11, p30 | p11, p30 | p11, p30 | ( t1 + t2<<1 + t3 + 2 ) >> 2 |
| p02, p21, p40 | p02, p21 | p02, p21, p40 | ( t2 + t3 + 1) >> 1 |
| p12, p31, p50 | p12, p31 | p12, p31, p50 | ( t2 + t3<<1 + t4 + 2 ) >> 2 |
| p03, p22, p41, p60 | p03, p22 | p03, p22, p41, p60 | ( t3 + t4 + 1) >> 1 |
| p13, p32, p51, p70 | p13, p32 | p13, p32, p51, p70 | ( t3 + t4<<1 + t5 + 2 ) >> 2 |
| p23, p42, p61 | p04, p23 | p04, p23, p42, p61 | ( t4 + t5 + 1) >> 1 |
| p33, p52, p71 | p14, p33 | p14, p33, p52, p71 | ( t4 + t5<<1 + t6 + 2 ) >> 2 |
| p43, p62 | p05, p24 | p05, p24, p43, p62 | ( t5 + t6 + 1) >> 1 |
| p53, p72 | p15, p34 | p15, p34, p53, p72 | ( t5 + t6<<1 + t7 + 2 ) >> 2 |
| p63 | p06, p25 | p06, p25, p44, p63 | ( t6 + t7 + 1) >> 1 |
| p73 | p16, p35 | p16, p35, p54, p73 | ( t6 + t7<<1 + t8 + 2 ) >> 2 |
| - | p07, p26 | p07, p26, p45, p64 | ( t7 + t8 + 1) >> 1 |
| - | p17, p36 | p17, p36, p55, p74 | ( t7 + t8<<1 + t9 + 2 ) >> 2 |
| - | p27 | p27, p46, p65 | ( t8 + t9 + 1) >> 1 |
| - | p37 | p37, p56, p75 | ( t8 + t9<<1 + t10 + 2 ) >> 2 |
| - | - | p47, p66 | ( t9 + t10 + 1) >> 1 |
| - | - | p57, p76 | ( t9 + t10<<1 + t11 + 2 ) >> 2 |
| - | - | p67 | ( t10 + t11 + 1) >> 1 |
| - | - | p77 | ( t10 + t11<<1 + t12 + 2 ) >> 2 |

Mode 8 – Horizontal-Up

For a prediction block size of MxN, where all t0 – tN and l0 – lM are available for prediction, then

| 4x8 block samples | 8x4 block samples | 8x8 block samples | predicted by |
| --- | --- | --- | --- |
| p00 | p00 | p00 | ( l0 + l1 + 1 ) >> 1 |
| p01 | p01 | p01 | ( l0 + l1<<1 + l2 + 2 ) >> 2 |
| p10, p02 | p10, p02 | p10, p02 | ( l1 + l2 + 1 ) >> 1 |
| p11, p03 | p11, p03 | p11, p03 | ( l1 + l2<<1 + l3 + 2 ) >> 2 |
| p20, p12 | p20, p12, p04 | p20, p12, p04 | ( l2 + l3 + 1 ) >> 1 |
| p21, p13 | p21, p13, p05 | p21, p13, p05 | ( l2 + l3<<1 + l4 + 2 ) >> 2 |
| p30, p22 | p30, p22, p14, p06 | p30, p22, p14, p06 | ( l3 + l4 + 1 ) >> 1 |
| p31, p23 | p31, p23, p15, p07 | p31, p23, p15, p07 | ( l3 + l4<<1 + l5 + 2 ) >> 2 |
| p40, p32 | p32, p24, p16 | p40, p32, p24, p16 | ( l4 + l5 + 1 ) >> 1 |
| p41, p33 | p33, p25, p17 | p41, p33, p25, p17 | ( l4 + l5<<1 + l6 + 2 ) >> 2 |
| p50, p42 | p34, p26 | p50, p42, p34, p26 | ( l5 + l6 + 1 ) >> 1 |
| p51, p43 | p35, p27 | p51, p43, p35, p27 | ( l5 + l6<<1 + l7 + 2 ) >> 2 |
| p60, p52 | p36 | p60, p52, p44, p36 | ( l6 + l7 + 1 ) >> 1 |
| p61, p53 | p37 | p61, p53, p45, p37 | ( l6 + l7<<1 + l7 + 2 ) >> 2 |
| p70, p62, p71, p63, p72, p73 | - | p70, p71, p72, p73, P74, p75, p76, p77, P62, p63, p64, p65, p66, p67, p54, p55, P56, p57, p46, p47 | l7 |

5.  Conclusion

This document proposes nine new intra prediction modes for the 8x8, 8x4 and 4x8 block sizes used in ABT.  These modes use less surrounding pixels for prediction, and thus, present fewer memory access problems.  In addition, these new modes reduce the worse-case computational complexity by 50%, while leaving the compression performance unharmed.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | RealNetworks, Inc. |  |  |
| Mailing address | P.O. Box 91123 Seattle, WA 98111-9223 |  |  |
| Country | USA |  |  |
| Contact person | Greg Conklin |  |  |
| Telephone | +1 206 674 2337 |  |  |
| Fax | +1 206 674 3580 |  |  |
| Email | gregc@real.com |  |  |
| Place and date of submission | Geneva, Switzerland, October 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Reducing Pixel Dependencies for Intra-Prediction with ABT |  |
| Contribution number |  | JVT-E112 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | Gregory Conklin |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: For purposes of this contribution, the terms of 2.2.1 above [and as detailed in document VCEG-O54 submitted by Gary Sullivan] are agreed to only as necessary to comply with the required portions of a “baseline” JVT video profile.