| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E122 Filename: JVT-E122.doc |
| --- | --- |

| Title: | Field Repetition and Timing Indications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dustin Green 1065 La Avenida Mountain View CA 94043 Gary Sullivan | Tel: Email: | +1.650.693.1371 dusting@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corporation |  |  |

_____________________________

# 1.0	Summary

We propose two additional values for pic_structure to allow 3-field pictures with the first field repeated.  This will allow straightforward use of the video stream for NTSC 2:3 pull-down in scenarios that expect precisely-regular field timing (without requiring the interpretation of missing fields or the insertion of field-structured pictures with all macroblocks skipped).  We show that the proposed syntax change does not increase bit count, and how to actually simplify the syntax flow descriptions despite adding two more pic_structure values.

We also seek to clarify the expected use of the temporal_reference() and clock_timestamp() timing information.  We expect the temporal_reference() timestamp to be the time tag that is used for HRD output time definition and for a/v sync definition at the system level, while the basic HH:MM:SS:FF parameters of clock_timestamp() are expected to be used to express timing for conventional "SMPTE timecode"-style purposes, with the offset parameter being used for fine adjustments to reflect ideal display time adjustments for such purposes as drift correction and inverse 2:3 pull-down. Finally, we propose some changes to the syntax and semantics of a few bits at the beginning of these SEI messages.  A few minor related essentially-editorial issues are also proposed to be fixed.

# 2.0	Minor Related Issues

We also propose placing a limit (such as 8 bytes) on the length of time_offset.

And explicitly disallowing values of pic_structure not shown in Table 7-4.

We propose changing "bit_count" to BitCount for consistency with the naming of other non-syntax-element variables.

We propose changing "equivalent_timestamp" to EquivalentTimestamp for consistency with the naming of other non-syntax-element variables.

And changing "(PayloadSize )" to "( PayloadSize )" – found in two places.

temporal_ref_value should have a fixed length specified in the sequence parameter set.

# 3.0	Three-Field Pictures

We propose adding two more elements to Table 7-4 as shown below:

Table 7-1 – Meaning of pic_structure

| - Value of pic_structure | - Meaning |
| --- | --- |
| - 0 | - Progressive frame picture |
| - 1 | - Top field picture |
| - 2 | - Bottom field picture |
| - 3 | - Interlaced frame picture, whose top field precedes its bottom field in time. |
| - 4 | - Interlaced frame picture, whose bottom field precedes its top field in time. |
| - 5 | - Frame picture whose fields are ordered in time as top, bottom, top. Note repetition of top field. |
| - 6 | - Frame picture whose fields are ordered in time as bottom, top, bottom. Note repetition of bottom field. |

Note that because the exp-Golomb code numbers 5 and 6 map to the same number of entropy-coded bits as code numbers 3 and 4, no additional quantity of data is needed for these indications.

It is proposed that everywhere in the syntax where “(pic_structure == 3 || pic_structure == 4)” is found (with parenthesis explicit or not), this be replaced simply with “pic_structure >= 3”.

The intent is that:

- pic_structure 3 and 4 remain unaffected,

- pic_structure 5 behaves like pic_structure 3 except that the top field is indicated to be repeated as a third field (allowing the presence of a third timestamp and establishing a nominal duration of three field periods in fixed-timing scenarios), and

- pic_structure 6 behaves like pic_structure 4 except that the bottom field is indicated to be repeated as a third field (per previous item)

- values of pic_structure >= 7 remain unused and should explicitly not be allowed.

# 4.0	Changes to temporal_reference() syntax & semantics

We propose to redefine the first two bits of the temporal_reference() message.  These bits are currently called progressive_scan and bottom_field.  We propose to rename them to be a single two-bit syntax element called tr_type, defined as follows:

- '00' indicates that the temporal_reference applies to the entire picture

- '01' indicates that the temporal_reference applies to the first field of the picture

- '10' indicates that the temporal_reference applies to the second field of the picture (not to be used for field pictures)

- '11' indicates that the temporal_reference applies to the third (repeated) field of the picture (only to be used for pic_structure equal to 5 or 6)

The temporal_reference() message should be the one that governs HRD output timing, should reflect the timing expected for the primary envisioned delivery environment (e.g., 59.94 or 50 Hz timing), and is expected to be the timing indicator used by the system (if any such timing indicator is used by the system) to establish expected a/v sync relationships.  Clarifying wording to that effect should be added to the text.

# 5.0	Changes to clock_timestamp() syntax & semantics

The clock_timestamp() SEI message is intended to be used for two purposes:

- The basic HH:MM:SS:FF parameters are expected to be used for conventional "SMPTE timecode"-style purposes

- With the offset parameter, the clock_timestamp() is capbable of being used for fine adjustments to reflect ideal display time for such purposes as inverse 2:3 pull-down and correction of HH:MM:SS:FF drift caused by fractional frame rates.

We propose to redefine the first four bits of the clock_timestamp() SEI message.  We propose to make these four bits a single syntax element called ct_type, defined as follows:

  - '0000' indicates that the EquivalentTimestamp applies to the entire picture and indicates that the picture scan was progressive (only used for frame pictures)

  - '0001' indicates that the EquivalentTimestamp applies to the first field of the picture and indicates that the picture scan was interlaced

  - '0010' indicates that the EquivalentTimestamp applies to the second field of the picture and indicates that the picture scan was interlaced (not used for field pictures)

  - '0011' indicates that the EquivalentTimestamp applies to the third field of the picture and indicates that the picture scan was interlaced (used only for three-field pictures).
NOTE: In this case this is an indication that the field repetition feature was used for coding efficiency rather than source timing indication purposes.

  - '0101' indicates that the EquivalentTimestamp applies to the first field of the picture and indicates that the picture scan was progressive

  - '0110' indicates that the EquivalentTimestamp applies to the second field of the frame and indicates that the picture scan was progressive (not used for field pictures)

  - '0111' indicates that the EquivalentTimestamp applies to the third field of the frame and indicates that the picture scan was progressive (used only for three-field pictures)

  - '1000' indicates that the EquivalentTimestamp applies to the entire picture and indicates that the picture scan type is not known with certainty

  - '1001' indicates that the EquivalentTimestamp applies to the first field of the picture and indicates that the picture scan type is not known with certainty

  - '1010' indicates that the EquivalentTimestamp applies to the second field of the frame and indicates that the picture scan type is not known with certainty

  - '1011' indicates that the EquivalentTimestamp applies to the third field of the frame and indicates that the picture scan type is not known with certainty (used only for three-field pictures)

# 6.0	Conclusions

We believe the addition of support of 3-field pictures with the first field repeated will enable straightforward use of the video stream for NTSC 2:3 pull-down in scenarios that expect precisely-regular field timing (without requiring the interpretation of missing fields or the insertion of field-structured pictures with all macroblocks skipped).  This should make system and codec designers more comfortable with our standard.  We believe that the adjustments and clarifications of the temporal_reference() and clock_timestamp() SEI messages advocated herein will aid in clarifying their intended use in the system and will enhance their value when used.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft Corporation |  |  |
| Mailing address | One Microsoft Way Redmond, WA 98052 USA |  |  |
| Country | USA |  |  |
| Contact person | Gary J. Sullivan |  |  |
| Telephone | +1 (425) 703-5308 |  |  |
| Fax | +1 (425) 706-7329 |  |  |
| Email | garysull@microsoft.com |  |  |
| Place and date of submission | 5th JVT meeting, Geneva, October 9-17, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT/H.264/MPEG4-AVC |  |
| Title |  | Advanced Video Coding |  |
| Contribution number |  | JVT-E122: Field repetition and timing information |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: