| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E123 Filename: JVT-E123.doc |
| --- | --- |

| Title: | Various Minor Clean-Up Issues |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary Sullivan One Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 (425) 703-5308 garysull@microsoft.com |
| Source: | Microsoft Corporation |  |  |

_____________________________

# 1.0	High-Level Syntax Refinements

I recommend that:

- motion_resolution should be deleted (assuming it will no longer have a purpose).

- entropy_coding_mode should be 1 bit and should be called a flag.

- The maximum number of sequence and picture parameter sets should be doubled, and these limits (and all other arbitrary hard numerical limits) should be stated in Annex A rather than in the main-body text.

- The duplicate entries in table E-1 (sample aspect ratio) should be consolidated.

- sar_width and sar_height should be 16 bits each.

- frame_cropping_flag and timing_information_flag should be u(1).

- Several items at the end of vui_seq_parameters() that are marked e(v) should be something else.

- filter_parameters_flag and constrained_intra_pred_flag should be moved from the sequence parameter set to the picture parameter set.

- non_stored_content_flag is redundant with nal_storage_idc and should therefore be removed.

# 2.0	Weighted Prediction Clean-up

## 2.1	Zero-valued weights?

Background: The pred_weight_table() syntax allows zero-valued weights to be transmitted.  This makes no sense (except perhaps as one ugly way to achieve something discussed in the next section).

Proposal: Use the descriptor me(v) for the weight parameters and define in the semantics section that the decoded value is the value for se(v) with the code number incremented by 1.

## 2.2	Duplicate List Entry Clarification

Background: In Klagenfurt we got rid of extra syntax at or below the macroblock level for selecting between different prediction weights, in favor of a weight selection using the list 0 and list 1 index values instead.  By doing so, we simplified the syntax and reduced the low-level overhead while sacrificing the ability to apply more than one weight to a particular reference picture during the decoding of a single slice.  Or so some of us may have thought.

Proposal: Specifically state that the same reference picture may appear more than once in the reference picture lists, and that the number of reference pictures active in each list may exceed the number of reference pictures in the buffer (neither of these things is specifically prohibited now, so the proposal is only to clearly state that something that is not prohibited now is definitely not prohibited).  As each list entry has its own weight, this enables different weights to be applied to the same reference picture without any extra syntax burden at the macroblock level or below.  This is possible with no change of syntax.

NOTE: I attribute this suggestion to Dustin Green.

## 2.3	Implicit Bi-Prediction with Long-Term Pictures

Do we really mean decoding order in 10.4.3, or do we mean display order?

How is implicit bi-prediction (10.4.3) applied to long-term pictures, for which decoding order should not be considered relevant?

Why would skipped macroblocks and direct mode use (2, -1) weights when the current picture falls temporally between the list 0 and list 1 references?  In my mental model, the (2, -1) weights make sense primarily when the picture getting the weight of 2 is temporally on the same side as the other reference picture and is temporally farther away than the other reference picture.

# 3.0	Loop Filter Clean-up

## 3.1	Clarification of deblocking filter control

Background: The draft says the filter adjustments of disable_deblocking_filter_flag, slice_alpha_c0_offset_div2, and slice_beta_offset_div2 apply to "the edges controlled by the macroblocks within the current slice".  But some edges controlled by the macroblocks within the current slice are also controlled by the macroblocks in other slices (by virtue of QP averaging for cross-slice boundary filtering).  So it is not clear what edges are controlled by these adjustment parameters.

Proposal: The author's interpretation is that for any edges between two different slices, the adjustment parameters of the macroblock to the right of a vertical edge apply, and the adjustment parameters of the macroblock below a horizontal edge apply.  This clarification should be added to the text.

## 3.2	Refinement of disable_deblocking_filter_flag definition

Background: Somehow I feel compelled to find out if the mood has changed on these issues.

Proposal: Rename disable_deblocking_filter_flag to disable_deblocking_filter_idc and give this indicator the following four defined values:

code_number = 0: Deblocking filter not disabled

code_number = 1: Operation of the deblocking filter is disabled for the edges controlled by the macroblocks within the current slice (defined as clarified above)

code_number = 2: Operation of the deblocking filter is disabled across slice boundaries for the edges controlled by the macroblocks within the current slice

code_number = 3: Operation of the deblocking filter is disabled across slice boundaries and across the boundaries between intra and inter macroblocks for the edges controlled by the macroblocks within the current slice

# 4.0	Sequence VUI Chroma Locations

All chroma locations that exist in a frame should also exist in fields, and vice versa.  This will allow switching between frame and field coding without indicating that the chroma sample geometry is moving around, and will eliminate any need to encode in one form or another solely to indicate its chroma sample locations.

# 5.0	Compression Ratio Requirement Decrease

The 4:1 compression ratio requirement should be reduced to allow for highly-detailed I frame coding with good fidelity for challenging input pictures.  The requirement should be restated to restrict the number of total bits for a frame rather than the ratio of bits to video resolution, allowing higher fidelity coding of lower-resolution pictures.

# 6.0	Macroblock-Level AFF

We should clarify that if the first macroblock of a macroblock pair is skipped, the deblocking filter operation for both macroblocks will use the QP value of the second macroblock of the pair.

# 7.0	HRD-Related Issues

pdb_cnt should not be allowed to exceed some value, such as 32.

A minimum frame interval should be defined in seconds per frame (conceptually as the inverse of the maximum frame rate) and a minimum field interval should be defined as half the minimum frame interval.  Decoding times should be restricted to require

- the time difference between the decoding time of a current frame-structured picture and the decoding time of the previous frame or field in decoding order shall be no smaller than the minimum frame interval, and similarly

- the time difference between the decoding time of a current field-structured picture and the decoding time of the previous frame or field in decoding order shall be no smaller than the minimum field interval.

Output times should be restricted to require that

- the absolute time difference between the output times of any pair of fields shall not be less the minimum field interval, and

- the absolute time difference between the output times of any pair of fields with the same parity shall not be less than the minimum frame interval.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft Corporation |  |  |
| Mailing address | One Microsoft Way Redmond, WA 98052 USA |  |  |
| Country | USA |  |  |
| Contact person | Gary J. Sullivan |  |  |
| Telephone | +1 (425) 703-5308 |  |  |
| Fax | +1 (425) 706-7329 |  |  |
| Email | garysull@microsoft.com |  |  |
| Place and date of submission | 5th JVT meeting, Geneva, October 9-17, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT/H.264/MPEG4-AVC |  |
| Title |  | Advanced Video Coding |  |
| Contribution number |  | JVT-E123: Various Clean-Up Issues |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: