| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E134 Filename: JVT-E134.doc |
| --- | --- |

| Title: | Cross-verification of MV Spread Restriction (JVT-E041) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Anthony Joch UB Video Inc. 400-1788 West 5th Ave Vancouver, BC, V6J 1P2, Canada | Tel: Email: | +1 604 737 2424 .33 anthony@ubvideo.com |
| Source: | UB Video Inc. |  |  |

_____________________________

This document provides cross-verification of a sub-set of the results presented in JVT-E041, “MV Spread in sub-8x8 Partitions”. Source code provided by the proponents was verified and run for the common conditions. Due to limited time, results were only generated for the “Hybrid” motion search case with a byte limit of 484 bytes. Results for the unrestricted software (Ref) and the 484 bytes limit (484B) are presented below. The results are consistent with those presented in JVT-E041, with a slight variation on the Foreman sequence, likely due to a difference in the input data.

| Sequence | frames | QP | Ref PSNR | Ref Bitrate | 484B PSNR | 484B Bitrate | BD-Bitrate | BD-PSNR [db] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 28 | 36.03 | 23.19 | 36.03 | 23.19 | 0.000% | -0.000 |
|  |  | 32 | 33.23 | 12.92 | 33.23 | 12.92 |  |  |
|  |  | 36 | 30.54 | 7.65 | 30.54 | 7.65 |  |  |
|  |  | 40 | 27.89 | 4.75 | 27.89 | 4.75 |  |  |
| foreman | 100 | 28 | 36.04 | 69.52 | 36.05 | 69.65 | 0.001% | -0.000 |
|  |  | 32 | 33.25 | 42.67 | 33.26 | 42.62 |  |  |
|  |  | 36 | 30.61 | 26.46 | 30.60 | 26.47 |  |  |
|  |  | 40 | 28.10 | 16.71 | 28.08 | 16.74 |  |  |
| mobile | 300 | 28 | 34.03 | 1490.12 | 34.03 | 1489.41 | 0.066% | -0.003 |
|  |  | 32 | 30.64 | 731.97 | 30.64 | 732.12 |  |  |
|  |  | 36 | 27.58 | 353.20 | 27.58 | 353.46 |  |  |
|  |  | 40 | 24.66 | 189.39 | 24.65 | 189.64 |  |  |
| news | 100 | 28 | 36.82 | 44.36 | 36.84 | 44.30 | 0.362% | -0.021 |
|  |  | 32 | 33.75 | 27.30 | 33.71 | 27.40 |  |  |
|  |  | 36 | 30.83 | 16.77 | 30.85 | 16.88 |  |  |
|  |  | 40 | 28.14 | 10.41 | 28.16 | 10.36 |  |  |
| paris | 150 | 28 | 35.50 | 311.17 | 35.49 | 310.94 | -0.122% | 0.006 |
|  |  | 32 | 32.42 | 180.38 | 32.43 | 180.28 |  |  |
|  |  | 36 | 29.51 | 100.87 | 29.51 | 100.69 |  |  |
|  |  | 40 | 26.85 | 57.15 | 26.84 | 57.16 |  |  |
| silent | 150 | 28 | 35.82 | 56.40 | 35.84 | 56.42 | 0.195% | -0.009 |
|  |  | 32 | 32.98 | 34.05 | 32.97 | 34.09 |  |  |
|  |  | 36 | 30.36 | 20.02 | 30.37 | 20.11 |  |  |
|  |  | 40 | 28.03 | 11.71 | 28.01 | 11.68 |  |  |
| tempete | 260 | 28 | 35.00 | 1130.14 | 35.00 | 1129.51 | 0.173% | -0.007 |
|  |  | 32 | 31.85 | 546.95 | 31.84 | 546.90 |  |  |
|  |  | 36 | 29.01 | 270.45 | 28.99 | 269.62 |  |  |
|  |  | 40 | 26.33 | 145.92 | 26.33 | 146.18 |  |  |
| Average |  |  |  |  |  |  | 0.096% | -0.004 |