| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E136 Filename: JVT-E136.doc |
| --- | --- |

| Title: | Demonstration of “FCD-Conformant” Baseline Real-Time Codec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Anthony Joch, Jaehan In and Faouzi Kossentini UB Video Suite 400 1788 West 5th Avenue Vancouver, BC, Canada V6J 1P2 | Tel: Email: | +1 604 737 2426 .33 anthony@ubvideo.com |
| Source: | UB Video Inc. |  |  |

_____________________________

# Overview

We present revised results from a computation-optimized JVT codec implementation that achieves near-real-time simultaneous encoding and decoding of CIF resolution video sequences on a Pentium III 800 MHz laptop. A demonstration of the codec will be available at the meeting. The codec supports all the syntax and coding features specified for the baseline profile of the JFCD. We welcome offers for bitstream exchange with interested parties in the near future.

# Description of Software

The UB Video codec is a highly optimized implementation on Pentium III and IV platforms that has been developed from scratch.  The main sources of the speed improvement relative to the TML software are the highly efficient design and efficient motion estimation and mode decision algorithms that are tailored for real-time, conversational video applications.  Additionally, through the additions to the draft and improved rate-distortion optimization, our latest encoder provides a significant improvement of both objective and subjective quality over the previous version that was demonstrated at Fairfax (JVT-C142).

# Simulations

Simulations were generated using the following system:

CPU: 		Pentium III, 800 MHz

Memory: 	128 MB SDRAM

All of the results presented use CAVLC entropy coding and just one reference frame.

# Performance

## Speed

Encoding Speed

The following table illustrates the average encoding speeds in frames per second at three bit rates using CIF resolution Akiyo, Foreman, News, Paris, and Silent sequences at 30Hz with the exception of 128Kbps, where sequences were converted to a 15Hz frame rate.

| Bit rate | Akiyo | Foreman | News | Paris | Silent |
| --- | --- | --- | --- | --- | --- |
| 128 Kbps (15Hz) | 65 fps | 33 fps | 55 fps | 52 fps | 44 fps |
| 384 Kbps | 56 fps | 36 fps | 53 fps | 54 fps | 50 fps |
| 768 Kbps | 44 fps | 31 fps | 40 fps | 45 fps | 39 fps |

Decoding Speed

The following table illustrates the average decoding speeds in frames per second when decoding above-encoded sequences.

| Bit rate | Akiyo | Foreman | News | Paris | Silent |
| --- | --- | --- | --- | --- | --- |
| 128 Kbps (15Hz) | 120 fps | 92 fps | 116 fps | 114 fps | 108 fps |
| 384 Kbps | 110 fps | 85 fps | 107 fps | 108 fps | 101 fps |
| 768 Kbps | 99 fps | 67 fps | 83 fps | 88 fps | 80 fps |

Combined Encoding/decoding Speed

The following table illustrates the average combined encoding/decoding speeds in frames per second when simultaneously encoding/decoding the above sequences.

| Bit rate | Akiyo | Foreman | News | Paris | Silent |
| --- | --- | --- | --- | --- | --- |
| 128 Kbps (15Hz) | 42 fps | 24 fps | 37 fps | 35 fps | 33 fps |
| 384 Kbps | 37 fps | 25 fps | 36 fps | 36 fps | 33 fps |
| 768 Kbps | 31 fps | 21 fps | 27 fps | 30 fps | 26 fps |

## Subjective Quality

The subjective quality of the video sequences encoded using UB Video’s JFCD encoder will be illustrated through the demonstration, which will be available during the meeting.