| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-Exxx Filename: JVT-Exxx.doc |
| --- | --- |

| Title: | Limits for High-level Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal and Discussion |  |  |
| Author(s) or Contact(s): | Toby Walker, Ali Tabatabai 3300 Zanker Road San Jose, CA, USA, 95134 | Tel: Email: | 1-408-955-4081 toby.walker@am.sony.com |
| Source: | Sony |  |  |

_____________________________

# Introduction

This contribution addresses limiting the size of variable length coded fields in the high-level syntax, especially in parameter sets and slices headers. In the worst case, these fields can be arbitrarily large. This contribution identifies problematic fields and proposes some alternative strategies for limiting the size of such fields.

## Problem Statement

In the current high-level syntax many elements are coded as variable length fields. Each of these fields is a potential problem in that, unless there is a restriction on the set of legal values that are encoded in the field, the size of these fields is potentially unbounded. This true for fields that are encoded as UVLC code points, such as delta encoding of picture order counts in slice orders, as well for fields using a configurable size, like the frame_num field in the slice header.

In this rest of this contribution we refer to these fields in the high-level syntax as unbounded fields.  For someone trying to implement a compliant decoder, unbounded fields are a potential problem from three perspectives.

- Unbounded Memory Consumption. A compliant decoder has to store all fields inside of a parameter set. If these fields are unbounded then the storage space required for parameter sets is also unbounded.

- Unbounded Length of Syntax Elements. A decoder must be prepared to parse parameter sets or slice headers whose length can be arbitrarily large up to the constraints impose by HRD compliance.

- Decoding Processing. A decoder must be prepared to perform computations on fields whose upper limit is not known. For example, a frame number of 1000 bits long is syntactically and semantically legal in the FCD (even when subject to HRD considerations).

## Possible Approaches

There are two obvious approaches to solving this problem: (1) a local approach in which we assign specific limits to each unbounded field; and (2) a global approach where we assign upper limits to the size of certain syntax elements but do not define field specific limits.

Using the global approach one could limit the size of major high-level syntax elements. For example, one could impose an upper limit on the size of a picture parameter set to be m bits. Or even for a global limit of n bytes for the storage of all sequence or all picture parameter sets. Such an approach would impose an implicit limit on every field in the constrained syntax element. A variation of this idea would be to impose limits on subparts of the high-level syntax. For example, limiting the size of the adaptive weighting prediction element to a total size of x bits.

While the global approach addresses the unbounded memory consumption and unbounded length problems it still does not address the decoding processing issue. Therefore we believe that a local approach is essential for all unbounded fields. It is also worth considering whether global limits should be combined with local limits. For example, it might be useful to define a maximum size a decoder must allocate to store parameter sets to, say, 16K for sequence parameter sets and 64K for picture parameter sets.

In the rest of this contribution we examine each of the high-level syntax elements – parameter sets, slice headers – to determine the unbounded fields. For each field we discuss the issues with limiting the field and propose some initial limits for discussion. Note that our intention here is not to advocate one particular limit but to start the discussion of appropriate limiting values.

SEI messages have similar issues but we did not have time to look into this part of the high-level syntax.

# Sequence Parameter Set Limits

In this section we review the sequence parameter set syntax and semantics to identify fields that are unbounded and discuss possible limits for these fields.

## Unbounded Fields

The following table shows all fields in the sequence parameter set syntax using variable length coding. Beside each field we indicate whether it is currently bounded or not.

| Field Name | Descriptor | Bounded | Limit |
| --- | --- | --- | --- |
| profile_idc | ue(v) | Yes? | Maximum defined value among current profile codes but no value defined for future profiles. |
| level_idc | ue(v) | Yes? | Maximum defined value implicitly for existing levels but no maximum for future levels. |
| seq_parameter_set_id | ue(v) | Yes? | Profile limits to maximum of 16 but it unclear if this is limited to a value between zero and 15. |
| log2_max_frame_num_minus4 | ue(v) | No |  |
| pic_order_cnt_type | ue(v) | Yes | Semantics. |
| log2_max_pic_order_cnt_minus4 | ue(v) | No |  |
| offset_for_non_stored_pic | se(v) | No |  |
| num_stored_frames_in_pic_order_cnt_cycle | ue(v) | No |  |
| offset_for_stored_frame[ i ] | se(v) | No |  |
| num_of_ref_frames | ue(v) | Yes | Annex A limits to maximum of 16. |
| pic_width_in_mbs_minus1 | ue(v) | Yes | Each level defines a maximum picture size in MBs. |
| pic_height_in_mbs_minus1 | ue(v) | Yes | Each level defines a maximum picture size in MBs. |

## Limiting Field Values

Most fields at the sequence parameter level are bounded by the limits specified in Annex A. The fields that are currently unbounded control the coding of frame numbers and picture order counts in slice headers.

### Frame Numbers Parameters

The log2_max_frame_num_minus4 field specifies the number of bits used to code the frame number in each slice header.

| Field | Limit | Rationale |
| --- | --- | --- |
| log2_max_frame_num_minus4 | 12 | Since frame numbers larger than 16 bits are unlikely, we suggest assigning a maximum value of this field of 12 (i.e. 16-4). |

### Picture Order Count Parameters

| Field | Limit | Rationale |
| --- | --- | --- |
| log2_max_pic_order_cnt_minus4 | [0,28] | Similar to frame numbers, this field specifies the number of bits used to code picture order counts in slice headers when POC mode 0 is used. We propose limiting the size of the picture_cnt field in the slice header to 32 bits because that is the maximum accuracy for the picture order count required by the decoder. This gives a limit for this field of 32-4 = 28. |
| offset_for_non_stored_pic | [-231, 231-1] | This field is the default offset for the POC value from the first non-stored frame after each stored frame. Again, limiting this to a maximum value that fits in the range of signed 32-bit number is reasonable given that POC values are maintained by a decoder as 32-bit unsigned values. |
| num_stored_frames_in_pic_order_cnt_cycle | 16 | This field indicates the length of the repeating pattern of differences between picture order counts. A value of 16 seems more than reasonable. |
| offset_for_stored_frame[ i ] | [-231, 231-1] | Same as for offset_for_non_stored_pic. |

# Picture Parameter Sets Limits

In this section we review the picture parameter set syntax and semantics to identify fields that are unbounded and discuss possible limits for these fields.

## Unbounded Fields

The following table shows all fields in the picture parameter set syntax using variable length coding. Beside each field we indicate whether it is currently bounded or not.

| Field Name | Descriptor | Bounded | Limit |
| --- | --- | --- | --- |
| pic_parameter_set_id | ue(v) | Yes? | Limit of 64 picture parameter sets. Assume that legal values are between 0 and 63. |
| seq_parameter_set_id | ue(v) | Yes? | Limit of 16 sequence parameter sets. |
| entropy_coding_mode | ue(v) | Yes | Semantics. |
| motion_resolution | ue(v) | Yes | Semantics. |
| num_slice_groups_minus1 | ue(v) | Yes | By profile defined limits on slice groups |
| mb_allocation_map_type | ue(v) | Yes | Semantics |
| run_length | ue(v) | Yes | Bounded by size of maximum picture in level. |
| slice_group_id | u(v) | Yes | By profile defined limits on slice groups. |
| top_left_mb | u(v) | Yes | Bounded by size of maximum picture in level. |
| bottom_right_mb | u(v) | Yes | Bounded by size of maximum picture in level. |
| slice_group_change_rate_minus1 | ue(v) | Yes. | Bounded by the maximum number of MBs in a picture. |
| num_ref_idx_l0_active_minus1 | ue(v) | Yes | Bounded by maximum # of reference frames (16). |
| num_ref_idx_l1_active_minus1 | ue(v) | Yes | Bounded by maximum # of reference frames (16). |
| slice_qp_minus26 /* relative to 26 */ | se(v) | Yes | Limited by legal range of QP (0,51). |
| slice_qp_s_minus26 /* relative to 26 */ | se(v) | Yes | Limited by legal range of QP (0,51). |

Thus we can see that currently all picture parameter fields are bounded.

# Slice Header Limits

In this section we review the slice header syntax and semantics, as well its contained elements, to identify fields that are unbounded and discuss possible limits for these fields.

## Unbounded Fields

The following tables show all fields in the slice header syntax, as well as contained elements that use variable length coding. Beside each field we indicate whether it is currently bounded or not.

### Slice Header

| Field | Descriptor | Bounded | Limit |
| --- | --- | --- | --- |
| pic_parameter_set_id | ue(v) | Yes | Maximum of 64 parameter sets. |
| frame_num | u(v) | No | Bounded by the value of this field specified in the sequence parameter set. |
| pic_structure | ue(v) | Yes | Semantics. |
| first_mb_in_slice | u(v) | Yes | Maximum number of MBs in level. |
| slice_type_idc | ue(v) | Yes | Semantics. |
| pic_order_cnt | u(v) | No | Bounded by the number of bits configured in picture set. |
| delta_pic_order_cnt | se(v) | No |  |
| redundant_pic_cnt | ue(v) | No |  |
| num_ref_idx_l0_active_minus1 | ue(v) | Yes | Bounded by maxmimum size of reference picture buffer (16). |
| num_ref_idx_l1_active_minus1 | ue(v) | Yes | Bounded by maxmimum size of reference picture buffer (16). |
| slice_qp_delta | se(v) | Yes | Bounded by limits on QP ([-51, 51]) |
| slice_qp_s_delta | se(v) | Yes | Bounded by limits on QP ([-51, 51]) |
| slice_alpha_c0_offset_div2 | se(v) | Yes | Semantics. |
| slice_beta_offset_div2 | se(v) | Yes | Semantics |
| slice_group_change_cycle | u(v) | Yes | Limited by value of slice group change configured in sequence parameter set. |

### Reference Picture Reordering

| Field | Descriptor | Bounded | Limits |
| --- | --- | --- | --- |
| remapping_of_pic_nums_idc | ue(v) | Yes | Semantics |
| abs_diff_pic_num_minus1 | ue(v) | No | Bounded by the value of MAX_PN in the picture parameter set. |
| long_term_pic_idx | ue(v) | Yes | Bounded by maximum picture buffer size (16). |
| remapping_of_pic_nums_idc | ue(v) | Yes | Semantics |
| long_term_pic_idx | ue(v) | Yes | Bounded by maximum picture buffer size (16). |

### Reference Picture Buffer Management

| Field | Descriptor | Bounded | Limits |
| --- | --- | --- | --- |
| memory_management_control_operation | ue(v) | Yes | Semantics. |
| difference_of_pic_nums_minus1 | ue(v) | No | Bounded by MAX_PN value in sequence parameter set. |
| long_term_pic_idx | ue(v) | Yes | Bounded by maximum picture buffer size (16). |
| max_long_term_pic_idx_plus1 | ue(v) | Yes | Bounded by maximum picture buffer size (16). |

### Prediction Weight Table

| Field | Descriptor | Bounded | Limit |
| --- | --- | --- | --- |
| luma_log_weight_denom | ue(v) | No |  |
| chroma_log_weight_denom | ue(v) | No |  |
| luma_weight_l0[i] | se(v) | Yes | Semantics |
| luma_offset_l0[i] | se(v) | No |  |
| chroma_weight_l0[i][j] | se(v) | Yes | Semantics |
| chroma_offset_l0[i][j] | se(v) | No |  |
| luma_weight_l1[i] | se(v) | Yes | No restrictions in |
| luma_offset_l1[i] | se(v) | No |  |
| chroma_weight_l1[i][j] | se(v) | Yes | Semantics |
| chroma_offset_l1[i][j] | se(v) | No |  |
| num_custom_bipred_weights | ue(v) | Yes? | Is this limited by square of number of maximum number of reference pictures? |
| irp_l0 | xe(v) | Yes | Bounded by maximum size of reference picture buffer defined in limits |
| irp_l1 | xe(v) | Yes | Bounded by maximum size of reference picture buffer defined in limits |
| luma_weight_bipred_l0[irp_l0][irp_l1] | se(v) | No | Sum of two weights limited but not individual weights. |
| luma_weight_bipred_l1[irp_l0][irp_l1] | se(v) | No |  |
| luma_offset_bipred[irp_l0][irp_l1] | se(v) | No |  |
| chroma_weight_bipred_l0[irp_l0][irp_l1][j] | se(v) | No |  |
| chroma_weight_bipred_l1[irp_l0][irp_l1][j] | se(v) | No |  |
| chroma_offset_bipred[irp_l0][irp_l1][j] | se(v) | No |  |

## Limiting Field Values

### Slice Header

| Field | Limit | Rationale |
| --- | --- | --- |
| frame_num | 216-1 | As discussed in limit for frame number size in sequence parameter set. |
| pic_order_cnt | [0, 232-1] | 32 bits is the maximum accuracy of the picture order count maintained by the decoder. |
| delta_pic_order_cnt | [-231, 231-1] | The maximum accuracy of a POC is 32 bits unsigned, delta values of half that range should be sufficient. |
| redundant_pic_cnt | 4? | It is unlikely that more than a "few" redundant pictures would be used. |

### Reference Picture Reordering

| Field | Limit | Rationale |
| --- | --- | --- |
| abs_diff_pic_num_minus1 | Same as bound for frame_number * 2. | Depends on the value of MAX_PN = MAX_FN * 2. The proposed limit for MAX_FN is discussed in the section on limits for sequence parameter sets. |

### Reference Picture Buffer Management

| difference_of_pic_nums_minus1 | [-MAX_PN, MAX_PN] | Depends on the value of MAX_PN = MAX_FN * 2. The proposed limit for MAX_FN is discussed in the section for limits for sequence parameter sets. |
| --- | --- | --- |

### Prediction Weight Table

We have briefly examined this syntax and point out that many elements appear to be unbounded by the semantics of these fields. However, the authors are aware that various improvements to this syntax will be proposed in Geneva and therefore have not suggested any limits for these fields pending a discussion of proposed syntax/semantic changes.

# Conclusion

We have conducted a detailed review of syntax elements in the high-level to identify potentially problematic elements whose value (and size) can be unbounded.

At Geneva we propose a two stage discussion:

- General discussion of right approach for limiting high-level syntax take place based on this contribution; and

- Detailed discussion on the appropriate limit values by the experts on each of those areas.