| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, Switzerland, 9-17 October, 2002 | Document: JVT-E141 Filename: JVT-E141.doc |
| --- | --- |

| Title: | On Spare Picture SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Tian TICSP Tampere Univ. of Technology P.O. Box 553 33101 Tampere Finland Miska M. Hannuksela Nokia Mobile Software P.O. Box 88 33721 Tampere Finland | Tel: Email: Tel: Email: | +358 3 3115 3853 tian@cs.tut.fi +358 7180 73151 miska.hannuksela@nokia.com |
| Source: | Tampere University of Technology and Nokia Corporation |  |  |

_____________________________

# Summary

This contribution proposes a small improvement on the spare picture SEI messages in JVT FCD.

The contribution is organized as follows: Section 2 introduces the problems in the definition of spare picture SEI message of JVT FCD. Section 3 presents the proposed changes relative to the JVT FCD.

# iNTRODUCTION

The spare picture SEI message indicates that certain macroblocks, called spare decoded macroblocks, in one or more decoded stored pictures resemble the co-located macroblocks in a certain decoded picture, called the target picture, so much that any of these spare decoded macroblocks can be used to replace a co-located incorrect decoded macroblock in the target picture in the multi-frame buffer and in decoder output. Decoded pictures that contain spare macroblocks are called spare pictures.

In current FCD the spare picture SEI message, delta_frame_num is defined to indicate the target picture number. Such a definition leads to a strong relationship between the spare picture SEI message and its associated picture, which may cause problems in storage and transmission, because the spare picture SEI message must rely on the storage place or transmission order to decode its target picture correctly.

# proposals

To decouple the relationship between the spare picture SEI message and the target picture, it is proposed to code the frame number of target picture as an absolute frame number. Thus, we propose replacing the delta_frame_num with target_frame_num in the syntax. See section 4 for the details on the syntax and semantics modifications.

# syntax and sematics modificaton on fcd

This section gives the modified spare picture SEI message syntax and semantics in a high-lighted style.

- D.2.11 Spare picture SEI message syntax

| spare_picture( PayloadSize ) { | Category | Descriptor |
| --- | --- | --- |
| target_frame_num | 7 | u(v) |
| num_spare_pics_minus1 | 7 | ue(v) |
| for( i = 0; i < num_spare_pics_minus1+1; i++ ) { |  |  |
| delta_spare_frame_num | 7 | ue(v) |
| ref_area_indicator | 7 | ue(v) |
| if( ref_area_indicator = = 1 ) |  |  |
| for( j = 0; j < number_of_mbs_in_pic; j++ ) |  |  |
| ref_mb_indicator | 7 | u(1) |
| else if( ref_area_indicator = = 2 ) { |  |  |
| MbCnt = 0 |  |  |
| do { |  |  |
| zero_run_length | 7 | ue(v) |
| MbCnt = MbCnt + zero_run_length + 1 |  |  |
| } while( MbCnt <= MaxMbAddress ) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

- D.3.11 Spare picture SEI message semantics

The spare picture SEI message indicates that certain macroblocks, called spare decoded macroblocks, in one or more decoded stored pictures resemble the co-located macroblocks in a certain decoded picture, called the target picture, so much that any of these spare decoded macroblocks can be used to replace a co-located incorrect decoded macroblock in the target picture in the multi-frame buffer and in decoder output. Decoded pictures that contain spare macroblocks are called spare pictures.

target_frame_num identifies the target picture whose spare pictures and macroblocks are specified later in the message. The target picture is the latest stored picture having frame_num equal to target_frame_num.

num_spare_pics_minus1 specifies the number of pictures which contain spare picture or macroblocks for the target picture.

delta_spare_frame_num specifies to which spare picture the following spare picture information in the current loop count belongs. For the first spare picture of the message, CandidateSpareFrameNum is set to target_frame_num -1, if target_frame_num is greater than 0 and MAX_FN – 1 otherwise. For later spare pictures, CandidateSpareFrameNum is the SpareFrameNum of the previous loop round minus 1 if SpareFrameNum is greater than 0 and MAX_FN – 1 otherwise. For each loop round, SpareFrameNum is calculated as follows:

SpareFrameNum = CandidateSpareFrameNum – delta_spare_frame_num
if( SpareFrameNum < 0 )
		SpareFrameNum = MAX_FN + SpareFrameNum 	(D-3)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | On Spare Picture SEI Message |  |
| Contribution number |  | JVT-E141 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. X |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: