# General

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Throughout | MICROSOFT | Editorial |  |  |

Context

Numbering of fields of frames in figures.

Question

Sometimes fields in figures are numbered 0 and 1, and sometimes 1 and 2.

Proposal

Use zero-based numbering convention as is used elsewhere throughout the text.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Throughout | MICROSOFT | Minor technical |  |  |

Context

Our understanding is that division operators are used in the following three places, all based on DiffPicOrderCount values:

1) Implicit weighted prediction weight computation (where the most significant problem lies due to the potential need to consider all pairwise combinations of positions in list 0 and list 1)

2) Temporal direct mode motion vector computation (where the number of required divisions required can easily be cut in half by replacing MV1 = (W * MV +128) >> 8 with MV1 = MV0 – MV).

3) Motion vector scaling for B slices (subclause 8.4.1.2.2 in JVT-E146d37)

Question

The use of division operations is a computational problem, particularly for implicit weighted prediction for decoding field mode macroblocks (when the maximum number of reference pictures doubles relative to frame mode coding).

Proposal

The need, accuracy requirements, and wordlength requirements for these division operations should be reviewed.  If feasible without impact on quality, the operations in question should be modified to avoid use of the division operator.  Consider the potential simplification for temporal direct mode motion as noted above.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Throughout | MICROSOFT | Major technical/editorial |  |  |

Context

Macroblock-adaptive frame/field decoding operation.

Question

This part of the design seems to lack clarity/completeness.

Proposal

Our understanding/position on specific aspects of MB-AFF is as follows.  We believe these points should address all remaining issues regarding clear specification of MB-AFF. The draft should be reviewed to ensure that it is consistent with this interpretation and that all aspects are clearly documented:

1) MB-AFF should be supported in Profiles Main and X.

2) The slice-level field picture flag and the sequence parameter set-level MB-AFF flag should be merged into a single syntax element in the picture parameter set

3) If Annex A specifies a maximum number of reference frames equal to T, the maximum range of num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 for frame pictures (including MB-AFF frame pictures) is at least T-1 (saying "at least" to allow multiple weights per picture to possibly  increase that number) and for field pictures is at least 2*T-1.

4) In a field macroblock pair, the maximum allowed value of ref_idx_l0 is 2*(num_ref_idx_l0_active_minus1+1)-1.  In a frame macroblock pair, the maximum allowed value of ref_idx_l0 is  num_ref_idx_l0_active_minus1.  Similar for ref_idx_l1.

5) If num_ref_idx_l0_active_minus1 = 0 and MB-AFF is in use, ref_idx_l0 is not sent for frame macroblocks but is sent for field macroblocks.

6) In a field macroblock pair, a value of ref_idx_l0 equal to 2n and 2n+1 refer to the same frame as for a frame macroblock pair with ref_idx_l0 equal to n.  There must be a definition of which field is numbered 2n and which is 2n+1.  (Idea #1: 2n is the one closest in POC order or the top field if the POCs are equal, Idea #2: 2n is the one with the same parity as the current field MB and 2n+1 is the one with opposite parity).

7) In explicit weighted prediction for MB-AFF, one weight is sent per reference frame index.  When an MB pair is coded in field mode, that same weight is applied to either field of that frame. (In any case there must be a definition of the association of weights to fields.)

8) In implicit weighted prediction for MB-AFF, frame mode MB pairs use weights calculated from the POC of the referenced frame and field mode MB pairs use weights calculated from the POC of the referenced field. (Note that this impacts the number of division operations required for implicit weighted bi-prediction, and the complexity of decoding process with regard to division operations should be evaluated and reduced if possible.)

10) Intra prediction for frame MB pairs should be based on neighbouring decoded samples constructed into a frame.  Intra prediction for field MB pairs should similarly be based on spatial relationships with data in the same field as the field being predicted.

11) The definition of co-located MB when the current MB pair is in different frame/field mode than the co-located MB pair must be clear.

12) Should the deblocking filter operate on the interior edges of a field MB pair in the mode of the MB pair (with the step size of the MB) or always in frame mode (with the average of the step sizes of the  two MBs of the pair)?

13) Should the deblocking filter operate on the exterior edges of the MB pair in the mode of the MB pair or always in frame mode?

14) The method of performing motion data prediction for MB-AFF must be clear. One idea is use a precise definition of spatial neighbors that works across differences in MB pair type and another is to predict motion data for field MB pairs only from other field MB pairs and to predict motion data for frame MB pairs only from other frame MB pairs.

15) The method of predicting intra modes for MB-AFF must be clear.  Does that operate across MB boundaries?

16) If any kind of other cross-macroblock prediction not discussed above is used, it must be clearly specified for MB-AFF (e.g., CABAC context issues)

17) MB location is defined properly in 3.60 of the SoFCD JVT-E146d37.  Text elsewhere should be changed if not consistent with that definition.

18) If a reference frame store has only one field occupied, it is not a candidate for use in prediction of an MB-AFF frame picture.

19) If reference re-ordering is done for an MB-AFF frame, the re-ordering is a re-ordering of frames.

20) "frame-structured picture" should be "coded frame" and "field-structured picture" should be "coded field".

# Clause 7

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7 (of JVT-E146d37) | MICROSOFT | Editorial |  |  |

Context

Syntax category numbers 2 and 3 are not used.

Question

This seems editorially sloppy and may confuse the reader.

Proposal

Change category numbers 4 through 7 to be 2 through 5.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.1 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

cabac_alignment_bit is equal to 0.

Question

Shouldn't it be equal to 1? (We set most such vacant things equal to 1 for start code emulation prevention except for things specifically designed to handle their relationship to start codes.)

It would also be helpful to give it a name that directly implies its value (cabac_alignment_one_bit).

Proposal

Rename cabac_alignment_bit to cabac_alignment_one_bit and set it equal to 1.

Also rename other similar syntax elements to imply their value (rbsp_stop_bit  rbsp_stop_one_bit, rbsp_alignment_bit  rbsp_alignment_zero_bit).

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1 (of JVT-E146d37) | MICROSOFT | Minor technical |  |  |

Context

There is no indication in the sequence parameter set to indicate whether FMO, ASO, and redundant slices may be present in the sequence.

Question

This makes it impossible to tell at the sequence level whether a Main profile decoder can decode a Baseline profile sequence.

Proposal

Add such flags.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.8 (of JVT-E146d37) | MICROSOFT | Editorial |  |  |

Context

Specification of bin count constraint for CABAC.

Question

There seems to be a lack of clarity regarding whether the bin count constraint applies to slice data partitions or only to slices without partitioning.

Proposal

Change:

"When entropy_coding_mode is equal to 1, the number of bins (excluding end_of_slice_flag) resulting from decoding the contents of the slice layer NAL unit shall not exceed 32*NumBytesInNALunit." (in subclause 7.4.2.8)

To:

"When entropy_coding_mode is equal to 1, the number of bins (excluding end_of_slice_flag) resulting from decoding the contents of any NAL unit containing RBSP slice trailing bits shall not exceed 32*NumBytesInNALunit."

This is to avoid the impression that the constraint does not apply to slice data partitions.

# Clause 8

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.1 & 8.2.2 of FCD | MICROSOFT | Editorial |  |  |

Context

NAL unit decoding order specification and parameter set decoding.

Question

The draft content should be reviewed to ensure that the content of these subclauses of the FCD text have been retained.

Proposal

The draft content should be reviewed to ensure that the content of these subclauses of the FCD text have been retained.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.2 & C.1.2.1 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

Use of DiffPicOrderCount.

Question

Problem: Mathematics problems that could be caused by DiffPicOrderCount becoming equal to zero and need to clarify the intended relation between time and picture order count.

Proposal:

Proposal

Change:

"The ordering of DPB output times for the sequence of decoded pictures is required to be the same as the ordering of the picture order counts for the sequence of decoded pictures (subclause 8.2.2)."  (in subclause C.1.2.1)

To:

"When pic_order_cnt_type is equal to 0 or 1, if the picture order count (TopPicOrderCnt or BottomPicOrderCnt) for some decoded field is less than the picture order count of another decoded field, the DPB output time of the decoded field in question shall not be greater than the DPB output time of the other decoded field.  When pic_order_cnt_type is equal to 2, the DPB output times of decoded fields shall be non-decreasing with decoding order."

And:

Specify wherever DiffPicOrderCount appears in the denominator of an equation (e.g., subclause 8.4.1.2.2 luma motion vector prediction in B slices and 8.4.1.4.2.3 temporal direct luma motion vector in B slices) that it shall not be equal to 0.

And:

Specify in subclause 8.2.2 that when pic_order_cnt_type is equal to 2, the output order shall be equal inferred to be the same as the decoding order..

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1 (of JVT-E146d37) | MICROSOFT | Minor technical |  |  |

Context

At levels 3.1 and higher, direct and bi-predictive block sizes smaller than 8x8 are not allowed.  However, there is a method of inferring 8x8 motion from 4x4 motion defined only for temporal direct mode.

Question

This appears to harm the ability to use spatial mode direct prediction (although spatial direct prediction seems to most often be the most effective type of direct prediction and although it is the prediction method most immune to unusual temporal relationships).

Proposal

The ability to infer 8x8 motion from 4x4 motion should apply not only to the temporal direct mode case but also to the spatial direct mode case (in the same fashion).

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.2.2.1 (of JVT-E146d37) | MICROSOFT | Minor technical |  |  |

Context

The two-sample average used in motion compensation does not add 1 before the right shift as was done in prior video standards.

Question

This is inconsistent with the definition of two-sample averaging defined for most video processors.

Proposal

The two-sample average used in motion compensation should add 1 before shifting right for consistency with B-picture reference block averaging and for compatibility with instruction sets that optimize for this style of averaging used in prior video standards.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.2.3.2 (of JVT-E146d37) | MICROSOFT | Minor technical |  |  |

Context

Implicit weighted prediction.

Question

There seems to be a problem in the details of implicit weighted prediction as specified.  The default value of LWD appears to be 7 where 6 would seem to make more sense.  The range of weight values producible by these equations should be examined.  Particular consideration should be given to cases in which both references are in the same picture order count direction.

Proposal

Examine this subclause and fix as necessary.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7 (of JVT-E146d37) | MICROSOFT | Minor technical |  |  |

Context

Deblocking operation with respect to PCM macroblocks.

Question

Deblocking operation with respect to PCM macroblocks must be clearly defined

Proposal

PCM macroblocks should be treated for deblocking purposes as inter macroblocks with QPY= 0, all-zero transform coefficients, and all-zero motion vectors.

# Annex D: SEI

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.1.9 & D.1.11 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

target_frame_num and subseq_frame_num are ue(v) while original_frame_num and frame_num are u(v).  All of these are frame numbers with range 0 to MaxFrameNum.

Question

Is there a good reason for the inconsistency in the way these are represented?

Proposal

We think these should all be u(v) with number of bits determined by log2_max_frame_num_minus4.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.1.10 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

scene_id and second_scene_id are u(8), while snapshot_id and progressive_refinement_id are u(32).

Question

This appears to significantly limit the degree to which an encoder can uniquely label scenes.

Proposal

scene_id and second_scene_id should perhaps be u(32) or ue(v) to allow more unique labeling of scenes.

# Annex E: VUI

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

In the indication of vertical and horizontal motion vector length limits (log2_max_mv_length_vertical and log2_max_mv_length_horizontal) in the sequence parameter set VUI, the vertical parameter appears first in the syntax.

Question

In all other places in the syntax where it makes no technical difference, horizontal parameters are sent before corresponding vertical parameters.

Proposal

Swap the order of these two syntax elements.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

max_bits_per_mb_denom has a default value of 1, which is the smallest expressible denominator (other than no limit) and corresponds to 384 bytes per MB.

Question

The range of values for this parameter (other than a "no limit" expression) does not go up to 400 bytes per MB (the limit imposed in every profile & level), and the default value could be violated by otherwise-conforming bitstreams.

Proposal

The default value should be 0 (no limit expressed) and/or the range of values for the parameter should be increased to encompass the 400 byte limit imposed by all profiles and levels.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

The default value of  max_bytes_per_pic_denom is 4 (at least 4:1 compression) and the interpretation of this parameter for coded fields indicates an additional factor of two in compression relative to that of two fields being coded as a frame.

Question

The range of values for this parameter (other than a "no limit" expression) does not go up to ½ of the maximum picture size supported by the profile & level (the limit imposed in every profile & level), and the default value could be violated by otherwise-conforming bitstreams.  The definition for fields appears to be a mistake.

Proposal

The default value should be 0 (no limit expressed) and/or the range of values for the parameter should be increased.  Also the limit imposed on fields should be consistent with that imposed on frames.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | MICROSOFT | Minor technical |  |  |

Context

Alternative chroma sample locations in sequence VUI are different for frames and fields, and the positions for frames conflict with some of the positions for fields.

Question

This seems to mean that if the encoder chooses to use picture-level adaptive frame/field coding, the indicated chroma sample positions will move.  This appears to conflict with the intent for the sampling positions to be fixed at the sequence level.

Proposal

Alternative chroma sample locations in sequence VUI should be for top fields and bottom fields rather than for frames and fields.  This way the indicated spatial chroma sample locations will not move when the picture coding method changes from a field to a frame or vice-versa, as the positions for frames will be the merger of the positions for their two constituent fields.  Sample locations corresponding to current frame sample locations mode 1, 2, and 3 of Figure E-1 should be supported.  The need for the other locations indicated in Figure E-2 should be reviewed.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | MICROSOFT | Editorial |  |  |

Context

Semantics of fixed_frame_rate_flag.

Question

The semantics of fixed_frame_rate_flag should be clarified with respect to 3:2 pull-down (for which some frames last three field times and some last two field times).

Proposal

Clarify such that fixed_frame_rate_flag indicates that the temporal distance between the output of the last field of one frame and the output first field of the next frame in output order as indicated in picture timing SEI is constant throughout the sequence.

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | MICROSOFT | Editorial |  |  |

Context

Semantics of num_units_in_tick and time_scale.

Question

The interpretation of zero values for these parameters seems confusing.

Proposal

Clarify that external timing information is needed to verify HRD conformance when num_units_in_tick and time_scale are equal to zero.  Consider limiting use of other syntax under these circumstances (e.g., not allowing picture timing SEI messages as they become uninterpretable).

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.2 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

cpb_cnt cannot be zero.

Question

Why bother allowing a value of zero to be sent syntactically but disallow it semantically?  Most such syntax elements are sent by sending the desired value minus 1, so the current form is inconsistent with the rest of the design.

Proposal

Rename cpb_cnt to cpb_cnt_minus1 and adjust semantics accordingly

| Clause | Company | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.3 (of JVT-E146d37) | MICROSOFT | Minor technical/editorial |  |  |

Context

Frame cropping rectangle of picture parameter set.

Question

It seems difficult to clearly determine how the frame cropping rectangle is intended to operate with respect to chroma, because in 4:2:0 video there is only one chroma sample per two luma samples vertically and horizontally.

Proposal

frame_cropping_rect_{left,right,top,bottom} parameters should be in units of two luma samples so that the relationship between chroma and luma is easily preserved through the cropping operation.