| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji,Japan, 5-15 December, 2002 | Document: JVT-F019-r1 Filename: JVT-F019-r1.doc |
| --- | --- |

| Title: | Adaptive Rate Control for JVT |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JVT] |  |  |
| Purpose: | [ Proposal] |  |  |
| Author(s) or Contact(s): | Zhengguo Li, Feng Pan, Keng Pang Lim, Genan Feng, Xiao Lin and Susanto Rahardja Signal Processing Program, Institute for InfoComm Research 21 Heng Mui Keng Terrace Singapore 119613 | Tel: Email: | +65 6874 6874 {ezgli,efpan,kplim,gnfeng,linxiao,rsusanto}@i2r.a-star.edu.sg |
| Source: | [Institute for InfoComm Research] |  |  |

_____________________________

(Begin text of document here: 11-point font is suggested for short documents, 10 for long ones)

# Introduction

In this proposal, we present an adaptive picture layer rate control scheme for JVT by introducing a linear model to predict the mean absolute difference (MAD) of current picture by that of previous picture. The target bit rate for the current picture is computed by adopting a fluid flow traffic model and simple tracking theory according to the removal interval between the previous picture and the current one, the current buffer occupancy, the target buffer level and the available channel bandwidth [6]. The corresponding quantization parameter is computed by using a quadratic rate-distortion (R-D) model [1,2]. The rate distortion optimization (RDO) is then performed for each macroblock (MB) in the current picture by using the quantization parameter. We focus on the variable bit rate (VBR) case while our scheme is also applicable to the constant bit rate (CBR) case. It should be emphasized that the VBR in our proposal means that the available channel bandwidth is time varying. A virtual buffer is used to store the generated bits and to help adjust the coding process according to the dynamics of the channel bandwidth. Since our scheme is directly based on the fluid flow traffic model and the bucket is not underflow   or overflow, our rate control is thus conformed to hypothetical reference decoder (HRD).

- Preliminary Knowledge

Since quantization parameters are involved in both rate control and RDO, there exists a problem when the rate control is implemented: to perform RDO for MBs in the current picture, a quantization parameter should be firstly determined for each MB by using the MAD of current picture and/or the MB [1,2,7,8]. However, the MAD of current picture and/or the MB is only available after performing the RDO. The following linear model (1) is proposed to solve this problem.

Suppose that the MADs of current picture and previous picture with the same type are denoted by MADcurpic and , respectively. The prediction model is then given by

MADcurpic=a1∗MADprepic+a2                                                                 (1)

where  is the actual MAD of previous picture, MADcurpic is the predicted MAD of current picture, a1 and a2are two coefficients of prediction model.

In the following description, the time base uses the num_units_in_tick and time_scale syntax in the Sequence Parameter Set to derive the difference of the removal time between the previous picture and the current picture. Particularly, let tc=num_units_in_tick÷ time_scale be the clock tick associate with the coding process [7].

Let Ngop denote the total number of pictures in a group of picture (GOP), ni,j(i=1,2,⋯,j=1,2,⋯,Ngop) denote the jth picture in the ith GOP, and Bc(ni,j) denote the occupancy of virtual buffer. It can be shown from the fluid flow traffic model that

Bc(ni,j+1)=minmax0,Bc(ni,j)+A(ni,j)−u(ni,j)∗tc∗pre_dec_removal_delay(ni,j+1),BsBc(n1,1)=Bc(t0)=Bs4Bc(ni+1,1)=Bc(ni,Ngop+1)        (2)

where A(ni,j) is the actual bits generated by the jth picture in the ith GOP, u(ni,j) is the available channel bandwidth which can be either VBR or CBR, pre_dec_removal_delay(ni,j+1) is the delay of the removal time for the (j+1)th picture in the ith GOP with respective to that of the jth picture in the ith GOP [7], and Bs is the buffer size and its maximum value is determined based on different level and different profile [7]. Note that the initial level of the virtual buffer is set to Bs/4.

The model is similar to the leaky bucket model in [7] whenF=3Bs4. The above model guarantees that the bitstream is contained in the leaky bucket. Therefore, when the bitstream is input to the virtual buffer with parameters R=u,B=Bs and F=3Bs4, the buffer will not overflow and underflow. In other words, our rate control scheme is conformed to HRD.

- Adaptive Rate Control

Our scheme is composed of two layers: GOP-layer rate control and picture-layer control.

  - GOP-Layer Rate Control

In this layer, we need to compute the total number of bits Tr for each GOP. Same as [5], we assume that the GOP structure is IBBPBBP... P or IPPP…P, with I being an intra-coded picture, P being a forward predicted picture and B being a bi-directional predicted picture. The length of a GOP is usually 15-50 [4].

The total number of bits allocated for the ith GOP is computed as follows:

Tr(1)=u(ni,1)Fr∗Ngop−(Bs4−Bc(ni−1,Ngop+1))                                                     (3)

where Fr is the predefined picture rate. The total number of bits is updated after coding each picture.

It can be shown from equation (3) that the coding results of the latter GOPs depend on those of the former GOPs. To ensure that all GOP has a uniform quality, each GOP should use its own budget. In other words, the buffer occupancy should be kept at Bs4 after coding each GOP.

Since the channel bandwidth may vary at any time, Tr is updated picture by picture as follows:

Tr(ni,j)=Tr(ni,j−1)+u(ni,j)−u(ni,j−1)Fr(Ngop−j)−A(ni,j−1)                                     (4)

  - Picture Layer Rate Control

In our scheme, the first I picture is coded by a predefined quantization parameter InitialQp, the first P picture and the first B picture are coded by (InitialQp+2) and (1.25*InitialQp), respectively. InitialQp is predefined based on the available channel bandwidth. The first P picture in other GOP is coded by (PtotalQp/Np-1) and the other I pictures are coded by (PtotalQp/Np-3) where Np is the total number of P pictures in a GOP and PtotalQp is the sum of quantization parameters for all P pictures in the previous GOP.

The picture layer rate control scheme consists of two stages: pre-encoding and post-encoding.

3.2.1 Pre-Encoding Stage:

The objective of this stage is to compute quantization parameter for each picture. This is achieved via the following two steps:

Step 1 Determine a target bit rate for each picture. Step 1 is composed of the following two sub-steps.

Step 1.1 Macroscopic control (budget allocation among pictures).

The bit allocation is implemented by predefining a target buffer level for each P picture.

After coding the first P picture in the ith GOP, we reset the initial value of target buffer level as

Target(2)=Bc(ni,3)                                                                            (5)

where Bc(ni,3) is the actual buffer occupancy after coding the first P picture in the ith GOP. The target buffer level for the subsequent picture is determined according to the following two cases:

Case 1. The jth picture in the ith GOP is a P picture. Its target buffer level is predefined and determined using the following equation:

Target(j+1)=Targetj+Wp(ni,j)Tr(ni,j)Wp(ni,j)Np,r(ni,j)+WB(ni,j)Nb,r(ni,j)−u(ni,j)Fr             (6)

where Wp(ni,j) is the average complexity weight of P pictures, Np,r(ni,j) is the number of remaining P pictures in the current GOP after coding the jth picture, WB(ni,j) is the average complexity weight of B pictures, Nb,r(ni,j) is the number of remaining B pictures after coding the jth picture,  and Target(j+1) is the target buffer level. Wp and Wb are computed by Wp(ni,j)=Wp(ni,j)8+7∗Wp(ni,j−1)8Wb(ni,j)=Wb(ni,j)8+7∗Wb(ni,j−1)8Wp(ni,j)=Sp(ni,j)Qp(ni,j)Wb(ni,j)=Sb(ni,j)Qb(ni,j)1.3636                                                            (7)

Sp and Sb are the number of bits generated by encoding the corresponding picture, and Qp and Qb are the quantization parameters.

Case 2. The jth picture is a B picture. Its target buffer level is predefined and determined using the following equation:

Target(j+1)=Targetj+Wb(ni,j)Tr(ni,j)Wp(ni,j)Np,r(ni,j)+WB(ni,j)Nb,r(ni,j)−u(ni,j)Fr              (8)

It can be easily shown that Target(Ngop)=Bs4 is about Bs4. Thus, if the actual buffer fullness is exactly the same as the predefined target buffer level, it means that each GOP uses its own budget. However, since the rate-distortion (R-D) model is not accurate [1,2], there usually exists a difference between the actual buffer fullness and the target buffer level. We therefore need to compute a target bit rate for each picture to attenuate this difference. This is achieved by the following microscopic control.

Step 1.2 Microscopic control (target bit rate computation).

Using linear tracking theory [6], the target bits allocated for the jth picture in the ith GOP is determined based on the target buffer level, the difference of the removal time between the previous picture and the current picture, the available channel bandwidth and the actual buffer occupancy as follows:

fni,j=u(ni,j)(ti,j+1−ti,j)+γ(Target(j)−Bc(ni,j))                        (9)

where  is a constant and its typical value is 0.25 for both P pictures and B pictures. Further adjustment by a weighted smoothing of the target bit rate is given according to the following two cases:

Case 1. The jth picture is a P picture.

f(ni,j)=β∗Wp(ni,j−1)Tr(ni,j)Wp(ni,j−1)Np,r(j−1)+Wb(ni,j−1)Nb,r(j−1)+(1−β)∗f(ni,j)                   (10)

where β is a constant and its typical value is 0.95 for both P pictures and B pictures.

Case 2. The jth picture is a B picture.

f(ni,j)=β∗Wb(ni,j−1)Tr(ni,j)Wp(ni,j−1)Np,r(j−1)+Wb(ni,j−1)Nb,r(j−1)+(1−β)∗f(ni,j)                    (11)

Step 2 Compute the quantization parameter and perform RDO.

The MAD of current picture is predicted by using model (1) and the MAD of previous picture.

The quantization parameter corresponding to the target bit rate is then computed by using the quadratic model provided in [1,2].  The details on this can be found in [1,2,5], it is thus not elaborated in this section. The final quantization parameter is adjusted according to the following two cases:

The final Qpc for P pictures is adjusted as follows:

Qpc=minQpp+DQuant,maxQpp−DQuant,Qpc,1,51                                     (12)

where DQuant is 2 if the number of successive B pictures is less than 2, and it is 3 otherwise.

The final quantization parameter Qbc for B pictures is adjusted as follows:

Qbc=minQbp+2,maxQbp−2,Qbc,51Qbc=minQpp,maxQpp−2,Qbc,1                                                     (13)

The quantization parameter is then used to perform RDO for each MB in the current picture by using the method provided in [7,8]. The coding mode is selected by minimizing the following performance index:

D(s,c,MODE|QP)+λmodeR(s,c,MODE|QP)                                               (14)

with

λmode=0.85×2QP3,             for I, P pictures;λmode=4×0.85×2QP3,        for B pictures.                                                  (15)

If the picture is P or B and the SAD is adopted as the criterion, the lambda in motion estimation is give by

λmotion=λmode                                                                             (16)

Post-encoding Stage:

There are two major tasks in this stage:  update the model parameters, and determine the number of skipped pictures.

After encoding a picture, the parameters of model (1), as well as that of the quadratic R-D model should be updated. A method similar to that in [1,2] is used where the window size is computed by using the method provided in [5] instead of that in [1,2].

After encoding a picture, the total number of actual generated bits A(ni,j)is added to the current buffer occupancy. To ensure that the updated buffer occupancy is not too high, the picture skipping parameter Npost is set to zero and increased until the following buffer condition is satisfied [1]:

Bc(ni,j+Npost)<Bs0.8                                                                        (17)

where the buffer fullness is updated as follows:

Bc(ni,j+l+1)=Bc(ni,j+l)−u(ni,j+l)∗tc∗Pre_enc_removal_delay(ni,j+l); 1≤l<Npost                (18)

- Experimental Results

We test our rate control scheme in both the VBR case and the CBR case, in both the case that RDO is on and the case that RDO is off.

Group 1 Variable bit rate (VBR)

Two groups of experiments are carried out. Test conditions are given in Table 1. The other experimental conditions are given as follows:

Testing platform: JM5.0;

Test sequences: news, foreman, children, weather, container, coastguard, and carphone;

Format: QCIF(4:2:0);

Input picture rate is 30 pictures/s, output picture rate is 10pictures/s;

100 pictures are used for each sequence;

QP for the first I picture is 17;

Bandwidth: 128000 bits/s (1≤j≤60), 192000 bits/s    (61≤j≤100).

Table 1 Test Conditions

| MV resolution | 1/4 pel |
| --- | --- |
| Hadamard | ON |
| RD optimization | Off |
| Search Range | 16 (QCIF) |
| Restrict Search Range | 2 |
| Reference Frames | 1 |
| Symbol Mode | CABAC |
| ABT Mode | OFF |
| GOP structure | IPP |

The experimental results for the sequences with QCIF size are given in the following table:

Table 2 Experimental Results for VBR and QCIF

| sequence | foreman | News | Container | coastguard | Weather | children | carphone |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average PSNR | 40.08dB | 45.03dB | 44.08dB | 35.56dB | 45.37dB | 37.59dB | 41.17dB |
| Resulted Bandwidth | 153.63kb/s | 153.52kb/s | 153.94kb/s | 153.47kb/s | 153.70dB | 153.71kb/s | 153.71kb/s |

Group 2 Constant bit rate (CBR)

The test conditions are given in Table 3 and the experimental conditions are given as follows:

Testing platform: JM5.0;

Test sequences: news, foreman, children, weather, container, coastguard, and carphone;

Format: QCIF(4:2:0);

Input picture rate is 30 pictures/s, output picture rate is 10pictures/s;

100 pictures are used for each sequence;

QP for the first I picture is 20;

Bandwidth: 64kbs/s

Table 4 Test Conditions

| MV resolution | 1/4 pel |
| --- | --- |
| Hadamard | ON |
| RD optimization | ON |
| Search Range | 16(QCIF), 32(CIF) |
| Restrict Search Range | 2 |
| Reference Frames | 5(QCIF),1(CIF) |
| Symbol Mode | CABAC |
| ABT Mode | OFF |
| GOP structure | IBBPBB |

The experimental results for the sequences with QCIF size are given in the following table:

Table 4 Experimental Results for 64kb/s, IBBPBB and RDO=ON

| sequence | foreman | News | Container | coastguard | weather | children | carphone |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average PSNR | 35.82dB | 40.41dB | 41.36dB | 32.00dB | 38.88dB | 31.40dB | 37.29dB |
| Resulted Bandwidth | 63.54kb/s | 64.19kb/s | 64.00kb/s | 64.02kb/s | 65.36kb/s | 63.72kb/s | 65.36kb/s |

Testing platform: JM5.0;

Test sequences: hall, goldfish, bike, mobile and paris;

Format: CIF(4:2:0);

Input picture rate is 30 pictures/s, output picture rate is 10pictures/s;

100 pictures are used for each sequence;

QP for the first I picture is 30;

Bandwidth: 128kb/s;

The experimental results for the sequences with CIF size are given in the following table:

Table 5 Experimental Results for sequence with CIF size

| Sequence | mobile | paris | bike | hall | goldfish |
| --- | --- | --- | --- | --- | --- |
| Average PSNR | 27.09dB | 32.66dB | 31.45dB | 38.43dB | 32.53dB |
| Resulted Bandwidth | 129.68kb/s | 128.39kb/s | 129.04kb/s | 128.31kb/s | 128.73kb/s |

- Discussion About Our Scheme

A picture layer rate control is provided in this proposal by introducing a linear prediction model. There are generally two performance indices for the rate control, the PSNR and the fluctuation of generated bits. In generally, a picture layer rate control could achieve a high PSNR and big bit fluctuation, while MB layer rate control could have small bit fluctuation, but with slight loss in PSNR. A good trade-off could be achieved by introducing the concept of basic unit.

Definition 1: Suppose that a picture is composed of Nmbpic MBs. A basic unit is defined to be a group of MBs that is composed of Nmbunit MBs where Nmbunit is a fraction of Nmbpic.

For example, a basic unit can be an MB, or a slice, or a field, or a picture, and so on. In this proposal, we consider the case that a basic unit is a picture.

If the basic unit is not selected as a picture, an additional basic unit layer rate control should be added to our scheme.

To reduce the number of bits used for Dquant , the difference among the quantization parameters of macroblocks, the syntax of JVT could be modified by inserting a flag in the beginning of the bit stream to indicate the exact number of MBs in the basic unit. We then only need to code the difference among the quantization parameters of basic unit instead of those of macroblocks.

- Conclusion.

An adaptive rate control scheme is provided for JVT by introducing a linear model to predict the MAD of current picture by that of previous one. The linear model is used to solve the problem arising from the coexistence of RDO and rate control algorithm in JVT. Our scheme is directly based on a fluid flow traffic model and is thus conformed to HRD. Moreover, it is applicable to both VBR case and CBR case. It is applicable to different choice of basic unit.

- References.

- H.J.Lee and T.H.Chiang and Y.Q.Zhang. Scalable Rate Control for MPEG-4 Video. IEEE Trans. Circuit Syst. Video Technology, 10: 878-894, 2000.

- Vetro, H.Sun and Y.Wang. MPEG-4 rate control for multiple video objects. IEEE Trans. Circuit Syst. Video Technology, 9: 186-199, 1999.

- J. Ribas-Corbera and S.Lei. Rate control in DCT video coding for low-delay communications. IEEE Trans. Circuit Syst. Video Technology, 9: 172-185, 1999.

- MPEG-2 Test Model 5, Doc. ISO/IEC JTC1/SC29 WG11/93-400. Apr.1993.

- Z. G. Li, Lin Xiao, C. Zhu and Pan Feng. A Novel Rate Control Scheme for Video Over the Internet. In Proceedings ICASSP 2002, Florida, USA, 2065--2068, May 13-17, 2002.

- Chi-Tsong Chen. Linear system theory and design. Rinehart and Winston, New York, 1984.

- Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG Document JVT-B118R2, 2002-03-25.

- Thomas Wiegand and Bernd Girod. Parameter Selection in Lagrangian Hybrid Video Coder Control, ICIP 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institute for InfoComm Research |  |  |
| Mailing address | 21 Heng Mui Keng Terrace Singapore 119613 |  |  |
| Country | Singapore |  |  |
| Contact person | Zhengguo LI |  |  |
| Telephone |  |  |  |
| Fax | +65 6774 6874 |  |  |
| Email | ezgli@lit.a-star.edu.sg |  |  |
| Place and date of submission | Singpaore, Dec. 2, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: