| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F029 Filename: JVT-F029.doc |
| --- | --- |

| Title: | Level and Profile Limits for the Maximum Number of Slices per Picture, Maximum Bitrates and Maximum Number of Slice Groups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng Zhong 3151 Zanker Road San Jose, CA 95134, USA Alexander (Sandy) MacInnis 3151 Zanker Road San Jose, CA 95134 USA | Tel: Email: Tel: Email: | 1-408-501-3885 szhong@broadcom.com 1-408-501-3801 macInnis@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

This document proposes a couple of level limits, one for the maximum number of slices per picture for levels 3 and above, and one for the maximum bitrate for levels 3.1, 3.2 and 4, and a profile limit for the maximum number of slice groups in the Baseline Profile.

Level Limit for the Maximum Number of Slices per Picture

The AVC standard allows a slice contains only one macroblock in the worst case, which results in many slices in a picture. Too many slices in a picture would decrease the coding efficiency because some tools such as motion vector prediction are not allowed between slice boundaries and overhead from slice headers would be significant. Too many slices in a picture would increase the decoding complexity as well because, for example, decoding the slice headers, resetting the contexts at the beginning of the slices and calculating the Z-factor for temporal direct mode MV calculation increase the load. This would be an issue for demanding applications such as HD decoding. Therefore it is suggested the following constraint be adopted.

For levels 3 and above, the maximum number of slices per picture should be no more than 3 times as many as the number of macroblock-rows in a picture.

Level Limit for the Maximum bitrate

It is proposed that the maximum video bitrate for level 4 be kept as 20 Mbps and that for level 3.1 or 3.2 be no more than 20 Mbps and preferably smaller than 20 Mbps proportionally.

The reason is that high bitrates would be a challenge for real time decoding of compliant worst case CABAC bitstreams, given the current constraint of the bin to bit ratio being no more than 4 averagely in a slice. It would be very difficult and expensive to design compliant HD decoders. In addition, 20 Mbps should be adequate for HD TV broadcasting and HD DVD applications. Other levels or profiles should address applications that desire a higher bitrate such as VTR.

Baseline Profile Limit for Maximum Number of Slice Groups

It is proposed that the maximum number of slice groups in the baseline profile be 4 for FMO.

By the four-color map theorem, 4 slice groups can be designed such that, for any macroblock in the picture, any of its neighboring macroblocks can be from a slice group that is different from the current slice group.  Smaller number of slice groups gives the benefit of less cost needed for context switch for FMO.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Broadcom Corporation |  |  |
| Mailing address | 3151 Zanker Road San Jose, CA 95134 |  |  |
| Country | USA |  |  |
| Contact person | Alexander (Sandy) MacInnis |  |  |
| Telephone | (+1) 408 501 3801 |  |  |
| Fax | (+1) 408 501 3880 |  |  |
| Email | macinnis@broadcom.com |  |  |
| Place and date of submission | San Jose, CA; 03October2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | AVC/JVT |  |
| Title |  | Level and Profile Limits for the Maximum Number of Slices per Picture, Maximum Bitrates and Maximum Number of Slice Groups |  |
| Contribution number |  | JVT-F029 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: