| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F030 Filename: JVT-F030.doc |
| --- | --- |

| Title: | Minimum Luma Block Size Level Limit for Direct mode Macroblocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng Zhong 3151 Zanker Road San Jose, CA 95134, USA Alexander (Sandy) MacInnis 3151 Zanker Road San Jose, CA 95134 USA | Tel: Email: Tel: Email: | 1-408-501-3885 szhong@broadcom.com 1-408-501-3801 macInnis@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

#### Introduction

The minimum luma block size for the direct mode macroblocks is restricted to be 8x8 in AVC.  See Annex A.3.1 of the JVT DIS document, JVT-E146d37ncm. This shall apply to all the direct mode macroblocks, including both the spatial and temporal direct modes.

However, DIS doesn’t specify the semantics of this limit for the spatial direct mode macroblocks motion vector derivation process. Furthermore, it doesn’t explicitly associate the limit with the semantics of the temporal direct mode macroblocks motion vector derivation process when direct_temporal_constrained_flag is ‘1’.

This document suggests

- The change of the terminology of the picture parameter set syntax element direct_temporal_constrained_flag to direct_constrained_flag. And the value of the flag decides the motion vector derivation process for both the spatial and temporal direct modes.

- The semantics for motion vector derivation for spatial direct mode macroblocks.

- The explicit statement in Annex A that direct_constrained_flag must be set to ‘1’ for levels 3.1 and above.

#### Spatial mode motion vector derivation

The following paragraph is suggested to replace the corresponding (6th) paragraph in 8.4.1.4.2.2 of DIS. And figure 8-10 below is to replace figure 8-10 in DIS.

If the reference index exists for either list, the decision of the associated motion vector values for that list, for each 4x4 block if direct_constrained_flag is ‘0’, or for each 8x8 block if direct_constrained_flag is ‘1’, of the current macroblock depends on the coded parameters, of the co-located 4x4 blocks if direct_constrained_flag is ‘0’, or of the corresponding corner 4x4 blocks of the co-located 8x8 blocks (as shown in Figure 8-10) if direct_constrained_flag is ‘1’, in the first picture in list 1, if both pictures have been coded using the same picture type (frame or field). If the current picture is coded in field mode and the list 1 picture in frame mode, only the same parity lines and their associated motion parameters are considered. Otherwise, if current picture is in frame mode and the first list 1 picture in field mode, then the corresponding frame mode picture containing the first list 1 picture is considered, if such exists. If the reference index of either list for the current block is zero, the associated list 1 picture is a short-term picture, and if all lines of the co-located (when direct_constrained_flag is ‘0’) or the corner (when direct_constrained_flag is ‘1’) 4x4 block exist and were predicted using list 0 prediction, reference picture index 0 and motion vector components in the range of -1 to 1 in sub-pixel units, inclusive, then the motion vector for this 4x4 block (when direct_constrained_flag is ‘0’) or 8x8 block (when direct_constrained_flag  is ‘1’) is set to (0, 0). Otherwise, the associated candidate motion vector for each list shall be obtained, using the 16x16 block motion vector prediction, as described in subclause Error: Reference source not found by using the reference picture index of each list.

Figure 8-1 – Double-dashed border to indicate each ‘corner’ 4x4 block of each co-located 8x8 submacroblock, from which each direct-mode 8x8 sub-macroblock derives its motion vectors if direct_constrained_flag is set to 1.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Broadcom Corporation |  |  |
| Mailing address | 3151 Zanker Road San Jose, CA 95134 |  |  |
| Country | USA |  |  |
| Contact person | Alexander (Sandy) MacInnis |  |  |
| Telephone | (+1) 408 501 3801 |  |  |
| Fax | (+1) 408 501 3880 |  |  |
| Email | macinnis@broadcom.com |  |  |
| Place and date of submission | San Jose, CA; 03October2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | AVC/JVT |  |
| Title |  | # Minimum Luma Block Size Level Limit for Direct mode Macroblocks |  |
| Contribution number |  | JVT-F030 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: