| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F035 Filename: JVT-F035.doc |
| --- | --- |

| Title: | SD Complexity Limitations Experiments |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Informational |  |  |
| Author(s) or Contact(s): | Jill Boyce and Cecile Zhang 2 Independence Way Princeton, NJ 08540 | Tel: Email: | +1 609 734 9859 jill.boyce@thomson.net |
| Source: | Thomson |  |  |

_____________________________

Summary: Experimental data is provided to support a complexity limitation of 32 motion vectors per 2 consecutive macroblocks for Level 3.  The sequences tested were those thought to be most impacted by such a restriction.  Minimal impact on BD-bitrate and BD-PSNR was observed.

Experiments:

Sequences were encoded in an unconstrained manner, and with 4x4 blocks disabled for B pictures, and compared using BD-bitrate and BD-PSNR.  Disabling 4x4 motion blocks for B pictures (or bipredictive macroblocks) is the simplest  and most restrictive way for an encoder to observe a limitation of 32 motion vectors per two consecutive macroblocks.   For the unconstrained encoder, counts of the number of motion vectors naturally occurring in two consecutive macroblocks were also gathered.

The sequences tested were Rugby, Canoa, and Fireworks.  Fireworks was downsampled 2:1, and then the center section selected to form a 720x480 sequence.

Five reference frames, a search range of +/- 32, MBAFF, UVLC, spatial direct mode were used, 2 B frames (PBB), at QP=28, 32, 36, and 40.

The delta bitrates and delta PSNRs between the constrained and unconstrained encoder are in the Table 1 below.

| sequences | Delta Bitrate | Delta PSNRY |
| --- | --- | --- |
| Fireworks | -0.12% | 0.006 dB |
| Rugby | -0.06% | 0.003 dB |
| Canoa | -0.05% | 0.002 dB |

Table 1. BD-Bitrate and BD-PSNR of Constrained vs. Unconstrained Encoder

When spatial direct mode was used, an unconstrained encoder never exceeded the proposed limit of 32 motion vectors per 2 consecutive macroblocks, for the tested sequences.

| sequences | Count | Percentage of MBs |
| --- | --- | --- |
| Fireworks (QP = 24) | 0 | 0% |
| Rugby | 0 | 0% |
| Canoa | 0 | 0% |

Table 2. Frequency of exceeding 32 MVs per 2 MBs for unconstrained encoder

Table 3 lists the 99th and 100th percentiles of the number of motion vectors used in two consecutive macroblocks for an unconstrained encoder.

| sequences | 99th percentile | 100th percentile |
| --- | --- | --- |
| Fireworks (QP = 24) | 14 | 25 |
| Rugby | 12 | 26 |
| Canoa | 15 | 26 |

Table 3. Percentiles of number of motion vectors used in two consecutive macroblocks for unconstrained encoder

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson |  |  |
| Mailing address | 2 Independence Way Princeton, NJ 08540 |  |  |
| Country | USA |  |  |
| Contact person | Jill Boyce |  |  |
| Telephone | +1 609 734 9859 |  |  |
| Fax | +1 609 734 7299 |  |  |
| Email | jill.boyce@thomson.net |  |  |
| Place and date of submission | Awaji, Japan, Dec 5, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | SD Complexity Limitations Experiments |  |
| Contribution number |  | JVT-F035 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: