| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F037 Filename: JVT-F037.doc |
| --- | --- |

| Title: | Verification Results on JVT-F033 (Simplified Block-size Adaptive MC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shun-ichi Sekiguchi, Yoshihisa Yamada 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 shun@isl.melco.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Summary

The complexity reduction technique that will be proposed in JVT-F033 [1] has been verified based on our independent implementation into JM5.0. JVT-F033 proposes a simplification of JVT-D110 [2], which provides a solution for the complexity reduction in terms of memory access following the “shorter MC filter” approach. A negative comment to this approach raised in Geneva meeting was to force the codec to use different MC filters that may complicate encoding/decoding process, however, JVT-F033 does not use any different MC filters than 6-tap filtering, but just to limit the accuracy of vertical motion vector component to integer-pel level at smaller MC blocks, where no MC filtering process is required. It has been confirmed that the performance loss by introducing JVT-F033 is negligible under the common test conditions for coding efficiency test (average less than 1% with maximum 2%).

# Implementation

The MC technique of JVT-F033 is just a simplification of JVT-D110 by applying the following modifications:

- Always use 1/4-pel accuracy MC with 6-tap filtering for horizontal motion vector component regardless of MC block size. No motion vector scaling at the entropy coding process is used for horizontal direction. JVT-D110 employed half-pel MC that requires 2-tap (different) MC filter.

- Integer-pel accuracy MC is applied to the vertical vector component of smaller MC blocks than 8x8 (not including 8x8 block). Apparently, no MC interpolation process is required for vertical direction in these MC blocks. Motion vector scaling at the entropy coding process is performed to reduce the bit budget for vertical motion vector component. (just two 2-bit shift operations at both encoder and decoder per each motion vector) JVT-D110 employed half-pel MC that requires 2-tap (additional) MC filter.

For more implementation details and draft normative text, please refer to the document JVT-F033 [1].

# Verification Results

Table 1 shows the test conditions for our experiment. All QCIF/CIF sequences are the common test sequences that have been used for coding efficiency tests. The first version of this contribution contains QCIF/CIF results only, however, HHR test results may be provided at the next revision. HHR sequences will be encoded with MB-AFF (option 3 in the encoder configuration file).

Table 1: Test Conditions

| Reference Software Version | JM 5.0 |
| --- | --- |
| Sequence | QCIF: Foreman, News, Container, Silent Voice CIF: Paris, Mobile, Tempete HHR: Canoa, Bus (on going) |
| GOP Structure | IPPP… IBBP…(all sequences are encoded with full frame rate) |
| QP | QPI=QPP= QPB= 24,28,32,36 |
| Entropy Coding | UVLC / CAVLC |
| RD Optimization | ON |
| Hadamard | YES |
| Number of Reference Frames | 3 for QCIF/CIF, 2 for HHR |
| MV Search Range | 32x32 |
| Restricted Search Range | 2 |
| Direct Mode | Spatial direct |
| Interlaced Coding (HHR only) | MB-AFF |

Table 2 and 3 illustrate the objective results in the form of BD value. In general, it can be said that the modified encoding process using JVT-F033 provides quite similar coding efficiency to the anchor (JM5.0). Average bitrate loss is less than 1% even in the case of IBBP, which can be regarded as negligible loss.

Table 2: BD-values for IPPP… coding case

| Sequence | PSNR[dB] | BRS[%] |
| --- | --- | --- |
| foreman | -0.02 | 0.35 |
| container | -0.01 | 0.23 |
| news | -0.04 | 0.63 |
| silent | 0.01 | -0.25 |
| paris | -0.04 | 0.68 |
| tempete | -0.04 | 0.94 |
| mobile | -0.06 | 1.28 |
| average | -0.03 | 0.55 |

Table 3: BD-values for IBBP… coding case

| Sequence | PSNR[dB] | BRS[%] |
| --- | --- | --- |
| foreman | -0.02 | 0.46 |
| container | -0.01 | 0.09 |
| news | -0.04 | 0.60 |
| silent | 0.00 | -0.01 |
| paris | -0.06 | 1.16 |
| tempete | -0.08 | 2.07 |
| mobile | -0.06 | 1.41 |
| average | -0.04 | 0.83 |

# Conclusion

This contribution provides our verification results of a simplified technique of JVT-D110 that will be proposed in JVT-F033 at the Awaji meeting. Since the number of pixels required for horizontal MC filtering is not a critical issue for today’s DRAM access interface, it can be said that JVT-F033 is promising technique that can reduce memory bandwidth to almost the same level as JVT-D110 while maintaining reasonable coding performance.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Mitsubishi Electric Corporation |  |  |
| Mailing address | 5-1-1, Ofuna, Kamakura |  |  |
| Country | JAPAN |  |  |
| Contact person | Yoshiaki Kato |  |  |
| Telephone | +81-467-41-2463 |  |  |
| Fax | +81-467-41-2486 |  |  |
| Email | kato@isl.melco.co.jp |  |  |
| Place and date of submission | Awaji/Japan, Dec.5th, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Verification Results on JVT-F033 (Simplified Block-size Adaptive MC) |  |
| Contribution number |  | JVT-F037 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: