| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F039 Filename: JVT-F039r1.doc |
| --- | --- |

| Title: | Final CABAC Cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Detlev Marpe, Heiko Schwarz, Gabi Blättermann, and Thomas Wiegand Image Processing Department Fraunhofer-Institute HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002 619/206/617 marpe@hhi.de, hschwarz@hhi.de, , blaetter@hhi.de, wiegand@hhi.de |
| Source: | Fraunhofer-Institute HHI |  |  |

_____________________________

# Summary

In this document, we first propose some changes for the harmonization of context modeling for transform coefficients and field-based alternate scanning mode. Secondly, we provide a cleanup of the initialization process for context models together with a new set of initialization parameters. In addition, a simplification of the current context model for coding of the reference frame index is proposed for decoupling the symbol parsing process and the decoding process. Finally, in order to limit the worst-case decoding complexity we propose to specify for all Levels a maximum number of bins per second by two times the maximum number of bits per second as specified for the maximum video bit-rate.

# Additional Context Models for Coding of Transform Coefficients in Field-based Coding Mode

## Background

At the Geneva meeting, the alternate scan for field-based coding both in picture and MB adaptive frame/field (AFF) coding mode has been adopted [2]. However, since context modeling of transform coefficients in CABAC depends on the scanning position, this technical change implies the need for revisiting the context models for field-based coding of transform coefficients as specified in the current draft [1]. In the case of MB AFF, where both scans can be used within the same slice, transform coefficients related to different frequencies in a transform block may be coded using the same context model. In addition, the initialization tables for transform coefficient related data originally were designed for the zig-zag scan only, and thus may no longer be accurate, when applied to scanning processes using the alternate scan. As a consequence, the performance of CABAC may drop for field-based coding. To avoid this loss in performance, we propose to add some separate context models as well as new initialization tables for field-based coding. The specification of the proposed new tables will be discussed in Section 3. In Section 2.2, we will present the proposed new context models for the alternate scan.

In order to keep the number of additional context models reasonably small, we re-examined only those syntax elements that do explicitly depend on the scanning position. These are the significant_coeff_flag and the last_significant_coeff_flag. For encoding these syntax elements, up to 15 contexts (depending on the context category CtxCat) are used for coding of each of both flags. For each category CtxCat, we use MaxNumCoeff-1 different contexts, which results in a total number of 2x61=122 context models for coding of both significant_coeff_flag and last_significant_coeff_flag.

## Proposed Changes

We propose the usage of new context identifiers CtxSigCoeffFld, CtxSigCoeffFrm, CtxLastCoeffFrm and CtxLastCoeffFld as a substitute for the context identifiers CtxSigCoeff and CtxLastCoeff, as specified in the current draft [1]. Each of these context identifiers is related to a separate set of context models uniquely determined by a range of CtxIdx. We propose the assignment of the range of context indices to context identifiers as specified in Table 1. The assignment process for CtxIdxInc remains the same as specified in subclause 9.3.3.1.3 of the current draft [1].

Table 1: Assignment of range of CtxIdx to context identifiers

| Context identifier | Range of CtxIdx |
| --- | --- |
| CtxSigCoeffFrm | 81 – 141 |
| CtxLastCoeffFrm | 142– 202 |
| CtxSigCoeffFld | 253 – 313 |
| CtxLastCoeffFld | 314 – 374 |

The selection process for the context identifier related to significant_coeff_flag and last_significant_coeff_flag is specified as follows:

If mb_adaptive_frame_field_flag is equal to 1 and the field_pic_flag is equal to 0, the choice of the context identifier depends on the value of mb_field_decoding_flag in the following way; if mb_field_decoding_flag is equal to 1, CtxSigCoeffFld and CtxLastCoeffFld are chosen for coding of significant_coeff_flag and last_significant_coeff_flag, respectively, if mb_field_decoding_flag is equal to 0, CtxSigCoeffFrm and CtxLastCoeffFrm are chosen. If the the field_pic_flag is equal to 1, CtxSigCoeffFld and CtxLastCoeffFld are chosen for coding of significant_coeff_flag and last_significant_coeff_flag, respectively. In all other cases the context identifiers CtxSigCoeffFrm and CtxLastCoeffFrm are chosen. To summarize the selection process, CtxSigCoeffFld and CtxLastCoeffFld are invoked if and only if the alternate scan is applied.

# Cleanup of Initialization Process for Context Models

In this section, we describe some cleanup changes for the initialization process. First, we propose to change the currently specified rule [1] for computing the initial context state and its MPS value given the quantization parameter QPY and the pair of table entries {m, n}. The intention of this change is to get rid of the redundant offset of -12 applied to QPY, and, more importantly, to allow a larger range of pre-states in the interval [1,126] for a more accurate determination of initial states. Next, we propose a complete new set of initialization tables for all context models and for all values of the indicator cabac_init_idc. Finally, we provide some missing initialization parameters: for B-slices separate initial values for CtxMbSkip are given and for CtxMbField the missing set of initial values for I-slices is provided. In addition, the modified slice header syntax table containing cabac_init_idc is provided. All detailed information related to the proposed changes of this section can be found in the Appendix B.

# Simplified Context Modeling for Coding of Reference Frame Index

In the current specification, coding of a given value of the reference frame index ref_idx_l0 or ref_idx_l1 in a current block C implies the evaluation of values of the reference frame index related to neighboring blocks of C as specified in subclause 9.3.3.1.1 of [1]. In B-slice decoding, however, it may happen that one of the neighboring blocks in the context template is coded in Direct mode, in which case the corresponding reference frame index is derived from the co-located blocks. Thus, according to the current specification, the decoding process has to be involved to derive the proper values of the reference index parameter of a neighboring block coded in Direct mode before the parsing process can be continued.

To decouple the decoding process from the symbol parsing process, we propose to simply use a default value of 0 for the reference frame index of a neighboring (macro)block in B_Direct_16x16 or B_Direct_8x8 mode in the context assignment process only.

# Proposed Limits on Bins per Second for all Levels

## Background

The currently specified process for limiting worst-case decoding complexity relies on a stuffing mechanism for adjusting the bin-to-bit ratio per slice in such a way that an upper bound of 4 is never exceeded [1]. This method has some serious drawbacks.

First problem is given by the fact that the stuffing mechanism may be invoked in cases where it is not really needed. For instance, in low bit-rate coding scenarios of slow-motion content, it typically often happens that B-slices are coded with much higher bin-to-bit ratios than the upper limit of 4 would permit. In some of these cases a quite significant loss in coding efficiency of up to 6% can be observed as a consequence of introducing stuffing bits, although the related video bit-rates are far below the corresponding Level limits meaning that the resulting complexity impact should be negligible [5].

Another problem may arise in cases where the source is non-stationary with respect to its bin-to-bit ratio and slices of different bin-to-bit ratios are combined within one picture. Here, for some slices the bin-to-bit bound may be exceeded, while other slices may show an uncritical behavior. Consider, for example, a case where the overall bin-to-bit ratio on a picture level may be compliant, while the usage of appropriately sized slices may introduce some stuffing overhead. This would result in the rather awkward behavior that stuffing bits would be written in case of using more than one slice per picture while in the case of one picture in a single slice no stuffing would be needed.

The third and main problem of the current specification is that it does not give a sufficiently tight bound on the maximum number of bins at the highest possible bit-rates. We have done a set of experiments for evaluating the bin-to-bit ratio at the maximum video bit-rates specified for each Level. A set of 3 – 5 sequences has been tested for each Level and the bin-to-bit ratio has been measured over a sliding time window of one second for coding of whole sequences. As a result of these experiments, we observed bin-to-bit ratios in the range of 1.15 – 1.41 at the maximum video bit-rate. More detailed results can be found in JVT-F039.xls.

## Proposed Limits

Given our observed experimental data, we would propose to specify a limit on the number of bins per second as follows:

The maximum number of bins per second shall be equal to two times the maximum number of bits per second for a given Level. At the same time, we propose to remove the current slice-based bound on the bin-to-bit ratio and its related stuffing mechanism. The explicit values of our proposed limits and the necessary modifications to the Level specification are given in Appendix A.

If desirable, an additional bound on the maximum number of bins per macroblock can be specified as proposed in JVT-E054 [6]. However, great care should be taken in the specification of such a limit for achieving a good compromise between complexity and coding efficiency.

# Experimental Results

For our experiments, the proposed changes related to Sections 2 – 4 have been integrated into the current test model, software version 5.0f. Note that this version already includes all CABAC related Geneva adoptions with the exception of JVT-E154. The detailed results of our experiments can be found in the Excel-document JVT-F039.xls.

In our first set of experiments the official test set of QCIF- and CIF-sequences has been evaluated. Coding parameters were chosen according to the common test conditions. For these tests, IBBP frame pattern has been used.

In a second experiment, the performance of the proposed CABAC changes were analyzed for a set of interlaced sequences including the set specified in VCEG-N85 [3]. Due to lack of time, only CIF-sized sequences cropped out of the center of the original sequences have been used for this experiment. For these tests the MB AFF interlaced coding option has been enabled and IBBP frame pattern has been used.

Table 2: Average bit rate savings in % for the QP-range = 20, 24, 28, 32, 36 using the common test set (QCIF/CIF for a performance comparison of the proposed changes relative to JM50f (positive values reflect a bit rate reduction).

| Container | Foreman | News | Silent | Paris | Tempete | Mobile | Average |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.09 | -0.14 | -0.60 | 0.07 | 0.06 | 0.13 | 0.07 | -0.05 |

Table 3: Average bit rate savings in % for the QP-range = 20, 24, 28, 32, 36 using CIF-sized cropped parts of a set of interlaced test sequences for a performance comparison of the proposed changes relative to JM50f (positive values reflect a bit rate reduction).

| Canoe | F1 | Rugby | Football | Mobile | Fun Fair | Tempete | Harp&Piano | Avg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.82 | 0.97 | 0.69 | 0.96 | 0.30 | 0.38 | 0.43 | 0.32 | 0.73 |

The results of our simulations related to the common test set and the interlaced test set are summarized in Table 2 and Table 3, respectively. For the common test set, we did not observe any significant change of the average performance relative to JM50f. Note, however, that for these simulations only the proposed changes related to Sections 3 and 4 are relevant. For the interlaced test set, however, to which also the additional context models as proposed in Section 2 were applied, an average bit rate reduction of approximately 0.7% could be achieved in comparison to the methods currently implemented in JM50f.

# References

- Th. Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)”, JVT-E146d37, Nov. 2002.

- B. Jeon et al., “Alternate Scan for non-ABT Coding”, JVT-E107, Oct. 2002.

- Peter Borgwardt, “Core Experiment on Interlaced Video Coding”, VCEG-N85, Oct. 2001.

- G. Sullivan et al., “Report of 5th JVT Meeting”, JVT-E001d1, Oct. 2002.

- D. Marpe et al., “Proposed Editorial Changes and Cleanup of CABAC”, JVT-D019, July 2002.

- E. Viscito, “Macroblock level limit on the number of binary symbols in CABAC”, JVT-E054, Dec. 2002.

# Appendices

# Proposed Text for Specification of Maximum Bin Rate per Level

Conformance to a particular Level shall be indicated by setting the syntax element level_idc equal to a value of ten times the level number specified in Table A-1.

- Table A-1 – Level Limits

| Level # | Max Sample Processing Rate (MB/s) | Max Frame Size MaxFS (MBs) | Decoded picture buffer memory MaxKB (1024 bytes) | Max Video Bitrate (1000 bits/sec) | Max Bin Rate (1000 bins/sec) | Max CPB Size (1000 bits) | Vertical MV Range (full samples) | Minimum luma Direct & Bi-predictive block size | Maximum number of MVs per two consecutive MBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 485 | 99 | 148.5 | 64 | 128 | 175 | [-64,+63.75] | - | - |
| 1.1 | 2 970 | 396 | 891.0 | 128 | 256 | 325 | [-128,+127.75] | - | - |
| 1.2 | 5 940 | 396 | 891.0 | 768 | 1536 | 2 000 | [-128,+127.75] | - | - |
| 2 | 11 880 | 396 | 891.0 | 2 000 | 4 000 | 2 000 | [-128,+127.75] | - | - |
| 2.1 | 19 800 | 792 | 1 782.0 | 4 000 | 8 000 | 4 000 | [-256,+255.75] | - | - |
| 2.2 | 20 250 | 1 620 | 3 037.5 | 4 000 | 8 000 | 4 000 | [-256,+255.75] | - | - |
| 3 | 40 500 | 1 620 | 3 037.5 | 10 000 | 20 000 | 8 000 | [-256,+255.75] | - | - |
| 3.1 | 108 000 | 3 600 | 6 750.0 | 20 000 | 40 000 | 14 000 | [-512,+511.75] | 8x8 | 16 |
| 3.2 | 216 000 | 5 120 | 7 680.0 | 20 000 | 40 000 | 20 000 | [-512,+511.75] | 8x8 | 16 |
| 4 | 245 760 | 8 192 | 12 288.0 | 50 000 | 100 000 | 25 000 | [-512,+511.75] | 8x8 | 16 |
| 5 | 786 432 | 32 768 | 61 440.0 | 150 000 | 300.000 | 450 000 | [-512,+511.75] | 8x8 | 16 |

[Ed.Note: Changed numbers for max. VBR taken from the Geneva meeting report [4].]

Levels with non-integer Level numbers in Table A-1 are refered to as “intermediate Levels”.  All Levels have the same status, but note that some applications may choose to use only the integer-numbered Levels.

The maximum video bitrate, the maximum bin rate, and maximum coded picture buffer (CPB) size in Table A-1 apply to VCL conformance. The maximum number of bins per second shall be equal to two times the maximum number of bits per second given by the maximum video bitrate. The equivalent limits for NAL conformance are 20% higher than these values.

# Proposed New Text for Description of Initialization Process

### Slice header syntax

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 4 | ue(v) |
| frame_num | 4 | u(v) |
| if( !frame_coding_only_flag ) { |  |  |
| field_pic_flag | 4 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 4 | u(1) |
| } |  |  |
| first_mb_in_slice | 4 | u(v) |
| slice_type | 4 | ue(v) |
| if( pic_order_cnt_type < 2 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 4 | se(v) |
| if( pic_order_present_flag = = 1 && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 4 | se(v) |
| } |  |  |
| if( redundant_slice_flag ) |  |  |
| redundant_pic_cnt | 4 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 4 | u(1) |
| num_ref_idx_active_override_flag | 4 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_l0_active_minus1 | 4 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 4 | ue(v) |
| } |  |  |
| } |  |  |
| ref_idx_reordering( ) | 4 |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_explicit_flag && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 4 |  |
| if( nal_unit_type = = 5 ) |  |  |
| non_ref_pic_reset_flag | 4 | u(1) |
| else |  |  |
| dec_pic_buffer_management( ) | 4 |  |
| if( entropy_coding_mode = = 1 && slice_type != I && slice_type != SI) |  |  |
| cabac_init_idc | 4 | ue(v) |
| slice_qp_delta | 4 | se(v) |
| if( slice_type = = SP ¦ ¦ slice_type = = SI ) { |  |  |
| if( slice_type = = SP ) |  |  |
| sp_for_switch_flag | 4 | u(1) |
| slice_qs_delta | 4 | se(v) |
| } |  |  |
| if( deblocking_filter_parameters_present_flag = = 1 ) { |  |  |
| disable_deblocking_filter_idc | 4 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 4 | se(v) |
| slice_beta_offset_div2 | 4 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && mb_slice_group_map_type >= 3 && mb_slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 4 | u(v) |
| } |  |  |

### Slice header semantics

….

non_ref_pic_reset_flag indicates how the non-reference pictures are treated for an IDR picture.  A value of 1 indicates that the non-reference pictures are removed from the decoded picture buffer. A value of 0 indicates that the non-reference pictures remain in the decoded picture buffer.

cabac_init_idc is only present when entropy_code_mode is equal to 1 and slice_type is not equal to I or SI. It specifies the index for addressing the initialisation table used in the initialisation process for contexts. The value of cabac_init_idc shall be in the range of 0 to 2, inclusive.

slice_qp_delta specifies the initial value of QPY to be used for all the macroblocks in the slice until modified by the value of mb_qp_delta in the macroblock layer. The initial QPY parameter for the slice is computed as:

QPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-1)

The value of slice_qp_delta shall be limited such that QPY is in the range of 0 to 51, inclusive.

….

#### Initialisation process for contexts

Input to this process is the quantisation parameter QPY.

Output of this process are the initialised CABAC context models indexed by CtxIdx and the context identifier.

Each context model is initialised with an initial state, which consists of a state number and the meaning of the most probable symbol (MPS) as further described in subclause Error: Reference source not found.  The actual initial state of a context model depends linearly on the (initial) quantisation parameter QPY of the given slice, such that for each context model a pair of table entries {m, n} is given, from which the initial state is computed in the following way

- Compute PreState = (( m QPY)>>4) + n;

- Limit PreState to the range of [1,126], i.e.,
PreState = min (126, max(1, PreState));

- Map PreState to {State, MPS} pair according to the following rule:
if (PreState <= 63) then {State = 63-PreState, MPS = 0} else {State = PreState-64, MPS = 1}

For syntax elements specifically used for P- and B-slices only, there are always three tables defined. The index for addressing the used table is given by the indicator cabac_init_idc.

Initialisation process for contexts – Table 9-13 contain the initialisation parameters {m, n} related to the context identifiers of all syntax elements except the end-of-slice flag.

For the context identifier CtxEosFlag with CtxIdx = 252 associated to the end_of_slice_flag, the initial state is given by State = 63 and MPS = 0.

Table 9-2 – Initialisation parameters for context identifiers CtxMbTypeI, CtxMbTypeSIPrefix, and CtxMbTypeSISuffix

| Context identifier | Initialisation parameter | CtxIdx |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| CtxMbTypeI | m | 20 | 2 | 3 | -28 | -23 | -6 | -1 | 7 | na | na | na |
|  | n | -15 | 54 | 74 | 127 | 104 | 53 | 54 | 51 | na | na | na |
| CtxMbTypeSIPrefix | m | 20 | 2 | 3 | na | na | na | na | na | na | na | na |
|  | n | -15 | 54 | 74 | na | na | na | na | na | na | na | na |
| CtxMbTypeSISuffix | m | na | na | na | 20 | 2 | 3 | -28 | -23 | -6 | -1 | 7 |
|  | n | na | na | na | -15 | 54 | 74 | 127 | 104 | 53 | 54 | 51 |

Table 9-3 – Initialisation parameters for context identifier CtxMbSkip (for P- and SP-slices only), CtxMbTypeP, CtxMbTypePSuffix, and CtxMbTypeBSuffix

| Context identifier |  | CtxMbSkip (for P- and SP- slices only) |  |  | CtxMbTypeP |  |  |  | CtxMbTypePSuffix, CtxMbTypeBSuffix |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value of cabac_init_idc | Initialisation parameter | CtxIdx |  |  |  |  |  |  |  |  |  |  |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 9 | 10 |  | 11 |
| 0 | m | -23 | -23 | -21 | 1 | 0 | -37 | 5 | -13 | -11 | 1 |  |
|  | n | 94 | 125 | 127 | 9 | 49 | 118 | 57 | 78 | 65 | 62 |  |
| 1 | m | -22 | -34 | -16 | -2 | 4 | -29 | 2 | -6 | -13 | 5 |  |
|  | n | 102 | 127 | 127 | 9 | 41 | 118 | 65 | 71 | 79 | 52 |  |
| 2 | m | -29 | -25 | -14 | -10 | -3 | -27 | 26 | -4 | -24 | 5 |  |
|  | n | 111 | 127 | 127 | 51 | 62 | 99 | 16 | 85 | 102 | 57 |  |

Table 9-4X – Initialisation parameters for context identifier CtxMbSkip for B-slices

| Context identifier |  | CtxMbSkip (for B-slices only) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value of cabac_init_idc | Initialisation parameter | CtxIdx |  |  |  |  |  |  |  |  |  |  |
|  |  | 0 | 1 | 2 |  |  |  |  |  |  |  |  |
| 0 | m | -18 | -9 | -29 |  |  |  |  |  |  |  |  |
|  | n | 63 | 84 | 127 |  |  |  |  |  |  |  |  |
| 1 | m | -26 | -19 | -40 |  |  |  |  |  |  |  |  |
|  | n | 93 | 105 | 127 |  |  |  |  |  |  |  |  |
| 2 | m | -20 | -20 | -29 |  |  |  |  |  |  |  |  |
|  | n | 87 | 117 | 127 |  |  |  |  |  |  |  |  |

Table 9-5 – Initialisation parameters for context identifier CtxMbTypeB, CtxB8ModeB, and CtxB8ModeP

| Context identifier |  | CtxMbTypeB |  |  |  |  |  | CtxB8ModeB |  |  |  | CtxB8ModeP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value of cabac_init_idc | Initialisation parameter | CtxIdx |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 4 | 5 | 6 | 7 | 8 | 12 | 13 | 14 | 15 | 12 | 13 | 14 |
| 0 | m | 26 | 16 | 9 | -46 | -20 | 1 | -6 | -17 | -6 | 9 | 12 | -4 | 17 |
|  | n | 67 | 90 | 104 | 127 | 104 | 67 | 86 | 95 | 61 | 45 | 49 | 73 | 50 |
| 1 | m | 57 | 41 | 26 | -45 | -15 | -4 | 6 | -13 | 0 | 8 | 9 | -3 | 10 |
|  | n | 2 | 36 | 69 | 127 | 101 | 76 | 69 | 90 | 52 | 43 | 50 | 70 | 54 |
| 2 | m | 54 | 37 | 12 | -32 | -22 | -2 | -6 | -14 | -6 | 4 | 6 | -17 | 14 |
|  | n | 0 | 42 | 97 | 127 | 117 | 74 | 93 | 88 | 44 | 55 | 57 | 73 | 57 |

Table 9-6 – Initialisation parameters for context identifiers CtxAbsMvdHoriz and CtxAbsMvdVert

| Context identifier |  | CtxAbsMvdHoriz |  |  |  |  |  |  | CtxAbsMvdVert |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value of cabac_init_idc | Initialisation parameter | CtxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 16 | 17 | 18 | 19 | 20 | 21 | 22 |  | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| 0 | m | -3 | -6 | -11 | 6 | 7 | -5 | 2 |  | 0 | -3 | -10 | 5 | 4 | -3 | 0 |
|  | n | 69 | 81 | 96 | 55 | 67 | 86 | 88 |  | 58 | 76 | 94 | 54 | 69 | 81 | 88 |
| 1 | m | -2 | -5 | -10 | 2 | 2 | -3 | -3 |  | 1 | -3 | -6 | 0 | -3 | -7 | -5 |
|  | n | 69 | 82 | 96 | 59 | 75 | 87 | 100 |  | 56 | 74 | 85 | 59 | 81 | 86 | 95 |
| 2 | m | -11 | -15 | -21 | 19 | 20 | 4 | 6 |  | 1 | -5 | -13 | 5 | 6 | -3 | -1 |
|  | n | 89 | 103 | 116 | 57 | 58 | 84 | 96 |  | 63 | 85 | 106 | 63 | 75 | 90 | 101 |

Table 9-7 – Initialisation parameters for context identifier CtxRefIdx and CtxMbField

| Context identifier |  | CtxRefIdx |  |  |  |  |  | CtxMbField |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value of cabac_init_idc | Initialisation parameter | CtxIdx |  |  |  |  |  |  |  |  |
|  |  | 30 | 31 | 32 | 33 | 34 | 35 | 46 | 47 | 48 |
| 0 | m | -7 | -5 | -4 | -5 | -7 | 1 | 0 | 13 | 7 |
|  | n | 67 | 74 | 74 | 80 | 72 | 58 | 45 | 15 | 34 |
| 1 | m | -1 | -1 | 1 | -2 | -5 | 0 | -4 | 7 | -9 |
|  | n | 66 | 77 | 70 | 86 | 72 | 61 | 78 | 51 | 88 |
| 2 | m | 3 | -4 | -2 | -12 | -7 | 1 | -3 | 2 | -20 |
|  | n | 55 | 79 | 75 | 97 | 50 | 60 | 96 | 80 | 127 |
| INTRA | m |  |  |  |  |  |  | 0 | 1 | 0 |
|  | n |  |  |  |  |  |  | 11 | 55 | 69 |

Table 9-8 – Initialisation parameters for context identifiers CtxMbQpDelta, CtxIpredChroma, CtxIpredLumaMpm, CtxIpredLumaRm

| Context label | CtxMbQpDelta |  | Context label | CtxIpredChroma |  | Context label | CtxIpredLumaMpm |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | m | n |  | m | n |  | m | n |
| 36 | 0 | 41 | 40 | -9 | 83 | 44 | -13 | 86 |
| 37 | 0 | 63 | 41 | 4 | 86 |  | CtxIpredLumaRm |  |
| 38 | 0 | 63 | 42 | 0 | 97 | 45 | 3 | 62 |
| 39 | 0 | 63 | 43 | -7 | 72 |  |  |  |

- Table 9-9 – Initialisation parameters for context identifiers CtxCbpLuma and CtxCbpChroma

| Context identifier | CtxIdx | I-slices |  | P- and B-slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cabac_init_idc = 0 |  | cabac_init_idc = 1 |  | cabac_init_idc = 2 |  |
|  |  | m | n | m | n | m | n | m | n |
| CtxCbpLuma | 49 | -17 | 127 | -27 | 126 | -39 | 127 | -36 | 127 |
|  | 50 | -13 | 102 | -28 | 98 | -18 | 91 | -17 | 91 |
|  | 51 | 0 | 82 | -25 | 101 | -17 | 96 | -14 | 95 |
|  | 52 | -7 | 74 | -23 | 67 | -26 | 81 | -25 | 84 |
| CtxCbpChroma | 53 | -21 | 107 | -28 | 82 | -35 | 98 | -25 | 86 |
|  | 54 | -27 | 127 | -20 | 94 | -24 | 102 | -12 | 89 |
|  | 55 | -31 | 127 | -16 | 83 | -23 | 97 | -17 | 91 |
|  | 56 | -24 | 127 | -22 | 110 | -27 | 119 | -31 | 127 |
|  | 57 | -18 | 95 | -21 | 91 | -24 | 99 | -14 | 76 |
|  | 58 | -27 | 127 | -18 | 102 | -21 | 110 | -18 | 103 |
|  | 59 | -21 | 114 | -13 | 93 | -18 | 102 | -13 | 90 |
|  | 60 | -30 | 127 | -29 | 127 | -36 | 127 | -37 | 127 |

- Table 9-10 – Initialisation parameters for context identifier CtxCodedBlock

| Value of CtxCat | CtxIdx | I-slices |  | P- and B-slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cabac_init_idc = 0 |  | cabac_init_idc = 1 |  | cabac_init_idc = 2 |  |
|  |  | m | n | m | n | m | n | m | n |
| 0 | 61 | -17 | 123 | -7 | 92 | 0 | 80 | 11 | 80 |
|  | 62 | -12 | 115 | -5 | 89 | -5 | 89 | 5 | 76 |
|  | 63 | -16 | 122 | -7 | 96 | -7 | 94 | 2 | 84 |
|  | 64 | -11 | 115 | -13 | 108 | -4 | 92 | 5 | 78 |
| 1 | 65 | -12 | 63 | -3 | 46 | 0 | 39 | -6 | 55 |
|  | 66 | -2 | 68 | -1 | 65 | 0 | 65 | 4 | 61 |
|  | 67 | -15 | 84 | -1 | 57 | -15 | 84 | -14 | 83 |
|  | 68 | -13 | 104 | -9 | 93 | -35 | 127 | -37 | 127 |
| 2 | 69 | -3 | 70 | -3 | 74 | -2 | 73 | -5 | 79 |
|  | 70 | -8 | 93 | -9 | 92 | -12 | 104 | -11 | 104 |
|  | 71 | -10 | 90 | -8 | 87 | -9 | 91 | -11 | 91 |
|  | 72 | -30 | 127 | -23 | 126 | -31 | 127 | -30 | 127 |
| 3 | 73 | -1 | 74 | 5 | 54 | 3 | 55 | 0 | 65 |
|  | 74 | -6 | 97 | 6 | 60 | 7 | 56 | -2 | 79 |
|  | 75 | -7 | 91 | 6 | 59 | 7 | 55 | 0 | 72 |
|  | 76 | -20 | 127 | 6 | 69 | 8 | 61 | -4 | 92 |
| 4 | 77 | -4 | 56 | -1 | 48 | -3 | 53 | -6 | 56 |
|  | 78 | -5 | 82 | 0 | 68 | 0 | 68 | 3 | 68 |
|  | 79 | -7 | 76 | -4 | 69 | -7 | 74 | -8 | 71 |
|  | 80 | -22 | 125 | -8 | 88 | -9 | 88 | -13 | 98 |

Table 9-11 – Initialisation parameters for context identifier CtxSigCoeffFrm

| Value of CtxCat | CtxIdx | I-slices |  | P- and B-slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cabac_init_idc = 0 |  | cabac_init_idc = 1 |  | cabac_init_idc = 2 |  |
|  |  | m | n | m | n | m | n | m | n |
| 0 | 81 | -7 | 93 | -2 | 85 | -13 | 103 | -4 | 86 |
|  | 82 | -11 | 87 | -6 | 78 | -13 | 91 | -12 | 88 |
|  | 83 | -3 | 77 | -1 | 75 | -9 | 89 | -5 | 82 |
|  | 84 | -5 | 71 | -7 | 77 | -14 | 92 | -3 | 72 |
|  | 85 | -4 | 63 | 2 | 54 | -8 | 76 | -4 | 67 |
|  | 86 | -4 | 68 | 5 | 50 | -12 | 87 | -8 | 72 |
|  | 87 | -12 | 84 | -3 | 68 | -23 | 110 | -16 | 89 |
|  | 88 | -7 | 62 | 1 | 50 | -24 | 105 | -9 | 69 |
|  | 89 | -7 | 65 | 6 | 42 | -10 | 78 | -1 | 59 |
|  | 90 | 8 | 61 | -4 | 81 | -20 | 112 | 5 | 66 |
|  | 91 | 5 | 56 | 1 | 63 | -17 | 99 | 4 | 57 |
|  | 92 | -2 | 66 | -4 | 70 | -78 | 127 | -4 | 71 |
|  | 93 | 1 | 64 | 0 | 67 | -70 | 127 | -2 | 71 |
|  | 94 | 0 | 61 | 2 | 57 | -50 | 127 | 2 | 58 |
|  | 95 | -2 | 78 | -2 | 76 | -46 | 127 | -1 | 74 |
| 1 | 96 | 1 | 50 | 11 | 35 | -4 | 66 | -4 | 44 |
|  | 97 | 7 | 52 | 4 | 64 | -5 | 78 | -1 | 69 |
|  | 98 | 10 | 35 | 1 | 61 | -4 | 71 | 0 | 62 |
|  | 99 | 0 | 44 | 11 | 35 | -8 | 72 | -7 | 51 |
|  | 100 | 11 | 38 | 18 | 25 | 2 | 59 | -4 | 47 |
|  | 101 | 1 | 45 | 12 | 24 | -1 | 55 | -6 | 42 |
|  | 102 | 0 | 46 | 13 | 29 | -7 | 70 | -3 | 41 |
|  | 103 | 5 | 44 | 13 | 36 | -6 | 75 | -6 | 53 |
|  | 104 | 31 | 17 | -10 | 93 | -8 | 89 | 8 | 76 |
|  | 105 | 1 | 51 | -7 | 73 | -34 | 119 | -9 | 78 |
|  | 106 | 7 | 50 | -2 | 73 | -3 | 75 | -11 | 83 |
|  | 107 | 28 | 19 | 13 | 46 | 32 | 20 | 9 | 52 |
|  | 108 | 16 | 33 | 9 | 49 | 30 | 22 | 0 | 67 |
|  | 109 | 14 | 62 | -7 | 100 | -44 | 127 | -5 | 90 |
| 2 | 110 | -13 | 108 | 9 | 53 | 0 | 54 | 1 | 67 |
|  | 111 | -15 | 100 | 2 | 53 | -5 | 61 | -15 | 72 |
|  | 112 | -13 | 101 | 5 | 53 | 0 | 58 | -5 | 75 |
|  | 113 | -13 | 91 | -2 | 61 | -1 | 60 | -8 | 80 |
|  | 114 | -12 | 94 | 0 | 56 | -3 | 61 | -21 | 83 |
|  | 115 | -10 | 88 | 0 | 56 | -8 | 67 | -21 | 64 |
|  | 116 | -16 | 84 | -13 | 63 | -25 | 84 | -13 | 31 |
|  | 117 | -10 | 86 | -5 | 60 | -14 | 74 | -25 | 64 |
|  | 118 | -7 | 83 | -1 | 62 | -5 | 65 | -29 | 94 |
|  | 119 | -13 | 87 | 4 | 57 | 5 | 52 | 9 | 75 |
|  | 120 | -19 | 94 | -6 | 69 | 2 | 57 | 17 | 63 |
|  | 121 | 1 | 70 | 4 | 57 | 0 | 61 | -8 | 74 |
|  | 122 | 0 | 72 | 14 | 39 | -9 | 69 | -5 | 35 |
|  | 123 | -5 | 74 | 4 | 51 | -11 | 70 | -2 | 27 |
|  | 124 | 18 | 59 | 13 | 68 | 18 | 55 | 13 | 91 |
| 3 | 125 | -8 | 102 | 3 | 64 | -4 | 71 | 3 | 65 |
|  | 126 | -15 | 100 | 1 | 61 | 0 | 58 | -7 | 69 |
|  | 127 | 0 | 95 | 9 | 63 | 7 | 61 | 8 | 77 |
| 4 | 128 | -4 | 75 | 7 | 50 | 9 | 41 | -10 | 66 |
|  | 129 | 2 | 72 | 16 | 39 | 18 | 25 | 3 | 62 |
|  | 130 | -11 | 75 | 5 | 44 | 9 | 32 | -3 | 68 |
|  | 131 | -3 | 71 | 4 | 52 | 5 | 43 | -20 | 81 |
|  | 132 | 15 | 46 | 11 | 48 | 9 | 47 | 0 | 30 |
|  | 133 | -13 | 69 | -5 | 60 | 0 | 44 | 1 | 7 |
|  | 134 | 0 | 62 | -1 | 59 | 0 | 51 | -3 | 23 |
|  | 135 | 0 | 65 | 0 | 59 | 2 | 46 | -21 | 74 |
|  | 136 | 21 | 37 | 22 | 33 | 19 | 38 | 16 | 66 |
|  | 137 | -15 | 72 | 5 | 44 | -4 | 66 | -23 | 124 |
|  | 138 | 9 | 57 | 14 | 43 | 15 | 38 | 17 | 37 |
|  | 139 | 16 | 54 | -1 | 78 | 12 | 42 | 44 | -18 |
|  | 140 | 0 | 62 | 0 | 60 | 9 | 34 | 50 | -34 |
|  | 141 | 12 | 72 | 9 | 69 | 0 | 89 | -22 | 127 |

Table 9-12 – Initialisation parameters for context identifier CtxLastCoeffFrm

| Value of CtxCat | CtxIdx | I-slices |  | P- and B-slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cabac_init_idc = 0 |  | cabac_init_idc = 1 |  | cabac_init_idc = 2 |  |
|  |  | m | n | m | n | m | n | m | n |
| 0 | 142 | 24 | 0 | 11 | 28 | 4 | 45 | 4 | 39 |
|  | 143 | 15 | 9 | 2 | 40 | 10 | 28 | 0 | 42 |
|  | 144 | 8 | 25 | 3 | 44 | 10 | 31 | 7 | 34 |
|  | 145 | 13 | 18 | 0 | 49 | 33 | -11 | 11 | 29 |
|  | 146 | 15 | 9 | 0 | 46 | 52 | -43 | 8 | 31 |
|  | 147 | 13 | 19 | 2 | 44 | 18 | 15 | 6 | 37 |
|  | 148 | 10 | 37 | 2 | 51 | 28 | 0 | 7 | 42 |
|  | 149 | 12 | 18 | 0 | 47 | 35 | -22 | 3 | 40 |
|  | 150 | 6 | 29 | 4 | 39 | 38 | -25 | 8 | 33 |
|  | 151 | 20 | 33 | 2 | 62 | 34 | 0 | 13 | 43 |
|  | 152 | 15 | 30 | 6 | 46 | 39 | -18 | 13 | 36 |
|  | 153 | 4 | 45 | 0 | 54 | 32 | -12 | 4 | 47 |
|  | 154 | 1 | 58 | 3 | 54 | 102 | -94 | 3 | 55 |
|  | 155 | 0 | 62 | 2 | 58 | 0 | 0 | 2 | 58 |
|  | 156 | 7 | 61 | 4 | 63 | 56 | -15 | 6 | 60 |
| 1 | 157 | 12 | 38 | 6 | 51 | 33 | -4 | 8 | 44 |
|  | 158 | 11 | 45 | 6 | 57 | 29 | 10 | 11 | 44 |
|  | 159 | 15 | 39 | 7 | 53 | 37 | -5 | 14 | 42 |
|  | 160 | 11 | 42 | 6 | 52 | 51 | -29 | 7 | 48 |
|  | 161 | 13 | 44 | 6 | 55 | 39 | -9 | 4 | 56 |
|  | 162 | 16 | 45 | 11 | 45 | 52 | -34 | 4 | 52 |
|  | 163 | 12 | 41 | 14 | 36 | 69 | -58 | 13 | 37 |
|  | 164 | 10 | 49 | 8 | 53 | 67 | -63 | 9 | 49 |
|  | 165 | 30 | 34 | -1 | 82 | 44 | -5 | 19 | 58 |
|  | 166 | 18 | 42 | 7 | 55 | 32 | 7 | 10 | 48 |
|  | 167 | 10 | 55 | -3 | 78 | 55 | -29 | 12 | 45 |
|  | 168 | 17 | 51 | 15 | 46 | 32 | 1 | 0 | 69 |
|  | 169 | 17 | 46 | 22 | 31 | 0 | 0 | 20 | 33 |
|  | 170 | 0 | 89 | -1 | 84 | 27 | 36 | 8 | 63 |
| 2 | 171 | 26 | -19 | 25 | 7 | 33 | -25 | 35 | -18 |
|  | 172 | 22 | -17 | 30 | -7 | 34 | -30 | 33 | -25 |
|  | 173 | 26 | -17 | 28 | 3 | 36 | -28 | 28 | -3 |
|  | 174 | 30 | -25 | 28 | 4 | 38 | -28 | 24 | 10 |
|  | 175 | 28 | -20 | 32 | 0 | 38 | -27 | 27 | 0 |
|  | 176 | 33 | -23 | 34 | -1 | 34 | -18 | 34 | -14 |
|  | 177 | 37 | -27 | 30 | 6 | 35 | -16 | 52 | -44 |
|  | 178 | 33 | -23 | 30 | 6 | 34 | -14 | 39 | -24 |
|  | 179 | 40 | -28 | 32 | 9 | 32 | -8 | 19 | 17 |
|  | 180 | 38 | -17 | 31 | 19 | 37 | -6 | 31 | 25 |
|  | 181 | 33 | -11 | 26 | 27 | 35 | 0 | 36 | 29 |
|  | 182 | 40 | -15 | 26 | 30 | 30 | 10 | 24 | 33 |
|  | 183 | 41 | -6 | 37 | 20 | 28 | 18 | 34 | 15 |
|  | 184 | 38 | 1 | 28 | 34 | 26 | 25 | 30 | 20 |
|  | 185 | 41 | 17 | 17 | 70 | 29 | 41 | 22 | 73 |
| 3 | 186 | 30 | -6 | 1 | 67 | 0 | 75 | 20 | 34 |
|  | 187 | 27 | 3 | 5 | 59 | 2 | 72 | 19 | 31 |
|  | 188 | 26 | 22 | 9 | 67 | 8 | 77 | 27 | 44 |
| 4 | 189 | 37 | -16 | 16 | 30 | 14 | 35 | 19 | 16 |
|  | 190 | 35 | -4 | 18 | 32 | 18 | 31 | 15 | 36 |
|  | 191 | 38 | -8 | 18 | 35 | 17 | 35 | 15 | 36 |
|  | 192 | 38 | -3 | 22 | 29 | 21 | 30 | 21 | 28 |
|  | 193 | 37 | 3 | 24 | 31 | 17 | 45 | 25 | 21 |
|  | 194 | 38 | 5 | 23 | 38 | 20 | 42 | 30 | 20 |
|  | 195 | 42 | 0 | 18 | 43 | 18 | 45 | 31 | 12 |
|  | 196 | 35 | 16 | 20 | 41 | 27 | 26 | 27 | 16 |
|  | 197 | 39 | 22 | 11 | 63 | 16 | 54 | 24 | 42 |
|  | 198 | 14 | 48 | 9 | 59 | 7 | 66 | 0 | 93 |
|  | 199 | 27 | 37 | 9 | 64 | 16 | 56 | 14 | 56 |
|  | 200 | 21 | 60 | -1 | 94 | 11 | 73 | 15 | 57 |
|  | 201 | 12 | 68 | -2 | 89 | 10 | 67 | 26 | 38 |
|  | 202 | 2 | 97 | -9 | 108 | -10 | 116 | -24 | 127 |

Table 9-13 – Initialisation parameters for context identifier CtxAbsLevel

| Value of CtxCat | CtxIdx | I-slices |  | P- and B-slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cabac_init_idc = 0 |  | cabac_init_idc = 1 |  | cabac_init_idc = 2 |  |
|  |  | m | n | m | n | m | n | m | n |
| 0 | 203 | -3 | 71 | -6 | 76 | -23 | 112 | -24 | 115 |
|  | 204 | -6 | 42 | -2 | 44 | -15 | 71 | -22 | 82 |
|  | 205 | -5 | 50 | 0 | 45 | -7 | 61 | -9 | 62 |
|  | 206 | -3 | 54 | 0 | 52 | 0 | 53 | 0 | 53 |
|  | 207 | -2 | 62 | -3 | 64 | -5 | 66 | 0 | 59 |
|  | 208 | 0 | 58 | -2 | 59 | -11 | 77 | -14 | 85 |
|  | 209 | 1 | 63 | -4 | 70 | -9 | 80 | -13 | 89 |
|  | 210 | -2 | 72 | -4 | 75 | -9 | 84 | -13 | 94 |
|  | 211 | -1 | 74 | -8 | 82 | -10 | 87 | -11 | 92 |
|  | 212 | -9 | 91 | -17 | 102 | -34 | 127 | -29 | 127 |
| 1 | 213 | -5 | 67 | -9 | 77 | -21 | 101 | -21 | 100 |
|  | 214 | -5 | 27 | 3 | 24 | -3 | 39 | -14 | 57 |
|  | 215 | -3 | 39 | 0 | 42 | -5 | 53 | -12 | 67 |
|  | 216 | -2 | 44 | 0 | 48 | -7 | 61 | -11 | 71 |
|  | 217 | 0 | 46 | 0 | 55 | -11 | 75 | -10 | 77 |
|  | 218 | -16 | 64 | -6 | 59 | -15 | 77 | -21 | 85 |
|  | 219 | -8 | 68 | -7 | 71 | -17 | 91 | -16 | 88 |
|  | 220 | -10 | 78 | -12 | 83 | -25 | 107 | -23 | 104 |
|  | 221 | -6 | 77 | -11 | 87 | -25 | 111 | -15 | 98 |
|  | 222 | -10 | 86 | -30 | 119 | -28 | 122 | -37 | 127 |
| 2 | 223 | -12 | 92 | 1 | 58 | -11 | 76 | -10 | 82 |
|  | 224 | -15 | 55 | -3 | 29 | -10 | 44 | -8 | 48 |
|  | 225 | -10 | 60 | -1 | 36 | -10 | 52 | -8 | 61 |
|  | 226 | -6 | 62 | 1 | 38 | -10 | 57 | -8 | 66 |
|  | 227 | -4 | 65 | 2 | 43 | -9 | 58 | -7 | 70 |
|  | 228 | -12 | 73 | -6 | 55 | -16 | 72 | -14 | 75 |
|  | 229 | -8 | 76 | 0 | 58 | -7 | 69 | -10 | 79 |
|  | 230 | -7 | 80 | 0 | 64 | -4 | 69 | -9 | 83 |
|  | 231 | -9 | 88 | -3 | 74 | -5 | 74 | -12 | 92 |
|  | 232 | -17 | 110 | -10 | 90 | -9 | 86 | -18 | 108 |
| 3 | 233 | -11 | 97 | 0 | 70 | 2 | 66 | -4 | 79 |
|  | 234 | -20 | 84 | -4 | 29 | -9 | 34 | -22 | 69 |
|  | 235 | -11 | 79 | 5 | 31 | 1 | 32 | -16 | 75 |
|  | 236 | -6 | 73 | 7 | 42 | 11 | 31 | -2 | 58 |
|  | 237 | -4 | 74 | 1 | 59 | 5 | 52 | 1 | 58 |
|  | 238 | -13 | 86 | -2 | 58 | -2 | 55 | -13 | 78 |
|  | 239 | -13 | 96 | -3 | 72 | -2 | 67 | -9 | 83 |
|  | 240 | -11 | 97 | -3 | 81 | 0 | 73 | -4 | 81 |
|  | 241 | -19 | 117 | -11 | 97 | -8 | 89 | -13 | 99 |
| 4 | 242 | -8 | 78 | 0 | 58 | 3 | 52 | -13 | 81 |
|  | 243 | -5 | 33 | 8 | 5 | 7 | 4 | -6 | 38 |
|  | 244 | -4 | 48 | 10 | 14 | 10 | 8 | -13 | 62 |
|  | 245 | -2 | 53 | 14 | 18 | 17 | 8 | -6 | 58 |
|  | 246 | -3 | 62 | 13 | 27 | 16 | 19 | -2 | 59 |
|  | 247 | -13 | 71 | 2 | 40 | 3 | 37 | -16 | 73 |
|  | 248 | -10 | 79 | 0 | 58 | -1 | 61 | -10 | 76 |
|  | 249 | -12 | 86 | -3 | 70 | -5 | 73 | -13 | 86 |
|  | 250 | -13 | 90 | -6 | 79 | -1 | 70 | -9 | 83 |
|  | 251 | -14 | 97 | -8 | 85 | -4 | 78 | -10 | 87 |

Table 9-14XX – Initialisation parameters for context identifier CtxSigCoeffFld

| Value of CtxCat | CtxIdx | I-slices |  | P- and B-slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cabac_init_idc = 0 |  | cabac_init_idc = 1 |  | cabac_init_idc = 2 |  |
|  |  | m | n | m | n | m | n | m | n |
| 0 | 253 | -6 | 93 | -13 | 106 | -21 | 126 | -22 | 127 |
|  | 254 | -6 | 84 | -16 | 106 | -23 | 124 | -25 | 127 |
|  | 255 | -8 | 79 | -10 | 87 | -20 | 110 | -25 | 120 |
|  | 256 | 0 | 66 | -21 | 114 | -26 | 126 | -27 | 127 |
|  | 257 | -1 | 71 | -18 | 110 | -25 | 124 | -19 | 114 |
|  | 258 | 0 | 62 | -14 | 98 | -17 | 105 | -23 | 117 |
|  | 259 | -2 | 60 | -22 | 110 | -27 | 121 | -25 | 118 |
|  | 260 | -2 | 59 | -21 | 106 | -27 | 117 | -26 | 117 |
|  | 261 | -5 | 75 | -18 | 103 | -17 | 102 | -24 | 113 |
|  | 262 | -3 | 62 | -21 | 107 | -26 | 117 | -28 | 118 |
|  | 263 | -4 | 58 | -23 | 108 | -27 | 116 | -31 | 120 |
|  | 264 | -9 | 66 | -26 | 112 | -33 | 122 | -37 | 124 |
|  | 265 | -1 | 79 | -10 | 96 | -10 | 95 | -10 | 94 |
|  | 266 | 0 | 71 | -12 | 95 | -14 | 100 | -15 | 102 |
|  | 267 | 3 | 68 | -5 | 91 | -8 | 95 | -10 | 99 |
| 1 | 268 | 10 | 44 | -9 | 93 | -17 | 111 | -13 | 106 |
|  | 269 | -7 | 62 | -22 | 94 | -28 | 114 | -50 | 127 |
|  | 270 | 15 | 36 | -5 | 86 | -6 | 89 | -5 | 92 |
|  | 271 | 14 | 40 | 9 | 67 | -2 | 80 | 17 | 57 |
|  | 272 | 16 | 27 | -4 | 80 | -4 | 82 | -5 | 86 |
|  | 273 | 12 | 29 | -10 | 85 | -9 | 85 | -13 | 94 |
|  | 274 | 1 | 44 | -1 | 70 | -8 | 81 | -12 | 91 |
|  | 275 | 20 | 36 | 7 | 60 | -1 | 72 | -2 | 77 |
|  | 276 | 18 | 32 | 9 | 58 | 5 | 64 | 0 | 71 |
|  | 277 | 5 | 42 | 5 | 61 | 1 | 67 | -1 | 73 |
|  | 278 | 1 | 48 | 12 | 50 | 9 | 56 | 4 | 64 |
|  | 279 | 10 | 62 | 15 | 50 | 0 | 69 | -7 | 81 |
|  | 280 | 17 | 46 | 18 | 49 | 1 | 69 | 5 | 64 |
|  | 281 | 9 | 64 | 17 | 54 | 7 | 69 | 15 | 57 |
| 2 | 282 | -12 | 104 | 10 | 41 | -7 | 69 | 1 | 67 |
|  | 283 | -11 | 97 | 7 | 46 | -6 | 67 | 0 | 68 |
|  | 284 | -16 | 96 | -1 | 51 | -16 | 77 | -10 | 67 |
|  | 285 | -7 | 88 | 7 | 49 | -2 | 64 | 1 | 68 |
|  | 286 | -8 | 85 | 8 | 52 | 2 | 61 | 0 | 77 |
|  | 287 | -7 | 85 | 9 | 41 | -6 | 67 | 2 | 64 |
|  | 288 | -9 | 85 | 6 | 47 | -3 | 64 | 0 | 68 |
|  | 289 | -13 | 88 | 2 | 55 | 2 | 57 | -5 | 78 |
|  | 290 | 4 | 66 | 13 | 41 | -3 | 65 | 7 | 55 |
|  | 291 | -3 | 77 | 10 | 44 | -3 | 66 | 5 | 59 |
|  | 292 | -3 | 76 | 6 | 50 | 0 | 62 | 2 | 65 |
|  | 293 | -6 | 76 | 5 | 53 | 9 | 51 | 14 | 54 |
|  | 294 | 10 | 58 | 13 | 49 | -1 | 66 | 15 | 44 |
|  | 295 | -1 | 76 | 4 | 63 | -2 | 71 | 5 | 60 |
|  | 296 | -1 | 83 | 6 | 64 | -2 | 75 | 2 | 70 |
| 3 | 297 | -7 | 99 | -2 | 69 | -1 | 70 | -2 | 76 |
|  | 298 | -14 | 95 | -2 | 59 | -9 | 72 | -18 | 86 |
|  | 299 | 2 | 95 | 6 | 70 | 14 | 60 | 12 | 70 |
| 4 | 300 | 0 | 76 | 10 | 44 | 16 | 37 | 5 | 64 |
|  | 301 | -5 | 74 | 9 | 31 | 0 | 47 | -12 | 70 |
|  | 302 | 0 | 70 | 12 | 43 | 18 | 35 | 11 | 55 |
|  | 303 | -11 | 75 | 3 | 53 | 11 | 37 | 5 | 56 |
|  | 304 | 1 | 68 | 14 | 34 | 12 | 41 | 0 | 69 |
|  | 305 | 0 | 65 | 10 | 38 | 10 | 41 | 2 | 65 |
|  | 306 | -14 | 73 | -3 | 52 | 2 | 48 | -6 | 74 |
|  | 307 | 3 | 62 | 13 | 40 | 12 | 41 | 5 | 54 |
|  | 308 | 4 | 62 | 17 | 32 | 13 | 41 | 7 | 54 |
|  | 309 | -1 | 68 | 7 | 44 | 0 | 59 | -6 | 76 |
|  | 310 | -13 | 75 | 7 | 38 | 3 | 50 | -11 | 82 |
|  | 311 | 11 | 55 | 13 | 50 | 19 | 40 | -2 | 77 |
|  | 312 | 5 | 64 | 10 | 57 | 3 | 66 | -2 | 77 |
|  | 313 | 12 | 70 | 26 | 43 | 18 | 50 | 25 | 42 |

Table 9-15XX – Initialisation parameters for context identifier CtxLastCoeffFld

| Value of CtxCat | CtxIdx | I-slices |  | P- and B-slices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cabac_init_idc = 0 |  | cabac_init_idc = 1 |  | cabac_init_idc = 2 |  |
|  |  | m | n | m | n | m | n | m | n |
| 0 | 314 | 15 | 6 | 14 | 11 | 19 | -6 | 17 | -13 |
|  | 315 | 6 | 19 | 11 | 14 | 18 | -6 | 16 | -9 |
|  | 316 | 7 | 16 | 9 | 11 | 14 | 0 | 17 | -12 |
|  | 317 | 12 | 14 | 18 | 11 | 26 | -12 | 27 | -21 |
|  | 318 | 18 | 13 | 21 | 9 | 31 | -16 | 37 | -30 |
|  | 319 | 13 | 11 | 23 | -2 | 33 | -25 | 41 | -40 |
|  | 320 | 13 | 15 | 32 | -15 | 33 | -22 | 42 | -41 |
|  | 321 | 15 | 16 | 32 | -15 | 37 | -28 | 48 | -47 |
|  | 322 | 12 | 23 | 34 | -21 | 39 | -30 | 39 | -32 |
|  | 323 | 13 | 23 | 39 | -23 | 42 | -30 | 46 | -40 |
|  | 324 | 15 | 20 | 42 | -33 | 47 | -42 | 52 | -51 |
|  | 325 | 14 | 26 | 41 | -31 | 45 | -36 | 46 | -41 |
|  | 326 | 14 | 44 | 46 | -28 | 49 | -34 | 52 | -39 |
|  | 327 | 17 | 40 | 38 | -12 | 41 | -17 | 43 | -19 |
|  | 328 | 17 | 47 | 21 | 29 | 32 | 9 | 32 | 11 |
| 1 | 329 | 24 | 17 | 45 | -24 | 69 | -71 | 61 | -55 |
|  | 330 | 21 | 21 | 53 | -45 | 63 | -63 | 56 | -46 |
|  | 331 | 25 | 22 | 48 | -26 | 66 | -64 | 62 | -50 |
|  | 332 | 31 | 27 | 65 | -43 | 77 | -74 | 81 | -67 |
|  | 333 | 22 | 29 | 43 | -19 | 54 | -39 | 45 | -20 |
|  | 334 | 19 | 35 | 39 | -10 | 52 | -35 | 35 | -2 |
|  | 335 | 14 | 50 | 30 | 9 | 41 | -10 | 28 | 15 |
|  | 336 | 10 | 57 | 18 | 26 | 36 | 0 | 34 | 1 |
|  | 337 | 7 | 63 | 20 | 27 | 40 | -1 | 39 | 1 |
|  | 338 | -2 | 77 | 0 | 57 | 30 | 14 | 30 | 17 |
|  | 339 | -4 | 82 | -14 | 82 | 28 | 26 | 20 | 38 |
|  | 340 | -3 | 94 | -5 | 75 | 23 | 37 | 18 | 45 |
|  | 341 | 9 | 69 | -19 | 97 | 12 | 55 | 15 | 54 |
|  | 342 | -12 | 109 | -35 | 125 | 11 | 65 | 0 | 79 |
| 2 | 343 | 36 | -35 | 27 | 0 | 37 | -33 | 36 | -16 |
|  | 344 | 36 | -34 | 28 | 0 | 39 | -36 | 37 | -14 |
|  | 345 | 32 | -26 | 31 | -4 | 40 | -37 | 37 | -17 |
|  | 346 | 37 | -30 | 27 | 6 | 38 | -30 | 32 | 1 |
|  | 347 | 44 | -32 | 34 | 8 | 46 | -33 | 34 | 15 |
|  | 348 | 34 | -18 | 30 | 10 | 42 | -30 | 29 | 15 |
|  | 349 | 34 | -15 | 24 | 22 | 40 | -24 | 24 | 25 |
|  | 350 | 40 | -15 | 33 | 19 | 49 | -29 | 34 | 22 |
|  | 351 | 33 | -7 | 22 | 32 | 38 | -12 | 31 | 16 |
|  | 352 | 35 | -5 | 26 | 31 | 40 | -10 | 35 | 18 |
|  | 353 | 33 | 0 | 21 | 41 | 38 | -3 | 31 | 28 |
|  | 354 | 38 | 2 | 26 | 44 | 46 | -5 | 33 | 41 |
|  | 355 | 33 | 13 | 23 | 47 | 31 | 20 | 36 | 28 |
|  | 356 | 23 | 35 | 16 | 65 | 29 | 30 | 27 | 47 |
|  | 357 | 13 | 58 | 14 | 71 | 25 | 44 | 21 | 62 |
| 3 | 358 | 29 | -3 | 8 | 60 | 12 | 48 | 18 | 31 |
|  | 359 | 26 | 0 | 6 | 63 | 11 | 49 | 19 | 26 |
|  | 360 | 22 | 30 | 17 | 65 | 26 | 45 | 36 | 24 |
| 4 | 361 | 31 | -7 | 21 | 24 | 22 | 22 | 24 | 23 |
|  | 362 | 35 | -15 | 23 | 20 | 23 | 22 | 27 | 16 |
|  | 363 | 34 | -3 | 26 | 23 | 27 | 21 | 24 | 30 |
|  | 364 | 34 | 3 | 27 | 32 | 33 | 20 | 31 | 29 |
|  | 365 | 36 | -1 | 28 | 23 | 26 | 28 | 22 | 41 |
|  | 366 | 34 | 5 | 28 | 24 | 30 | 24 | 22 | 42 |
|  | 367 | 32 | 11 | 23 | 40 | 27 | 34 | 16 | 60 |
|  | 368 | 35 | 5 | 24 | 32 | 18 | 42 | 15 | 52 |
|  | 369 | 34 | 12 | 28 | 29 | 25 | 39 | 14 | 60 |
|  | 370 | 39 | 11 | 23 | 42 | 18 | 50 | 3 | 78 |
|  | 371 | 30 | 29 | 19 | 57 | 12 | 70 | -16 | 123 |
|  | 372 | 34 | 26 | 22 | 53 | 21 | 54 | 21 | 53 |
|  | 373 | 29 | 39 | 22 | 61 | 14 | 71 | 22 | 56 |
|  | 374 | 19 | 66 | 11 | 86 | 11 | 83 | 25 | 61 |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Fraunhofer-Institute HHI |  |  |
| Mailing address | Einsteinstraße 37 10587 Berlin |  |  |
| Country | Germany |  |  |
| Contact person | Detlev Marpe |  |  |
| Telephone |  |  |  |
| Fax | +49-30-392 72 00 |  |  |
| Email | marpe@hhi.de |  |  |
| Place and date of submission | Awaji, Island, JP, 5-13 December, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | H.264 ¦ AVC |  |
| Title |  | Final CABAC Cleanup |  |
| Contribution number |  | JVT-E039 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: