| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji Island, JP, 5-13 December, 2002 | Document: JVT-F041 Filename: JVT-F041.doc |
| --- | --- |

| Title: | Frame Doubling and Frame Tripling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marie Dahlqvist Teracom AB PO Box 17666 SE-11892 Stockholm Sweden Lars Haglund SVT Sveriges Television AB SVT Corporate Development Technology Oxenstiernsgatan 26 SE-105 10 Stockholm Sweden | Tel: Email: Tel: Email: | +46 8 5554 2091 marie.dahlqvist@teracom.se +46 8 784 6511 lars.haglund@svt.se |
| Source: | Teracom AB, SVT Sveriges Television AB |  |  |

_____________________________

# Introduction

The interest for progressive High Definition Television Broadcasting is increasing, for example within the EBU (see Technical Information I34-2002) where the 720p format is being promoted to overcome the quality problems observed on PDPs for Digital TV. With the “very high-end” HDTV format for the future being 1080p, progressive techniques within broadcasting will be even more common in the future. The progressive techniques use 50 Hz and 60 Hz (59.94), which makes an addition to H.264 needed to also be able to use progressive frame rates, such as 24p, 25p and 30p.

# Proposed Text Changes

We propose two additions to Table D1 in annex D to be able to signal to the decoder to display progressive frames twice or three times, frame doubling and frame tripling.  These two additional modes will make it possible for the decoder to reconstruct 50p from 25p broadcasted and 60p (59.94) from 30p (29.97) using frame doubling and also 60p (59.94) from 24p (23.98), using frame doubling in combination with frame tripling on every other frame.

This addition will result in the following addition to Table D-1:

Table D-1 – Interpretation of pic_structure

| Value | Indicated display of picture | Restrictions | NumClockTS |
| --- | --- | --- | --- |
| 0 | frame | field_pic_flag must be 0 | 1 |
| 1 | top field | field_pic_flag must be 1, bottom_field_flag must be 0 | 1 |
| 2 | bottom field | field_pic_flag must be 1, bottom_field_flag must be 1 | 1 |
| 3 | top field, bottom field, in that order | field_pic_flag must be 0 | 2 |
| 4 | bottom field, top field, in that order | field_pic_flag must be 0 | 2 |
| 5 | top field, bottom field, top field repeated, in that order | field_pic_flag must be 0 | 3 |
| 6 | bottom field, top field, bottom field repeated, in that order | field_pic_flag must be 0 | 3 |
| 7 | frame doubling | field_pic_flag must be 0 | 2 (?) |
| 8 | frame tripling | field_pic_flag must be 0 | 3 (?) |
| 9-31 | reserved |  |  |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Teracom AB |  |  |
| Mailing address | PO Box 17666 SE-11892 Stockholm |  |  |
| Country | Sweden |  |  |
| Contact person | Marie Dahlqvist |  |  |
| Telephone | +46 8 5554 2091 |  |  |
| Fax | +46 8 5554 2082 |  |  |
| Email | marie.dahlqvist@teracom.se |  |  |
| Place and date of submission | Awaji Island, December 5 – 13 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | ## Frame Doubling and Frame Tripling |  |
| Contribution number |  | JVT-F041 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | SVT Sveriges Television AB |  |  |
| Mailing address |  | SVT Corporate Development Technology Oxenstiernsgatan 26 SE-105 10 Stockholm |  |  |
| Country |  | Sweden |  |  |
| Contact person |  | Lars Haglund |  |  |
| Telephone |  | +46 8 784 6511 |  |  |
| Fax |  | +46 8 784 8817 |  |  |
| Email |  | lars.haglund@svt.se |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: