| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F064 Filename: JVT-F064.doc |
| --- | --- |

| Title: | FCD Ballot Comment from SGNB |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal / Comments |  |  |
| Author(s) or Contact(s): | T.K.Tan (Acting HoD) CS Lim (MRTC MPEG TC,Singapore) | Email: Tel: Email: | ttk@pacific.net.sg +65-6482-5493 cslim@psl.com.sg, |
| Source: | SGNB |  |  |

_____________________________

The following are ballot comments submitted by the SGNB on the ISO/IEC FCD 14496-10, document number: ISO/IEC JTC 1/SC 29 N 4979.

As the ballot due date is the 2002-12-09, the Singapore National Body is submitting this document to the JVT meeting so that the comments can be dealt with earlier should the schedule permit.

Note that the comments are with respect to the Study of the FCD produced at the Geneva JVT meeting in October.  The document number is JVT-E146d37.

Subject 1

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| All | SGNB | T |  |  |

Context

There are many references to external specifications using or setting bits in the syntax of the specification.

The following is a list of examples:

forbidden_bit

There is no apparent usage of this bit except that it can be used to signal an error.  It is not clear how this will be helpful.  This specification does not specify the behaviour of a decoder when decoding a bitstream containing errors.  It does not affect the normative decoding.

nal_ref_idc

The usage by external specification does not affect the decoding of the picture.  Signaling of the priority in the transport is of no importance to the decoder unless it is specified.  If this information has reached the decoder the information is useless anyway as it has already been through the transport layer.

If the usage is not normative or specified, it could lead to confusion as different implementation will try to use it differently and this can lead to incompatibility and misinterpretation of the bits and therefore affect interoperability.

required_frame_num_update_behaviour_flag

aspect_ratio_idc = 0

Question

Should any reference to external specifications be made here?  If the specification specifies a value then it should be helpful for this specification.  The specification should not embed information that the decoder is supposed to use in a non-normative way.

Proposal

Signaling should be done by other means and not by embedding information in the bitstream syntax that does not affect the normative decoding of the picture.  Remove all descriptions pertaining to external standards/specification, which do not affect the behavior of the normative decoders or the behavior of the encoders related to the decoders.

Subject 2

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.1 | SGNB | E |  |  |

Context

nal_reference_idc is not used in the rest of the document

Proposal

Change all nal_reference_idc to nal_ref_idc.

Subject 3

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.2.1 | SGNB | T |  |  |

Context

mb_adaptive_frame_field_flag should be conditional on the frame_coding_only_flag.

Proposal

| frame_coding_only_flag | 0 | u(1) |
| --- | --- | --- |
| if( !frame_coding_only_flag ) { |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |

Subject 4

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.5 | SGNB | E |  |  |

Context

The function residual ( ) requires an argument

Proposal

Change to residual (mb_type)

Subject 5

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.1 | SGNB | T |  |  |

Context

1 bit is sufficient to indicate the context of nal_ref_idc however 2 bits is provided for in the syntax.

Question

What is the purpose of providing 2 bits?

Proposal

Change to nal_ref_flag

Subject 6

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1 | SGNB | T |  |  |

Context

Define the maximum value of num_ref_frames in relation to DPBcapacity defined in Annex A.

Proposal

Set maximum value of num_ref_frames to DPBcapacity – 1 in Annex A.  The reason is to include the working memory for the current picture (nal_ref_idc equal 1).  Due to the nature of the MMCO commands, which is applied after the decoding of the current picture, the current decoded picture need to be stored in DPB before applying the MMCO commands.  Therefore the maximum number of reference pictures in DPB should always be DPBcapacity minus one frame.

Question.

Does this affect the values defined in Annex A, if the maximum values for number of reference frames have reduced by one from DPBcapacity?

Subject 7

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.3.1 | SGNB | T |  |  |

Context

non_ref_pic_reset_flag should also be the same for all slices.

Question

Should these values be the same for both field pairs?

Proposal

The value of pic_parameter_set_id, frame_num, field_pic_flag, bottom_field_flag, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], non_ref_pic_reset_flag, sp_for_switch_flag, and slice_group_change_cycle shall be the same in all slice headers of a redundant or primary coded frame, field or field pairs. Moreover, the parameter values included in the dec_pic_buf_management( ) function shall remain unchanged in all slice headers of a redundant or primary coded frame, field or field pairs.

Subject 8

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.3.1 | SGNB | T |  |  |

Context

It is mandated that:

"

The value of pic_parameter_set_id, frame_num, field_pic_flag, bottom_field_flag, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], non_ref_pic_reset_flag, sp_for_switch_flag, and slice_group_change_cycle shall be the same in all slice headers of a redundant or primary coded frame, field or field pairs. Moreover, the parameter values included in the dec_pic_buf_management( ) function shall remain unchanged in all slice headers of a redundant or primary coded frame, field or field pairs.

"

Question

It may be desirable not to send these parameters in some cases.  The slice header is sent in two places:  slice_layer_no_partitioning_rbsp() and slice_data_partition_a_layer_rbsp().  Should the encoder be given the option to do this?

Proposal

It is proposed that an abbreviated slice header is sent in cases where the values of these parameters can be derived from slice headers that were previously sent.

Two possible ways of doing this is:

To create two new NAL unit types for abbreviated_slice_layer_no_partitioning_rbsp() and abbreviated_slice_data_partition_a_layer_rbsp() where the abbreviated version of the slice header is sent.

Send a flag in the slice header to indicate if it is an abbreviated slice header or not.  The following semantics may be used:

abbreviated_slice_header_flag indicates if the following syntax elements that must be the same for all slices are present in the slice header.  When the abbreviated_slice_header_flag is 0, the following syntax elements, field_pic_flag, bottom_field_flag, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], non_ref_pic_reset_flag, sp_for_switch_flag, and slice_group_change_cycle are present.  When the abbreviated_slice_header_flag is 1, the values of the above syntax elements are derived from the previously decoded slice header belonging to the same picture.  The abbreviated_slice_header_flag shall have a value of 0 for the first slice of a picture to be decoded and the first slice data partitioning A of a picture to be decoded.

Subject 9

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5 | SGNB | T |  |  |

Context

Table 7-11, the heading in the last column is incorrect.

Proposal

- Table 7-1 – Macroblock type values 0 to 4 for P and SP slices

| mb_type | Name of mb_type | num_mb_part ( mb_type ) | mb_part_pred_mode ( mb_type, 0 ) | mb_part_pred_mode ( mb_type, 1 ) |
| --- | --- | --- | --- | --- |

Subject 10

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.7.2 | SGNB | T |  |  |

Context

Sliding window decoded picture buffer management process is defined as below:

If N equals DPBcapacity, S shall be greater than zero, and the short-term reference frame that was decoded first is marked as “unused for reference”.

Proposal

Change the sentence to “If N equals DPBcapacity minus one, S shall be greater than zero, and the short-term reference frame that was decoded first is marked as “unused for reference”.”.  The reason is because N should be DPBcapacity minus one after the current decoded picture is added to the DPB.

Subject 11

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2 and 8.3.3 | SGNB | E |  |  |

Context

There is no such syntax element constrained_intra_pred.

Proposal

Use constrained_intra_pred_flag

Subject 12

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.4 | SGNB | T |  |  |

Context

This mode does not need I, J, K, L and M to be available

Proposal

Remove the constraint on I, J, K, L and M

Subject 13

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.8 | SGNB | T |  |  |

Context

This mode does not need I, J, K, L and M to be available

Proposal

Remove the constraint on I, J, K, L and M

Subject 14

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.9 | SGNB | T |  |  |

Context

This mode does not need A, B, C, D and M to be available

Proposal

Remove the constraint on A, B, C, D and M.

Subject 15

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.4.2.3 | SGNB | T |  |  |

Context

For case 3 or Figure 8-15, the second weight W is not equal to Z –256.

W = Z-256 = TDB x 256 / TDD  - 256 = (TDB –TDD) x 256 / TDD.  However from figure 8-15, the distance between List 1 and current picture is not (TDB – TDD).

Proposal

Calculate W as TDA x 256/ TDD where TDA is the DiffPicOrderCount (current_picture, ref_idx_l1) for this case.

Subject 16

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.2.3.2 | SGNB | T |  |  |

Context

The weighted prediction is broken for both luma and chroma prediction block when weighted_bipred_implicit_flag equal to 1.

- The chroma prediction block equation is broken.

- LWD and CWD should not be 7.

- The calculation is not constrained to 16-bit precision if one of weights exceeds +/- 128.

Proposal

Revise the weighted prediction equations for the case weighted_bipred_implicit_flag equal to 1.  Consider adopt JVT-F080 as the solution to the problem.

Subject 17

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.2.1 | SGNB | E/T |  |  |

Context

field_pic_flag and mb_adaptive_frame_field_flag (see the comment on 7.3.2.1) should be conditional on the frame_coding_only_flag, so frame_coding_only_flag should be restricted here.

Proposal

remove:

c) Pictures with field_pic_flag equal to 0

d) Pictures with mb_adaptive_frame_field_flag equal to 0

add:

c) frame_coding_only_flag equal to 1

Subject 18

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.2 | SGNB | T |  |  |

Context

The sequence of operations for the decoded picture buffer (DPB) verifier is defined differently from the sequence of operations for decoded picture buffer defined in 8.2.7.1.  The step to mark pictures as “unused for reference” (C.1.2.3) is later than the step to store pictures in DPB (C.1.2.2.1 and C.1.2.2.2).

Proposal

Change the order of C.1.2.3 with C.1.2.2 so that the sequence of operations of decoded picture buffer in Annex C is the same as defined in 8.2.7.1.

Subject 19

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.1 | SGNB | E |  |  |

Context

Duplicate semantics for seq_parameter_set_id

Proposal

Remove one of the semantics.

Subject 20

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.2 | SGNB | E |  |  |

Context

For semantics of cpb_removal_delay, the length in bits is given by cpb_removal_delay_length.  There is no such syntax element.

Proposal

Change to cpb_removal_delay_length_minus1+1