| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F067 Filename: JVT-F067.doc |
| --- | --- |

| Title: | Preliminary Results for Verification Test |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki and Yoichi Yagasaki Gate City Osaki East Tower 20F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

In MPEG Shanghai meeting, new HDTV materials of HDTV (1080i) was proposed and 4 sequences are adopted preliminary. In this contribution, preliminary encoding results for those materials are reported.

# New 1080i Materials

1080i test materials were distributed from Sony for the verification test of H.264|MPEG-4 AVC. The following four sequences were adopted as a candidate of the test materials.

| Name | Description |
| --- | --- |
| Kayak | Detail texture, Fast motion |
| Flamingo | Detail texture, Random motion |
| Mountain | Slow panning, Many trees |
| Cross Road | Detail texture, Random motion |

# Preliminary Encoding Results

We preliminary encoded those materials and report the results in this document.

## Test Conditions

Software: JM50 + Rate Control, JM50 Fixed QP

Entropy coding: CABAC

Adaptive Frame/Field Coding

Search Range +-39

RDOpt: On

Direct Spatial Mode

Number of Reference Frames: 3

## Results

(Remarks: Empty column will be filled by the meeting.)

|  | Bitrate (Mbps) | SNRY (db) | SNR Cb (db) | SNR Cr (db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| Cross road | 10 |  |  |  |  |
| Kayak | 10 |  |  |  |  |
| Flamingo | 10 |  |  |  |  |
| Mountain | 10 |  |  |  |  |
| Cross road | 20 |  |  |  |  |
| Kayak | 20 |  |  |  |  |
| Flamingo | 20 |  |  |  |  |
| Mountain | 20 |  |  |  |  |

Table 1: Preliminary encoding results

# Discussions

In the email discussion, it was suggested use wider search range for 1080i materials. However, the improvement was very small while encoding time is quite longer. The following is an example.

Sequence: Kayak

Cabac, MBAFF, Direct Spatial

1) SR: +-127, Ref frame: 4

0(I)	683792	33	33.7182

3(P)	593656	33	33.1181

1(B)	261344	35	30.5495

2(B)	299840	35	30.5894

6(P)	647624	33	32.9062

4(B)	282024	35	30.2761

5(B)	328976	35	30.2907

encoding time: 12000 sec/frame (20 days/150frames)

2) SR +-63, Ref frame: 3

0(I)	683296	33	33.7215

3(P)	597160	33	33.1409

1(B)	261560	35	30.5210

2(B)	300752	35	30.5665

6(P)	648632	33	32.9158

4(B)	283536	35	30.2768

5(B)	328888	35	30.2616

encoding time: 2800 sec/frame (5 days/150 frames)

Since the encoding time is quite long, SR+-127 is not a realistic value. Therefore, I propose to use +-63 or smaller search range. +-39 is a reasonable value in terms of encoding time.

There was also suggestion to encode at least 10 sec. (300 frames) However, even if we encode 150 frames, encoding time is quite long and it is not realistic to encode 300 frames. Rec. 500 recommends to use 10 sec for formal subjective test. Meanwhile, it is suggested that repeating 5 sec sequence two times could be alternative if 10 sec sequence is not available. Since this approach was used in formal verification test in past, I would like to propose to use 150 frames.

# Conclusions

H.264 | MPEG-4 AVC supports HDTV and the performance of codec should be tested using HDTV as well as lower resolution video. In this contribution, the preliminary results of new 1080i materials adopted as a candidate of the test material are reported. It is found during the encoding that large SR and long test sequence is not realistic in terms of encoding burden. The test conditions should be determined taking into account those issues.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F 1-11-1 Osaki, Shinagawa-ku, Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan, Oct. 4th, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Study of Direct Mode |  |
| Contribution number |  | E071 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: