| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F069 Filename: JVT-F069.doc |
| --- | --- |

| Title: | Consideration on Intra Prediction for Pipeline Processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato, Teruhiko Suzuki and Yoichi Yagasaki Gate City Osaki East Tower 20F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Fax: Email: | +81-3-5435-3669 +81-3-5435-3891 kazushi@av.crl.sony.co.jp teruhiko@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

- Introduction

As encoding/decoding of SD/HD sequences require more macroblocks to be processed, any kind of technique for speeding up is indispensable for implementation. Pipeline processing is an effective tool for that purpose. However, we need to introduce additional constraint of intra prediction modes at the encoder side, which will cause a loss in coding efficiency at a severe level.

In this contribution, we propose an additional method that enables us to encode/decode with pipeline at ignorable loss in coding efficiency.

- Problem Statement

In the current JVT standard, the decoding order of 4x4 blocks within a macroblock is as shown in Fig 2.1. Assume that block 1 in Fig 2.1 is now being processed. In the case that the decoding process is executed sequentially as shown in Fig 2.2(a), neighboring pixels I to L, which belong to block 0, are available as the decoding of block 0 is finished when the decoding of block 1 is started.

However, for the purpose of speeding up, the pipeline processing technique may be used as shown in Fig 2.2(b). In this case, neighboring pixels I to L will not be available for intra prediction when the decoding of block 1 is started.

Therefore we should employ constraint as shown in Table 2.1 and 2.1 at the encoder side, but it will cause a loss in coding efficiency.

Fig 2.1

(a)

(b)

Fig 2.2

Fig 2.3

| Location of 4x4 Block (in Fig 2. ) | Neighboring Pixels which are not available |
| --- | --- |
| 0,4,8,12 | N/A |
| 2,6,10,14 | Upper_Right (E,F,G,H) |
| 1,3,5,7,9,11,13,15 | Left (I,J,K,L) |

Table 2.1

| Intra_Pred Mode | Left Unavailable | Upper_Right Unavailable |
| --- | --- | --- |
| 0: Vertical |  |  |
| 1: Horizontal | X |  |
| 2: DC | X |  |
| 3: Diagonal_Down_Left |  | X |
| 4: Diagonal_Down_Right | X |  |
| 5: Vertical_Right | X |  |
| 6: Horizontal_Down | X |  |
| 7: Vertical_Left |  | X |
| 8: Horizontal_Up | X |  |

X = Unavailable Mode

Table 2.2

- Proposed Algorithm

First, we propose to change the decoding order of 4x4 blocks within a macroblock in zigzag-scan order, as shown in Fig 3.1. That is, 4x4LumaBlkIdx is defined as shown in Table 3.1

Fig 3.1

| 4x4LumaBlkIdx | 4x4LumaBlkLoc[4x4LumaBlkIdx][0] | 4x4LumaBlkLoc[4x4LumaBlkIdx][1] |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 1 | 0 |
| 2 | 0 | 1 |
| 3 | 2 | 0 |
| 4 | 1 | 1 |
| 5 | 0 | 2 |
| 6 | 3 | 0 |
| 7 | 2 | 1 |
| 8 | 1 | 2 |
| 9 | 0 | 3 |
| 10 | 3 | 1 |
| 11 | 2 | 2 |
| 12 | 1 | 3 |
| 13 | 3 | 2 |
| 14 | 2 | 3 |
| 15 | 3 | 3 |

Table 3.1

In addition, we introduce an additional flag “constrained_intra_flag_for_pipeline_processing” which will be contained either in sequence parameter set or in picture parameter set.

When constrained_intra_flag_for_pipeline_processing = 1 the intra prediction process will be changed as follows:

If the current 4x4 block being processed is located directly right to the previous 4x4 block, neighboring pixels I to L in Fig 2.3 will be replaced with the ones as shown in Fig 3.2(a) if they are available. This will be applied when 4x4LumaBlkIdx = 1 or 15.

Similarly, if the current 4x4 block being processed is located directly down-left to the previous 4x4 block, neighboring pixels E to H in Fig 2.3 will be replaced with the ones as shown in Fig 3.2(b) if they are available. This will be applied when 4x4LumaBlkIdx = 2, 4, 5, 7, 8, 9, 11, 12 or 14.

Fig 3.2

- Simulation

4.1 Simulation Condition

| Codec | JM50 (JM) JM50+Restriction in Table 2.1&2.2 (Const) JM50+Proposed Method (Prop) |
| --- | --- |
| Sequence | “F1” (720x576x25i, 220frames) “Canoa” (720x576x25i, 220frames) “Rugby” (720x576x25i; 220frames) “Tempete” (720x480x30i; 260frames) “Football” (720x480x30i; 260frames) [Specified as Interlace Test Sets] |
| GOP | I-pict Only |
| QP | 28,32,36,40 |
| EntropyCoding | CABAC |
| RDOptimization | On |
| UseHadamard | On |
| InterlaceCodingOption | 3 (MB-AFF) |

Table 4.1

4.2 Simulation Result

F1

JM:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.25 | 40.78 | 41.35 | 9563.7 |
| QP=32 | 33.27 | 39.29 | 39.72 | 5916.78 |
| QP=36 | 30.53 | 37.62 | 37.84 | 3466.89 |
| QP=40 | 28.26 | 37.1 | 37.6 | 1759.09 |

Const.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.16 | 40.79 | 41.35 | 10219.19 |
| QP=32 | 33.18 | 39.31 | 39.73 | 6408.46 |
| QP=36 | 30.46 | 37.61 | 37.83 | 3805.27 |
| QP=40 | 27.98 | 36.43 | 36.61 | 2180.8 |

Prop.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.25 | 40.78 | 41.35 | 9649.98 |
| QP=32 | 33.26 | 39.3 | 39.72 | 5954.55 |
| QP=36 | 30.53 | 37.63 | 37.83 | 3495.67 |
| QP=40 | 28.06 | 36.44 | 36.58 | 1890.35 |

Canoa

JM:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 37.06 | 41.13 | 41.72 | 7326.58 |
| QP=32 | 34.15 | 39.77 | 40.33 | 4732.73 |
| QP=36 | 31.38 | 38.2 | 38.73 | 2929.39 |
| QP=40 | 28.76 | 37.1 | 37.6 | 1759.09 |

Const.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 37.01 | 41.12 | 41.72 | 8331.1 |
| QP=32 | 34.08 | 39.76 | 40.33 | 5339.19 |
| QP=36 | 31.3 | 38.21 | 38.73 | 3232.13 |
| QP=40 | 28.69 | 37.11 | 37.6 | 1884.83 |

Prop.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 37.06 | 41.13 | 41.72 | 7498.1 |
| QP=32 | 34.15 | 39.76 | 40.33 | 4847.27 |
| QP=36 | 31.37 | 38.2 | 38.74 | 2991.48 |
| QP=40 | 28.74 | 37.11 | 37.6 | 1785.49 |

Rugby

JM:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.17 | 39.92 | 41.1 | 8860.77 |
| QP=32 | 33.32 | 38.51 | 39.67 | 5567.28 |
| QP=36 | 30.75 | 36.94 | 38.09 | 3356.26 |
| QP=40 | 28.45 | 35.86 | 36.97 | 1997.45 |

Const.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.06 | 39.93 | 41.11 | 9614.67 |
| QP=32 | 33.23 | 38.52 | 39.69 | 6070.1 |
| QP=36 | 30.66 | 36.94 | 38.1 | 3660.2 |
| QP=40 | 28.36 | 35.87 | 36.98 | 2156.02 |

Prop.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.16 | 39.92 | 41.1 | 8977.77 |
| QP=32 | 33.31 | 38.51 | 39.68 | 5645.15 |
| QP=36 | 30.75 | 36.94 | 38.08 | 3403.9 |
| QP=40 | 28.44 | 35.85 | 36.98 | 2022.73 |

Tempete

JM:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 35.29 | 36.97 | 37.85 | 10436.13 |
| QP=32 | 32.22 | 35.31 | 36.66 | 6724.84 |
| QP=36 | 29.27 | 33.73 | 35.43 | 4145.69 |
| QP=40 | 26.63 | 32.72 | 34.62 | 2546.74 |

Const.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 35.15 | 36.96 | 37.83 | 11127.43 |
| QP=32 | 32.09 | 35.31 | 36.64 | 7227.64 |
| QP=36 | 29.15 | 33.72 | 35.42 | 4477.66 |
| QP=40 | 26.54 | 32.71 | 34.62 | 2734.35 |

Prop.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 35.29 | 36.97 | 37.85 | 10547.43 |
| QP=32 | 32.22 | 35.32 | 36.66 | 6814.05 |
| QP=36 | 29.28 | 33.74 | 35.44 | 4215.1 |
| QP=40 | 26.63 | 32.72 | 34.62 | 2583.86 |

Football

JM:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.75 | 39.99 | 40.85 | 5329.59 |
| QP=32 | 34.26 | 38.47 | 39.54 | 3281.68 |
| QP=36 | 32.02 | 36.84 | 38.18 | 1947.6 |
| QP=40 | 30.01 | 35.52 | 37.28 | 1169.09 |

Const.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.68 | 39.99 | 40.84 | 5872.18 |
| QP=32 | 34.19 | 38.46 | 39.54 | 3636.28 |
| QP=36 | 31.95 | 36.82 | 38.18 | 2163.84 |
| QP=40 | 29.93 | 35.5 | 37.27 | 1282.9 |

Prop.:

|  | PSNRY | PSNRU | PSNRV | Bitrate@30Hz |
| --- | --- | --- | --- | --- |
| QP=28 | 36.75 | 39.99 | 40.84 | 5416.78 |
| QP=32 | 34.25 | 38.47 | 39.54 | 3332.42 |
| QP=36 | 32.02 | 36.83 | 38.19 | 1977.6 |
| QP=40 | 30 | 35.55 | 37.28 | 1187.69 |

Table 4.1

|  | Const. |  | Prop. |  |
| --- | --- | --- | --- | --- |
|  | BD-PSNR [dB] | BD-Bitrate [%] | BD-PSNR [dB] | BD-Bitrate [%] |
| F1 | -0.563 | 12.63 | -0.087 | 2.23 |
| Canoa | -0.679 | 12.79 | -0.133 | 2.31 |
| Rugby | -0.530 | 10.80 | -0.077 | 1.51 |
| Tempete | -0.570 | 9.69 | -0.084 | 1.37 |
| Football | -0.522 | 12.49 | -0.074 | 1.69 |
| Average | -0.573 | 11.68 | -0.091 | 1.822 |

Table 4.2

Fig 4.1

Fig 4.2

Fig 4.3

Fig 4.4

Fig 4.5

- Conclusion

Pipeline processing is an indispensable tool for implementing JVT encoder/decoder especially for SD/HD sequences. However, we need to introduce additional constraint of intra prediction modes at the encoder side for pipeline processing, which will cause a loss in coding efficiency at a severe level.

In this contribution we propose to change the decoding order of 4x4 blocks within a macroblock into zigzag-san. In addition we propose to introduce “constrained_intra_flag_for_pipeline_processing” which will go into either in sequence parameter set or picture parameter set. When constrained_intra_flag_for_pipeline_processing=1, the definition of neighboring pixels I to L and E to H will be changed so that pipeline processing will be enabled. The proposed algorithm will introduce an ignorable level of loss in coding efficiency.

References

[1] T.Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)”, Geneva modifications draft 37, JVT-E146d37ncm.doc, Nobember 17, 2002

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Kazushi Sato |  |  |
| Telephone | +81 3 5435 3669 |  |  |
| Fax | +81 3 5435 3891 |  |  |
| Email | kazushi@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | F069 |  |
| Title |  | Consideration on Intra Prediction for Pipeline Processing |  |
| Contribution number |  | JVT-E069 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| x | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: