| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F077 Filename: JVT-F077.doc |
| --- | --- |

| Title: | Simplification of the Weighted Prediction and Verification result |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal/Information |  |  |
| Author(s) or Contact(s): | Yoshihiro Kikuchi Takeshi Chujoh | Tel: Email: | +81 44 549 2288 yoshihiro.kikuchi@toshiba.co.jp |
| Source: | Toshiba |  |  |

_____________________________

# Introduction

After the JVT Geneva meeting in October, many comments of concerns were raised on the reflector to the weighted prediction in JVT-E060[1] and JVT-E146d37[2]. On the other hands, there were valuable comments for fixes, simplifications and possible improvement with minor modifications. In this contribution, we propose a simplification of the weighted prediction based on the proposals on the reflector with modifications; simplified implicit mode with single multiplication and combination of the implicit bi-prediction and the explicit single prediction. We also compare various methods for the implicit bi-predictive weighting including JVT-E146d37, JVT-E060 and the proposed simplified method. We evaluate the explicit mode to verify the performance compared to this combined mode.

# Simplified implicit mode

## Implicit bi-prediction with single multiplication

The implicit mode of JVT-E060 and JVT-E146d37 uses two multiplications per pixel to generate the prediction signal. This is to share the same operation with the explicit weighted bi-prediction. However, the prediction signal for the implicit mode is basically derived by single multiplication by adequately forming the equation like follows:

P = P0+(( P1- P0) ·W1 +2LWD-1)>>LWD

The equation above is arithmetically the same as the current implicit mode using two multiplications.

We propose an implicit mode prediction signal generation formula using only single multiplication per pixel. There was a similar proposal on the reflector, but slightly modified as follows to further simply the operation.

- Fixed amount shift operation

The variable shift operation is used in the proposal on the reflector. This is to derive the optimum precision, but it introduces a variable amount shift operation for each MC block. To simplify the operation, fixed amount shift operation is used in this contribution.

- Exception rule

To guarantee 16-bit precision calculation, some limitation is necessary on the range of the weighting factor. In the proposal on the reflector, the weighting factor is set to perform Pred_L1 in such case. In this contribution, the weighting factor to generate average of P0 and P1 is used. This is the same operation as the exception rule for the long-term memory, thus unify the operation.

The following is the formula for the proposal:

P = Clip1 (P0+(( P1- P0) ·W1 +64)>>7))

W1 is calculated as follows:

W1 = (256 · DiffPicOrderCount(ref_idx_l0, current_pic) / DiffPicOrderCount(ref_idx_l0 ,ref_idx_l1))>>1

except if DiffPicOrderCount(ref_idx_l0, ref_idx_l1) is equal to zero or if W1 is less than -128 or if W1 is more than 127 or if one or both reference pictures is a long term reference picture, in which case, W1 = 64.

W1 above is equal to W>>1 where W is the scaling factor for the temporal direct mode.

## Combination of explicit single WP and implicit bi-predictive WP

Recently, there was a proposal on the reflector to allow the combination of the implicit weighting for bi-prediction and the explicit weighting for the single prediction (Pred_L0 and Pred_L1). It was observed that the explicit weighting parameter suited for the single prediction would not be suited for the bi-prediction, and vice versa. This is an essential problem of the explicit mode of JvT-E060 and JVT-E146d37 that shares the weighting factor for the single and the bi-predictive predictions. It is possible to send multiple weighting factors for a reference picture by the reference index multiplication mechanism, but it will cost overhead to send the weighting table for each slice. This problem would be solved by introducing the combined mode of the implicit weighting for bi-prediction and the explicit weighting for the single prediction.

Both the single prediction weighting and the simplified implicit mode above requires only one multiplication. On the other hands, the explicit mode requires two multiplications per pixel. By removing the explicit weighting for the bi-prediction, two multiplications will be completely eliminated from the weighting prediction, thus much simplifies the implementation. This may affect the coding efficiency, but the degradation may not so much by introducing the combined mode

# Implementation consideration

Table 1 summarizes the necessary operations per pixel to calculate the prediction signal for the weighted bi-predictive prediction of various documents, software and proposals.

The implicit modes of JVT-E060 and JVT-E148d37 do not satisfy 16-bit operation, or harm the extrapolation when restriction is applied to the range of the weighting factor. The shift amount (LWD value) should be corrected adequately. W0 and W1 should be restricted within the range of +/- 128 to satisfy 16-bit operation. To allow scaling up to 2, LWD must be set to 5 for both JVT-E146d37 and JVT-E060.

JVT-E146d37 requires two more shift and two more addition operations than JVT-E060, jm50g and the explicit mode of the FCD. It is because rounding operations ((xx+64)>>7) are applied to both P0*W0 and P1*W1 terms. It was claimed that this equation eases to share the module for single and bi-predictive predictions. However, it is questionable because the bi-prediction requires two weighting operations per pixels while the single requires once, thus it requires twice the speed of the module for the sharing.

The implicit mode of the FCD (JVT-E022) requires no multiplication operation. This method does not require any division operation to derive the weighting factor whereas other method requires divisions of DPOC. If there is a serious problem on this division operation related to DPOC, we may choose this implicit mode.

To summarize the implementation consideration:

- JVT-E146d37 is worst in the number of operations (two multiplications, two more addition and shift operations than JVT-E060), while there is no advantage on the range of the weighting factor from JVT-E060 and the proposed simplified method.

- The proposed method requires only one multiplication per pixel while others except for the implicit mode of the FCD (JVT-E022) require two multiplications.

Table 1 Comparison of necessary operation for weighted bi-prediction

|  | equation |  | * |  | + - |  | >> |  | Clip |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FCD implicit | P=(P0+P1+1)>>1 |  | 0 |  | 2 |  | 1 |  | 0 |  |
| (JVT-E022) | P=clip1(2*P0 – P1) |  | 0 |  | 1 |  | 1 |  | 1 |  |
| FCD explicit (JVT-E022) |  | P=clip1(((P0*W0+P1*W1+2LWD)>> (LWD+1))+BDO) |  | 2 |  | 3 |  | 1 |  | 1 |
|  |  | CP=clip1(128+(((CP0-128)*CW0 +(CP1-128)*CW1+2CWD)>>(CWD+1))+CBDO ) |  | 2 |  | 6 |  | 1 |  | 1 |
| JVT-E060 | P=clip1(P0*W0+P1*W1+128)>>8 |  | 2 |  | 2 |  | 1 |  | 1 |  |
| JVT-E146d37 | P=clip1(((P0*W0+64)>>7) +((P1*W1+64)>>7)+1)>>1) |  | 2 |  | 4 |  | 3 |  | 1 |  |
|  | *CP=clip1((((P0-128)*W0+64)>>7+128) +(((P1-128)*W1+64)>>7)+128+1)>>1) |  | 2 |  | 6 |  | 3 |  | 1 |  |
| JM50g | P=clip1((P0*W0+P1*W1+128)>>8) |  | 2 |  | 2 |  | 1 |  | 1 |  |
|  | CP=clip1((CP0-128)*W0 +(CP1-128)*W1+128)>>8+128 |  | 2 |  | 5 |  | 1 |  | 1 |  |
| Proposed method | P=clip1(P0+((P1-P0)*W1+64)>>7) |  | 1 |  | 3 |  | 1 |  | 1 |  |

* “+128” is necessary

# Simulations

Computer simulations were conducted to evaluate the proposed method. Table 2 summarizes the simulation conditions. These follow the common test condition (VCEG-N81)[4]. Table 3 shows the test sequences and fade patterns used for the simulation. Various fade types were used; fade from black/white, linear/quadratic/S-curve, cross-fade.

Since there seems to be bugs and performance problem on the latest version of the JM (jm50g), we used two programs as listed in Table 4. Jm50g1 is basically based on JVT-E060. The combined mode is introduced in jm50f_wp where the proposed simplified implicit is used for the bi-prediction and the explicit weighting is used for the single prediction. This mode requires only one multiplication per pixel while the explicit mode requires two multiplications for weighted bi-prediction. Another major differences except for the stability are the implicit mode and the weight and offset detection algorithm for the explicit mode at the encoder.

Table 2  Test condition

| Base program | JM5.0g1/JM5.0f+WP |
| --- | --- |
| R-D optimization | used |
| MV search range | 32 |
| CABAC | used |
| Slice, Data partitioning, FMO | not used |
| Hadamard | used |
| Number of reference pictures | 5 |
| QP | 28, 32, 36, 40 |
| Picture type structure | IPPPPP, IBBPBBP, IBBPBBBs |

Table 3  Fade patterns and test sequences

| Fade type | Video sequence | Fade duration |
| --- | --- | --- |
| Linear fade from black | Container(QCIF 10Hz) News(QCIF 10Hz) Foreman(QCIF 10Hz) | 2 sec. |
| Quadratic curve fade from black |  |  |
| Sigmoid (S-curve) fade from black |  |  |
| Linear fade from white |  |  |
| Quadratic curve fade from white |  |  |
| Sigmoid (S-curve) fade from white |  |  |
| Cross fade | Container to News (QCIF 10Hz) | 2 sec. |

Table 4  Programs used for the simulation

|  |  | Implicit | Explicit |
| --- | --- | --- | --- |
| jm50g1 | modified version of jm50g (provided by Jill Boyce from Thomson) | JVT-E060 based (two multiplication) | JVT-E060 based |
| jm50f_wp | Toshiba’s implementation of Weighted Prediction based on jm50f | proposed method (single multiplication) | modified weight and offset detection (encoder) |

Detailed simulation results are summarized in the excel file (JVT-F077.xls).

The performance of three implicit modes, JVT-E146d37, JVT-E060 and the proposed are compared. (See sheet “implicit comparison in “ JVT-F077.xls) The proposed simplified method is slightly better than JVT-E146d37 with corrected LWD value=5 (0.3% in average). The differences among three implicit modes are not significant; advantage in some cases and disadvantage in other cases, with at most a few percents differences. No clear advantage was observed on JVT-E146d37 from others.

We also verified the proposed method with changing LWD=7 to 6 to allow wider range of scaling. The performance degradation is negligible of 0.09% in average and up to less than 1%.

Comparing the weight table detection algorithm for the explicit modes of the two programs, jm50f_wp outperforms jm50g1 in almost all sequences and fade types except for foreman, M=3. The average differences are about 25%(2.0dB) for PPPP and 6.2%(0.5dB) for PBBP. Especially, there are significant differences for fade from white of about 44% (3.4dB) for PPPP and 13% (0.9dB) for PBBP. There are also gains for cross fade, PPPP of about 15% (1.0dB). These are because the encoder of jm50f_wp detects both weight (W0 and W1) and offset (O0 and O1) in the weighting table whereas jm50g1 detects only weight (offset is set to zero).

The implicit mode of jm50g, where the weighted prediction is applied only to the bi-prediction, is worse than the explicit mode for non-linear fades; about 2.7%(0.2dB) in average and 11.9%(1.1dB) at maximum. On the other hands, the degradation of the combined mode in jm50f_wp is small or even better compared to the explicit mode requiring two multiplications. The combined mode outperforms the explicit mode about 4.7%(0.3dB) in average. The gain of the combined mode is large especially for fade from white and cross fade, regardless of the fade curve types (linear, quadratic or S-curve); 7.0%(0.47dB) for fade from white and 17.7%(1.1dB) for cross-fade. Degradation is observed for fade from black, especially for non-linear fade, but not significant as the implicit mode versus the explicit mode in jm50g1; 1.9%(0.1dB) in average.

The performance of BsBBBs structure was also evaluated. The gain of the implicit mode in jm50g1 is 24%(1.3dB) in average while the gain of the combined mode is 32%(1.9dB) in average. This verifies the effectiveness of the weighted prediction in Bs-picture.

The simulation results are summarized as follows:

- The performance difference between the implicit mode of JVT-E146d37 and the proposed simplified implicit mode as well as JVT-E060 is marginal.

- The combined mode of implicit for bi-prediction and explicit for single prediction performs best in average among all modes verified. The gain from the non-WP is high enough of ave:38%/2.4dB and max:53%/4.4dB. The explicit mode outperforms the combined mode for fade from black, but the gain is small (1.9%/0.1dB)

- The  explicit mode of jm50f_wp using both weight and offset in the weigh table outperforms that in jm50g1 where the offset is to zero, especially for fade from white and cross fade (up to 44%/3.4dB). This verifies the effectiveness of the explicit mode with offset values.

# Conclusion

We studied various methods for the weighted prediction signal generation method for the bi-predictive prediction. We found no advantage in performance on the method of JVT-E146d37 form others, while it requires two multiplications as well as two more addition and two more shift operations than those of JVT-E060 and the explicit mode of JVT-E022. Therefore:

- We seen no reason to adopt the weighted bi-predictive prediction signal generation method as in JVT-E146d37, instead of that in JVT-E060.

A simplified form of the implicit weighted bi-prediction using only one multiplication was proposed. The combined mode with the explicit weighting for the single prediction was also proposed and evaluated. This combined mode uses only one multiplication per pixel, thus much simplifies the implementation. Simulation result showed that that the overall performance of the combined mode is even better than the explicit mode except for small degradation for fade from black of 1.9% in average.

Therefore, we recommend the following:

- Adopt the implicit weighted bi-prediction with the proposed simplified form using only one multiplication.

- Allow combination of the explicit weighting for the single prediction and the implicit weighting for the bi-predictive prediction in B-slice.

- Remove the explicit weighting for the bi-predictive prediction requiring two multiplications per pixel.

It should be noted that the weighted prediction with the proposed simplifications still performs extremely high coding gains for all fade types (ave:38%(2.4dB), max:53%(4.4dB)) while much simplifies the implementation by eliminating two multiplications per pixel.

# References

[1] Jill Boyce, “Changes to Adaptive Reference Picture Weighting”, JVT-E060, Oct. 2002.

[2] Thomas Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Geneva modifications draft 37”, JVT-E146d37, Nov. 2002.

[3] Thomas Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Draft 7”, JVT-E022d7, Oct. 2002.

[4] G. Sullivan, G. Bjontegaard, "Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material", VCEG-N81, Sep. 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | TOSHIBA CORPORATION |  |  |
| Mailing address | 1, Komukai Toshiba-Cho, Saiwai-Ku, Kawasaki 212-8582, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Yoshihiro Kikuchi |  |  |
| Telephone | +81 44 549 2288 |  |  |
| Fax | +81 44 520 1267 |  |  |
| Email | yoshihiro.kikuchi@toshiba.co.jp |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: