| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F080 Filename: JVT-F080.doc |
| --- | --- |

| Title: | Implicit Weighted Prediction using Single Weight |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chong-Soon Lim Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Teck-Wee Foo Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Sheng-Mei Shen Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Shinya Kadono 1006 Kadomo, Kadomo, Osaka 573-8501 Japan | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | (65) 6550-5487 cslim@psl.com.sg (65) 6550-5474 twfoo@psl.com.sg (65) 6550-5466 shen@psl.com.sg +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Panasonic Singapore Laboratories Pte. Ltd. |  |  |

_____________________________

# Summary

The objectives of this proposal are to address the problems of the implicit mode adaptive reference picture weighting described in the document JVT-E146d37 and provide possible solutions to the problems.  The proposal JVT-E060 was adopted in the last meeting, which describes a method of adaptive reference picture weighting based on the picture distances.  However there are some problems with the equations involved and these have been reflected on the reflector.  A simplified equation for implicit mode adaptive reference picture weighting for both luma and chroma is proposed in this document.

# Problems

The current implicit mode adaptive reference picture weighting is described below:

When bi-prediction is used, the luma prediction block is generated as:

P = Clip1 (((P0·W0 +2 LWD-1)>>LWD)  + O0 + ((P1·W1 + 2LWD-1) >> LWD) + O1 + 1)>>1)	       (1)

If weighted_bipred_implicit_flag is equal to one,

LWD = 7

O0 = 0

O1 = 0

W0 = (128* DiffPicOrderCount(ref_idx_l1, current_pic) / DiffPicOrderCount (ref_idx_l1, ref_idx_l0)

W1 = (128* DiffPicOrderCount(current_pic,ref_idx_l0) / DiffPicOrderCount(ref_idx_l1 ,ref_idx_l0)

except if DiffPicOrderCount(ref_idx_l1,ref_idx_l0) is equal to zero or if one or both reference pictures is a long term reference picture, in which case, W0 = W1 = 128.

When bi-prediction is used, the chroma prediction block is generated as

P = Clip1 (((((CP0 – 128) · CW0 +2 CWD-1)>>CWD)  + CO0 +
	   (((CP1– 128) · CW1 + 2 CWD-1) >> CWD) + CO1 + 1) >> 1)			       (2)

where

CP0 = chroma reference prediction block from list 0

CP1 = chroma reference prediction block from list 1

If weighted_bipred_implicit_flag is equal to one,

CWD = 7

CO0 = 0

CO1 = 0

CW0 = (128* DiffPicOrderCount(ref_idx_l1, current_pic) / DiffPicOrderCount (ref_idx_l1, ref_idx_l0)

CW1 = (128* DiffPicOrderCount(current_pic,ref_idx_l0) / DiffPicOrderCount(ref_idx_l1 ,ref_idx_l0)

except if DiffPicOrderCount (ref_idx_l1, ref_idx_l0)  is equal to zero or if one or both reference pictures is a long term reference,  in which case, CW0 = CW1 = 128.

Problem 1:

The values of CWD and LWD should be 6.  The reason is because if the W1 and W0 are both 64, P should be approximately equal to (P1+P0)/2.  Similarly, if DiffPicOrderCount(ref_idx_l1,ref_idx_l0) is equal to zero or if one or both reference pictures is a long term reference picture, in which case, W0, W1, CW0 and CW1 should be 64 and not 128.

Problem 2:

The equation for chroma prediction block is broken.  The correct equation should be:

P = Clip1 ((((((CP0 – 128) · CW0 +2 CWD-1)>>CWD)  + CO0 +
	   (((CP1– 128) · CW1 + 2 CWD-1) >> CWD) + CO1 + 1) >> 1)+128)		       (3)

Problem 3:

The intermediate operations of equation (1) and (2) will exceed 16 bits if the weights are greater than +/- 128.  This is very common in the extrapolation case for bi-predictive weighting.  Figure 1 shows the case where W1 is greater than 128.

Prediction block 1 (P0)

Prediction block 2 (P1)

T0

T1

T

Combined block  (P)

P1*W1

P0*W0

.

# Proposed Solution

Part 1:

We would like to propose a simplified equation for the implicit mode adaptive reference picture weighting.  Based on the equation of W0, it is possible to prove the relationship of W0 and W1.

W0 = 128*(T1-T)/(T1-T0) = (128*(T1-T0+T0-T))/(T1-T1)

= 128 – W1.

Where W1 = (128*(T-T0))/(T1-T0), T1 is the POC of list 1 reference picture, T0 is POC of list0 reference picture and T is POC of current picture.

Therefore it is possible to simply equation (1) and (3) to form equation (4) and (5), respectively.

P = Clip1 (P0·+ ((P1-P0)·W1 + 64) >> 7))						       (4)

P = Clip1 (CP0·+ ((CP1-CP0)·W1 + 64) >> 7))					       (5)

W1 = (256*(T0-T)/(T0-T1))>>1							       (6)

We would like to change the definition of W1 to equation 6 so as to use a similar weight as defined in temporal direct mode.

Part 2:

To solve problem 3, some constraints need to be applied to prevent W1 to be greater than 127 or less than –128.  Thus we proposed to add an additional constraint that if the calculated W1 is greater than 127 or less than –128, W1 is defined as 64.

# Conclusion

A solution to the problems for the current implicit mode adaptive reference picture weighting is presented in this document.  The proposed solution simplifies the equations for the implicit mode adaptive reference picture weighting and thus reduces the complexity needed.  Therefore we would like to propose to adopt these changes to solve the existing bugs in adaptive reference picture weighting.

# References

[1]	Thomas Wiegand, “Editor's Proposed Draft Text Modifications for Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), Geneva modifications draft 37”, JVT-E146d37, JVT Meeting, Geneva, Switzerland, October 2002.

[2]	Jill Boyce, “Changes to Adaptive Reference Picture Weighting”, JVT-E060r1, JVT Meeting, Geneva, Switzerland, October 2002.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9699 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Implicit Weighted Prediction using Single Scaled Weight |  |
| Contribution number |  | JVT-F080 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: