| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F085 Filename: JVT-F085.doc |
| --- | --- |

| Title: | # Unified Interpolation Scheme within H.264/AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter List | Tel: Email: | +49 6151 83-3878 Peter.List@t-systems.com |
| Source: | Deutsche Telekom, T-Systems |  |  |

_____________________________

The Status:

At several places throughout H.264/AVC integer averaging is used. For instance, when calculating Intra predictors, we do use rounding. For instance:

8.5.1.6	Mode 5: vertical-right prediction

## This mode shall be used only if all A, B, C, D, I, J, K, L, M are

inside the slice.  This is a 'diagonal' prediction.

–	a, j are predicted by 			(M + A + 1)  1

–	b, k are predicted by			(A + B + 1)  1

when calculating non weighted bi-prediction, we also use rounding:

##### 8.4.2.3.1  Default weighted sample prediction process

…

For macroblocks or partitions coded using bi-prediction, including skipped macroblocks and direct mode macroblocks or partitions, if weighted_bipred_explicit_flag is equal to zero and weighted_bipred_implicit_flag is equal to zero, the two predictions are combined into a single predictor using the following formula.

P = ( P0 + P1 + 1 ) >> 1

where

P0 = list 0 reference prediction block [Ed. Note: Not a defined term.  What does it mean?]

P1 = list 1 reference prediction block

while for quarter sample luma interpolation" we do not use rounding:

8.4.2.1 Quarter sample luma interpolation"

....

–	The samples at quarter sample positions labelled as

‘a’, ‘c’, ‘d’, ‘n’, ‘f’, ‘i’, ‘k’ and ‘q’ shall be obtained by averaging with

truncation the two nearest samples at integer and half sample positions

using: a=(G+b)1, c=(H+b)1, d=(G+h)1, n=(M+h)1, f=(b+j)1,

i=(h+j)1,  k=(j+m)1 and q=(j+s)1.

....

_____________________________

This seems a very minor issue, since from a mathematical point of view the accuracy is the same either way. Still, things should be done consistently throughout the whole document.

Besides the consistency point there is an other more important one. Most (maybe all) of the general purpose processors or DSP’s on the market, that provide an averaging instruction within their instruction set, calculate:

reg0 = (reg1 + reg2 + 1) / 2

For instance, the Intel-Pentium processor has an MMX instruction called “pavgb”, calculation the above including the rounding 1. Since one MMX register carries 8 byte, these 8 byte can be averaged in one cycle.

Doing the same with non dedicated instructions gets difficult, since the intermediate result (reg1 + reg2) has 9 bit and can therefore not be calculated in parallel for 8 bytes, but only for 4 bytes. Additionally unpack/pack operations are needed to unpack the 8 byte pixels into a 16 byte reprensentation and back. The whole code function becomes quite complex and ends up to be at least 6 times slower than using the dedicated averaging instruction.

An other issue is the fact, that up to now “quarter-pel interpolation” and “non weighted bi-prediction” have exactly the same algorithm except for the rounding 1. So, if not for this difference, an identical function or piece of hardware could be used for these two operations.

The Proposal:

It is proposed to include the “rounding 1” in all averaging operations in H264/AVC, and in particular in the quarter-pel interpolation.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

Submitting Organization or Person:. Organization name; Deutsche Telekom. Mailing address; Am Kavalleriesand 3 64859 Darmstadt. Country; Germany. Contact person; Peter List. Telephone; +49 6151 83-3878. Fax; +49 6151 83-4842. Email; Peter.List@t-systems.com. Place and date of submission. Relevant Recommendation ¦ Standard and, if applicable, Contribution:. Name (ex: “JVT”); AVC/JVT. Title; # Unified Interpolation Scheme within H.264/AVC. Contribution number; JVT-F085.

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: