| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 6th Meeting: Awaji, Island, JP, 5-13 December, 2002 | Document: JVT-F089 Filename: JVT-F089.doc |
| --- | --- |

| Title: | USNB Comments on the MPEG-4 Part 10 AVC FCD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | National Body Comments |  |  |
| Author(s) or Contact(s): | On behalf of the United Status National Body Ali Tabatabai Toby Walker | Tel: Email: | ali.tabatabai@am.sony.com toby.walker@am.sony.com |
| Source: | USNB |  |  |

_____________________________

# Introduction

Note: This is a draft of USNB comments. The final version and official USNB comments will be submitted through the SC29 Secretariat.

The USNB National Body is pleased to submit its comments on the Final Committee Draft for Advanced Video Coding (ISO/IEC 14496-10), M4920). The comments in this document follow the template in N3326.

The USNB submits its comments on the FCD text in section 2. These comments were created prior to the Geneva and have not been modified since that time. Where the Geneva Study has already addressed these comments, they have been so marked.

Further, based on the considerable technical improvements made in the Geneva Study, (JVT-E146d37ncm.doc), the USNB requests that the Awaji meeting adopt the changes made in that study and use that document as the basis for the FDIS document to be created at the JVT Awaji meeting.

Moreover, after reviewing the Geneva Study text, and based on its recommendation to adopt that text as the starting point for the Awaji meeting, the USNB submits additional comments on issues found in the Geneva text (JVT-E146d37ncm.zip) in section 3.  The USNB comments in section 3 should be treated as modifications to the request to adopt the Geneva text as the starting point in Awaji.

Note that there may be some overlap between the FCD and Geneva Text comments and that some FCD comments were already addressed in the Geneva Study.

# Comments on the ISO/IEC 14496-10 FCD

This section contains USNB comments based on the ISO/IEC 14496-10 FCD ballot text. All references to sections are made with respect to that document.

## General Comments

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| All |  | M | USNB-FCD-1 | Note: Addressed in Geneva Study. |

Context

Whole standard.

Question

Features not in profiles should be removed. Examples include 1/8 pel motion interpolation.

Proposal

Remove 1/8 pel motion interpolation, intra at 8x8 level (for predicted macroblocks), bi-prediction below 8x8.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| All |  | T | USNB-FCD-2 |  |

Context

Whole standard.

Question

What provisions are made for future extensions and modifications of the standard that will support backward compatibility?

Proposal

Ensure that SEI messages or some equivalent supplemental information can be defined in the future with a scope applicable to sequence, picture and slice levels in a backward compatible way.

## Clause 0 Introduction

## Clause 3: Definitions

## Clause 6: Source coder

## Clause 7: Syntax and semantics

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.2.9 | USNB | T | USNB-FCD-3 | Note: Addressed in Geneva Study. |

Context

Cabac_stuff_bit---this close relates to the bit-to-bin ratio in CABAC, which is actually a constraint for CABAC.

Question

Current FCD says, “When entropy_coding_mode is equal to 1, the number of bins resulting from decoding the contents of the slice layer NAL unit shall not exceed 0.5*NumBytesInNALunit”.

This is not consistent with the adopted proposal JVT-D019, according to which the average bit-to-bin ratio in a slice should be upper bounded by 4. The current value gives an upper bound of 1/16, which is wrong.

Proposal

Change to, “When entropy_coding_mode is equal to 1, the number of bins resulting from decoding the contents of the slice layer NAL unit shall not exceed 32*NumBytesInNALunit”.

And accordingly change “Ebins=2*Ebins” to Ebins=Ebins/4” in the pseudo-code.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.5 | USNB | E | USNB-FCD-4 | Obsolete |

Context

Table is missing closing parenthesis in statement immediately before dq_quant

Proposal

Add parenthesis

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.5.1 | USNB | T | USNB-FCD-5 |  |

Context

Macroblock prediction syntax.

It is not stated that the j indices for mvd_l0[i][j] and mvd_l1[i][j] correspond to j=0 is the the horizontal component of the vector and j=1 is the vertical component of the vector.

Proposal

Either rewrite the syntax to make this clear or specify it in the semantics in clause 7.4.5.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.5.2 | USNB | T | USNB-FCD-6 |  |

Context

Sub macroblock prediction syntax.

It is not stated that the k indices for mvd_l0[i][j][k] and mvd_l1[i][j][k] correspond to k=0 is the the horizontal component of the vector and k=1 is the vertical component of the vector.

Proposal

Either rewrite the syntax to make this clear or specify it in the semantics in clause 7.4.5.2.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4 | USNB | E | USNB-FCD-7 | Obsolete |

Context

Semantics section, subclause 7.4, and decoder section 8.2.1.

Question

A subclause 8.2.1 was added to attempt to describe some bitstream constraints, but this is in the wrong place.  It should probably be part of subclause 7.4.1.

Proposal

It is proposed to move 8.2.1 into 7.4.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.1 | USNB | T | USNB-FCD-8 |  |

Context

High-level bitstream organization

Question

The philosophy behind the VCL/NAL split is not adequately described.  The distinction is not mentioned at all in 8.2.1, which lists constraints on bitstream order, even the construction of the VCL is quite different from that of the NAL.

Proposal

Provide an explanation of the split in 7.4.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.1 | USNB | E | USNB-FCD-9 | Note: Addressed in Geneva Study. |

Context

Paragraph below table 7-1.  2nd Sentence: "An IDR picture contains only I or SI slices, and IDR slice type shall be used for all slices of an IDR picture".

Proposal

There is no “IDR slice type”. Assume this intends to say “IDR NAL Unit Type”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1, 7.4.2.2 | USNB | M | USNB-FCD-10 |  |

Context

Whole section

Question

Many of the parameters in both the picture and parameter sets, and in the slice header are currently coded using variable length codes, meaning that these values are unbounded. For example, the value of MAX_FN could require a decoder to support 64 or more bit frame numbers. It also means that the size of some information in a slice header is also unbounded.

Proposal

Define limits for the values of syntax elements. For syntax elements that have only a few defined values, there should be an explicit statement saying that all other values are reserved.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1, 7.4.2.2 | USNB | T | USNB-FCD-11 |  |

Context

Definitions of seq_parameter_set_id and pic_parameter_set_id. : Undefined/ambiguous decoder behavior.

Question

What happens when a new sequence or picture parameter set is received that uses the same set ID value as a previously received parameter set (including possibly the currently used set).  Is it legal?  Does it automatically replace it?  What are the editing considerations here?  Do parameter sets need to be bundled with all IDR pictures so that editors (which aren't JVT aware) don't bundle separately encoded sequences together without their proper parm sets?

Are sequences parameters set IDs greater than 15 allowed? (same for IDs greater than 63 for picture parameter sets)

Proposal

Clarify use of parameter sets.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5 | USNB | T | USNB-FCD-12 | Note: Addressed in Geneva Study. |

Context

Mixing of intra/inter coding in the same MB.

Question

Is a mixture of inter and intra coded blocks allowed with a single MB? In the CD, mixed intra/inter 8x8 blocks was indicated as not supported in any profile in annex A. In the FCD, there seems to be no comment indicating definitely whether this is or is not allowed.

Proposal

Explicitly disallow mixing of inter and intra 8x8 blocks within the same macroblock by removing corresponding syntax.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5 | USNB | E | USNB-FCD-13 | Note: Addressed in Geneva Study. |

Context

Second bullet below Table 7-10: “Imode numbers from 6 and upwards represent 16x16 intra coding”

Proposal

This is not true. Error noted in JVT-E022d7

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5.1 | USNB | T | USNB-FCD-14 | Note: Addressed in Geneva Study. |

Context

“Reconstructed motion vectors shall be clipped to +/- 19 integer samples outside of the picture”

Question

Implies clipping vectors for motion comp AND(?) for predicting of future motion vectors. SW does not clip for predicting future vectors

Proposal

Remove the restriction.

## Clause 8: Decoding Process

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8 | USNB | M | USNB-FCD-16 |  |

Context

Whole section

Question

There is no overview of the decoding process indicating how the various decoding processes (transforms, dequantitization, deblocking filter), are combined together. Most of the technical content describing the decoding process is missing here.

Proposal

Rewrite section including an introduction and description of a generic decoder architecture (block diagram). Example:

An incoming macroblock in the bitstream is either coded using spatial prediction (I) or motion-compensated (MC) prediction (P or B). I macroblocks must undergo intra-prediction decoding. Predicted frames or blocks must undergo motion vector prediction decoding using data from buffered frames. In either case, these blocks are first entropy decoded (VLC or CABAC), scaled/transformed, prediction updated (either by I-prediction or MC), and sent to video out. Decoder control information (not shown in the diagram) flows from the entropy decode to all boxes. Decoding of SP/SI frames is not fully accounted for in this diagram (see clause xxx).

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.1 | USNB | T | USNB-FCD-17 | Note: Addressed in Geneva Study. |

Context

Order of decoding process

Question

FCD states the order as the following

1.	Parsing of syntax elements using VLC/CAVLC (see subclause Error: Reference source not found) or CABAC (see subclause Error: Reference source not found)

2.	Motion compensation (see subclause Error: Reference source not found) or Intra prediction (see subclause Error: Reference source not found)

3.	Transform coefficient decoding (see subclause Error: Reference source not found)

4.	Deblocking Filter (see subclause Error: Reference source not found)

Item 3 above refers to subclause 8.6 and mainly handles inverse quantization and transforms. Except 8.6.3, which adds the residue to the prediction, the rest of 8.6 can be parallel or prior to item 2. So it’s not proper to specify the order as it is now.

Proposal

Add a statement at the beginning of the decoding process section to clarify that:

The decoding process is defined such that all decoders shall produce numerically identical results. Any decoding process that produces identical results to the process described here, by definition, conforms to this specification.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2.1 | USNB | T | USNB-FCD-18 | Note: Addressed in Geneva Study. |

Context

Mode 0 and error resilience.

Question

In mode 0, the decoder needs to maintain the correct value of PicOrderCntOffset in order correctly decode picture order count. If more than two or more picture are not decoded (through error loss or through the ability to skip frames provided by enhanced GOPs) then it possible for the value of PicOrderCntOffset to become incorrect. Unlike POC mode 1, this can happen even if the pictures are not stored. This breaks the property of the AVC decoder design that the decoder does not need to decode non-stored frames to correctly maintain its state.

Proposal

Fix POC mode 0 so that it does not require non-stored frames to be decoded to maintain the correct value of PicOrderCntOffset.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2.2 | USNB | T | USNB-FCD-19 | Note: Addressed in Geneva Study. |

Context

POC mode 1

Question

The following problems exist with the following definition:

- The calculation of POC does not handle IDR frames correctly. The value of PicOrderCnt for IDR frames is always equal to offset_time_delta[0]. It should be zero.

- The calculation of POC for a non-stored frame should be the frame number of the immediately prior frame number when calculating the expected picture order count (i.e. FN-1 for the non-stored frame).

Proposal

Fix the calculations.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2.2 | USNB | T | USNB-FCD-20 |  |

Context

Initialization of repeated values.

Question

The current design of POC is designed so that regular frame structures can be coded extremely efficiently. However, when common frame structures like IPBBPBBP (assuming all frame stored) there are initial irregularities

Proposal

Add support for signaling initial POC delta offsets for use until the cycle of repeated POC deltas is used.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2 | USNB | E | USNB-FCD-21 | Note: Addressed in Geneva Study. |

Context

Interlace coding and POC

Question

Is it not clear that two fields for the same frame can have different values of POC.

Proposal

Add a note or comment that for interlace coding the POC of the top and bottom can be different and indicates the output order of those two fields with respect to each other.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2 | USNB | E | USNB-FCD-22 | Note: Addressed in Geneva Study. |

Context

POC Overview

Question

It is difficult to understand the use of the picture order count in the JVT codec from the short introduction in this section (first paragraph).

Proposal

Add a clearer description of the concept of a POC. An example would also help. If possible, an informative comment on when to use either mode 0 or mode 1 should be included.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.1 | USNB | E | USNB-FCD-23 |  |

Context

Median prediction of motion vectors.

Question

The description of media prediction is incomplete and difficult to understand. In particular, the result obtained depends on the order in which the rules are applied but the order is not clear.

Proposal

State the order in which the rules apply, e.g. that the fourth rule shall be applied after the first three. Clarify the median prediction for MB AFF.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.5.1.6 8.5.1.8 | USNB | T | USNB-FCD-24 | Note: Addressed in Geneva Study. |

Context

Section headers are wrong.  Mode 5 = vert-right, Mode 7 = vert-left.

Proposal

Fix. Error noted in JVT-E022d7

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.5.2.4 | USNB | T | USNB-FCD-25A | Note: Addressed in Geneva Study. |

Context

Section headers are wrong.  Mode 5 = vert-right, Mode 7 = vert-left.

Proposal

Fix. Error noted in JVT-E022d7

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.6.2.2 | USNB | T | USNB-FCD-25 | Note: Addressed in Geneva Study. |

Context

Equations 8-46 and 8-47 (8-42 and 8-43 in E022d7) are incorrect in stating that the range for i,j = 0..3. (2x2 chroma DC block)

Proposal

Change range to i,j = 0,1

## Clause 9: Entropy coding

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.1.5.2 | USNB | E | USNB-FCD-26 |  |

Context

Incorrect reference to section 9.2.1.6

Proposal

The correct reference (in the FCD) is Table 7-14 in section 7.4.5.  Also, the final sentence of this section should clarify that the ac_flag is set only for the 16 blocks of ac coefficients, and not for ac coefficients present in the 4x4 DC coefficient block associated with the MB.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.1.6 | USNB | E | USNB-FCD-27 |  |

Context

Paragraph after the numbered list – incorrect reference to section 9.4.1

Proposal

Correct reference is 8.6.1

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.1.6.2 | USNB | E | USNB-FCD-28 |  |

Context

Clarification: value of N (number of coeffs) does not include DC coeffs in Intra 16x16 and Chroma MBs, because they are coded separately.  So in those modes N=# of coded AC coeffs, not purely the number of residual coeffs, which the text seems to imply.

Proposal

Clarify value of number of coefficients.

## Clause 10: Decoding process for B-slices

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 10.3.3.1 | USNB | T | USNB-FCD-29 |  |

Context

Spatial technique of obtaining the direct mode motion parameters

Questions

The third paragraph is quoted, “If a preliminary candidate reference index exists for either list, decision of the final candidate reference indices and associated motion vector values for that list for each 4x4 block of the current macroblock depends on the coded parameters of the co-located 4x4 blocks in the first picture in list 1. If the first picture in list 1 is a short-term picture and if all lines of the co-located 4x4 block were predicted using list 0 prediction with reference picture index 0 and motion vector components in the range of -1 to 1 inclusive, then the final candidate reference picture index for the list of the current 4x4 block shall be 0 and shall be associated with a candidate motion vector value of (0, 0). Otherwise, the final candidate reference picture index for the list shall be the preliminary candidate reference picture index for the list and the associated candidate motion vector shall be obtained, using the 16x16 block motion vector prediction, as described in subclause Error: Reference source not found by using the final reference picture index.”

Two questions:

- The first sentense, as being bold-highlighted above, seems to indicate that MVs are to be derived for each 4x4 blocks. However, the last sentense containing “using the 16x16 blcok motion vector prediction” is confusing because it seems to say the MV is for the whole macroblock.

- For the sentense containing “motion vector components in the range of -1 to 1 inclusive”, is the MV component in integer pixel unit or in ¼-pel unit?

Proposal

FCD needs to clarify on these two points specifically.

## Clause 11: Decoding process for SP and SI slices

## Clause 12: Adaptive block size transforms

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.2 | USNB | M | USNB-FCD-30 | Note: Addressed in Geneva Study. |

Context

General

Question

There are still concerns about the performance of ABT in relation with other tools. Furthermore the development of a 16 bit implementation of ABT transforms is desirable.

Proposal

- Re-verify the performance of ABT with the latest reference software.

- Look at the results of Alternate scan ad-hoc group. If Alternate Scans shows significant improvement then re-evaluate the gains obtained by ABT over the case when Alternate scan is used in non-ABT situation.

- Clarify the impact of Quantization bias for predictive frames (1/3 for Non-ABT and 1/6 for ABT) used in the reference software

- Evaluate the performance of ABT when MB-AFF is used.

- Consider 16 bit implementations of ABT transforms.

## Annex A: Profile and level definitions

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A | USNB | M | USNB-FCD-31 |  |

Context

Profile definition

Question

We should have Baseline defined such that it is common to ALL other profiles.

Proposal

New Baseline  = Current Baseline minus (ASO+FMO+Redundant Slices)

Profile W = Current Baseline

Profile X = No Change

Main = No Change

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.2 | USNB | T | USNB-FCD-32 |  |

Context

Minimum compression ratio.

Question

The minimum compression ratio of 4:1 may be too high for I-frames.

Proposal

Consider decreasing ratio to 2:1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.2 | USNB | T | USNB-FCD-33 |  |

Context

Maximum bitrate @L4.

Question

The maximum bit rate for Level 4 may not be appropriate for applications that require “short-GOP” and/or visual transparency.

Proposal

Consider raising the maximum bit rate.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.2 | USNB | T | USNB-FCD-34 |  |

Context

Level 5.

Question

Maximum bit rate, vertical motion vector range, and maximum HRD buffer for Level 5 are not defined.

Proposal

Either define appropriate numbers or remove level 5.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annex A&C | USNB | T | USNB-FCD-35 |  |

Context

HRD & Levels

Question

It is not clear to what entity the HRD related Level constraints apply, since the HRD can be applied to both the VCL and the NAL.

Proposal

Specify that the Annex A bitrate and buffer size limits apply to the VCL and that the NAL bitrate limit may be 1.2x the VCL bitrate limit.    Also specify that the NAL buffer size limit is bigger than the VCL buffer limit (by an amount tbd).

## Annex B: Byte stream format

## Annex C: Hypothetical reference decoder

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annex C, D, E | USNB | T | USNB-FCD-36 |  |

Context

HRD

Question

Conformance of a bitstream to the HRD is mandatory, unless both the nal_hrd_flag and vcl_hrd_flag are equal to ‘0’.  But carriage of HRD information is currently optional.  This may encourage stream producers to create non-conformant streams.

Proposal

Insert text to the semantic constraints subclause (currently 8.2.1) stating that carriage of HRD information is mandatory in the byte stream format, and indicating where it should be placed.  For VCL conformance, HRD information may be supplied “out of band.”

Clean up “bitstream” and “byte stream” terminology.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annex C, D, E | USNB | E | USNB-FCD-37 |  |

Context

HRD

Question

Annex E refers in a general way to default values for HRD parameters.  But for some HRD parameters, default values are not appropriate.  Needs to be cleaned up.

Proposal

Change “each” to “some” in the second paragraph of E.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.2 | USNB | M | USNB-FCD-38 |  |

Context

Post-decoder buffer.

Question

The management of the reference picture buffer and the post-decoder buffer is not clear. For example, this section states that "Picture memory is marked unused in the post–decoder buffer when it is virtually displayed or when it is no longer needed as a reference picture". What does "no longer needed as a reference picture" mean?

The operation of the post-decoder buffer does not use the use of exact decoding times as specified  in the HRD. Use of picture order counts is sufficient for the encoder to determine whether the stream can be reordered within the limited of the "reordering buffer".

Proposal

Clarify the semantics and operation of the post-decoder buffer (including re-ordering for presentation).

- Decode current picture at DTS

- Mark pictures in the buffer as “unused for future reference” according to “reference picture buffering mode” (e.g. FIFO), MMCO commands, and IDR indication (i.e. marking all pictures in the buffer as “unused”).

- Remove each picture in the buffer that satisfies both of the two following rules: a) it is marked as “unused for future reference”; b) its presentation time is smaller or equal to the DTS of the current picture.

- If the current picture is not marked as “unused for future reference” (as per nal_storage_idc in the NAL unit header) or if its presentation time is larger than its DTS, add the current picture to the buffer.

After step 4 the total amount of data in the buffer shall not exceed the limits defined by profile@level.

Note: Separate post-decoder removal times are required for the two fields of an interlaced frame.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annex C | USNB | T | USNB-FCD-39 |  |

Context

HRD

Question

The buffer fullness curve in Figure C4 contains features that are not easy to understand.

Proposal

Provide additional wording along the following lines in the paragraph following Figure C4.

“It can be noted that data enters the buffer continuously at rate R until t=14.5 seconds.  At that time, fifteen pictures have entered the buffer, and the sixteenth picture cannot enter because its earliest possible arrival time is t=15.  Thus there is a 0.5 second flat spot from 14.5 to 15 seconds.  The reason it takes until t=14.5 for this to occur is that the initial picture took five picture periods to enter the buffer.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.2.1 | USNB | E | USNB-FCD-39A |  |

Context

Figure C-4.

Question

The Figure C-4 for the HRD buffer fullness example may not be correct or need more explanation. Since the slope of the graph in  Figure C-4 corresponds to the arriving bit rate, Figure C-4 shows that the bitstream is arriving at two rates. One is at the channel bit rate, the other is at zero bits/second when the bit rates seems to be "clipped" (e.g. three times between time 15 and 20). Is it intentional to have two bit rates in the example? If having two bit rates is intentional, what is the scenario that it would happen?

Proposal

Clarify.

## Annex D: Supplemental enhancement information

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.2 | USNB | T | USNB-FCD-40 | Note: Addressed in Geneva Study. |

Context

Clock Timestamp SEI

Question

- What is the time that is being represented here? The semantics says, "Indicates the displayer or capture time". These are different, which is intended?

- Semantics of counting_type is confusing. What does "If counting type is '0010' and count_dropped is 1, nframes shall be 1 and the value of nframes for the last previous picture in displayer shall not be equal to 0 unless discontinuity_flag is equal to 1" mean?  Similar comments apply to other paragraphs in the semantics of nframes.

Proposal

- Clarify what time is being represented by this message. If both capture and display times need to be represented make this information explicit in the syntax of the message.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.3 | USNB | T | USNB-FCD-41 | Note: Addressed in Geneva Study. |

Context

The semantics of pan/scan.

Question

The pan/scan as documented is too general;  it even allows mirror operations implicitly if left > right, for example.

Proposal

Please insert the text: “Top must be <= bottom, and right >= left. Applications or specific environments may restrict the use of pan/scan.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.9 | USNB | E | USNB-FCD-42 | Note: Addressed in Geneva Study. |

Context

The meaning of the broken_link flag.

Question

The text is not entirely clear.

Proposal

This needs to be clearer.  The broken-link flag needs to say that the decoded pictures may not be correct even when starting from the most recent IDR.  Whether or not this flag is set, the picture will be correct after the frame indicated by post-roll count,  if a decoder starts at the frame indicated by pre-roll count,

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.10 | USNB | T | USNB-FCD-43 |  |

Context

Reference Picture Buffer Management Repetition SEI

Question

Where in the decoding order should this SEI message be placed?

Proposal

This message should occur only after the last data partition/slice of the stored picture(s) having the original frame number.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.12 | USNB | T | USNB-FCD-44 |  |

Context

Scene Information SEI

Question

- The intended use of this SEI message is not clear.

- Scene Transition Type: The semantics of these transitions are not well defined. For example, "Fade-out" is a fade out to what color? What is the difference between types 1, 2, and 3? What is a "wipe"? Etc.

- How are cuts indicated in this scheme?

Proposal

- State the intended use clearly in the semantics, e.g., indicate that it is for a hint for error concealment.

- Give a definition for each of the different kinds of scene transition types.

- Indicate how cut (instantaneous transition) should be represented. Assume that it is indicated as two pictures before and after cut that have different scene identifiers both with scene_transition_type of 0 (no transition – slightly odd). Suggest renaming "transition type" to "effect" type to make this clearer.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.12 | USNB | E | USNB-FCD-45 |  |

Context

The meaning of this SEI.

Question

There are a number of aspects of this which need clarification.

Proposal

This seems seriously under-documented.   It needs to say that it is descriptive of how the sequence was formed, rather than prescriptive (even optionally) for the decoder.  The period of the transition needs to be identified (possibly using a frame count like RAP).  Fade-in and fade-out should be specified as from/to black, or something?

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.13 to D.3.15 | USNB | M | USNB-FCD-46 |  |

Context

Technical issues with this section.

Question

It seems that for presentations which consist of sub-sequences which repeat, the sub-sequence ID will grow, as it cannot be re-used.  A technique which allows this to cycle is desirable.  In addition, the interaction with the HRD is unclear.  Finally, the design seems fairly complex and it may be desirable to search for simpler solutions in both the SEI and file format.

Proposal

For these reasons, we recommend that consideration be given to deferring these SEI messages to a future version of the specification.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.3.14/15 | USNB | T | USNB-FCD-46B |  |

Context

Sub-sequence characteristics SEI

Question

Average bit rate semantics are not clear, for instance how the duration of sequence calculated to determine the average bit rate.

Proposal

Clarify the semantics.

## Annex E: Video usability information

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| ? | USNB | E | USNB-FCD-47 | Obsolete |

Context

Carriage of film-originated content using AVC

Question

Currently there is no mechanism that can be used to signal that coded pictures originated from film nor SEI messages to a display unit how to reconstruct the video timing associated with that film-originated content.

Proposal

Add syntax, probably supplemental enhancement information, possibly in the spirit of the picture display extension of JVT-B048.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Annex E | USNB | T | USNB-FCD-48 | Obsolete |

Context

VUI data

Question

The wording “complies with” is used where “conforms to” is appropriate (two places).

Proposal

Change the wording.

# Comments on the Geneva Study of ISO/IEC 14496-10 FCD

The comments contained in this section are based on the Geneva Study text (JVT-E146d37ncm.doc). All references to sections are made with respect to that document.

## General

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Throughout | USNB | E/T | USNB-G-0 |  |

Context

All

Question

Editor';s notes should not appear in the final FDIS specification text.

Proposal

Remove all editors' notes from the specification. This includes addressing and resolving all issues indicated in editor's notes.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Throughout | USNB | E | USNB-G-1 |  |

Context

Numbering of fields of frames in figures.

Question

Sometimes fields in figures are numbered 0 and 1, and sometimes 1 and 2.

Proposal

Use zero-based numbering convention as is used elsewhere throughout the text.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Throughout | USNB | T | USNB-G-2 |  |

Context

Our understanding is that division operators are used in the following three places, all based on DiffPicOrderCount values:

1) Implicit weighted prediction weight computation (where the most significant problem lies due to the potential need to consider all pairwise combinations of positions in list 0 and list 1)

2) Temporal direct mode motion vector computation (where the number of required divisions required can easily be cut in half by replacing MV1 = (W * MV +128) >> 8 with MV1 = MV0 – MV).

3) Motion vector scaling for B slices (subclause 8.4.1.2.2 in JVT-E146d37)

Question

The use of division operations is a computational problem, particularly for implicit weighted prediction for decoding field mode macroblocks (when the maximum number of reference pictures doubles relative to frame mode coding).

Proposal

The need, accuracy requirements, and wordlength requirements for these division operations should be reviewed.  If feasible without impact on quality, the operations in question should be modified to avoid use of the division operator.  Consider the potential simplification for temporal direct mode motion as noted above.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| Throughout | USNB | M | USNB-G-3 |  |

Context

Macroblock-adaptive frame/field decoding operation.

Question

This part of the design seems to lack clarity/completeness.

Proposal

Our understanding/position on specific aspects of MB-AFF is as follows.  We believe these points should address all remaining issues regarding clear specification of MB-AFF. The draft should be reviewed to ensure that it is consistent with this interpretation and that all aspects are clearly documented:

1) MB-AFF should be supported in Profiles Main and X.

2) The slice-level field picture flag and the sequence parameter set-level MB-AFF flag should be merged into a single syntax element in the picture parameter set

3) If Annex A specifies a maximum number of reference frames equal to T, the maximum range of num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 for frame pictures (including MB-AFF frame pictures) is at least T-1 (saying "at least" to allow multiple weights per picture to possibly increase that number) and for field pictures is at least 2*T-1.

4) In a field macroblock pair, the maximum allowed value of ref_idx_l0 is 2*(num_ref_idx_l0_active_minus1+1)-1.  In a frame macroblock pair, the maximum allowed value of ref_idx_l0 is  num_ref_idx_l0_active_minus1.  Similar for ref_idx_l1.

5) If num_ref_idx_l0_active_minus1 = 0 and MB-AFF is in use, ref_idx_l0 is not sent for frame macroblocks but is sent for field macroblocks.

6) In a field macroblock pair, a value of ref_idx_l0 equal to 2n and 2n+1 refer to the same frame as for a frame macroblock pair with ref_idx_l0 equal to n.  There must be a definition of which field is numbered 2n and which is 2n+1.  (Idea #1: 2n is the one closest in POC order or the top field if the POCs are equal, Idea #2: 2n is the one with the same parity as the current field MB and 2n+1 is the one with opposite parity).

7) In explicit weighted prediction for MB-AFF, one weight is sent per reference frame index.  When an MB pair is coded in field mode, that same weight is applied to either field of that frame. (In any case there must be a definition of the association of weights to fields.)

8) In implicit weighted prediction for MB-AFF, frame mode MB pairs use weights calculated from the POC of the referenced frame and field mode MB pairs use weights calculated from the POC of the referenced field. (Note that this impacts the number of division operations required for implicit weighted bi-prediction, and the complexity of decoding process with regard to division operations should be evaluated and reduced if possible.)

10) Intra prediction for frame MB pairs should be based on neighbouring decoded samples constructed into a frame.  Intra prediction for field MB pairs should similarly be based on spatial relationships with data in the same field as the field being predicted.

11) The definition of co-located MB when the current MB pair is in different frame/field mode than the co-located MB pair must be clear.

12) Should the deblocking filter operate on the interior edges of a field MB pair in the mode of the MB pair (with the step size of the MB) or always in frame mode (with the average of the step sizes of the  two MBs of the pair)?

13) Should the deblocking filter operate on the exterior edges of the MB pair in the mode of the MB pair or always in frame mode?

14) The method of performing motion data prediction for MB-AFF must be clear. One idea is use a precise definition of spatial neighbors that works across differences in MB pair type and another is to predict motion data for field MB pairs only from other field MB pairs and to predict motion data for frame MB pairs only from other frame MB pairs.

15) The method of predicting intra modes for MB-AFF must be clear.  Does that operate across MB boundaries?

16) If any kind of other cross-macroblock prediction not discussed above is used, it must be clearly specified for MB-AFF (e.g., CABAC context issues)

17) MB location is defined properly in 3.60 of the SoFCD JVT-E146d37.  Text elsewhere should be changed if not consistent with that definition.

18) If a reference frame store has only one field occupied, it is not a candidate for use in prediction of an MB-AFF frame picture.

19) If reference re-ordering is done for an MB-AFF frame, the re-ordering is a re-ordering of frames.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| General | USNB | E | USNB-G-4 |  |

20) "frame-structured picture" should be "coded frame" and "field-structured picture" should be

"coded field".

Context

Random access points in HRD text

Question

There is an editor’s note that should be removed.

Proposal

Remove editor’s note at the top of C.1.1.  The issue has been addressed.

## Clause 0

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 0.2 | USNB | E | USNB-0-2 |  |

Context

Question

‘Insternational’ should be “International”

Proposal

Fix

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 0.4 | USNB | E | USNB-0-3 |  |

Context

“Ed. Note”

Question

This paragraph says that flags are used in the bitstream to signal the use of other syntax elements.  It does not say that this _all_ syntax elements’ use is signaled by flags.

Proposal

Remove the Editor Note

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 0.4 | USNB | E | USNB-0-4 |  |

Context

In Paragraph 2, it states “These and other related terms are formally defined in clause 4”.

Question

Clause 4 does not talk about profiles and levels.

Proposal

Change “Clause 4” to “Annex A”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 0.5.1 | USNB | E | USNB-0-5 |  |

Context

In Paragraph 2, flexible application of picture type in a sequence is mentioned.

Question

It would introduce new types of dependencies on reference pictures.

Proposal

Add an example Temporal Dependency diagram.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 0.5.3 | USNB | E | USNB-0-6 |  |

Context

In Paragraph 2, macroblock partition is discussed for inter prediction.

Question

Macroblock partition is also used for intra prediction. The first sentence of paragraph 2 makes it sound if it is only for inter prediction.

Proposal

Mention intra prediction as well.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 0.6 | USNB | editorial | USNB-0-7 |  |

Context

“Ed. Note”

Question

The Editor Note does not belong in the final specification.

Proposal

Replace the Editor Note with

“The Annexes define and explain system-level information that may be necessary to ensure system interoperability between multiple encoders, processors, and decoders.”

## Clause 3: Definitions

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.4 | USNB | E | USNB-3-1 |  |

Context

bi-predictive slice

Question

Replace ‘;’ with ‘:’ after the term bi-predictive slice

Proposal

Fix

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.23 | USNB | E | USNB-3-2 |  |

Context

DC coefficient

Question

Remove Editor Note

Proposal

Define “frequency index” to mean “one of the two (vertical and horizontal) indices used to identify elements within a block of transform coefficients”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.26 | USNB | E | USNB-3-3 |  |

Context

decoded picture buffer management

Question

Remove Editor Note and fix up this definition

Proposal

?

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.49 | USNB | E | USNB-3-5 |  |

Context

macroblock

Question

Put in an ‘s’:  “The division of a slice or macroblock pair into macroblocks…”

Proposal

Fix

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.64 | USNB | E | USNB-3-7 |  |

Context

NAL unit

Question

italicize ‘RBSP’

Proposal

Fix

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.67 | USNB | E | USNB-3-8 |  |

Context

non-reference picture

Question

Remove the Editor Note.

Proposal

Remove the entire definition?

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 3.69 | USNB | E | USNB-3-9 |  |

Context

Output order is defined.

Question

How about display order? It is mentioned in Clause 8.2.2.3 and Clause 9.3.4.

Proposal

Define display order and state it is the same as output order.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.82 | USNB | editorial | USNB-3-11 |  |

Context

primary coded picture

Question

Append the following:

“

see “redundant coded picture”

“

Proposal

Fix

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 3.88 | USNB | E | USNB-3-12 |  |

Context

Quantisation parameter

Question

Remove Editor Note and keep the definition

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 3.95 | USNB | E | USNB-3-13 |  |

Context

redundant coded picture

Question

replace the given definition with

“

a redundant coded representation of a picture that shall not be used for decoding unless the primary coded picture is missing or corrupted.  The decoded picture generated from the redundant coded picture may not cover the entire picture area.  There should be no noticeable difference between co-located areas of the decoded picture generated from the primary coded picture and the decoded picture generated from the redundant coded picture.

“

Proposal

Either adopt this text or delete the definition entirely and give similar text to the definition of redundant coded slice.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.105 | USNB | E | USNB-3-14 |  |

Context

reference list 1 motion vector

Question

Remove Editor Note

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.109 | USNB | E | USNB-3-15 |  |

Context

residual

Question

Do we need definitions for both ‘prediction residual’ and ‘residual’?

Proposal

Delete definition of ‘prediction residual’?

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 3.110 | USNB | E | USNB-3-16 |  |

Context

“run” is defined and block scan is mentioned.

Question

Block scan is not defined.

Proposal

Define block scan in Clause 3 or replace block scan by “zig-zag scan or field scan”.

Remove the ‘Editor Note”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.115, 3.117 | USNB | E | USNB-3-17 |  |

Context

slice, slice group

Question

Remove the Editor Notes.  For the first Editor Note, we can substitute “macroblocks or macroblock pairs” for ‘macroblocks’.

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.118 | USNB | E | USNB-3-18 |  |

Context

slice header

Question

This is a terrible definition.  Just delete it?

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 3.121, 3.122, 3.123, 3.124, 3.125 | USNB | E | USNB-3-19 |  |

Context

sections 3.111 through 3.115

Question

3.114 is not in alphabetical order, and SODB appears twice

Proposal

Fix

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 3.126 | USNB | E | USNB-3-20 |  |

Context

sub-macroblock

Question

replace “one quarter of the samples of the samples” with “One quarter of the samples”

Replace “aligned to macroblock boundaries…” with “that shares a corner with its containing macroblock”

Remove Editor Note

Proposal

## Clause 4: Abbreviations

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 4.26 | USNB | E | USNB-4-1 |  |

Context

Question

VUI is referenced in clause 7.4.2.1 and Annex E.

Proposal

Add VUI: Video Usability Information

## Clause 5: Convention

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 5.1 | USNB | T | USNB-4-2 |  |

Context

The list of arithmetic operators is missing the floor function. This function is used in section 9.3.1.4.

Question

Proposal

Add “ x       Floor. The greatest integer less than or equal to x.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 5.5 | USNB | E | USNB-4-3 |  |

Context

x=a..b

Question

I could not find any use of this—let’s remove this definition.  (If it is used somewhere, let’s replace the notation with
x  [a, b]

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 5.6 | USNB | E | USNB-4-4 |  |

Context

Equation 5-7 for Inverse Raster Scan

Question

Proposal

Change InverseRasterScan(a,b,c,d,e) to InverseRasterScan( Index, width, height, pitch, y_dim ) or explain how those arguments may represent in an example.

## Clause 6: Data formats

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 6.3 | USNB | E | USNB-6-2 |  |

Context

The paragraph starting “When mb_adaptive_frame_field_flag…”

Question

Needs clarification.

Proposal

Replace with

“

When mb_adaptive_frame_field_flag (which can be found in the sequence parameter set) is equal to 1, the slices of the picture consist of a sequence of macroblock pairs.  A macroblock pair, as shown in figure 6-4, consists of two macroblocks that are located one above the other.

“

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 6.3 | USNB | E | USNB-6-3 |  |

Context

Paragraph starting “Macroblocks or sub-macroblocks…”

Question

replace ‘paritions’ with ‘partitions’

fix printing of figure 6-5 and remove Editor Note

replace ‘quadrats’ with ‘quadrants’ (three times)

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.3 | USNB | E | USNB-6-4 |  |

Context

Spatial subdivision.

Question

How is MB numbered when mb_adaptive_frame_field_flag equals 0 or 1? Based on Clauses 6.4.1 and 6.4.2, they are numbered in the same way.

Proposal

Add MB indices to both figures for clarification.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.3 | USNB | E | USNB-6-6 |  |

Context

In the paragraph below Figure 6-4, it states:

"Macroblocks or sub-macroblocks may be partitioned and the paritions are
scanned for inter prediction as shown in Figure 6-5. The outer quadrats
refer to the samples in a macroblock or sub-macroblock, respectively.
The rectangles or quadrats refer to the partitions. The numbers in each
quadrat or rectangle specify the index of the inverse macroblock
partition scan or inverse sub-macroblock partition scan.".

Question

- "paritions" is mispelled once; "quadrats" is misspelled three times. Presumably they should be "partitions" and "quadrants" respectively.

- What does the term "outer quadrats" mean?

Proposal

- Fix spelling

- clarify "outer quadrants" or replace the highlighted sentence by

The outer boundaries of rectangles and quadrants represent a macroblock or sub-macroblock.

- Specify sub-macroblock partitions are only applicable to the 8x8 macroblock partition.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4 | USNB | E | USNB-6-7 |  |

Context

Inverse scanning processes

Question

There is no easy way to refer to equations in this clause.

Proposal

Add equation numbers to each equation.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 6.4.1 | USNB | E | USNB-6-8 |  |

Context

Ed. Note

Question

Fix and remove

Proposal

?

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4.1 | USNB | E | USNB-6-9 |  |

Context

Inverse macroblock scanning process

Question

MbLumaLoc[ ][ ] is not defined.

Proposal

MbLumaLoc[ MbIdx ][ 0 ]: x-coordinate of upper-left luma sample for MB with index MbIdx

MbLumaLoc[ MbIdx ][ 1 ]: y-coordinate of upper-left luma sample for MB with index MbIdx

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 6.4.2 | USNB | E | USNB-6-10 |  |

Context

Ed. Note

Question

Fix and remove

Proposal

?

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4.2 | USNB | T | USNB-6-11 |  |

Context

MbPairLumaLoc[ MbPairIdx ][ 0 ] = InverseRasterScan( PartIdx, 16, 16, Width, 0 )

MbPairLumaLoc[ MbPairIdx ][ 1 ] = InverseRasterScan( PartIdx, 16, 16, Width, 1 )

Question

MbPairLumaLoc[][] is not defined.

Macro-block pairs are 16x32. Doesn’t that change the equation to?

MbPairLumaLoc[ MbPairIdx ][ 0 ] = InverseRasterScan( PartIdx, 16, 32, Width, 0 )

MbPairLumaLoc[ MbPairIdx ][ 1 ] = InverseRasterScan( PartIdx, 16, 32, Width, 1 )

I guess the answer depends on the definition of SliceGrolupMbPairIdxList[]…

Proposal

MbPairLumaLoc[ MbIdx ][ 0 ]: x-coordinate of upper-left luma sample for MB pair with index MbIdx

MbPairLumaLoc[ MbIdx ][ 1 ]: y-coordinate of upper-left luma sample for MB pair with index MbIdx

Verify the formulae for MbPairLumaLoc[] are correct.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4.3 | USNB | E | USNB-6-12 |  |

Context

Inverse macroblock in macroblock pair scanning process

Question

The equations can be further simplified to something trivial.

MbInMbPairLumaLoc[ MbInMbPairIdx ][ 0 ] = InverseRasterScan( MbInMbPairIdx, 16, 16, 16, 0 )

= (MbInMbPairIdx % 1) * 16 = 0

MbInMbPairLumaLoc[ MbInMbPairIdx ][ 1 ] = InverseRasterScan( MbInMbPairIdx, 16, 16, 16, 1 )

= (MbInMbPairIdx / 1) * 16 = 16 * MbInMbPairIdx

Proposal

Simplify the equations

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| section 6.4.4, 6.4.5 | USNB | E | USNB-6-13 |  |

Context

Ed. Note

Question

Remove the four Editor Notes

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4.6 | USNB | E | USNB-6-14 |  |

Context

Inverse 4x4 luma block scanning process

Question

It is not clear how all the 4x4 blocks are numbered in a macroblock.

Proposal

Add figure 6-6, “Ordering of blocks for coded_block_pattern”, in the July 2002 FCD to this sub-clause.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4.7 | USNB | T | USNB-6-15 |  |

Context

In Paragraph 4, three conditions are used to check if MbNeighbourIdx should be –1.

Question

The 3rd condition, for x-dimension checking against frame width, uses the x coordinate of its upper-left luma location plus location difference. The upper bound should be frame width minus one macroblock width.

Proposal

Change the 3rd condition to:

MbLumaLoc[ MbIdx ][ 0 ] + LumaSampleLocDiff[ 0 ] > frame_width_in_mbs_minus1 * 16 - 1

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 6.4.8 | USNB | T | USNB-6-16 |  |

Context

In Paragraph 4, three conditions are used to check if MbPairNeighbourIdx should be –1.

Question

The 3rd condition, for x-dimension checking against frame width, uses the x coordinate of its upper-left luma location plus location difference. The upper bound should be frame width minus one macroblock width.

Is “frame_width_in_MbPairs_minus1” same as “frame_width_in_mbs_minus1”?

Proposal

Change the 3rd condition to:

MbPairLumaLoc[ MbPairIdx ][ 0 ] + LumaSampleLocDiff[ 0 ] > frame_width_in_MbPairs_minus1 * 16 – 1

## Clause 7: Syntax and Semantics

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7 | USNB | E | USNB-7-1 |  |

Context

Syntax category numbers 2 and 3 are not used.

Question

This seems editorially sloppy and may confuse the reader.

Proposal

Change category numbers 4 through 7 to be 2 through 5.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.2 | USNB | E | USNB-7-2 |  |

Context

In the first paragraph after the function definitions, it states:

“Categories (labelled in the table a C) specify the slice data partition for slice data into three partitions….”

Question

Only categories 4, 5 and 6 are defined here. Other categories (0, 1, 7, 8 and 9) are not defined.

Proposal

Add a reference to Table 7-1, where all the categories are specified.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.2.2 | USNB | E | USNB-7-3 |  |

Context

Pix parameter set

Question

Proposal

delete  "if( pic_order_cnt_type  <  2 )"

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.3.3 | USNB | T | USNB-7-4 |  |

Context

field_pic_flag

Question

Field_pic_flag is undefined when frame_coding_only_flag is 1, but its used in subsequence lines.and in slice data semantic.

Proposal

Add an else as following,

| if( !frame_coding_only_flag ) { |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| field_pic_flag |  |  |  | 4 | u(1) |  |  |
| if( field_pic_flag ) |  |  |  |  |  |  |  |
| bottom_field_flag |  |  |  | 4 | u(1) |  |  |
| } else |  |  |  |  |  |  |  |
| field_pic_flag = 0 |  |  |  |  |  |  |  |
| first_mb_in_slice |  |  |  |  |  |  |  |
| Clause | National Body | Type | Reference |  |  | Disposition |  |
| 7.3.4 | USNB | T | USNB-7-5 |  |  |  |  |

Context

There are bugs in the table.

Question

There is a problem with incrementing MbIdx.

Proposal

| slice_data( ) { |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| if( entropy_code_mode = = 1 ) |  |  |  |  |  |
| while( !byte_aligned( ) ) |  |  |  |  |  |
| cabac_alignment_bit |  |  |  |  |  |
| if( mb_frame_field_adapative_flag && !field_pic_flag ) { |  |  |  |  |  |
| MbPairY = first_mb_in_slice / (frame_width_in_mbs_minus1 + 1) |  |  |  |  |  |
| MbPairX = first_mb_in_slice % (frame_width_in_mbs_minus1+1) |  |  |  |  |  |
| MbIdx = ( MbPairY << 1 ) * (frame_width_in_mbs_minus1 + 1) + MbPairX |  |  |  |  |  |
| } else |  |  |  |  |  |
| MbIdx = first_mb_in_slice |  |  |  |  |  |
| MoreDataFlag = 1 |  |  |  |  |  |
| MbFieldDecodingPresentFlag = 1 |  |  |  |  |  |
| do { |  |  |  |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |  |  |  |
| if( entropy_coding_mode = = 0 ) { |  |  |  |  |  |
| mb_skip_run |  |  |  |  |  |
| MbIdx = MbIdx + mb_skip_run  does the MbIdx need to be updated? |  |  |  |  |  |
| MoreDataFlag = more_rbsp_data( ) |  |  |  |  |  |
| } else { |  |  |  |  |  |
| mb_skip_flag |  |  |  |  |  |
| MoreDataFlag = !mb_skip_flag |  |  |  |  |  |
| } |  |  |  |  |  |
| if( MoreDataFlag ) { |  |  |  |  |  |
| if( mb_adaptive_frame_field_flag && !field_pic_flag ) { |  |  |  |  |  |
| if( MbIdx % 2 = = 0 ¦ ¦ (MbIdx % 2 = = 1 && first_non_skip_mb_in_pair( ) ) |  |  |  |  |  |
| MbFieldDecodingPresentFlag = 1 |  |  |  |  |  |
| if( MbFieldDecodingPresentFlag ) { |  |  |  |  |  |
| mb_field_decoding_flag |  |  |  |  |  |
| MbFieldDecodingPresentFlag = 0 |  |  |  |  |  |
| } else |  |  |  |  |  |
| mb_field_decoding_flag = 0 |  |  |  |  |  |
| } |  |  |  |  |  |
| macroblock_layer( ) |  |  |  |  |  |
| } is this is missing? |  |  |  |  |  |
| if( entropy_coding_mode = = 0 ) |  |  |  |  |  |
| MoreDataFlag = more_rbsp_data( ) |  |  |  |  |  |
| else { |  |  |  |  |  |
| if( mb_adaptive_frame_field_flag && !field_pic_flag && MbIdx%2 != 0 ) &&  extra && or missing condition? |  |  |  |  |  |
| MoreDataFlag = 1 |  |  |  |  |  |
| else { |  |  |  |  |  |
| end_of_slice_flag |  |  |  |  |  |
| MoreDataFlag = !end_of_slice_flag |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| MbIdx++ [Ed. Note: is this correct with FMO ?] |  |  |  |  |  |
| } while( MoreDataFlag ) |  |  |  |  |  |
| } |  |  |  |  |  |
| Clause | National Body | Type | Reference | Disposition |  |
| 7.3.4 | USNB | T | USNB-7-7 |  |  |

Context

Slice data syntax, coding of end_of_slice_flag.

Question

If mb_type is PCM, the CABAC decoding is terminated. It is not initialized and restarted until starting a slice. Also, how is end_of_slice_flag decoded if mb_type is PCM?

Proposal

Eliminate PCM encoding or re-think when CABAC is terminated and restarted.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.1 (of JVT-E146d37) | USNB | E | USNB-7-9 |  |

Context

After "An IDR picture shall not immediately follow another IDR picture in

decoding order."

Question

Proposal

Add "The output order of an IDR picture shall exceed that of all previously

decoded pictures."

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.1 (of JVT-E146d37) | USNB | T/E | USNB-7-10 |  |

Context

cabac_alignment_bit is equal to 0.

Question

Shouldn't it be equal to 1? (We set most such vacant things equal to 1 for start code emulation prevention except for things specifically designed to handle their relationship to start codes.)

It would also be helpful to give it a name that directly implies its value (cabac_alignment_one_bit).

Proposal

Rename cabac_alignment_bit to cabac_alignment_one_bit and set it equal to 1.

Also rename other similar syntax elements to imply their value (rbsp_stop_bit  rbsp_stop_one_bit, rbsp_alignment_bit  rbsp_alignment_zero_bit).

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1 | USNB | T | USNB-7-12 |  |

Context

There is no indication in the sequence parameter set to indicate whether FMO, ASO, and redundant slices may be present in the sequence.

Question

This makes it impossible to tell at the sequence level whether a Main profile decoder can decode a Baseline profile sequence.

Proposal

Add such flags.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.1 | USNB | T | USNB-7-13 |  |

Context

frame_coding_only_flag and mb_frame_field_adaptive_flag

Question

Can  frame_coding_only_flag = 1 at the same time as mb_frame_field_adaptive_flag = 1 ?

Proposal

Define clearly the legal values of these two parameters with respect to each other.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.3.1 | USNB | E | USNB-7-15 |  |

Context

In the last sentence of the first paragraph, it states: “SEI payload types are specified in Annex C.”

Question

The reference to the Annex is wrong.

Proposal

“SEI payload types are specified in Annex D.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.2.8 | USNB | E | USNB-7-16 |  |

Context

Specification of bin count constraint for CABAC.

Question

There seems to be a lack of clarity regarding whether the bin count constraint applies to slice data partitions or only to slices without partitioning.

Proposal

Change:

"When entropy_coding_mode is equal to 1, the number of bins (excluding end_of_slice_flag) resulting from decoding the contents of the slice layer NAL unit shall not exceed 32*NumBytesInNALunit." (in subclause 7.4.2.8)

To:

"When entropy_coding_mode is equal to 1, the number of bins (excluding end_of_slice_flag) resulting from decoding the contents of any NAL unit containing RBSP slice trailing bits shall not exceed 32*NumBytesInNALunit."

This is to avoid the impression that the constraint does not apply to slice data partitions.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.3 | USNB | E | USNB-7-18 |  |

Context

frame_num parameter.

Question

The definition is unclear and can be interpreted multiple ways. There is no definition of what happens with pictures having same frame number in the long-term reference decoded picture buffer.

Proposal

Define process of assigning it for each “image"

* Frame number serves as a unique identifier for each reference frame in decoder reference buffers.

* IDR pictures are assigned frame number 0.

* Frame number for a reference frame is calculated by incrementing, modulo MAX_FN, frame number of the previous reference frame in decoding order.

* Frame number of a non-reference frame is equal to frame number of the previous reference frame in decoding order.

* Non-reference pictures with the same frame number are decoded in increasing picture order count.

* Both fields of the same frame are assigned the same frame number.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.4 | USNB | E | USNB-7-19 |  |

Context

Mb_skip_flag description, 3rd paragraph, 2nd sentence

Question

One other question related to this section concerns the definition of mb_skip_flag.  This section never specifies what values mb_skip_flag should be.  Is it 1 when a block is skipped and 0 otherwise, or vice-versa?

Proposal

I think the 2nd sentence should be: When field_pic_flag is equal to 1 (instead of 0),  the decoded field of the same parity (top or bottom) with ref_idx_l0 equal to 0, which….

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5 | USNB | T | USNB-7-20 |  |

Context

Definition: “pcm_alignment_bit is a bit equal to 1”

Question

rbsp_alignment_bit, which is also used for alignment when terminating CABAC, is defined as 0 (sections 7.3.2.9 and 7.4.2.9). Would it be more consistent to use 0 for alignment here?

Also, section 9.3.4.3.6 figure 9-16 indicates that 0 is used for alignment in all cases.

Proposal

Change the definition to “pcm_alignment_bit is a bit equal to 0”

Alternately, the definition can remain as 1, and section 9.3.4.3.6 figure 9-16 can be modified so that the “PutBit(0)” block instead does “PutBit(0)” if coding end_of_slice_flag and “PutBit(1)” if coding the bit indicating PCM mode.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5 | USNB | E | USNB-7-21 |  |

Context

How intra prediction blocks are specified in P and B slices

Question

Proposal

- I think the sentence should be: The mb_type values 0 to 4 are specified in Error: Reference source not found and the mb_type values 5 to 29 are specified in Error: Reference source not found, indexed by adding (instead of subtracting) 5 from the value of mb_type.

- I think the sentence should be: The mb_type values 0 to 22 are specified in Error: Reference source not found and the mb_type values 23 to 47 are specified in Error: Reference source not found, indexed by adding (subtracting) 23 from the value of mb_type.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5 | USNB | T | USNB-7-22 |  |

Context

Table 7-14

Question

Is Table 7-14 used to binarize coded_block_pattern?  If so, is it only used when UVLC is used?  If CABAC is used, Table 9-23 and Sec 9.3.1.6 says that the Coded block pattern for Luma is coded with a different context model and coded separately from the Coded block pattern for chroma.  So binarization of coded_block_pattern does not seem to make sense if CABAC is used.

Proposal

Clarify.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 7.4.5 | USNB | T | USNB-7-23 |  |

Context

mb_type semantics for intra macroblocks.

Question

At the Geneva meeting, a PCM macroblock type was introduced. However, there is no entry for this type in the intra mb_type table.

Proposal

- Add PCM entry using type 25 into Table 7-9.

- The indexing offsets for P, B, SI, SP macroblock types should be updated.

- Alternatively, we could remove PCM mode.

## Clause 8: Decoding Process

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.1 | USNB | T | USNB-8-2 |  |

Context

Whole subclause.

Question

No text is available. Proposed text for this subclause is given below.

Proposal

Input to this process are NAL units.

Output of this process depends on the type of NAL unit. The output for each type is further specified below.

When any Coded Slice NAL unit (NAL unit types 1 to 5) is encountered, it shall be decoded according to subclause 8.2.

When a Sequence Parameter Set NAL unit is encountered, the decoder shall store the parameter set into a memory buffer that holds all Sequence Parameter Sets for decoding of subsequent coded slices. This parameter set replaces an existing parameter set in the buffer with the same seq_parameter_set_id.

When a Picture Parameter Set NAL unit is encountered, the decoder shall store the parameter set into a memory buffer that holds all Picture Parameter Sets for decoding of subsequent coded slices. This parameter set replaces an existing parameter set in the buffer with the same pic_parameter_set_id.

When a Picture Delimiter NAL unit is encountered, the decoder shall expect that the next coded slice belongs to a new picture of type given by the pic_type field.

When a Filler Data NAL unit is encountered, the decoder should discard the NAL unit. [Question: Does the length of the filler data need to be accounted for in the HRD?]

When a Supplemental Enhancement Information NAL unit is encountered, the decoder shall discard the NAL unit if it does not support SEI. If SEI is supported, the decoder shall process the SEI messages according to Annex D and store all relevant data from the messages to aid the decoding (and presentation) of coded slices received after this NAL unit.

When a NAL unit with reserved NAL unit type code is encountered, the decoder shall discard the NAL unit.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.1 | USNB | T | USNB-8-3 |  |

Context

Conditions for declaring a new picture.

Question

If a picture delimiter NAL unit is received, the next coded slice belongs to a new picture.

Proposal

Add this condition to the list. Proposed text as follows:

“7. A picture delimiter NAL unit was received prior to the current slice, but after the previous slice.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.1 & 8.2.2 of FCD | USNB | E | USNB-8-4 |  |

Context

NAL unit decoding order specification and parameter set decoding.

Question

The draft content should be reviewed to ensure that the content of these subclauses of the FCD text have been retained.

Proposal

The draft content should be reviewed to ensure that the content of these subclauses of the FCD text have been retained.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.2 & C.1.2.1 (of JVT-E146d37) | USNB | T/E | USNB-8-5 |  |

Context

Use of DiffPicOrderCount.

Question

Problem: Mathematics problems that could be caused by DiffPicOrderCount becoming equal to zero and need to clarify the intended relation between time and picture order count.

Proposal:

Proposal

Change:

"The ordering of DPB output times for the sequence of decoded pictures is required to be the same as the ordering of the picture order counts for the sequence of decoded pictures (subclause 8.2.2)."  (in subclause C.1.2.1)

To:

"When pic_order_cnt_type is equal to 0 or 1, if the picture order count (TopPicOrderCnt or BottomPicOrderCnt) for some decoded field is less than the picture order count of another decoded field, the DPB output time of the decoded field in question shall not be greater than the DPB output time of the other decoded field.  When pic_order_cnt_type is equal to 2, the DPB output times of decoded fields shall be non-decreasing with decoding order."

And:

Specify wherever DiffPicOrderCount appears in the denominator of an equation (e.g., subclause 8.4.1.2.2 luma motion vector prediction in B slices and 8.4.1.4.2.3 temporal direct luma motion vector in B slices) that it shall not be equal to 0.

And:

Specify in subclause 8.2.2 that when pic_order_cnt_type is equal to 2, the output order shall be equal inferred to be the same as the decoding order.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.2.2.3 | USNB | E | USNB-8-6 |  |

Context

Table 8-4

Question

Proposal

column 5: bottom 3 numbers (32 26 28) should be  33  27  29

column 7: bottommost 2  should be  -2

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.x.x | USNB | E | USNB-8-7 |  |

Context

The following words appear: availability, macroblock (6 times), ouput (10 times), above, and subcaluse.

Question

They are misspelled.

Proposal

Change to "availability", "macroblock", "output", "above", "subclause" respectively.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.1.1 | USNB | T | USNB-8-8 |  |

Context

Derivation process for the available samples for macroblocks

Question

"The sample d has the x-component of the location 4x4LumaBlkLoc[ 4x4LumaBlkIdx ][ 0 ] + 4 "

Isn't it at 4x4LumaBlkLoc[ 4x4LumaBlkIdx ][ 0 ] + 3 ?

Proposal

Verify and fix.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.1.2 | USNB | E | USNB-8-10 |  |

Context

The whole clause.

Question

It's not clear where the reference block came from. It seems that for both fields predicted from the top field.

Proposal

A diagram may be helpful.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.1.2 | USNB | E | USNB-8-11 |  |

Context

When the current macroblock pair is coded frame decoding mode...

and

When the current macroblock pair is coded field decoding mode...

Question

Those sentences seem to be incorrect.

Proposal

When the current macroblock pair is coded in frame mode...

and

When the current macroblock pair is coded in field mode...

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.2.3.1 | USNB | E | USNB-8-12 |  |

Context

Step 2 of the Derivation process for Intra4x4LumaPred Mode.

Question

In paragraph 3, Intra4x4LumaPredModeA is not defined.

Proposal

Change it to Intra4x4LumaPredMode.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.2.3.1 | USNB | Technical | USNB-8-13 |  |

Context

When MbIdxA > -1 and the mb_type of the macroblock with index MbIdxA is not equal to Intra_4x4 or SI, the variable Intra4x4LumaPredModeA is derived in a process equivalent to:

for( i = 0, i < 4, i++ )

ExtIntra4x4LumaPredMode[ (i+1)*5 ] = 2

Question

What happens in the case that   MbIdxA < 0 (i.e. no neighboring macro-block)?

Proposal

Does this sentence really mean to say,

When MbIdxA < 0 or the mb_type of the macroblock with index MbIdxA is not equal to Intra_4x4 or SI, the variable Intra4x4LumaPredModeA is derived in a process equivalent to

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.2.3.2 | USNB | E | USNB-8-14 |  |

Same as the previous context and question, but for Intra4x4LumaPredModeB.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.2.4.1 | USNB | E | USNB-8-15 |  |

Context

For the derivation process in subclause Error: Reference source not found, for the bottom macroblock of the current macroblock pair with index MbPairIdx, the top macroblock of the current macroblock pair with index MbPairIdx is assigned to MbIdxB as the input. The ouput is assigned to ExtIntra4x4LumaPredModeBottom.

Question

Is subclause Error: Reference source not found right one or subclause Error: Reference source not found2 ?

Proposal

Change the reference from section Error: Reference source not found to  subclause Error: Reference source not found2 .

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.2.5 | USNB | E | USNB-8-16 |  |

Context

The pseudo code defining the derivation of Intra4x4LumaPredMode[4x4LumaBlkIdx] – “for( 4x4LumaBlkIdx = 0,…”

Questions

- Where is the internal variable i defined? Is it the loop variable?

- What does the variable rem_intra4x4_pred_mode (defined in section 7.4.5.1) encode?

- Where is the variable intra_pred_mode_prediction defined?

- “Ext4x4LumaBlkIdx = 4x4LumaBlkIdx – RasterTo4x4LumablkOffset(4x4LumaBlkIdx)”  - this seems to be the mapping of the modes from Intra4x4LumaPredMode to Ext Intra4x4LumaPredMode. This mapping is not unique, e.g. for 4x4LumaBlkIdx=0,2 Ext4x4LumaBlkIdx=0.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.4 | USNB | T | USNB-8-17 |  |

Context

Intra_4x4_Diagonal_Down_Left Description

Question

Text requires the wrong predicted samples to be available.

Proposal

Replace “This mode shall be used only if all A, B, C, D, I, J, K, L, M are available.” with “This mode shall be used only if all A, B, C, D, E, F, G, H are available.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.4 | USNB | E | USNB-8-18 |  |

Context

In the sentence: "This mode shall be used only if all A, B, C, D, I, J, K, L, M are available."

Question

The pixels I, J, K, L, M are not used in calculations, but E, F, G, H are.

Proposal

Change the list to the following: A, B, C, D, E, F, G, H.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.6 | USNB | E | USNB-8-19 |  |

Context

In the sentence: "This mode shall be used only if all A, B, C, D, I, J, K, L, M are available."

Question

The pixel L is not used in calculations.

Proposal

Remove L.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.7 | USNB | E | USNB-8-20 |  |

Context

In the sentence: "This mode shall be used only if all A, B, C, D, I, J, K, L, M are available."

Question

The pixel D is not used in calculations.

Proposal

Remove D.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.8 | USNB | E | USNB-8-21 |  |

Context

In the sentence: "This mode shall be used only if all A, B, C, D, I, J, K, L, M are available."

Question

The pixels I, J, K, L, M are not used in calculations, but E, F, G are.

Proposal

Change the list to the following: A, B, C, D, E, F, G.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.1.3.9 | USNB | E | USNB-8-22 |  |

Context

In the sentence: "This mode shall be used only if all A, B, C, D, I, J, K, L, M are available."

Question

The pixels A, B, C, D, M are not used in calculations.

Proposal

Remove A, B, C, D.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2, 8.3.3 | USNB | T | USNB-8-23 |  |

Context

Intra_16x16 prediction process

Question

"constrained_intra_pred" should be constrained_intra_pred_flag.

Proposal

Fix.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2 | USNB | E | USNB-8-24 |  |

Context

In the sentence: " Let Pred(x,y)  x,y = 0..15..."

Question

The sequence should begin with "-1".

Proposal

Change the sequence to: "-1..15".

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2.3 | USNB | E | USNB-8-25 |  |

Context

Formula (8-11)

Question

Formula (8-11) can be simplified like (8-20)

Proposal

Simplify.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.2.4 | USNB | T | USNB-8-27 |  |

Context

Formulas (8-16), (8-17), (8-18), (8-19)

Question

The variable names are not self-explanatory.

Proposal

Rename: "H" to "horizontal_tilt" or "h_tilt"; "V" to "vertical tilt" or "v_tilt";  "b" to "h_tilt_normalized" or "h_tilt_norm"; "c" to "v_tilt_normalized" or "v_tilt_norm"; "a" to "dc".

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.3 | USNB | E | USNB-8-28 |  |

Context

Chroma intra prediction description

Question

Edits provided during the post-Geneva editing process were somehow not incorporated.

Proposal

Use the following text as the basis for Section 8.3.3 in Awaji.

### Intra prediction process for chroma blocks

This process is invoked for I and SI macroblock types. It specifies how the Intra predicted chroma samples for the current macroblock are derived.

Input to this process are the syntax elements intra_chroma_pred_mode and decoded samples prior to the deblocking process from neighboring luma blocks (if available).

Output of this process are Intra predicted chroma samples for the current macroblock.

[Ed.Note: what about MB-AFF ???]

The chroma in intra macroblocks is predicted in a manner very similar to the luma block in Intra_16x16 macroblock type (subclause Error: Reference source not found), using one of four prediction modes.  The same prediction mode is applied to both chroma blocks, but it is independent of the prediction mode used for the luma.

NOTE - If any portion of the luma macroblock is coded in intra mode, the entire chroma macroblock is coded intra.

Let P(-1,y), y=-1,0,...,7 and P(x,-1), with x=-1,0,…,7 denote the 17 neighbouring samples immediately above and/or to the left of the 8x8 chroma block.  Let Pred(x,y) with x,y = 0..7 denote the prediction for the 8x8 chroma block samples.  There are four different prediction modes as specified in subclauses 8.3.3.1 to 8.3.3.4.  Only Mode 3 uses P(-1,-1).

Samples P(x,-1) or P(-1,y) shall be considered not available if they are outside the picture or outside the current slice, or if they are in a non-intra macroblock and constrained_intra_pred is 1.  If none of the neighbouring samples are available, Mode 2 (DC prediction) is the only possible 8x8 chroma prediction mode.

#### Mode 0: DC prediction process

The 8x8 block of chroma samples is predicted in four separate 4x4 sub-blocks, labeled A-D as shown in Figure 8-7.

Figure 8-7: Chroma 4x4 blocks A, B, C and D; and predictors S0-S3.

In Figure 8-7, each of S0-S3 represents the sum of the four reconstructed samples bordering the chroma block at the location indicated in the figure.  The predictions for all chroma samples within a block (A, B, C or D) are dependent upon the presence of the pixels used to compute S0-S3, and are given in Table 8-3.  Note that S0-S3 can be unavailable by being outside the picture or slice or because constrained intra prediction is in use and the neighboring blocks are not intra coded.

Table 8-3: Chroma intra predictions in DC Mode

| Block | S0-S3 available | S0-S1 available | S2-S3 available | None available |
| --- | --- | --- | --- | --- |
| A | (S0 + S2 + 4)/8 | (S0 + 2)/4 | (S2 + 2)/4 | 128 |
| B | (S1 + 2)/4 | (S1 + 2)/4 | (S2 + 2)/4 | 128 |
| C | (S3 + 2)/4 | (S0 + 2)/4 | (S3 + 2)/4 | 128 |
| D | (S1 + S3 + 4)/8 | (S1 + 2)/4 | (S3 + 2)/4 | 128 |

#### Mode 1: horizontal prediction process

This mode shall be used only if all neighbouring samples P(-1, y), with y = 0..7 are available:

Pred(x, y) = P(-1, y), with x, y = 0..7		(8-20)

#### Mode 2: vertical prediction process

This mode shall be used only if all neighbouring samples P(x, -1), with x = 0..7 are available:

Pred(x, y) = P(x, -1), with x, y = 0..7		(8-21)

#### Mode 3: plane prediction

This mode shall be used only if all neighbouring samples P(x, -1) with x = -1..7 and P(-1, y) with y = 0..7 are available.

For the plane mode, the prediction is formed as:

Pred(x,y) = Clip1( (a + b·(x-3) + c·(y-3) +16) >> 5 ),  with x, y = 0,…,7	(8-21)

where:

a = 16·(P(-1,7) + P(7,-1))		(8-22)

b = (17*H+16)>>5		(8-23)

c = (17*V+16)>>5		(8-24)

and H and V are specified as:

H=x=14x⋅P3+x,−1−P(3−x,−1)	(8-25)

V=y=14y⋅P−1,3+y−P(−1,3−y)	(8-26)

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.3 | USNB | E | USNB-8-29 |  |

Context

Second paragraph:  ... neighboring luma blocks ...

Question

It should be: "neighboring chroma blocks"

Proposal

Fix.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.3 | USNB | E | USNB-8-30 |  |

Context

4th paragraph: In the sentence: "The same prediction mode is applied to both chroma blocks, but it is independent of the prediction mode used for the luma."

Question

This sentence is ambiguous. Because of the previous sentence, the reader may think that the chroma prediction mode must be the same as luma.

Proposal

Replace the sentence to the following two:  "The luma and chroma prediction modes are independent of each other. But both chroma blocks (Cr and Cb) must use the same prediction mode.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.3 | USNB | E | USNB-8-31 |  |

Context

Last paragraph.

Question

The "constrained_intra_pred" is undefined.

Proposal

Change to "constrained_intra_pred_flag".

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.3.3.4 | USNB | T | USNB-8-32 |  |

Context

Chroma plane prediction mode

Question

When can this mode be used?

Proposal

This mode can be used only if all neighbouring samples P(-1,y), P(x,-1) x,y=-1,..,7 are available.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4 | USNB | E | USNB-8-33 |  |

Context

Second paragraph, beginning “Input to this process…”

Question

Proposal

This sentence is a big list.  Fix the use of “and” and of the commas.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1 | USNB | E | USNB-8-34 |  |

Context

Paragraph beginning “When decoding a slice.” (Approximately paragraph 9.)

Question

Proposal

Fix usage of “and” and commas.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1 | USNB | T | USNB-8-35 |  |

Context

At levels 3.1 and higher, direct and bi-predictive block sizes smaller than 8x8 are not allowed.  However, there is a method of inferring 8x8 motion from 4x4 motion defined only for temporal direct mode.

Question

This appears to harm the ability to use spatial mode direct prediction (although spatial direct prediction seems to most often be the most effective type of direct prediction and although it is the prediction method most immune to unusual temporal relationships).

Proposal

The ability to infer 8x8 motion from 4x4 motion should apply not only to the temporal direct mode case but also to the spatial direct mode case (in the same fashion).

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.2.1 | USNB | E | USNB-8-36 |  |

Context

It states in 8.4.1.2.1

When mb_adaptive_frame_field_flag is equal to 1, the following additional rules apply:

- If any of the blocks A, B, C, and D (indicated as X in the following) is coded in field decoding mode, such block is assigned, in addition to its own field MV, two frame MVs for the purpose of motion vector prediction. The first assigned frame MV is the field MV of the block with vertical motion vector component multiplied by 2. The second assigned frame MV is the field MV of the block in the same geometric location as block X in the second macroblock of the macroblock pair with vertical motion vector component multiplied by 2. If any of the blocks A, B, C, and D (indicated as X in the following) is coded in frame mode, such block is assigned, in addition to its own frame MV, two field MVs for the purpose of motion vector prediction. The first assigned field MV is the frame MV of the block with vertical motion vector component divided by 2. The second assigned field MV is the frame MV of the block in same geometric location as block X in the second macroblock of the macroblock pair with vertical motion vector component divided by 2.

Question

When encoding the first macroblock of a macroblock pair with block X is coded in field decoding mode, and block X is contained in the same macroblock as E, how can a value be determined for the second assigned frame MV.  The second assigned frame MV is the field MV of the block in the same geometric location as block X in the second macroblock of the macroblock pair with vertical motion vector component multiplied by 2, and this has not yet been coded.

When encoding the first macroblock of a macroblock pair with block X is coded in frame decoding mode, and block X is contained in the same macroblock as E, how can a value be determined for the second assigned field MV. The second assigned field MV is the frame MV of the block in same geometric location as block X in the second macroblock of the macroblock pair with vertical motion vector component divided by 2, and this has not yet been coded.

Proposal

- If any of the blocks A, B, C, and D (indicated as X in the following) is coded in field decoding mode, and is not contained in the same macroblock pair containing block E, then, this block is assigned two additional frame MVs for the purpose of motion vector prediction. The first assigned frame MV is the field MV of the block with vertical motion vector component multiplied by 2. The second assigned frame MV is the field MV of the block in the same geometric location as block X in the second macroblock of X’s macroblock pair with vertical motion vector component multiplied by 2. If any of the blocks A, B, C, and D (indicated as X in the following) is coded in frame mode, and is not contained in the same macroblock pair containing block E, then, this block is assigned two additional field MVs for the purpose of motion vector prediction. The first assigned field MV is the frame MV of the block with vertical motion vector component divided by 2. The second assigned field MV is the frame MV of the block in same geometric location as block X in the second macroblock of X’s macroblock pair with vertical motion vector component divided by 2.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.2.1 | USNB | E | USNB-8-37 |  |

Context

It states in 8.4.1.2.1

- If any of the blocks A, B, C, and D (indicated as X in the following) is coded in field mode, such block is assigned, in addition to its own field reference number, two reference frame numbers for the purpose of motion vector prediction. The first assigned reference frame number is the reference field number of the block divided by 2. The second assigned reference frame number is the reference field number, divided by 2, of the block in same geometric location as block X in the second macroblock of the macroblock pair. If any of the blocks A, B, C, and D (indicated as X in the following) is decoded in frame mode, such block is assigned, in addition to its own frame reference number, two reference field numbers for the purpose of motion vector prediction. The first assigned reference field number is the reference frame number of the block multiplied by 2. The second assigned reference field number is the reference frame number, multiplied by 2, of the block in same geometric location as block X in the second macroblock of the macroblock pair.

Question

When encoding the first macroblock of a macroblock pair with block X is coded in field decoding mode, and block X is contained in the same macroblock as E, how can a value be determined for the second assigned reference frame number. The second assigned reference frame number is the reference field number, divided by 2, of the block in same geometric location as block X in the second macroblock of the macroblock pair, and this has not yet been coded.

When encoding the first macroblock of a macroblock pair with block X is coded in frame decoding mode, and block X is contained in the same macroblock as E, how can a value be determined for the second assigned field MV.  The second assigned reference field number is the reference frame number, multiplied by 2, of the block in same geometric location as block X in the second macroblock of the macroblock pair, and this has not yet been coded.

Proposal

- If any of the blocks A, B, C, and D (indicated as X in the following) is coded in field mode, and is not contained in the same macroblock pair containing block E, then, this block is assigned two additional reference frame numbers for the purpose of motion vector prediction. The first assigned reference frame number is the reference field number of the block divided by 2. The second assigned reference frame number is the reference field number, divided by 2, of the block in same geometric location as block X in the second macroblock of X’s macroblock pair. If any of the blocks A, B, C, and D (indicated as X in the following) is decoded in frame mode, and is not contained in the same macroblock pair containing block E, then, this block is assigned two additional reference field numbers for the purpose of motion vector prediction. The first assigned reference field number is the reference frame number of the block multiplied by 2. The second assigned reference field number is the reference frame number, multiplied by 2, of the block in same geometric location as block X in the second macroblock of X’s macroblock pair.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.2.2 - 8.4.1.4.2.3 | USNB | E | USNB-8-38 |  |

Context

Before every equation for Z:

Question

Proposal

- Add "In the calculations for Z, if the denominator = 0, then set Z=256.

- Product Z*MV shall not exceed 2^31 in absolute value.

- Also, replace equations involving W  by  MVsub1 = MVsub0 – MV

- Also, correct Figs. 8-11 to 8-15.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.2.2 | USNB | T | USNB-8-39 |  |

Context

The second “-“ paragraph, concluding with “Otherwise, if a neighbouring block has both motion vectors using a reference picture in the same temporal direction as the reference picture associated with MVi but they point to different reference pictures, then the motion vector with the associated temporally closest reference picture is used.”

Question

Does “temporally closest” mean closest to the reference picture associated with MVi, or closest to the current picture? If the former, the two candidates may be equally “close” and a means for breaking the tie is needed.

Proposal

Change “then the motion vector with the associated temporally closest reference picture is used” to “then the motion vector referencing the picture closer to the current picture is used.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.2.2 | USNB | E | USNB-8-40 |  |

Context

Figure 8.8

Question

Is Figure 8.8 referred to in the text?

Proposal

Make appropriate reference.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.3 | USNB | E | USNB-8-41 |  |

Context

Second sentence in the main paragraph ends “the prediction is specified in subclause.”

Question

Proposal

Specify subclause 8.4.1.3.2.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.4.2.2 | USNB | E | USNB-8-42 |  |

Context

The third paragraph begins with:

The reference picture indices for the neighbouring blocks A, B, C, and D within the current slice for the current luma macroblock E, for both Direct_16x16 macroblock and Direct_8x8 sub-macroblock types, as described in subclause Error: Reference source not found and shown in Error: Reference source not found, shall be used to determine the reference picture index for each list.

Question

The text refers to luma macroblock E.  This is the only time E is referred to as a macroblock.  The diagram in figure 8.7 refers to E as a block.

Proposal

Change “luma macroblock E” to “luma block E”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.4.2.3 | USNB | E | USNB-8-43 |  |

Context

Labels to figures 8-11, 8-12,  8-13, 8-14 and 8-15

Question

The figures contain labels MVF0 , MVB0 etc,  however the text refers to MV0  , MV1

Proposal

Modify figures to reflect the text.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.1.4.2.3 | USNB | E | USNB-8-44 |  |

Context

Brackets on equations

Question

Consistent use of brackets in equations.

Proposal

Be consistent.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.2.2.1 (of JVT-E146d37) | USNB | T | USNB-8-45 |  |

Context

The two-sample average used in motion compensation does not add 1 before the right shift as was done in prior video standards.

Question

This is inconsistent with the definition of two-sample averaging defined for most video processors.

Proposal

The two-sample average used in motion compensation should add 1 before shifting right for consistency with B-picture reference block averaging and for compatibility with instruction sets that optimize for this style of averaging used in prior video standards.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.4.2.3.2 | USNB | T | USNB-8-48 |  |

Context

Implicit weighted prediction.

Question

There seems to be a problem in the details of implicit weighted prediction as specified.  The default value of LWD appears to be 7 where 6 would seem to make more sense.  The range of weight values producible by these equations should be examined.  Particular consideration should be given to cases in which both references are in the same picture order count direction.

Proposal

Examine this subclause and fix as necessary.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.5.2 | USNB | E | USNB-8-49 |  |

Context

Wrong references to Equations

Question

There are 7 references to Equations in the text. They are all wrong. In all but one case, the correct equation number should be three less than that specified in the text.

Proposal

Fix.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.5.2.3 | USNB | E | USNB-8-50 |  |

Context

Wrong references to Equations

Question

There are 6 references to Equations in the text. They are all wrong. The correct equation number should be three less than that specified in the text.

Proposal

Fix.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.5.3 | USNB | E | USNB-8-51 |  |

Context

Wrong reference to Equation 8-55.

Question

It should be Equation 8-52.

Proposal

Fix.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7 (of JVT-E146d37) | USNB | T | USNB-8-52 |  |

Context

Deblocking operation with respect to PCM macroblocks.

Question

Deblocking operation with respect to PCM macroblocks must be clearly defined

Proposal

PCM macroblocks should be treated for deblocking purposes as inter macroblocks with QPY= 0, all-zero transform coefficients, and all-zero motion vectors.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7 | USNB | T | USNB-8-54 |  |

Context

Deblocking filtering process

Questions

The deblocking filtering is based on the frame samples even when the MB-AFF is on and the current MB pair is field coded. This causes undesirable artifacts for interlaced video coding using MB-AFF. The motion between the two fields is blurred.

Question:

The deblocking filtering in the case of MB-AFF needs to be re-designed. Simply reverse to the FCD solution has its own issues too. Is there a solution?

Proposal

This needs to be fixed. A contribution (JVT-F027) on this with a proposed solution to the Awaji JVT meeting is made.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7 | USNB | T | USNB-8-55 |  |

Context

Chroma filtering is not described well.

Question

If ap and aq do not have to be computed, it should be described more clealy, especially in Section 8.7.3.

Proposal

Add a few sentences on chroma filtering. For example, at the end of Section 8.7.3, you could add, “With this approach, ap and aq does not need to be calculated.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.1 | USNB | T | USNB-8-56 |  |

Context

In the second paragraph, the text says “A special procedure with even stronger filtering may be applied on macroblock boundaries with both macroblocks coded using Intra macroblock prediction mode (Bs=4).”

Question

There is a discrepancy between the above text and Figure 8-22.  Figure 8-22 suggests that Bs = 4 is obtained on macroblock boundaries if any sample of the neighboring macroblocks is coded using Intra macroblock prediction mode. It does not require that both macroblocks be coded using Intra macroblock prediction mode.

Proposal

Eliminate the discrepancy by modifying the text to match Figure 8-22.  The text could say, “A special procedure with even stronger filtering may be applied on macroblock boundaries if either of the neighboring macroblocks were coded using Intra macroblock prediction mode (Bs=4).”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.1 | USNB | T | USNB-8-57 |  |

Context

The last condition for Bs=1 says “any corresponding pair of motion vectors from the two neighbouring macroblocks is referencing the same picture and either component of this pair has a difference of more than one sample.”

Question

There is a discrepancy between the above text and Figure 8-22.

Proposal

In order to match with Figure 8-22, the last condition should read “any corresponding pair of motion vectors from the two neighboring macroblocks is referencing the same picture and either component of this pair has a difference of one sample or more”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.1 | USNB | E | USNB-8-58 |  |

Context

Description of neighbors for each edge

Question

The text uses the term “neighboring macroblocks” which does not correspond to Figure 8-22’s use of the term “neighboring 4x4 luma blocks”.  The use of macroblocks is also confusing since the intent is to examine neighboring 4x4 luma blocks not the 16x16 macroblock.

Proposal

Change all the references in the text that use “neighboring macroblocks” to “neighboring 4x4 luma blocks”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.1 | USNB | E | USNB-8-59 |  |

Context

The caption for Figure 8-22 is confusing.

Question

The notation for motion vectors, V, is not used elsewhere.  What does “for i= 1 to 1,2 or 4” mean?

Proposal

Use consistent notation for motion vectors. Also, define more precisely what “for i= 1 to 1,2 or 4” means or at least reference where a precise definition can be found.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.2 | USNB | E | USNB-8-60 |  |

Context

First line of first paragraph

Question

Typo.  Currently: “the input sample values a single line”

Proposal

Correction: “the input sample values for a single line”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.2 | USNB | E | USNB-8-62 |  |

Context

Text under equation 8-78

Question

Type.  Currently: “chroma blocks used as input to to Equation 8-78”

Proposal

Correction: “chroma blocks used as input to Equation 8-78”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.2 | USNB | E | USNB-8-63 |  |

Context

End of first paragraph, last sentence reads “However, p1 and p2 may indicate samples that have been modified by the filtering of a previous block edge.”

Question

The sample P3 may also have been modified.

Proposal

The sentence could read “However, p1 , p2 , and p3  may indicate samples that have been modified by the filtering of a previous block edge.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.2 | USNB | T | USNB-8-64 |  |

Context

There is no discussion on how QPav is determined in cases where macroblock skipping occurs.

Question

In the case where MBs are skipped, how is  QPav computed? Also, in the case of MB pairs where one of the MBs is skipped, how is QPav computed?

Proposal

The section should include a discussion dealing with MB skipping. Maybe a NOTE would suffice.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.3 | USNB | E | USNB-8-65 |  |

Context

First paragraph

Question

Reference source for IndexA not found

Proposal

Should refer to Table 8-10

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 8.7.4 | USNB | T | USNB-8-67 |  |

Context

The text, “For luma and for chroma samples, for which the condition described in Equation 8-68 does not hold, the filter described in Equation 8-72 shall be applied.”

Question

Does chroma utilize the filter in 8-82 always?

Proposal

If yes, the text should read  “For chroma samples, and for luma samples for which the condition described in Equation 8-78 does not hold, the filter described in Equation 8-82 shall be applied.”

## Clause 9: Entropy Coding

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.2 | USNB | T | USNB-9-1 |  |

Context

Third paragraph in 9.2, indicating syntax elements coded using ce(v).

Question

‘trailing_ones_sign’ is included in the list; it is coded using u(1).

Proposal

Remove ‘trailing_ones_sign’ from the sentence.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.2.2 | USNB | E | USNB-9-2 |  |

Context

Last paragraph of section 9.2.2.

Question

The number of “x”’s is incorrect in two places.

Proposal

Change “19 bits, four “x””s” to “19 bits, three “x”’s”

Change “28-bits, 12 “x”’s” to “28 bits, eleven “x”’s”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3 | USNB | T | USNB-9-3 |  |

Context

Context selection for CABAC decoding process

Question

The CABAC context for decoding a syntax element is usually determined by the neighboring MBs. However, when a neighboring MB doesn’t contain the same syntax element, what value should be used for the missing element of the neighboring MB?  An example is the CtxAbsMvdComp[] calculation.

Proposal

The text should specify for each of such cases the value that should be used for context calculation when a neighboring MB doesn’t have the same sysntax element.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3 | USNB | T | USNB-9-4 |  |

Context

MB availability for CABAC context calculation

Question

- What’s the definition of MB availability for CABAC context calculation?

- Is a skipped MB considered available if it’s in the same slice as the current MB?

Proposal

Please explicitly specify these definitions.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3 | USNB | T | USNB-9-5 |  |

Context

CABAC context selection when MB-AFF is enabled

Question

What are the adjacent MBs for the purpose of context selection, for each of  the following cases:

- frame coded Current MB-pair, and frame coded upper MB-pair,

- frame coded Current MB-pair, and field coded upper MB-pair,

- frame coded Current MB-pair, and frame coded left MB-pair,

- frame coded Current MB-pair, and field coded left MB-pair,

- field coded Current MB-pair, and frame coded upper MB-pair,

- field coded Current MB-pair, and field coded upper MB-pair,

- field coded Current MB-pair, and frame coded left MB-pair,

- field coded Current MB-pair, and field coded leftr MB-pair,

Proposal

Specify the neighboring MBs for each of the above cases for the purpose of CABAC context calculation

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.1 | USNB | E | USNB-9-6 |  |

Context

Last two sentences of 9.3.1.

Question

The paragraph references have 9.3.1.5 in the second group. It should be in the first group.

Proposal

Change “9.3.1.1 – 9.3.1.4” to “9.3.1.1 – 9.3.1.5”;
Change “9.3.1.5 – 9.3.1.8” to “9.3.1.6 – 9.3.1.8”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.1.1 | USNB | E | USNB-9-7 |  |

Context

Third paragraph of 9.3.1.1, saying “For a code symbol C it consists of |C| 1 bits”…

Question

C must be positive, so use of “|C|” is redundant and confusing. In the following paragraphs, the value of C is simply referred to as “C”.

Proposal

Change “consists of |C| 1 bits” to “consists of C 1 bits”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.1.4 | USNB | T | USNB-9-8 |  |

Context

The equation “L = log2 Cmax + 1” has the floor function in the wrong place.

Question

Proposal

Replace the equation with “L =  log2 Cmax  + 1”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.1.5 | USNB | E | USNB-9-9 |  |

Context

Table 9-21

Question

Proposal

For the binarization for P slices, mb_type = P_8x8 should have a value of 3 (instead of 4) and mb_type=P_8x8ref0 should have a value of 4 (instead of 6) as seen in Table 7-11.  Intra, prefix only then should have values ranging between 5 to 30 (instead of 7-32).

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.1.7 | USNB | E | USNB-9-10 |  |

Context

Near the middle of the main paragraph, a sentence begins with “If this is the case…” The condition referred to by “this” is not in the previous sentence.

Question

Proposal

Replace “If this is the case,” by “If significant_coeff_flag[i] has the value of one,”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.2 | USNB | E | USNB-9-11 |  |

Context

Table 9-23

Question

Proposal

For the syntax element coeff_absolute_value_minus_1, range of context label should be 203-251 (instead of 203-252)

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.2 | USNB | E | USNB-9-12 |  |

Context

Table 9-23 under the heading of “Type of binarization”

Question

What is subclause 0 in Table 9-23?  It is listed for both mb_skip_flag and mb_field_decoding_flag.

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.2.1 | USNB | T | USNB-9-13 |  |

Context

Table 9-25, entry for Context variable CtxCbpLuma[i] says “X: neighbouring 8x8 block of i-th block” but “i” indexes a bit, not a block.

Question

Proposal

Delete “of i-th block” or replace “of i-th block” with “of C”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.2.1 | USNB | T | USNB-9-14 |  |

Context

Table 9-25, entry for Context variable CtxCodedBlock. X is defined as “neighbouring block of same BlockType”

Question

The following paragraph lists the 6 relevant block types; table 9-27 has 12 entries under BlockType.

Proposal

Change “neighbouring block of same BlockType” to “neighbouring block of same type”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.2.1 | USNB | T | USNB-9-15 |  |

Context

Table 9-25, entry for Context variable CtxCodedBlock, under “if X not available” says “1, if X is INTRA; 0, otherwise”

Question

X may be unavailable because the neighbouring macroblock does not exist in the slice, or because the neighbouring macroblock does not contain a block of the “same type”. In the first case, what value is used for CondTerm(X)? In the second case, does “is INTRA” refer to the neighbouring macroblock?

Proposal

Change “1, if X is INTRA; 0, otherwise” either to

“1 if the current macroblock is INTRA; 0, otherwise” or to

“1 if the neighbouring macroblock is available and is INTRA; 0, otherwise”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.2.1 | USNB | T | USNB-9-16 |  |

Context

End of paragraph following Table 9-25.

Question

The second-to-last sentence indicates that frame mode is to be assumed when determining the context for decoding mb_skip_flag. The last sentence is an example that seems to indicate that field mode is assumed.

Proposal

Change the last sentence:

“That means, for instance, that for context determination the macroblock on top of the current skipped macroblock in a (potential) top field position is always chosen as the bottom field MB of the MB pair above the current macroblock.”

to

“That means, for instance, that when decoding mb_skip_flag for a (potential) top field position of a MB, the bottom frame MB of the MB pair above the current MB is always chosen for determining context.”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.2.1 | USNB | T | USNB-9-17 |  |

Context

Table 9-25

Question

-For context variable ctxRefIdx[1], if  the neighboring block is coded as Intra or coded as Direct_16x16, ref_idx does not exist for the neighboring block.  Do we assume in this case that condTerm(X) is 0 even though X = neighboring_block does exist?

-For context variable CtxIpredChroma[1], if the neighboring block is coded as Inter, then intra_chroma_pred_block does not exist for the neighboring block.  Do we assume in this case that condTerm(X) is 0 even though X = neighboring_block does exist?

In general, if the neighboring block does exist but does not have the desired syntax element to determine the context template, do we assume condTerm(X) = 0 (except for the case of CtxCodedBlock) ?

Proposal

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.3.1 | USNB | E | USNB-9-19 |  |

Context

Derivation process for the CtxIdx

Question

The fifth paragraph contains “MaxBinIdxEqProb”, which has no definition and should be “MinBinIdxEqProb”.

Proposal

Change “MaxBinIdxEqProb” to “MinBinIdxEqProb”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.4.2.6 | USNB | T | USNB-9-20 |  |

Context

The text that is as follows: When the decision S is equal to 1 no renormalization is carried out and CABAC decoding is terminated, and its associated figure, Figure 9-8

Question

Suppose CABAC encoding is used and a MB is of type PCM.  In this case, the second bit of the binarization of mb_type is 1 and decode_terminate is used to decode the symbol.  Since S=1 in this case, that means the CABAC decoding is terminated. Does that mean on the corresponding encoding side, the CABAC encoder has to be flushed out at this point when it has to encodes this symbol (S=1)?

After decoding the PCM MB, there is still the end_of_slice_flag that needs to be decoded using CABAC.  Since the CABAC decoding was terminated, how is the end_of_slice_flag decoded?  Is the CABAC decoder re-initialized?  But according to Sec 9.3.3.1, it seems like the decoder is only reinitialized at the beginning of each slice.

If several MBs are coded as mb_type = PCM, does that mean the CABAC decoder has to terminate and be re-initialized for each of those MB since some of the syntax that comes before the PCM encoding is encoded using CABAC?

Proposal

Remove PCM mode or clarify CABAC behavior with PCM mode.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.4.3.1 | USNB | E | USNB-9-21 |  |

Context

Encoding process for a decision (informative)

Question

The use of 0x1FE as the initial value for ‘R’ guarantees that the first bit output from the CABAC is a 0.  The initialization of BC to 8 properly suppresses this initial bit.  Some explanation of this behavior would be very helpful.

Proposal

After “set to …0x01FE” add

“

The initialisation of BC to 8 causes the first bit of the CABAC not to be output (see Figure 9-13).  This is proper behavior, as the informative algorithm described in section 9.3.4.3 generates an initial ‘0’ bit that should not be sent to the decoder.

“

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| 9.3.4.3.6 | USNB | T | USNB-9-22 |  |

Context

“stop_bit” is mentioned, but not defined. (See also 9.3.4.2.6)

Question

Is “rbsp_stop_bit” meant?

Proposal

Change “stop_bit” to “rbsp_stop_bit”.

Also, add a note to figure 9-16, indicating that the “PutBit(1)” block is inserting rbsp_stop_bit.

## Annex A: Profiles and Levels

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3 | USNB | E | USNB-A-8 |  |

Context

sub-paragraph g)

Question

The reference to subclause A.6.3 is wrong.

Proposal

Change it to subclause A.3.2

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.1 and 8.4.1.4.2.2 | USNB | T | USNB-A-9 |  |

Context

The level limit on the minimum luma block size for direct mode macroblocks is 8x8 for levels 3.1 and above.

Question

DIS doesn’t specify the semantics of this limit for the spatial direct mode. How is the limit applied to the spatial direct mode MBs?

Proposal

Specify the semantics of the limit for spatial direct mode. A separately contribution (JVT-F030) with solution to the Awaji AVC/JVT meeting is made.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.1 | USNB | T | USNB-A-10 |  |

Context

Maximum Video Bitrate in Table A-1

Question

According to the Geneva meeting report (page 13), it was agreed that the maximum video bitrate for Levels 3 and 4 would be changed to 10 Mbit/s and 50 Mbit/s respectively. These values are not reflected in the table.

Proposal

Update the table with new bit rates.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.1 | USNB | E | USNB-A-11 |  |

Context

Table A-1  column 3

Question

Proposal

Change MBs to Macroblocks

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| A.3.3 | USNB | E | USNB-A-12 |  |

Context

Note 4 for Table A-3.

Question

The second part of the sentence is incorrect because the “MB High” column has been removed from the table.

Proposal

“Frame rates given are correct for progressive scan modes, and for interlaced if the Sample Height is divisible by 32.”

## Annex B: Byte Stream Format

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| B.2 | USNB | E | USNB-B-1 |  |

Context

5th paragraph

Question

Missing reference to another subclause.

Proposal

It should be subclause 7.4.1.

## Annex C: Hypothetical Reference Decoder

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1 | USNB | E | USNB-C-1 |  |

Context

Hypothetical Reference Decoder ….

Question

Packet stream not defined

Proposal

Remove “(either a byte stream or a packet stream)” from the third paragraph of C.1.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.1 | USNB | E | USNB-C-2 |  |

Context

HRD

Question

The language describing the bits associated with a picture for HRD purposes is unclear.

Proposal

Replace “associated with primary” in the second sentence of C.1.1 with “containing primary”.

Replace “The first bit of” in the second sentence of C.1.1.1 with “The first bit associated with”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.1.2 | USNB | clarification | USNB-C-4 |  |

Context

Timing of coded picture removal

Question

Proposal

After Eq. (C-6) make the following correction

- At this time, the coded data for the  n’th picture is removed from the CPB

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.1.3 | USNB | T | USNB-C-5 |  |

Context

HRD

Question

The “maximum removal rate” should be increased by a factor of two.  The present restriction unnecessarily limits flexibility in selecting CPB removal times.

Proposal

Replace “the ratio of the number of macroblocks” in the sentence below equation C-9 with “the ratio of half the number of macroblocks”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.1.3 | USNB | T | USNB-C-6 |  |

Context

Removal time consistency

Question

Equation (C-8) is not correct – should be …

Proposal

Change Eq. (C-8) to

initial_cpb_removal_delay  90000 ×(tr(n) -  tai(n))

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.1.3 | USNB | T | USNB-C-7 |  |

Context

Big Picture Removal time, ….

Question

The case applies only to low delay: i.e. low_delay_hrd_flag = 1;

Proposal

After “Big Picture ….. Underflow Prevention:” add “If low_delay_hrd_flag =1 and the final arrival time taf(n) …..”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.2.1 | USNB | T | USNB-C-8 |  |

Context

Removal of pictures from the DPB

Question

The explanation applies only to dec_pic_buffering_mode 1. No explanation for the case of

the sliding window

Proposal

After eq. (C-10) add the following

If dec_pic_buffering_mode = 0, the picture which is stored in the DPB the longest is removed, if the DPB fullness is equal to the size of the DPB buffer. Else ….

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.2.1 | USNB | T | USNB-C-9 |  |

Context

Removal of pictures from the DPB

Question

Exact definition of removal time needed

Proposal

###### Before last paragraph of C.1.2.1 add the following

The removal time from the DPB obeys the following rule:

If dec_pic_buffering_mode = 0: at the decoding time of the reference picture (which is inserted

into the DPB instead)

If dec_pic_buffering_mode = 1:

- at the decoding time  which states the picture as “unused for reference”, when the output time already elapsed, or

- at the output time, when the picture was already marked as “unused for reference”

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.2.2 | USNB | T | USNB-C-10 |  |

Context

Picture decoding

Question

This subsection relates to operation of the DPB – so this needs to be related to that

Proposal

After the removal step from the CPB, picture n is decoded. If picture n is stored in the DPB, it enters the DPB at its removal time tr(n).

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| C.1.2.3 | USNB | E | USNB-C-11 |  |

Context

Question

Proposal

should be 8.2.X.X.X instead of 8.3.X.X.X …

## Annex D: SEI

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.1.9 & D.1.11 (of JVT-E146d37) | USNB | T/E | USNB-D-1 |  |

Context

target_frame_num and subseq_frame_num are ue(v) while original_frame_num and frame_num are u(v).  All of these are frame numbers with range 0 to MaxFrameNum.

Question

Is there a good reason for the inconsistency in the way these are represented?

Proposal

We think these should all be u(v) with number of bits determined by log2_max_frame_num_minus4.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.1.10 (of JVT-E146d37) | USNB | T/E | USNB-D-2 |  |

Context

scene_id and second_scene_id are u(8), while snapshot_id and progressive_refinement_id are u(32).

Question

This appears to significantly limit the degree to which an encoder can uniquely label scenes.

Proposal

scene_id and second_scene_id should perhaps be u(32) or ue(v) to allow more unique labeling of scenes.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.1 | USNB | T | USNB-D-3 |  |

Context

Initial_cpb_removal_delay

Question

Coded data associated with the first picture in the case that HRD pertains to NAL ? this should include all NAL data including the buffering period SEI message)

Proposal

The definition should be:

initial_cpb_removal_delay: This syntax element represents the delay between the time of arrival in the CPB of the first bit of the coded data associated with the first picture following the buffering period SEI message (including the buffering period SEI message and all NAL data in the case that the HRD pertains to the NAL)

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.2 | USNB | T | USNB-D-4 |  |

Context

pic_structure semantics

Question

In the Geneva meeting, codepoints 5 and 6 were added to support repeat fields due to the 3:2 pulldown conversion process from 24 frames per second film material to 60 fields per second interlaced video.

Progressive formats such as 1280x720p and 720x480p may operate at 60 frames per second. To display 24 frames per second film material for these formats, successives frames from the film source are displayed either three times or twice to achieve a frame rate of 60 times per second.

Proposal

Add two code points to represent display of a frame twice and three times. Suggested additions to Table D-1.

| Value | Indicated display of picture | Restrictions | NumClockTS |
| --- | --- | --- | --- |
| 7 | Frame displayed twice | field_pic_flag must be 0 | 2 |
| 8 | Frame displayed three times | field_pic_flag must be 0 | 3 |

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.2 | USNB | E | USNB-D-5 |  |

Context

nframes semantics

Question

In this section, it is unclear what the term “last previous picture in output order” means. This term appears four times in the text.

Proposal

Change “last previous” to “previous”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.2 | USNB | E | USNB-D-6 |  |

Context

seconds_value, minutes value and hours_value semantics

Question

All three sections use the term “last previous xxx_value”.

Proposal

Change “last previous” to “previous”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.3 | USNB | E | USNB-D-8 |  |

Context

5th, 6th and 7th paragraphs.

Question

These paragraphs refer to the frame cropping parameters in the VUI. However, the parameters names are wrong – cropping_rect_xxxx, where xxxx = top, bottom, left or right.

Proposal

Change cropping_rect_xxxx to frame_cropping_rect_xxxx.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.7 | USNB | E | USNB-D-9 |  |

Context

exact_flag semantics

Question

- The first sentence uses the term “last prior IDR point”.

- In the second paragraph, it states: “…. all references to unavailable reference pictures shall be inferred as references to sample values given by Y=Cb=Cr=128 (mid-level grey) in intra macroblocks for the purposes of determining the conformance of the value of exact_match_flag”.
The term “in intra macroblocks” has nothing to do with the generation of the pixels.

Proposal

- Change “last prior” to “previous” in first question.

- Remove the term “in intra macroblocks”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.7 | USNB | E | USNB-D-10 |  |

Context

broken_link semantics

Question

The second sentence uses the term “last previous IDR point”. What does it mean?

Proposal

Change “last previous” to “previous”.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.10 | USNB | T | USNB-D-11 |  |

Context

Scene Information SEI message usage.

Question

- When a scene information SEI is received, does it take effect at the next coded picture boundary?

- Does this scene information persist until the next scene information SEI is received?

Proposal

Clarify in the description of this message.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| D.2.11-D.2.13 | USNB | T | USNB-D-12 |  |

Context

The use of sub-sequence SEI messages

Question

From the description of the semantics of the three sub-sequence messages, it is unclear how these messages should be used.

Proposal

Insert an informative section describing how sub-sequences work, and how the sub-sequence messages are used.

## Annex E: VUI

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | USNB | T/E | USNB-E-1 |  |

Context

In the indication of vertical and horizontal motion vector length limits (log2_max_mv_length_vertical and log2_max_mv_length_horizontal) in the sequence parameter set VUI, the vertical parameter appears first in the syntax.

Question

In all other places in the syntax where it makes no technical difference, horizontal parameters are sent before corresponding vertical parameters.

Proposal

Swap the order of these two syntax elements.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | USNB | T/E | USNB-E-2 |  |

Context

max_bits_per_mb_denom has a default value of 1, which is the smallest expressible denominator (other than no limit) and corresponds to 384 bytes per MB.

Question

The range of values for this parameter (other than a "no limit" expression) does not go up to 400 bytes per MB (the limit imposed in every profile & level), and the default value could be violated by otherwise-conforming bitstreams.

Proposal

The default value should be 0 (no limit expressed) and/or the range of values for the parameter should be increased to encompass the 400 byte limit imposed by all profiles and levels.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | USNB | T/E | USNB-E-3 |  |

Context

The default value of  max_bytes_per_pic_denom is 4 (at least 4:1 compression) and the interpretation of this parameter for coded fields indicates an additional factor of two in compression relative to that of two fields being coded as a frame.

Question

The range of values for this parameter (other than a "no limit" expression) does not go up to ½ of the maximum picture size supported by the profile & level (the limit imposed in every profile & level), and the default value could be violated by otherwise-conforming bitstreams.  The definition for fields appears to be a mistake.

Proposal

The default value should be 0 (no limit expressed) and/or the range of values for the parameter should be increased.  Also the limit imposed on fields should be consistent with that imposed on frames.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | USNB | T/E | USNB-E-4 |  |

Context

Alternative chroma sample locations in sequence VUI are different for frames and fields, and the positions for frames conflict with some of the positions for fields.

Question

This seems to mean that if the encoder chooses to use picture-level adaptive frame/field coding, the indicated chroma sample positions will move.  This appears to conflict with the intent for the sampling positions to be fixed at the sequence level.

Proposal

Alternative chroma sample locations in sequence VUI should be for top fields and bottom fields rather than for frames and fields.  This way the indicated spatial chroma sample locations will not move when the picture coding method changes from a field to a frame or vice-versa, as the positions for frames will be the merger of the positions for their two constituent fields.  Sample locations corresponding to current frame sample locations mode 1, 2, and 3 of Figure E-1 should be supported.  The need for the other locations indicated in Figure E-2 should be reviewed.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | USNB | E | USNB-E-5 |  |

Context

Semantics of fixed_frame_rate_flag.

Question

The semantics of fixed_frame_rate_flag should be clarified with respect to 3:2 pull-down (for which some frames last three field times and some last two field times).

Proposal

Clarify such that fixed_frame_rate_flag indicates that the temporal distance between the output of the last field of one frame and the output first field of the next frame in output order as indicated in picture timing SEI is constant throughout the sequence.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 (of JVT-E146d37) | USNB | E | USNB-E-6 |  |

Context

Semantics of num_units_in_tick and time_scale.

Question

The interpretation of zero values for these parameters seems confusing.

Proposal

Clarify that external timing information is needed to verify HRD conformance when num_units_in_tick and time_scale are equal to zero.  Consider limiting use of other syntax under these circumstances (e.g., not allowing picture timing SEI messages as they become uninterpretable).

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 | USNB | T | USNB-E-7 |  |

Context

video_full_range_flag semantics

Question

- The round() function is not defined. Does it mean round up or round down to nearest integer value?

- Should the Clip3() function be used to ensure resulting values are clipped to a valid range based on the flag value?

Proposal

- Define round() function in this subclause or Clause 5.

- Apply Clip3() function to equations if appropriate.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 | USNB | E | USNB-E-8 |  |

Context

transfer_characteristics semantics

Question

Missing reference.

Proposal

It should be Table E-4.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.1 | USNB | E | USNB-E-9 |  |

Context

matrix_coefficients semantics

Question

- Missing reference should Table E-5

- The left-side of the second equation (E’PB) is wrong. It should be E‘PR.

Proposal

Fix.

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.2 (of JVT-E146d37) | USNB | T/E | USNB-E-10 |  |

Context

cpb_cnt cannot be zero.

Question

Why bother allowing a value of zero to be sent syntactically but disallow it semantically?  Most such syntax elements are sent by sending the desired value minus 1, so the current form is inconsistent with the rest of the design.

Proposal

Rename cpb_cnt to cpb_cnt_minus1 and adjust semantics accordingly

| Clause | National Body | Type | Reference | Disposition |
| --- | --- | --- | --- | --- |
| E.2.3 (of JVT-E146d37) | USNB | T/E | USNB-E-11 |  |

Context

Frame cropping rectangle of picture parameter set.

Question

It seems difficult to clearly determine how the frame cropping rectangle is intended to operate with respect to chroma, because in 4:2:0 video there is only one chroma sample per two luma samples vertically and horizontally.

Proposal

frame_cropping_rect_{left,right,top,bottom} parameters should be in units of two luma samples so that the relationship between chroma and luma is easily preserved through the cropping operation.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: