| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 7th Meeting: Pattaya II, Thailand, 7-14 March, 2003 | Document: JVT-G024 Filename: JVT-G024.doc |
| --- | --- |

| Title: | 2nd Choice Regarding G023: A New Baseline Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yasser Syed, Mukta Kar, et al. 400 Centennial Pkwy Louisvillle, Co 80027 USA | Tel: Email: | 303-664-8149/303-661-3478 y.syed@cablelabs.com, m.kar@cablelabs.com |
| Source: | Cablelabs, Motorola, Scientific-Atlanta, Broadcom |  |  |

____________________________

# Summary

If the non-hierarchical AVC/H.264 profile structural concerns are not fully addressed by the partial interoperability in the current profile structures (JVT-G023), then a new baseline profile (BP) is proposed. It meets the requirements of applications that need the tools of current    BP    except the error resiliency (ER) tools. This profile also meets the requirements of hierarchical profile structure mentioned in Terms of Reference (TOR).

# Comment

Three error resiliency (ER) tools (ASO, FMO and RS) provide little or no benefit to the main profile (MP) targeted applications as mentioned in JVT G-023. On the other hand it adds complexity to the silicon and lot more complexity to the conformance testing. All these lead to significant extra cost which is an unnecessary burden to MP targeted applications. After much discussion at the Klagenfurt meeting in July 2003 an agreement has been reached where 3 error resilient (ER) tools (ASO, FMO and RS) have been removed from the Main Profile (MP). Removal of ER tools from MP makes BP no longer a true subset of MP and gave rise to a non-hierarchical profile structure but could be addressed by the partial interoperability which allows the creation of bitstreams decodable by all existing profiles. According to the Terms of Reference (TOR), the agreement between ITU and ISO/MPEG, there should be a baseline profile which is a common subset of all other profiles, and this BP should be Royalty-free.

However questions have arisen in the last two meetings about the non-hierarchical H.264 profile structure that does not meet ITU and ISO/MPEG TOR. Also current BP does not meet the requirement of some applications that only need BP compression tools and not the ER tools.

# Proposal

As a response to the above concerns and only if the concerns are strongly felt to not be addressed by current profile structures, we propose to create a fourth profile: Current Baseline minus the ER tools ( = current BP –    (ASO, FMO, RS) ) which is a profile that is a common subset to all other profiles. This can be done by setting the sequence parameter set flags for the current baseline profile: 1)arbitrary_slice_order_allowed_flag , 2) more_than_one_slice-group_allowed_flag, and 3) redundant_pictures_allowed_flag to be all permanently equal to zero.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int    Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.    JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.    If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.    The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.    Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: