| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 7th Meeting: Pattaya II, Thailand, 7-14 March, 2003 | Document: JVT-G028 Filename: JVT-G028.doc |
| --- | --- |

| Title: | Normalized Computational Cost (NCC): A Platform-Independent Speed-up Measurement Metric for Fast Block-Matching Motion Estimation in JVT/H.264 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kai-Kuang Ma and Gang Qiu School of Electrical Electronic Engineering Nanyang Technological University Block S2, Nanyang Avenue Singapore 639798 | Tel: Email: | +65-6790-6366 ekkma@ntu.edu.sg |
| Source: | Nanyang Technological University, Singapore |  |  |

_____________________________

# Introduction

Currently, the computational time is considered in the JVT community as the yardstick for evaluating and comparing the speed-up of fast block-matching motion estimation. However, the computational time is not a suitable measurement for conducting a fair comparison of various motion estimation algorithms performed in JVT/H.264 framework. Because, it highly depends on not only the hardware platforms but also the software implementations (e.g., code efficiency). In the previous video coding standards (e.g., MPEG-4), the number of search points (per macroblock) has been widely used as the objective measurement for gauging the achieved speed-up in fast motion estimation. Unfortunately, it is invalid to use the search point in the JVT/H.264 encoding framework, because JVT/H.264 involves various block sizes (or modes) and multi-frame-based motion estimation. A new metric, called normalized computational cost (NCC), is suggested here to circumvent the above-mentioned problem by adding a weighting factor and normalization. Our experiments conducted in JVT JM encoders [1][2][3] have shown that the proposed metric is platform independent and suitable for objective computational cost measurement for comparing fast block-matching motion estimation algorithms.

# Normalized Computational Cost (NCC)

For each block search, a platform-independent measurement yardstick, called normalized computational cost (NCC) and denoted as C(mode), is proposed as follows.

C(mode)=Npts×W(mode)Nmode×Nref

where Npts denotes the total number of search points incurred, and W(mode) denotes the weight of the computational cost incurred for this block mode. Nmode is the total number of modes involved (in JM-6.1a encoder, Nmode = 7), and Nref is the total number of reference frames involved.

Since the computation of SAD is directly proportionate with the block size and occupies the majority of computational load in computing the Lagrangian cost functional J(m,MOTION), the computational costs per search point for a 1616 block is first defined as one unit cost. Then, the weight W(mode) becomes the ratio of the area of current block mode to that of the 1616 mode. That is, for a 1616 mode, the weight is 1, and accordingly, W=1/2 for the 168 mode or the 816 mode, W=1/4 for the 88 mode, W=1/8 for the 84 mode and the 48 mode, and W=1/16 for the 44 mode.

Another great advantage of our proposed normalized computational cost is that the computed results can be directly compared with the existing, which is already expressed in terms of search points per macroblock search.

# Conclusion

## In conclusion, our proposed normalized computational cost (NCC) has the following key merits: 1) hardware platform independent, 2) software code efficiency independent, and 3) search point equivalent. Hence, we recommend the NCC to be used for measuring the speed-up performance of fast motion estimation in the JVT/H.264 platform.

# Reference

- K.-K. Ma and G. Qiu, “An improved adaptive rood pattern search for fast block-matching motion estimation in JVT/H.26L,'' 2003 IEEE International Symposium on Circuits and Systems (ISCAS 2003), Bangkok, Thailand, May 2003. (Accept)

- K.-K. Ma and G. Qiu,  “Unequal-arm adaptive rood pattern search for fast block-matching motion estimation in JVT/H.26L,” 2003 IEEE International Conference on Image Processing (ICIP 2003), Barcelona, Spain. (Submitted)

- K.-K. Ma and G. Qiu,  “A new adaptive rood pattern search for fast block-matching motion estimation in the JVT/H.264”, document JVT-G029, Pattaya, Thailand, March 2003.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: