| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 8th Meeting: Geneva, May 20-26,2003 | Document: JVT-H017 Filename: JVT-H017.doc |
| --- | --- |

| Title: | Proposed Draft of Adaptive Rate Control |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JVT] |  |  |
| Purpose: | [ Proposal] |  |  |
| Author(s) or Contact(s): | Institute of Computing Technology, Chinese Academy of Sciences Institute for InfoComm Research, Singapore. | Tel: Email: |  |
| Source: |  |  |  |

_____________________________

(Begin text of document here: 11-point font is suggested for short documents, 10 for long ones)

1.	Rate Control	1

1.1.	Introduction to Rate Control	1

1.2.	Technical Description of Rate Control	2

1.2.1.	Definition of A Basic Unit	2

1.2.2.	A Fluid Flow Traffic Model	2

1.2.3.	A Linear Model for MAD Prediction	2

1.2.4.	HRD Consideration	3

1.2.5.	Algorithm Description	3

1.2.5.1.	GOP Layer Rate Control	4

1.2.5.2.	Frame Layer Rate Control	4

1.2.5.3.	Basic Unit Layer Rate Control	8

1.3.     Conclusions.	10

1.4.     Special Acknowledgements	11

1.5.     Contact Information	11

# Rate Control

### Introduction to Rate Control

Rate control plays a very important role in video coding, although it’s not a normative tool for any video coding standard. However, unlike the existing video coding standards, rate control in AVC standard becomes quite difficult because both rate control and rate-distortion optimization (RDO) will involve the quantization parameters. In rate control, a quantization parameter should be first determined for each MB to perform RDO by using the mean absolute difference (MAD) of current frame or MB. However, the MAD of current frame or MB is only available after the RDO. However, the MAD of current frame or MB is only available after RDO. Here an efficient rate control algorithm for AVC standard is provided through considering both rate control and RDO. In the adaptive rate control scheme, a concept of basic unit is introduced. The basic unit can be a frame, a slice, or an MB. A linear model is used to predict the MAD of current basic unit in the current frame by that of the basic unit in the co-located position of the previous frame. The adaptive rate control scheme is designed with HRD considerations, which would meet both VBR and CBR with buffer constraints.

### Technical Description of Rate Control

### Definition of A Basic Unit

The concept of a basic unit is defined by

Definition 1 Suppose that a frame is composed of Nmbpic MBs. A basic unit is defined to be a group of contiguous MBs which is composed of Nmbunit MBs where Nmbpic is a fraction of Nmbpic.

Denote the total number of basic units in a frame by Nunit, which is computed by

Nunit=NmbpicN                                      (1)

Examples of a basic unit can be an MB, a slice, a field, or a frame. For example, consider a video sequence with QCIF size, Nmbpic is 99.  According to Definition 1,t Nmbunit  can be 1, 3, 9, 11, 33, or 99. The corresponding Nunit is 99, 33, 11, 9, 3, and 1, respectively.

It is noted that by employing a bigger basic unit, a higher PSNR can be achieved while the bit fluctuation is also bigger. On the other hand, by using a small basic unit, the bit fluctuation is less severe, but with slight loss in PSNR.

### A Fluid Flow Traffic Model

A fluid flow traffic model is used to compute the target bit for the current coding frame.  Let Ngop denote the total number of frames in a group of picture (GOP), ni,j (I=1,2,….;j=1,2,…Ngop ) denote the jth frame in the ith GOP, and Bc(ni,j) denote the occupancy of virtual buffer after coding the jth frame. So,

Bc(ni,j+1)=Bc(ni,j)+b(ni,j)−u(ni,j)FrBc(n1,1)=0Bc(ni+1,0)=Bc(ni,Ngop)		              	(2)

where b(ni,j) is the number of bits generated by the jth frame in the ith GOP, u(ni,j) is the available channel bandwidth which can be either a VBR or a CBR,  and Fr is the predefined frame rate.

### A Linear Model for MAD Prediction

A linear model is used to predict the MADs of current basic unit in the current frame

by that of the co-located basic unit of the previous frame. Suppose that the predicted MAD of current basic unit in the current frame and the actual MAD of the co-located basic unit in the previous frame are denoted by MADcb and MADpb, respectively. The linear prediction model is then given by

MADcb=a1×MADpb+a2                                 (3)

where a1 and a2 are two coefficients of prediction model. The initial value of a1 and a2 are set to 1 and 0, respectively. They are updated after coding each basic unit. The linear model (3) is proposed to solve the chicken and egg dilemma between rate control and RDO.

### HRD Consideration

In this subsection, a lower bound and a upper bound for the target bits of each frame are determined by considering the hypothetical reference decoder (HRD).  The lower bound and the upper bound for the nth frame are denoted by L(ni,j)and U(ni,j), respectively. It is also shown that the HRD is conformed if the actual frame size is always within the bound L(ni,j),  U(ni,j).

tr (ni,j) denote the removal time of the jth frame in the ith GOP. be(t) is the bit equivalent of a time t, with the conversion factor being the buffer arrival rate. The initial values of the upper bound and the lower bound are given as follows:

L(ni,1)=Tr(ni,0)+u(ni,0)FrU(ni,1)=(Tr(ni,0)+be(tr(n1,1)))×ϖ                                (4)

where Tr(ni,0) is the remaining bits of the (i-1)th GOP and Tr(n1,0). The value of ω is 0.9.

L(ni,j)and U(ni,j) are computed iteratively as follows:

L(ni,j)=L(ni,j−1)+u(ni,j−1)Fr−b(ni,j−1)U(ni,j)=U(ni,j−1)+(u(ni,j−1)Fr−b(ni,j−1))×ϖ                         (5)

where Bs is the buffer size of CPB, the maximum value of which is determined based on different level and different profile.

### Algorithm Description

With the concept of basic unit, models (2) and (3), the steps of rate control are given as follows:

1.  Compute a target bit for the current frame by using the fluid traffic model (2), linear tracking theory and the bounds (4) and (5).

2.  Allocate the remaining bits to all non-coded basic units in the current frame equally.

3.  Predict the MAD of current basic unit in the current frame by the linear model (3) using the actual MAD of basic unit in the co-located position of previous frame.

4.  Compute the corresponding parameter by using the quadratic R-D model.

- Perform RDO for each MB in the current basic unit by the quantization parameter derived from

step 4.

The adaptive rate control scheme is composed of two layers: GOP layer rate control and frame layer rate control if the basic unit is selected as a frame. Otherwise, an additional basic unit layer rate control should be added. They will be presented in detail in the following sections.

##### GOP Layer Rate Control

In the beginning of the ith GOP, the total number of bits allocated for the ith GOP is computed as follows:

Tr(ni,0)=u(ni,1)Fr×Ngop−Bc(ni−1,Ngop)                                      (6)

Since the channel bandwidth may vary at any time, Tr is updated frame by frame as follows:

Tr(ni,j)=Tr(ni,j−1)+u(ni,j)−u(ni,j−1)Fr×(Ngop−j)−b(ni,j−1)                  (7)

In the case of CBR, i.e. u(i,j) = u(i,j-1), Equation (7) is simplified as

Tr(ni,j)=Tr(ni,j−1)−b(ni,j−1)                                              (8)

In other words, Equation (7) is also applicable to the CBR case.

The starting quantization parameter of the first GOP is a predefined quantization parameter QP0. The I frame and the first P frame of the GOP are coded by QP0. QP0 is predefined based on the available channel bandwidth and the GOP length. Normally, a small QP0 should be chosen if the available channel bandwidth is high and a big QP0 should be used if it is low. Under the same bandwidth, QP0 reduces by 1 if the GOP length increases by 30. QP0 can be set manually or computed by:

QP0=40 bpp≤l130 l1<bpp≤l220 l2<bpp≤l3                                     (9)

bpp=u(n(0,0))FrNpixel

where bpp is the bits per pixel. Npixel is the number of pixel in a frame. Recommended values l1=0.15,l2=0.45,l3=0.9 for QCIF/CIF and l1=0.6,l2=1.4,l3=2.4 for the image size bigger than CIF.

The starting quantization parameter QPst of other GOPs is computed by

QPst=SumPQPNp−8×Tr(ni−1,Ngop)Tr(ni,0)−min                       (10)

where Np is the total number of P frame in the previous GOP and SumPQP is the sum of quantization parameters for all P frames in the previous GOP.

The I frame and the first P frame are coded using QPst.

##### Frame Layer Rate Control

The frame layer rate control scheme consists of two stages: pre-encoding and post-encoding.

Pre-Encoding Stage:

The objective of this stage is to compute quantization parameter for all frames.

The following simple method is used to compute the quantization parameters of B frames.

Suppose that the number of successive B frames between two P frames is L and the quantization parameters are QP1 and QP2, respectively. The quantization parameter of the ith B frame is calculated according to the following two cases:

Case 1. L=1. In other words, there is only one B frame between two P frames. The quantization parameter is computed by

QB1=QP1+QP2+22 if QP1≠QP2QP1+2Otherwise                                 (11)

Case 2  L>1. In other words, there are more than one B frame between two P frames. The quantization parameters are computed by

QBi=QP1+α+maxmin(QP2−QP1)L−1,2×(i−1),−2×(i−1)               (12)

where α is the difference between the quantization parameter of the first B frame and QP1, and is given by

α=−3−2−10QP2−QP1≤−2×L−3QP2−QP1=−2×L−2QP2−QP1=−2×L−1QP2−QP1=−2×L12QP2−QP1=−2×L+1Otherwise                                   (13)

The case that QP2 – QP1 <-2xL+1can only occur at time instant the video sequence switches from one GOP to another GOP.

The final quantization parameter QBi is further adjusted by

QBi=minmaxQBi,1,51                                       (14)

The quantization parameters of P frames are computed via the following two steps:

Step 1 Determine a target bit for each P frame.

Step 1 is composed of the following two sub-steps.

The bits allocated to the current picture should be adjusted according to the current buffer occupancy and the picture complexity.

Step 1.1 Determination of target buffer occupancy.

A target buffer level is predefined for each frame according to the frame sizes of the first I frame and the first P frame, and the average picture complexity. The function of the target buffer level is to compute a target bit for each P frame, which is then used to compute the quantization parameter. Since the quantization parameter of the first P frame is given at the GOP layer, we only need to predefine target buffer levels for other P frames in each GOP.

After coding the first P frame in the ith GOP, the initial value of target buffer level is reset as follows:

Tbl(ni,2)=Bc(ni,2)                                      (15)

where Bc(ni,2) is the actual buffer occupancy after coding the first P frame in the ith GOP.

The target buffer level for the subsequent P frames is determined by

Tbl(ni,j+1)=Tbl(ni,j)−Tbl(ni,2)Np−1+Wp(ni,j)×(L+1)×u(ni,j)Fr×(Wp(ni,j)+Wb(ni,j)×L)−u(ni,j)Fr      (16)

where  Wp(ni,j) is the average complexity weight of P pictures, Wb(ni,j) is the average complexity weight of B pictures,  and Tbl(ni,j) is the target buffer level. Wp and Wb are computed by:

Wp(ni,j)=Wp(ni,j)8+7×Wp(ni,j−1)8Wb(ni,j)=Wb(ni,j)8+7×Wb(ni,j−1)8Wp(ni,j)=b(ni,j)×QPp(ni,j)Wb(ni,j)=b(ni,j)×QPb(ni,j)1.3636                               (17)

QPp and QPb are the corresponding quantization parameters.

In the case that there is no B frame between two P frames, Equation (15) can be simplified as

Tbl(ni,j+1)=Tbl(ni,j)−Tbl(ni,2)Np−1                                 (18)

It can be easily shown that Tbl(ni,Ngop,) is about 0. Thus, if the actual buffer fullness is exactly the same as the predefined target buffer level, it can be ensured that each GOP uses its own budget. However, since the rate-distortion (R-D) model and the MAD prediction model are not accurate, there usually exists a difference between the actual buffer fullness and the target buffer level. We therefore need to compute a target bit for each frame to reduce the difference between the actual buffer fullness and the target buffer level. This is achieved by the following microscopic control.

Step 1.2 Microscopic control (target bit rate computation).

The target bits allocated for the jth frame in the ith GOP is determined based on the target buffer level, the frame rate, the available channel bandwidth and the actual buffer occupancy as follows:

f(ni,j)=u(ni,j)Fr+γ×(Tbl(ni,j)−Bc(ni,j))                        (19)

wherein  is a constant and its typical value is 0.5 when there is no B frame and 0.25 otherwise. If the actual number of generated bits is around the target, it can be easily shown that

Bc(ni,j+1)−Tbl(ni,j+1)≈(1−γ)×(Bc(ni,j)−Tbl(ni,j))                (20)

Therefore, a tight buffer regulation can be achieved by choosing a large .

Meanwhile, the number of remaining bits should also be considered when the target bit is computed.

f(ni,j)=Wp(ni,j−1)×Tr(ni,j)Wp(ni,j−1)×Np,r(j−1)+Wb(ni,j−1)×Nb,r(j−1)                    (21)

If the last frame is complex and uses excessive bits, more bits should be assigned to this frame.

The target bit is a weighted combination of f(ni,j) and f(ni,j):

f(ni,j)=β×f(ni,j)+(1−β)×f(ni,j)                                   (22)

wherein β is a constant and its typical value is 0.75 when there is no B frame and is 0.9 otherwise. Normally, most frame will overuse its budget. To smooth the visual quality, the target bit is further adjusted by

f(ni,j)=(1−L×0.05)×f(ni,j)                                 (23)

To be confirmed with the HRD, the target bit is further bounded by

f(ni,j)=maxL(ni,j),f(ni,j)f(ni,j)=minU(ni,j),f(ni,j)                                    (24)

Step 2 Compute the quantization parameter and perform RDO.

The MAD of current P frame is predicted by model (2) using the actual MAD of previous P frame.

The quantization parameter Qpc corresponding to the target bit is then computed by using the quadratic model provided in [1,2].  The details on this can be found in [1,2,5], it is thus not elaborated in this section. To maintain the smoothness of visual quality among successive frames, the quantization parameter Qpc is adjusted by

Qpc=minQpp+2,maxQpp−2,Qpc                          (25)

where Qpp is the quantization parameter of the previous P frame.

The final quantization parameter Qpc is further bounded by

Qpc=minQpp+DQuant,maxQpp−DQuant,Qpc,1,51                                     (26)

The quantization parameter is then used to perform RDO for each MB in the current frame.

Post-encoding Stage:

There are three major tasks in this stage:  update the parameters a1 and a2 of model (3), the parameters of quadratic R-D model, and determine the number of frames needed to be skipped.

After encoding a picture, the parameters of model (3), as well as those of quadratic R-D model are updated. A method similar to that of R-D model in [1,2] is used where the window size is computed by using the method provided in [5] instead of that in [1,2].

After encoding a frame, the actual of bits generated, b(ni,j) is added to the current buffer occupancy. To ensure that the updated buffer occupancy is not too high, the frame skipping parameter Npost is set to zero and increased until the following buffer condition is satisfied:

Bc(ni,j+Npost)<Bs0.8                                       (27)

wherein the buffer fullness is updated as follows:

Bc(ni,j+l+1)=Bc(ni,j+l)−u(ni,j+l)∗tc∗Pre_enc_removal_delay(ni,j+l); 1≤l<Npost                  (28)

##### Basic Unit Layer Rate Control

If the basic unit is not selected as a frame, an additional basic unit layer rate control for P frame should be added to our scheme.

Same as the frame layer rate control, the I frame is coded by single quantization parameter. It is computed in the same way as that at frame layer. The B frame is also coded by single quantization parameter. It is computed almost the same as that at the frame layer provided that QP1 and QP2 are replaced by the average values of quantization parameters of all basic units in the corresponding frame.

Same as the frame layer, a target bit for each P frame is determined firstly. The bits are then allocated to each basic unit. Since the MADs of all non-coded basic units in the current frame are not known, the remaining bits are allocated to all non-coded basic units in the current frame equally.

The basic unit layer rate control selects the values of quantization parameters of all basic units in a frame, so that the sum of generated bits is close to the frame target f(ni,j).

The following is a step-by-step description of this method.

Step 1 Compute the number of texture bits Rt,l for the current basic unit. This step is composed of the following three substeps:

Step 1.1 Compute the target bit for the current basic unit.

Let frb(ni,j) and Nub denote the number of remaining bits for the all non-coded basic unit in the current frame and the number of non-coded basic units, respectively. The initial values of frb(ni,j) and Nub are f (ni,j) and Nunit, respectively. The target bit for the current basic unit is given

by frb/Nub.

Step 1.2 Compute the average number of header bits generated by all coded basic units:

mhdr,l=mhdr,l−1×(1−1l)+mhdr,llmhdr=mhdr,l×lNunit+mhdr,1×(1−lNunit)                                  (29)

where mhdr,l is the actual number of header bits generated by the lth basic unit in the current frame, mhdr,1 is the  estimate from all basic units in the previous frame.

Step 1.3 Compute the number of texture bits Rt,l.

Rt,l=frbNub−mhdr                                          (30)

Step 2  Predict the MAD of current basic unit in the current frame by model (3) using the actual MAD of basic unit in the same position of previous frame.

Step 3 Compute the quantization parameter of current basic unit by using the quadratic R-D model. We need to consider the following three cases:

Case 1 The first basic unit in the current frame.

Qcb=Qapf                                                (31)

where Qapf is the average value of quantization parameters for all basic units in the previous frame.

Case 2 frb<0. In this case, the quantization parameter should be greater than that of previous basic unit such that the sum of generated bits is close to f(ni,j),i.e.

Qcb=Qpb+DQuant                                       (32)

where Qpb is the quantization parameter of previous basic unit. To reduce the blocking artifacts, DQuant is 1 if Nunit is greater than 8 and is 2 otherwise.

To maintain the smoothness of visual quality, the quantization parameter is further bounded by

Qcb=max1,Qapf−Δ,min51,Qapf+Δ,Qcb                                        (33)

where  Δ is 3 if Nmbunit is less than the total number of MBs in a row, and 6 otherwise.

Case 3 Otherwise. In this case, we shall first compute a quantization parameter Qcb by using the quadratic model. Similar to case 2, it is bounded by

Qcb=maxQpb−DQuant,minQcb,Qpb+DQuant                          (34)

The objective is to reduce the blocking artifacts. Meanwhile, to maintain the smoothness of visual quality, it is further bounded by

Qcb=max1,Qapf−Δ,min51,Qapf+Δ,Qcb                        (35)

Step 4 Perform RDO for all MBs in the current basic unit.

Step 5 Update the number of remaining bits and the number of non-coded basic units for the current frame.

Step 6 After coding a whole frame, Qapf is updated.

To obtain a good trade-off between average PSNR and bit fluctuation, Nmbunit is recommended to be the number of MBs in a row for real time video communication, and Nunit is recommended to be 9 for other applications.

## 1.3.    Conclusions.

The adaptive rate control scheme can reach accurate rate control while keeping good PSNR. The target bit is bounded by two values derived from the consideration of HRD, and the scheme conforms to HRD. Moreover, it is applicable in both the VBR case and the CBR case.

References.

[1]   H.J.Lee and T.H.Chiang and Y.Q.Zhang. Scalable Rate Control for MPEG-4 Video. IEEE Trans. Circuit Syst. Video Technology, 10: 878-894, 2000.

[2 ]	A.Vetro, H.Sun and Y.Wang. MPEG-4 rate control for multiple video objects. IEEE Trans. Circuit Syst. Video Technology, 9: 186-199, 1999.

## 1.4.   Special Acknowledgements

The following individuals are greatly acknowledged: Dr. Feng Wu, from Microsoft Research China; Dr. Chul-Woo Kim from McubeWorks; Dr. Pankaj Topiwala from FastVDO.

## Contact Information

Prof. Wen Gao, Institute of Computing Technology, Chinese Academy of Sciences, Beijing, 100080, China, wgao@jdl.ac.cn;

Prof. Susanto Rahardja, Institute for InfoComm Research, 21 Heng Mui Keng Terrace, 119613 Singapore, rsusanto@i2r.a-star.edu.sg.

- (Append for Proposal Documents)

### JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institute for Computing Technology and Institute for InfoComm Research |  |  |
| Mailing address | Chinese Academy of Sciences 21 Heng Mui Keng Terrace Beijng, 10080, China Singapore 119613 |  |  |
| Country | P.R. China Singapore |  |  |
| Contact person | Wen Gao and Susanto Rahardja |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email | wgao@jdl.ac.cn; rsusanto@i2r.a-star.edu.sg |  |  |
| Place and date of submission | Singapore, May 12, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: