| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 8th Meeting: Geneva, Switzerland, 23-27 May, 2003 | Document: JVT-H018 Filename: JVT-H018.doc |
| --- | --- |

| Title: | Color Format Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dae-Sung Cho, and Hyun-Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

- Introduction

This proposal introduces a new coding technology that supports 4:4:4 format in YCbCr, XYZ and RGB color space while maintaining core technologies of the existing JVT codec. The proposed codec supports YCbCr and XYZ format by extending the 4:2:0 format while treating CbCr components as dependent luminance components. In other words, the full resolution CbCr components use the same mode as the luminance component does while the mode decision is executed using Luma component only including motion estimation. Green and Y components are treated as Luma in RGB and XYZ space respectively. For RGB coding we increased the coding efficiency greatly by applying DPCM coding to the residual images of R and B color components using the G component residual as a predictor. It works well for both the intra and the inter cases. To maintain the decoder compatibility we add a flag to signal the DPCM coding of residual images since only the RGB coding uses this mode among the 4:4:4 formats. The results show the comparable coding efficiency as that of the proposed YCbCr 4:4:4 format. So it can avoid the possible information loss due to the color space conversion while maintaining the coding efficiency. This makes the proposed JVT codec a good candidate for applications that require the high fidelity of colors such as digital cinema, digital archives, surveillance and so on.

- Proposed Syntax & Semantics Changes

We propose to change the document JVT-G050 to support 4:4:4 color format and various color space (YCbCr, RGB, XYZ, etc) as in the following sub-sections. Some syntax and semantics in subclause 7 shall be rewritten as in the following subclauses. The added parts are marked as blue colored sentences.

  - Subclause : 7.3 Syntax in tabular form

    - Subclause: 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| reserved_zero_5bits /* equal to 0 */ | 0 | u(5) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| color_space | 0 | ue(v) |
| chroma_format | 0 | u(2) |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

[ED. Note: color_space and chroma_format shall be skipped other than in Prof. Ext.]

    - Subclause: 7.3.2.2 Picture parameter set syntax

| pic_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 1 | ue(v) |
| seq_parameter_set_id | 1 | ue(v) |
| entropy_coding_mode_flag | 1 | u(1) |
| pic_order_present_flag | 1 | u(1) |
| num_slice_groups_minus1 | 1 | ue(v) |
| if( num_slice_groups_minus1 > 0 ) { |  |  |
| slice_group_map_type | 1 | ue(v) |
| if( slice_group_map_type = = 0 ) |  |  |
| for( iGroup = 0; iGroup <= num_slice_groups_minus1; iGroup++ ) |  |  |
| run_length_minus1[ iGroup ] | 1 | ue(v) |
| else if( slice_group_map_type = = 2 ) |  |  |
| for( iGroup = 0; iGroup < num_slice_groups_minus1; iGroup++ ) { |  |  |
| top_left[ iGroup ] | 1 | ue(v) |
| bottom_right[ iGroup ] | 1 | ue(v) |
| } |  |  |
| else if( slice_group_map_type = = 3 ¦ ¦ slice_group_map_type = = 4 ¦ ¦ slice_group_map_type = = 5 ) { |  |  |
| slice_group_change_direction_flag | 1 | u(1) |
| slice_group_change_rate_minus1 | 1 | ue(v) |
| } else if( slice_group_map_type = = 6 ) { |  |  |
| pic_size_in_map_units_minus1 | 1 | ue(v) |
| for( i = 0; i <= pic_size_in_map_units_minus1; i++ ) |  |  |
| slice_group_id[ i ] | 1 | u(v) |
| } |  |  |
| } |  |  |
| num_ref_idx_l0_active_minus1 | 1 | ue(v) |
| num_ref_idx_l1_active_minus1 | 1 | ue(v) |
| weighted_pred_flag | 1 | u(1) |
| weighted_bipred_idc | 1 | u(2) |
| pic_init_qp_minus26 /* relative to 26 */ | 1 | se(v) |
| pic_init_qs_minus26 /* relative to 26 */ | 1 | se(v) |
| chroma_qp_index_offset | 1 | se(v) |
| deblocking_filter_control_present_flag | 1 | u(1) |
| constrained_intra_pred_flag | 1 | u(1) |
| redundant_pic_cnt_present_flag | 1 | u(1) |
| if (chroma_format==3) /* 4:4:4 format */ |  |  |
| chroma_residue_pred_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 1 |  |
| } |  |  |

    - Subclause: 7.3.5.1 Macroblock predicton syntax

| mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ) |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if (chroma_format!=3) /* 4:2:0 or 4:2:2 */ |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

    - Subclause: 7.3.5.3 Residual data syntax

| residual( ) { | C | Descriptor |
| --- | --- | --- |
| if( !entropy_coding_mode_flag ) |  |  |
| residual_block = residual_block_cavlc |  |  |
| else |  |  |
| residual_block = residual_block_cabac |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block( Intra16x16DCLevel, 16 ) | 3 |  |
| for( i8x8 = 0; i8x8 < 4; i8x8++ ) /* each luma 8x8 block */ |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) /* each 4x4 sub-block of block */ |  |  |
| if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block( Intra16x16ACLevel[ i8x8 * 4 + i4x4 ], 15 ) | 3 |  |
| else |  |  |
| residual_block( LumaLevel[ i8x8 * 4 + i4x4 ], 16 ) | 3 ¦ 4 |  |
| } else { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| Intra16x16ACLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| else |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| } |  |  |
| if (chroma_format==1) { /* 4:2:0 */ |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| if( CodedBlockPatternChroma & 3 ) /* chroma DC residual present */ |  |  |
| residual_block( ChromaDCLevel[ iCbCr ], 4 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 4; i++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ i ] = 0 |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) |  |  |
| if( CodedBlockPatternChroma & 2 ) /* chroma AC residual present */ |  |  |
| residual_block( ChromaACLevel[ iCbCr ][ i4x4 ], 15 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| ChromaACLevel[ iCbCr ][ i4x4 ][ i ] = 0 |  |  |
| } else if (chroma_format==3) { /* 4:4:4 */ |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| if( CodedBlockPatternChroma & 3 ) /* chroma DC residual present */ |  |  |
| residual_block( Chroma16x16DCLevel[iCbCr], 16 ) | 3 |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| for( i8x8 = 0; i8x8 < 4; i8x8++ ) /* each chroma 8x8 block */ |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) /* each 4x4 sub-block of block */ |  |  |
| if( CodedBlockPatternChroma & 2 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block( Chroma16x16ACLevel[iCbCr][i8x8*4 + i4x4 ],15) | 3 |  |
| else |  |  |
| residual_block( ChromaLevel[iCbCr][ i8x8 * 4 + i4x4 ], 16 ) | 3 ¦ 4 |  |
| } else { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| Chroma16x16ACLevel[iCbCr][ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| else |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| ChromaLevel[iCbCr][ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

  - Subclause : 7.4 Semantics

    - Subclause: 7.4.2.1 Sequence parameter set RBSP semantics

color_space is the color space as specified in the Table V-1. If it is set to 1, the component representing green color is coded in the same way as the luminance component and the blue and red components are coded in the same way as the chrominance components. That is, the green component shall be treated as the Y component, the blue component shall be as the Cb component, and the red component shall be as the Cr component. If it is set to 2, the Y component is coded in the same way as the luminance component and the X and Z components are coded in the same way as the chrominance components. That is, the Y component shall be treated as the Y component, the X component shall be as the Cb component, and the Z component shall be as the Cr component. When color_space is not present, it shall be inferred to be 0.

Table V-1 – meaning of color_space.

| color_space | Color space |
| --- | --- |
| 0 | YCbCr |
| 1 | RGB |
| 2 | XYZ |

chroma_format is the chrominance format as specified in the Table V-2. When chroma_format is not present, it shall be inferred to be 1.

Table V-2 – meaning of chroma format.

| chroma_format | Chroma formt |
| --- | --- |
| 0 | reserved |
| 1 | 4:2:0 |
| 2 | 4:2:2 |
| 3 | 4:4:4 |

    - Subclause: 7.4.2.2 Picture parameter set RBSP semantics

chroma_residue_pred_flag equal to 0 indicates that the chroma residue prediction is not applied to I, P, B, SI, and SP slices. chroma_residue_pred_flag to equal to 1 indicates that the chroma residue prediction is applied to I, P, B, SI and SP slices (see subclause XXX). When chroma_residue_pred_flag is not present, it shall be inferred to be 0.

    - Subclause: 7.4.5 Macroblock layer semantics

mb_type specifies the macroblock type. The semantics of mb_type depend on the slice type.

Tables and semantics are specified for the various macroblock types for I, SI, P, SP, and B slices. Each table presents the value of mb_type, the name of mb_type, the number of macroblock partitions used (given by the NumMbPart( mb_type ) function), the prediction mode of the macroblock (if it is not partitioned) or the first partition (given by the

…

If chroma_format is set to 1, to each Intra_16x16 prediction macroblock, an Intra16x16PredMode is assigned, which specifies the Intra_16x16 prediction mode, CodedBlockPatternChroma contains the coded block pattern value for chroma as specified in Error: Reference source not found. CodedBlockPatternLuma specifies whether for the luma component non-zero AC transform coefficient levels are present.

If chroma_format is set to 3 (chroma 4:4:4), to each Intra_16x16 prediction macroblock, an Intra16x16PredMode, CodedBlockPatternChroma444 as specified in Table V-3, and Coded BlockPatternLuma are present.

CodedBlockPatternLuma equal to 0 specifies that there are no AC transform coefficient levels in the luma component of the macroblock.  CodedBlockPatternLuma equal to 15 specifies that at least one AC transform coefficient level is in the luma component of the macroblock, requiring scanning of AC transform coefficient levels for all 16 of the 4x4 blocks in the 16x16 block.

    - Subclause: 7.4.5.3 Residual data semantics

The syntax structure residual_block( ), which is used for parsing the transform coefficient levels, is assigned as follows.

- If entropy_coding_mode_flag is equal to 0, residual_block is set equal to residual_block_cavlc, which is used for parsing the syntax elements for transform coefficient levels.

- Otherwise (entropy_coding_mode_flag is equal to 1), residual_block is set equal to residual_block_cabac, which is used for parsing the syntax elements for transform coefficient levels.

Depending on mb_type, luma or chroma, the syntax structure residual_block( coeffLevel, maxNumCoeff ) is used with the arguments coeffLevel, which is a list containing the maxNumCoeff transform coefficient levels that are parsed in residual_block( ), and maxNumCoeff as follows.

- If MbPartPredMode( mb_type, 0 ) is equal to Intra_16x16, the transform coefficient levels are parsed into the list Intra16x16DCLevel and into the 16 lists Intra16x16ACLevel[ i ]. Intra16x16DCLevel contains the 16 transform coefficient levels of the DC transform coefficient levels for each 4x4 luma block. For each of the 16 4x4 luma blocks indexed by i = 0..15, the 15 AC transform coefficients levels of the i-th block are parsed into the i-th list Intra16x16ACLevel[ i ].

- Otherwise (MbPartPredMode( mb_type, 0 ) is not equal to Intra_16x16), for each of the 16 4x4 luma blocks indexed by i = 0..15,  the 16 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel[ i ].

- If chorma_format is set to 1 (chroma 4:2:0),

  - For each chroma component, indexed by iCbCr = 0..1, the 4 DC transform coefficient levels of the 4x4 chroma blocks are parsed into iCbCr-th list ChromaDCLevel[ iCbCr ].

  - For each of the 4x4 chroma blocks, indexed by i4x4 = 0..3, of each chroma component, indexed by iCbCr = 0..1, the 15 AC transform coefficient levels are parsed into the  i4x4-th list of the iCbCr-th chroma component ChromaACLevel[ iCbCr ][ i4x4 ].

- If chorma_format is set to 3 (chroma 4:4:4),

  - If MBPartPredMode( mb_type, 0) is equal to Intra_16x16, For each chroma component, indexed by iCbCr=0..1, the transform coefficient levels are parsed into iCbCr-th list Chroma16x16DCLevel[iCbCr] and into the 16 iCbCr-th lists Chroma16x16ACLevel[iCbCr][i]. For iCbCr-th chroma component, Chroma16x16DCLevel[iCbCr] contains the 16 transform coefficient levels of the DC transform coefficient levels for each 4x4 chroma block. For each of the 16 4x4 chroma blocks, indexed by i = 0..15, of each chroma component, indexed by iCbCr=0..1, the 15 AC transform coefficients levels are parsed into the i-the list of the iCbCr-the chroma component Chroma16x16ACLevel[iCbCr][i].

  - Otherwise (MBPartPredMode( mb_type, 0) is not equal to Intra_16x16), for each of the 16 4x4 chroma blocks, indexed by i = 0..15, of each chroma component, indexed by iCbCr=0..1, the 16 transform coefficient levels are parsed into the i-th list of the iCbCr-th chroma component ChromaLevel[iCbCr][i].

- Proposed Decoding Process

The details of our proposed decoding process for chroma format extensions are described by extending the draft text of FDIS document (JVT-G050). Some decoding process in subclause 8: shall be rewritten to support 4:4:4 format for YCbCr, XYZ and RGB color format while maintaining core technologies of the existing JVT 4:2:0 format. The added parts are marked as blue colored sentences.

  - Intra prediction process

Both Intra_4x4 prediction and Intra_16x16 prediction can be applied equally on all color components. Both shall not be used at the same time for each component, and each component shall not use different intra prediction mode. For example, when Intra_4x4 prediction is used, all color components shall use only one mode of Intra_4x4 prediction. Intra prediction process for chroma samples (subclause 8.3.3) can be rewritten as follows:

    - Subclause 8.3.3: Intra prediction process for chroma samples

This process is invoked for I and SI macroblock types. It specifies how the Intra prediction chroma samples for the current macroblock are derived.

Inputs to this process are constructed samples prior to the deblocking process from neighbouring chroma blocks (if available).

Outputs of this process are Intra prediction chroma samples for the current macroblock predCb[ x, y ] and predCr[ x, y ].

If chroma_format is set to 3, the current chroma 16x16 array predCb[ x, y ] and predCr[ x, y ] shall use the same intra prediction mode for luma component samples. If Intra_4x4 prediction is applied for luma samples, Intra_4x4 prediction is also applied for chroma samples. If Intra_16x16 prediction is applied for luma samples, Intra_16x16 prediction is also applied for chroma samples. The same prediction mode is applied both to Luma and chroma samples. For each chroma blocks the intra prediction is performed as in subclause 8.3.1 and 8.3.2, substituting luma block with chroma blocks.

If chroma_format is set to 1, the intra prediction for chroma samples is described in the remainder of this subclause..

Both chroma blocks (Cb and Cr) of the macroblock shall use the same prediction mode.  The prediction mode is applied to each of the chroma blocks separately. The process specified in this subclause is invoked for each chroma block. In the remainder of this subclause, chroma block refers to one of the two chroma blocks and the subscript C is used as a replacement of the subscript Cb or Cr.

The 17 neighbouring samples p[ x, y ] that are constructed chroma samples prior to deblocking, with x = -1, y = -1..7 and with x = 0..7, y = -1, are derived as follows. …..

  - Inter prediction process

All color components shall use same motion vector.

For the fractional sample interpolation process, the Luma sample interpolation process for YUV 4:2:0 format shall be used for all color componets. Inter prediction process for chroma samples (subclause 8.4.1.4 and subclase 8.4.2.2.2) can be rewritten as follows:

    - Subclase: 8.4.1.4 Derivation process for chroma motion vectors

Inputs to this process are a luma motion vector mvLX and a reference index refIdxLX.

Outputs of this process are a chroma motion vector mvCLX.

If chroma_format is set to 3 (4:4:4 format), chroma blocks shall use the corresponding luma motion vectors. The chroma motion vector is exactly same as the corresponding luma motion vector.

If chroma_format is set to 1 (4:2:0 format), A chroma motion vector is derived as described in the remainder of this subclause.

A chroma motion vector is derived from the corresponding luma motion vector.  Since the accuracy of luma motion vectors is one-quarter sample and chroma has half resolution compared to luma, the accuracy of chroma motion vectors is one-eighth sample, i.e., a value of 1 for the chroma motion vector refers to a one-eighth sample displacement.

NOTE - For example when the luma vector applies to 8x16 luma samples, the corresponding chroma vector applies to 4x8 chroma samples and when the luma vector applies to 4x4 luma samples, the corresponding chroma vector applies to 2x2 chroma samples.

For the derivation of the motion vector mvCLX, the following applies. …

    - Subclause: 8.4.2.2.2 chroma sample interpolation process

Inputs to this process are

- a chroma location in full-sample units ( xIntC, yIntC ),

- a chroma location offset in fractional-sample units ( xFracC, yFracC ), and

- chroma component samples from the selected reference picture refPicLXC.

Output of this process is a predicted chroma sample value predPartLXC[ xC, yC ].

In case that chroma_format is 3 (4:4:4 format), the luma sample interpolation process in subclause 8.4.2.2.1 shall be used, substituting 16x16 luma sample array predPartLXL[ xL, yL ] with 16x16 chroma sample array predPartLX C[ xC, yC ]. In case that chroma_format is 1 (4:2:0 format), the interpolation process for 8x8 chroma sample arrary predPartLX C[ xC, yC ] is described in the remainder of this subclause.

In Error: Reference source not found, the positions labelled with A, B, C, and D represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. …

  - Subclause: 8.5 Transform coefficient decoding process and picture construction process prior to deblocking filter process

Inputs to this process are Intra16x16DCLevel (if available), Intra16x16ACLevel (if available), LumaLevel (if available), ChromaDCLevel(if available), ChromaACLevel(if available),  Chroma16x16DCLevel (if available), Chroma16x16ACLevel (if available), ChromaLevel (if available) and available Inter or Intra prediction sample arrays for the current macroblock for the applicable component predL, predCb, or predCr. …..

    - Subclause: 8.5.3 Specification of transform decoding process for chroma samples

When chroma_format is 3 (4:4:4 format) and the current macroblock prediction mode is not equal to Intra_16x16, the variable ChromaLevel[iCbCr], with iCbCr set equal to 0 for Cb and iCbCr set equal to 1 for Cr, contains the levels for both components of the chroma transform coefficients. For a 4x4 chroma block indexed by chroma4x4BlkIdx = 0..15, the following ordered steps are specified.

- The inverse transform coefficient scanning process as described in subclause Error: Reference source not found is invoked with ChromaLevel[iCbCr][ chroma4x4BlkIdx ] as the input and the two-dimensional array c as the output.

- The scaling and transformation process for residual chroma 4x4 blocks as specified in subclause Error: Reference source not found is invoked with c as the input and x’’ as the output.

- If chroma_residue_pred_flag is equal to 1, x''' is derived as

x''' = x’’+ x’’luma		(8-x)

where x’’luma is the ouput of the step 2 in subcluase 8.5.1. Otherwise, x''' = x’’.

- The position of the upper-left sample of a 4x4 chroma block with index chroma4x4BlkIdx inside the macroblock is derived by invoking the inverse 4x4 chroma block scanning process in subclause Error: Reference source not found with chroma4x4BlkIdx as the input and the output being assigned to ( xO, yO ).

- The 4x4 array u with elements uyx for x, y = 0..3 is derived as

uyx = Clip1( predL[ xO + x, yO + y ] + x'''yx ) 	(8-x)

- The picture construction process prior to deblocking filter process in subclause Error: Reference source not found is invoked with chroma4x4BlkIdx, u as the input and S’ as the output.

When chroma_format is 3 (4:4:4 format) and the current macroblock prediction mode is equal to Intra_16x16, the variables Chroma16x16DCLevel[iCbCr] and Chroma16x16ACLevel[iCbCr], with iCbCr set equal to 0 for Cb and iCbCr set equal to 1 for Cr, contain the levels for both components of the chroma transform coefficients. The transform coefficient decoding proceeds in the following ordered steps:

- The 4x4 chroma DC transform coefficients of all 4x4 chroma blocks of the macroblock are decoded.

  - The inverse transform coefficient scanning process as described in subclause Error: Reference source not found is invoked with Intra16x16DCLevel as the input and the two-dimensional array c as the output.

  - The scaling and transformation process for chroma DC transform coefficients for Intra_16x16 macroblock type as specified in subclause Error: Reference source not found is invoked with c as the input and dcUV as the output.

- For a 4x4 chroma block indexed by chroma4x4BlkIdx = 0..15, the following ordered steps are specified.

  - The variable chromaList, which is a list of 16 entries, is derived. The first entry of chromaList is the corresponding value from the array dcUV. Error: Reference source not found shows the assignment of the indices of the array dcUV to the chroma4x4BlkIdx. The two numbers in the small squares refer to indices i and j in dcUVij, and the numbers in large squares refer to chroma4x4BlkIdx.

The elements in chromaList with index k = 1..15 are specified as

chromaList[ k ] = Intra16x16ACLevel[ chroma4x4BlkIdx ][ k - 1 ] 	(8-x)

  - The inverse transform coefficient scanning process as described in subclause Error: Reference source not found is invoked with chromaList as the input and the two-dimensional array c as the output.

  - The scaling and transformation process for residual chroma 4x4 blocks as specified in subclause Error: Reference source not found is invoked with c as the input and x’’ as the output.

  - If chroma_residue_pred_flag is equal to 1, x''' is derived as

x''' = x’’+ x’’luma		(8-x)

where x’’luma is the ouput of the step 2.c in subcluase 8.5.2. Otherwise, x''' = x’’.

  - The position of the upper-left sample of a 4x4 chroma block with index chroma4x4BlkIdx inside the macroblock is derived by invoking the inverse 4x4 chroma block scanning process in subclause Error: Reference source not found with chroma4x4BlkIdx as the input and the output being assigned to ( xO, yO ).

  - The 4x4 array u with elements uyx for x, y = 0..3 is derived as

uyx = Clip1( predL[ xO + x, yO + y ] + x'''yx )	(8-x)

  - The picture construction process prior to deblocking filter process in subclause Error: Reference source not found is invoked with chroma4x4BlkIdx, u as the input and S’ as the output.

When chroma_format is 1 (4:2:0 format), for each chroma component, the variables ChromaDCLevel[ iCbCr ] and ChromaACLevel[ iCbCr ], with iCbCr set equal to 0 for Cb and iCbCr set equal to 1 for Cr, contain the levels for both components of the chroma transform coefficients. For each chroma component, the transform decoding proceeds separately in the following ordered steps:…

  - Deblocking filter process

The deblocking filter process is invoked for the luma and chroma components separately. For each macroblock, vertical edges are filtered first, from left to right, and then horizontal edges are filtered from top to bottom. The luma deblocking filter process is performed on four 16-sample edges, and the deblocking filter process for each chroma components is performed on two 8-sample edges when chroma_format is 1 (4:2:0 format), and on four 16-sample edges when chroma_format is 3 (4:4:4 format), for the horizontal direction as shown on the left side of Error: Reference source not found and for the vertical direction as shown on the right side of Error: Reference source not found [Ed. Note: The Error: Reference source not found should be modified accordingly].

    - Subclause: 8.7.1 Filtering process for block edges

…

The variable nE is derived as follows.

- If chromaEdgeFlag is equal to 0, nE is 16;

- If chromaEdgeFlag is equal to 1 and chroma_format is 3, nE is 16;

- Otherwise (chromaEdgeFlag is equal to 1 and chroma_format is 1), nE is 8.

…

- Experimental Results

  - Test Conditions

- Test sequences: Crew, Harbour, Preakness, and Sailormen (1280x720@60p)

- QP: 12, 18, 24, and 30 for I- and P-slices and 14, 20, 26, and 32 for B-slice

- Search range: 32

- Entropy coding: CABAC

- RD-optimized mode selection: On

- Number of reference frames: 4

- GOP structure: N 30, M 3

- Number of slice groups: 1

- Direct mode: Spatial

- Weighted prediction: No

- Stored B-picture: No

  - Results

The simulation results show the RD-curves for proposed coding methods for the YUV 444 and RGB format with and without chroma residual prediction method. It clearly shows that the introduction of DPCM for residual image has greatly improved the coding efficiency in RGB space.

We also compared the coding efficiency using one mode for the entire components and that of using independent mode for each color components. It shows the comparable results. So it supports the versatility of the proposed method for any format in any color space.

- Conclusions

A new coding technology that supports 4:4:4 format in YCbCr, XYZ and RGB color space has been proposed. It uses the same mode for each component by using Luma componet (Green in RGB space and Y in XYZ space) as a principal component. The Luma component decides all the coding mode in intra and inter cases including motion estimation. It decodes the current 4:2:0 format by adding the chroma_format in sequence header. Simulation results show that the coding efficiency of the proposed 4:4:4 format is comparable to that of the inherent 4:4:4 format that can be implemented by extending the resolution of Chroma components. So we can decrease the complexity of the encoder by avoiding the computation required for Chroma mode decision. The coding efficiency of the RGB coding is greatly improved using the DPCM coding of Chroma residue. By turning on the Chroma_residue_pred_flag, the proposed method maintains the high coding efficiency both in RGB and YCbCr space. It helps maintain the fidelity of the color with the comparable coding efficiency when color space conversion is executed to decorrelate the redundancy resident in color components. It makes the proposed JVT codec a complete package for various needs of video compression.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Multimedia |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Geneva, CH, 23-27 May, 2003 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Color Format Extension |  |
| Contribution number |  | JVT-H018 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: